| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D117 |
| --- | --- |

| Title: | CE9: Cross verification about the adaptation and modification (3.2.c) of the set of predictors |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Patrice ONNO Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: :+33 (0)2 99 87 68 92 Email: patrice.onno@crf.canon.fr |  |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution contains the coding efficiency results of the second tester regarding the case 3.2.c of CE9 described in section 3.2 of the JCTVC-C509_r3 document (JUNG & BROSS, 2010). The experiment described in section 3.2.c mainly focuses on the reordering of the set of motion predictors to evaluate the impact in terms of coding efficiency.

# Introduction

## AMVP principle

Currently in the TMuC 0.9, the motion vector predictors are taken from an ordered list of motion vectors of the neighboring block. The motion prediction is based on 5 motion vector predictors which are used in a particular order.

The motion vector predictor list is ordered as follows: the Left block, the Above block, the Corner block (Top Left or Above Right or Bottom Left), the Median block (median vector between the Above, the Left and the Corner) and the temporally Collocated block. However, due to the tree structure and position in the frame, all motion vector predictors are not available.

Then, if some duplicated motion vector predictors exist, they are simply removed from the list. The same principle is applied at the decoder side.

Then, depending on the position in the list, an entopic code is assigned for the index of the motion vector predictor as presented in Table 1.

| Amount of predictors in the list | AMVP Current implementation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Index |  |  |  |  | Max bits |
|  | 1 | 2 | 3 | 4 | 5 |  |
| 1 | X |  |  |  |  | 0 |
| 2 | 0 | 1 |  |  |  | 1 |
| 3 | 0 | 10 | 11 |  |  | 2 |
| 4 | 0 | 10 | 110 | 111 |  | 3 |
| 5 | 0 | 10 | 110 | 1110 | 1111 | 4 |

Table 1: Codewords used for the predictor index in AMVP

# Modification tested

The source code modifications provided by Sony (SATO, 2010) for tis core experiment correspond to the case explained in Table 3.1.1. of the contribution JCTVC-C166. The principle is to change the order of the motion vector predictor candidates in the list in such a way that the temporally collocated motion vector predictor is the first element to be considered in the list. Currently in in TMuC v0.9, it is considered as the last element.

According to the Table 1 above and with this new order, the temporally collocated block will be coded with the shortest codeword.

# Results

The results of our encoding/decoding tests are also fully consistent with the data provided by Sony in document JCTVC-D301 (SATO, 2011) and can be summarized by the following Table 2.

## Simulation results when MRG=0 vs TMuC 0.9 anchors

Simulation suggested by Sony was to set MRG=0 for the method to be tested.

Our results with that particular setting (MRG=0) demonstrate that this modification does not bring any coding efficiency improvement. The following results show a decrease of performance from 0.1% to 1.5%.

|  |  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
|  | Class A | 0,2 | 0,2 | 0,2 | -0,2 | 0,0 | 0,0 |
|  | Class B | 0,5 | 0,3 | 0,2 | 0,3 | 0,0 | -0,2 |
|  | Class C | 0,4 | 0,3 | 0,3 | 0,1 | 0,2 | 0,1 |
|  | Class D | 0,1 | 0,3 | 0,4 | 0,0 | -0,2 | 0,0 |
|  | Class E |  |  |  |  |  |  |
|  | All | 0,3 | 0,3 | 0,3 | 0,1 | 0,0 | 0,0 |
|  |  | Low delay |  |  | Low delay LoCo |  |  |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
|  | Class A |  |  |  |  |  |  |
|  | Class B | 0,9 | 0,3 | -0,1 | 1,6 | 0,5 | 0,5 |
|  | Class C | 0,7 | 0,4 | 0,5 | 1,1 | 0,6 | 0,6 |
|  | Class D | 0,7 | -0,3 | 0,3 | 1,3 | 0,7 | 0,4 |
|  | Class E | 0,7 | 1,6 | 0,7 | 1,9 | 0,7 | 0,9 |
|  | All | 0,8 | 0,4 | 0,3 | 1,5 | 0,6 | 0,6 |

Table 2: Results of Sony modification with MRG=0.

# Additional results

## Simulation results when MRG=1 vs TMuC 0.9 anchors

Additional simulations have been done with MRG=1 for the Sony method to be tested. The goal was to really evaluate the impact of the change provided by Sony in JCTVC-C166 while keeping MRG=1 as for the TMuC v0.9 anchors. The corresponding results are presented inTable 3.

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0,2 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| Class B | -0,2 | -0,1 | -0,1 | 0,0 | 0,0 | -0,1 |
| Class C | -0,2 | -0,1 | -0,2 | 0,1 | 0,2 | 0,1 |
| Class D | -0,2 | -0,2 | 0,1 | 0,1 | 0,0 | 0,1 |
| Class E |  |  |  |  |  |  |
| All | -0,2 | -0,1 | -0,1 | 0,0 | 0,1 | 0,0 |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0,0 | -0,1 | -0,3 | 0,1 | -0,1 | 0,0 |
| Class C | -0,1 | -0,2 | -0,1 | 0,1 | -0,1 | 0,1 |
| Class D | -0,1 | -0,4 | -0,3 | -0,1 | -0,1 | -0,1 |
| Class E | -0,5 | -1,1 | -1,3 | 0,6 | 0,2 | 0,3 |
| All | -0,1 | -0,4 | -0,4 | 0,1 | -0,1 | 0,0 |

Table 3: Results of Sony modification with MRG=1 (as for the TMuC v0.9)

# Conclusion

According to the results presented here, we can conclude that the simple reordering of the collocated motion vector predictor as presented in Table 2 does not bring any improvement since a loss ranging from 0.1% to 1.5% has been observed.

However, when using only the modification of Sony with MRG=1 presented in Table 3, a slight gain is observed but it does not exceed 0.2% gain in average.

# Reference

JUNG, J., & BROSS, B. (2010). Core Experiment 9: Motion Vector Coding. Joint Collaborative Team on Video Coding (JCT-VC). JCTVC-C509_r3.

SATO, K. (2010). TE11: Study on motion vector coding (experiment 3.3.a & 3.3.c). Joint Collaborative Team on Video Coding (JCT-VC) - JCTVC-C166r1.

SATO, K. (2011). Results of CE9 3.2.C. Joint Collaborative Team on Video coding (JCTVC) - JCTVC-D301.