| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D119 |
| --- | --- |

| Title: | CE8 Subset2: A Joint Proposal on Improving the Adaptive Loop Filter in TMuC0.9 by MediaTek, Qualcomm, and Toshiba |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Authors: | Ching-Yeh Chen1 Chih-Ming Fu1 Chia-Yang Tsai1 Yu-Wen Huang1 Shawmin Lei1 Marta Karczewicz2 In Suk Chong2 Tomoo Yamakage3 Takeshi Chujoh3 Takashi Watanabe3 | Emails: | chingyeh.chen@mediatek.com chihming.fu@mediatek.com chiayang.tsai@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com martak@qualcomm.com ichong@qualcomm.com tomoo.yamakage@toshiba.co.jp takeshi.chujoh@toshiba.co.jp takashi39.watanabe@toshiba.co.jp |
| Source: | MediaTek Inc.1, Qualcomm2, and TOSHIBA Corporation3 |  |  |

_____________________________

# Abstract

This contribution describes MediaTek, Qualcomm, and Toshiba’s adaptive loop filter (ALF), namely MQT_ALF. The proposed MQT_ALF allows local adaptation by dividing a picture into equal-size filter units (FUs) in raster scan. Each FU can choose one method for quality enhancement. The first method is pixel adaptation (PA), which is used in TMuC0.9. Pixels are classified into groups, and each group of pixels is applied with one filter. The second method is single filter (SF), which is a degenerative case of the PA method when only one group is used and no pixel classification is required. The third method is adaptive offset (AO), which is a simplified PA method with simpler pixel classification and only offset adjustment for each group. Moreover, special attention is also paid to reduce the number of encoding passes from 16, for the ALF in TMuC0.9, to 1, for our MQT_ALF, which can dramatically save power consumption due to significantly less external memory accesses. Simulation results are compared with the JCTVC-C500 anchor. The MQT_ALF can achieve 0.4% and 1.9% bitrate reductions for HE-RA and HE-LD, respectively. The encoding time is decreased by 2%, and the decoding time is decreased by 15%.

- Introduction

At the 3rd JCT-VC meeting held in Guangzhou in October 2010, Toshiba, Qualcomm, and MediaTek all proposed their own ALF solutions in JCTVC-C082 [1], JCTVC-C113 [2], and JCTVC-C143 [3], respectively. All the three proposals tried to reduce the number of encoding passes in comparison with the 16-pass Qualcomm ALF (QC_ALF) in TMuC0.7 [4]. However, they all suffer coding efficiency loss in comparison with the JCTVC-B300 anchor [5] that uses the 16-pass QC_ALF. During the Guangzhou meeting, the consensus of tool experiment 10 (TE10) [6] was to keep the 16-pass QC_ALF in TMuC0.9 for further study, and TMuC0.9 will eventually become HEVC test model 1.0 (HM1.0) [7] right after clean-up. In addition, core experiment 8 (CE8) [8] was established for in-loop filtering, and the subset2 of CE8 focuses on ALF.

In this proposal, the three companies collaboratively provide an improved ALF solution, called MQT_ALF, with better coding efficiency and less encoding/decoding time in comparison with the JCTVC-C500 anchor [9] that uses the 16-pass QC_ALF. Improved features include filter unit (FU) and filter set (FS) concepts, switch among single filter (SF), pixel adaptation (PA), and adaptive offset (AO) methods for each FU, and single-pass encoding. The rest of this input document is organized as follows. The proposed MQT_ALF algorithm will be described in Section 2, simulation results will be shown in Section Error: Reference source not found, and finally conclusions will be given in Section 4.

# Algorithm description

Using single or multiple filters per picture with localization of filter selection and filter usage (on/off) can achieve significantly better coding efficiency than using single filter per picture without any localization. Nevertheless, in prior, many ALF algorithms require high computation complexity and large external memory access for accessing current/reconstructed/filtered picture buffers due to process of filter coefficient training, picking one filter from multiple candidate filters, or localization of filter usage [10]. Therefore, in this work, we focus on not only enabling different localized filtering methods but also reducing computation complexity. For luma, the concepts of filter unit (FU) and filter set (FS) will be introduced in Section 2.1, followed by different methods that can be selected in each FS, including single filter (SF), pixel adaptation (PA), and adaptive offset (AO) in Section 2.2, Section 2.3, and Section 2.4, respectively. As for chroma, the process in MQT_ALF is exactly the same as that in TMuC. Our syntax design will be described in Section 2.5, fast distortion estimation, the key to reduction of encoding passes, will be reviewed in Section 2.6, and finally, the encoding pass will be explained in Section 2.7.

## Filter unit and filter set

The basic unit for choosing a filter set (FS) is called a filter unit (FU). Each FU is a rectangle consisting of one or multiple LCUs horizontally and vertically, and each picture is composed of one or multiple FUs of the same size except for those FUs on the picture boundaries of the right side and lower side. Figure 1 shows three examples of different FU sizes for one picture. The FU size information is transmitted in the slice header. It is an encoder-only issue to decide a proper FU size. Before sending information in FUs, an encoder first derives one or multiple filter sets, and then each FU can select one FS for quality enhancement. In each FS, one of the four methods including SF, PA, AO, and OFF can be used.

(a)                         (b)                           (c)

Figure 1  Illustration of FUs for one 832x480 picture with LCU size equal to 64x64; (a) FU size = 3 LCUs x 2 LCUs; (b) FU size = 6 LCUs x 4 LCUs; (c) FU size = picture size.

FUs are transmitted in raster scan order. After the FU localization, the coding unit (CU) level filter usage control, i.e. decision of a filter on/off flag for one leaf CU whose size is equal to or larger than the AlfMinCuSize or for multiple leaf CUs of one parent CU whose size equal to the AlfMinCuSize, still can be further applied.

## Single filter

The single filter (SF) method can choose one of the four options of filter shapes and sizes including 3x3-square (3x3S), 5x5-square (5x5S), 7x7-diamond (7x7D), and 9x9-diamond (9x9D). Because the 1/2 symmetric property of filter coefficients is applied and one additional offset is also used, the numbers of filter coefficients to be transmitted are 6, 14, 14, and 22 for 3x3S, 5x5S, 7x7D, and 9x9D, respectively. The central filter coefficient is always spatially predicted by using the sum of non-central filter coefficients, and non-central filter coefficients can be temporally predicted by those of the same filter shape and size from prior pictures. However, when time-delayed filtering is enabled, it is not necessary to send filter coefficients, and only filter buffer index is required. As improvements of the SF method in [3], 7x7-square (7x7S) and 9x9-square (9x9S) are replaced by 3x3S and 5x5S, respectively, and new filters in non-reference pictures are disallowed for time-delayed filtering and filter coefficient prediction in following pictures.

## Pixel adaptation

The pixel adaptation (PA) method is the method adopted in the current TMuC [7] in which FU partitioning of a picture is not applied. Sum of Laplacian is used to classify all pixels of a picture into 16 groups, and therefore each pixel can implicitly select one filter. The 16 groups can be merged in order to reduce side information of filter coefficients. In the current design, modifications are made as follows. First, the filter shape and size can be 3x3S, 5x5S, 7x7D, and 9x9D in the new PA method. Second, the window size for calculating sum of Laplacian is reduced from 7x7 to 3x3 without losing coding efficiency. Third, instead of always sending filter coefficients for each group after merging, one flag can be used to disable filtering for each group.

## Adaptive offset

The adaptive offset (AO) method can be regarded as a special case to perform only offset adjusting. The AO method first classifies pixels into different groups and then sends an offset for each group. In this work, we propose to integrate AO with ALF. When combined with the SF method and the PA method as an option of each FS, the proposed AO method can not only increase the coding efficiency of the entire ALF because of more encoding options but also somewhat decrease the average decoding time due to the lower complexity of offset in comparison with filtering. In the proposed AO method, two AO types, band offset (BO) and edge offset (EO), are supported. BO and EO will be briefly introduced in the next two paragraphs.

Band correction (BDC) was first proposed in JCTVC-A124 [11]. The intensity range (e.g. [0, 255] for 8-bit pixels) is equally divided into 16 bands for pixel classification. In JCTVC-B077 [12] and JCTVC-C147 [13], BO was proposed as an improved BDC with localization of each picture. Moreover, the intensity range can be divided into 16 uniform bands or 12 non-uniform bands. In this proposal, BO is modified as follows. First, the intensity range is equally divided into 32 bands. Next, the central 16 bands (e.g. bands spanning [64, 191] for 8-bit pixels) are defined as the first group, and the rest 16 bands as the second group. Finally, only one group of offsets is transmitted. The reason for this change is that increasing the number of bands tends to decrease the number of pixels in each band and thus increase the possibility of nonzero offsets. Meanwhile, in order to avoid spending too many bits on zero offsets, only the group with more nonzero offsets is sent. The group selection is an encoder-only issue.

Extreme correction (EXC) was first proposed in JCTVC-A124 [11]. A 4-neighbor 2-D cross pattern is used for pixel classification. In JCTVC-B077 [12] and JCTVC-C147 [13], EO was proposed as an improved EXC with localization of each picture. Moreover, a 4-neighbor 2-D diagonal cross pattern is added for pixel classification. In this proposal, EO is further improved with new pixel classification options. Four 2-neighbor 1-D patterns are added (i.e. 0-degree, 45-degree, 90-degree, and 135-degree). In comparison with the 2-D patterns, the 1-D patterns require much lower complexity on the decoder side. Meanwhile, pixel classification partial results of the 1-D patterns can be fully reused for 2-D patterns due to the overlapped footprints, and the encoding time is increased only a little bit.

For more details, readers can refer to JCTVC-D122 [14]. There is only one main difference between the AO method of this proposal and that of JCTVC-D122. In this proposal FU is used for localization while in JCTVC-D122 picture quadtree (PQ) is used for localization.

## Syntax description

MQT_ALF syntax design is shown in Table 1. Modifications are highlighted with yellow backgrounds. As can be seen, several general ALF parameters are transmitted in the sequence parameter set (SPS) while the rest ALF parameters are all sent in the slice header. In the current TMuC, only one slice per picture is supported, and ALF parameters of an entire picture are derived after deblocking each picture. In the future when multiple slices per picture are supported, the locations of ALF parameters in the bitstream may have to be changed accordingly.

Table 1  Modified syntax elements for MQT_ALF

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| alf_td_buf_size_minus1 | 1 | ue(v) |
| alf_pa_fu_size_flag | 1 | u(1) |
| alf_pa_max_group_minus1 | 1 | ue(v) |
| alf_default_fu_width_in_lcu_minus1 | 1 | ue(v) |
| alf_default_fu_height_in_lcu_minus1 | 1 | ue(v) |
| alf_fu_size_syntax_mode | 1 | u(1) |
| AlfCoeffNum[ 0 ] = 6 // 3x3S, SF |  |  |
| AlfCoeffNum[ 1 ] = 14 // 5x5S, SF |  |  |
| AlfCoeffNum[ 2 ] = 14 // 7x7D, SF/PA |  |  |
| AlfCoeffNum[ 3 ] = 22 // 9x9D, SF |  |  |
| AlfOffsetNum[ 0 ] = 4 // 1-D 0-degree pattern EO |  |  |
| AlfOffsetNum[ 1 ] = 4 // 1-D 90-degree pattern EO |  |  |
| AlfOffsetNum[ 2 ] = 4 // 1-D 135-degree pattern EO |  |  |
| AlfOffsetNum[ 3 ] = 4 // 1-D 45-degree pattern EO |  |  |
| AlfOffsetNum[ 4 ] = 6 // 2-D cross pattern EO |  |  |
| AlfOffsetNum[ 5 ] = 6 // 2-D diagonal cross pattern EO |  |  |
| AlfOffsetNum[ 6 ] = 16 // central bands BO |  |  |
| AlfOffsetNum[ 7 ] = 16 // side bands BO |  |  |
| … |  |  |
| } |  |  |

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| alf_param( ) | 2 |  |
| } |  |  |

| alf_param( ) { | C | Descriptor |
| --- | --- | --- |
| alf_reset_td_flag | 2 | u(1)¦ae(v) |
| if( alf_reset_td_flag ) { |  |  |
| for( i = 0; i < 4; i++ ) |  |  |
| AlfTdBufCnt[ i ] = 0 |  |  |
| } |  |  |
| alf_flag | 2 | u(1)¦ae(v) |
| if( alf_flag ) { |  |  |
| alf_fu_size_param( ) |  |  |
| if( alf_fu_size_flag ) |  |  |
| AlfFsNum = alf_fs_num_minus1 + 1 | 2 | ue(v)¦ae(v) |
| else |  |  |
| AlfFsNum = 1 |  |  |
| for( idx = 0; idx < AlfFsNum; idx ++ ) |  |  |
| alf_fs_param( idx ) | 2 |  |
| AlfFuNumW=(PicWidthInLcu+AlfFuWidthInLcu–1)/AlfFuWidthInLcu |  |  |
| AlfFuNumH=(PicHeightInLcu+AlfFuHeightInLcu–1)/AlfFuHeightInLcu |  |  |
| if( alf_fu_size_flag ) { |  |  |
| alf_enable_fu_merge | 2 | u(1)¦ae(v) |
| for( j = 0; j < AlfFuNumH; j++ ) |  |  |
| for( i = 0; i < AlfFuNumW; i++ ) |  |  |
| alf_fs_selection_param( j, i ) | 2 |  |
| } |  |  |
| alf_chroma_idc | 2 | u(2)¦ae(v) |
| … |  |  |
| alf_cu_control_flag | 2 | u(1)¦ae(v) |
| … |  |  |
| } |  |  |
| } |  |  |

| alf_fu_size_param( ) { | C | Descriptor |
| --- | --- | --- |
| if( alf_fu_size_flag ) { | 2 | u (1)¦ae(v) |
| if( alf_change_fu_size ) { | 2 | u(1)¦ae(v) |
| if( alf_fu_size_syntax_mode ) { |  |  |
| alf_ fu_width_in_lcu_minus1 | 2 | ue(v)¦ae(v) |
| alf_fu_height_in_lcu_minus1 | 2 | ue(v)¦ae(v) |
| AlfFuWidthInLcu = alf_fu_width_in_lcu_minus1 + 1 |  |  |
| AlfFuHeightInLcu = alf_fu_height_in_lcu_minus1 + 1 |  |  |
| } else { |  |  |
| alf_fu_size_ratio_minus2 | 2 | ue(v)¦ae(v) |
| AlfRatio = alf_fu_size_ratio_minus2 + 2 |  |  |
| AlfFuWidthInLcu = alf_default_fu_width_in_lcu_minus1*AlfRatio |  |  |
| AlfFuHeightInLcu = alf_default_fu_height_in_lcu_minus1*AlfRatio |  |  |
| } |  |  |
| } else { |  |  |
| AlfFuWidthInLcu = alf_default_fu_width_in_lcu_minus1 + 1 |  |  |
| AlfFuHeightInLcu = alf_default_fu_height_in_lcu_minus1 + 1 |  |  |
| } |  |  |
| } else { |  |  |
| AlfFuWidthInLcu = PicWidthInLcu |  |  |
| AlfFuHeightInLcu = PicHeightInLcu |  |  |
| } |  |  |
| } |  |  |

| alf_fs_param( idx ) { | C | Descriptor |
| --- | --- | --- |
| alf_method[ idx ] | 2 | u(2)¦ae(v) |
| if( alf_method = = SF ) { |  |  |
| alf_sf_shape[ idx ] | 2 | u(1)¦ae(v) |
| alf_sf_size_flag[ idx ] | 2 | u(1)¦ae(v) |
| AlfFilterType = ( alf_sf_shape << 1 ) + alf_sf_size_flag |  |  |
| if( AlfTdBufCnt[ AlfFilterType ] > 0 ) { |  |  |
| if( alf_sf_td_filter[ idx ] ) { | 2 | u(1)¦ae(v) |
| if( AlfTdBufCnt[ AlfFilterType ] > 1 ) |  |  |
| alf_sf_td_filter_idx[ idx ] | 2 | u(v)¦ae(v) |
| } else { |  |  |
| if( alf_sf_td_pred[ idx ] && AlfTdBufCnt[ AlfFilterType ] > 1 ) | 2 | u(1)¦ae(v) |
| alf_sf_td_pred_idx[ idx ] | 2 | u(v)¦ae(v) |
| for( i = 0; i < AlfCoeffNum[ AlfFilterType ]; i++ ) |  |  |
| alf_sf_coeff[ idx ][ i ] | 2 | se(v)¦ae(v) |
| } |  |  |
| } |  |  |
| } else if ( alf_method = = PA ) { |  |  |
| alf_pa_shape[ idx ] | 2 | u(1)¦ae(v) |
| alf_pa_size_flag[ idx ] | 2 | u(1)¦ae(v) |
| AlfFilterType = ( alf_pa_shape << 1 ) + alf_pa_size_flag |  |  |
| alf_pa_group[ idx ][ 0 ] = 1 |  |  |
| AlfGroupCnt = 1 |  |  |
| for( i = 1; i <= alf_pa_max_group_minus1; i++ ) { |  |  |
| alf_pa_group[ idx ][ i ] | 2 | u(1)¦ae(v) |
| AlfGroupCnt += alf_group[ idx ][ i ] |  |  |
| } |  |  |
| for( i = 0; i < AlfGroupCnt; i++ ) { |  |  |
| alf_pa_group_off[ idx ][ i ] | 2 | u(1)¦ae(v) |
| if( !alf_pa_group_off[ idx ][ i ] ) { |  |  |
| for( j = 0; j < AlfCoeffNum[ AlfFilterType ]; j++ ) |  |  |
| alf_pa_coeff[ idx ][ i ][ j ] | 2 | se(v)¦ae(v) |
| } |  |  |
| } |  |  |
| } else if ( alf_method = = AO ) { |  |  |
| AlfOffsetType = alf_ao_type[ idx ] | 2 | ue(v)¦ae(v) |
| for( i = 0; i < AlfOffsetNum[ AlfOffsetType ]; i++ ) |  |  |
| alf_ao_offset[ idx ][ i ] | 2 | se(v)¦ae(v) |
| } |  |  |
| } |  |  |

| alf_fs_selection_param( r, c ) { | C | Descriptor |
| --- | --- | --- |
| if( c = = 0 && r = = 0 ) { |  |  |
| AlfFuFilterSetIdx[ r ][ c ] = 0 |  |  |
| } else { |  |  |
| if( alf_enable_fu_merge ) { |  |  |
| if ( c = = 0 ) { |  |  |
| if( alf_merge_flag ) | 2 | u(1)¦ae(v) |
| AlfFuFilterSetIdx[ r ][ c ] = AlfFuFilterSetIdx[ r – 1 ][ c ] |  |  |
| else if ( AlfFsNum > 1 ) |  |  |
| AlfFuFilterSetIdx[ r ][ c ] = alf_fu_filter_set_idx | 2 | u(v)¦ae(v) |
| else |  |  |
| AlfFuFilterSetIdx [ r ][ c ] = 0 |  |  |
| } else if ( r = = 0 ) { |  |  |
| if( alf_merge_flag ) | 2 | u(1)¦ae(v) |
| AlfFuFilterSetIdx[ r ][ c ] = AlfFuFilterSetIdx[ r ][ c – 1 ] |  |  |
| else if ( AlfFsNum > 1 ) |  |  |
| AlfFuFilterSetIdx[ r ][ c ] = alf_fu_filter_set_idx | 2 | u(v)¦ae(v) |
| else |  |  |
| AlfFuFilterSetIdx [ r ][ c ] = 0 |  |  |
| } else { |  |  |
| if( alf_merge_flag ) { | 2 | u(1)¦ae(v) |
| if( AlfFuFilterSetIdx[ r–1 ][ c ] = = AlfFuFilterSetIdx[ r ][ c–1 ] ){ |  |  |
| AlfFuFilterSetIdx[ r ][ c ] = AlfFuFilterSetIdx[ r ][ c – 1 ] |  |  |
| } else { |  |  |
| if( alf_merge_up_flag ) | 2 | u(1)¦ae(v) |
| AlfFuFilterSetIdx[ r ][ c ] = AlfFuFilterSetIdx[ r – 1 ][ c ] |  |  |
| else |  |  |
| AlfFuFilterSetIdx[ r ][ c ] = AlfFuFilterSetIdx[ r ][ c – 1 ] |  |  |
| } |  |  |
| } else { |  |  |
| if ( AlfFsNum > 1 ) |  |  |
| AlfFuFilterSetIdx[ r ][ c ] = alf_fu_filter_set_idx | 2 | u(v)¦ae(v) |
| else |  |  |
| AlfFuFilterSetIdx[ r ][ c ] = 0 |  |  |
| } |  |  |
| } |  |  |
| } else { |  |  |
| if ( AlfFsNum > 1 ) |  |  |
| AlfFuFilterSetIdx[ r ][ c ] = alf_fu_filter_set_idx | 2 | u(v)¦ae(v) |
| else |  |  |
| AlfFuFilterSetIdx[ r ][ c ] = 0 |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

alf_td_buf_size_minus1 denotes the maximum number of filters that can be stored in the filter buffer for each filter type of the SF method. There are four filter types (3x3S, 5x5S, 7x7D, 9x9D). The filters in the buffer can be used for time-delayed filtering or filter coefficient prediction. When the filter buffer is full, the first-in-first-out (FIFO) scheme is adopted for filter buffer update.

alf_pa_fu_size_flag equal to 0 denotes that the PA method can only be used in each FS when the FU size is equal to the picture size; equal to 1 denotes that the PA method can be used in each FS regardless of the FU size.

alf_pa_max_group_minus1 plus 1 denotes the maximum number of groups for the pixel classification in an FU that selects the PA method. Each pixel in the FU is classified into one group, and pixels of the same group can be processed by one filter or not filtered at all.

alf_default_fu_width_in_lcu_minus1 plus 1 denotes the default FU width in LCUs.

alf_default_fu_height_in_lcu_minus1 plus 1 denotes the default FU height in LCUs.

alf_fu_size_syntax_mode equal to 0 denotes that the FU width and height of a picture are multiples of the default FU width and height, respectively, when the FU size is not the picture size; equal to 1 denotes that the FU width and height of a picture are directly transmitted when the FU size is not the picture size.

alf_param( ) includes all picture-level ALF parameters, which is located in the slice header for now under the assumption that each picture contains only one slice.

alf_reset_td_flag equal to 0 denotes that the filter buffer is not reset; equal to 1 denotes that the filter buffer is reset. An encoder should carefully control this flag in order to support random access (e.g. set this flag to 1 for each intra picture).

alf_flag equal to 0 denotes that ALF is disabled for the entire picture; denotes to 1 denotes that ALF is enabled for at least a portion of the current picture.

alf_fu_size_param( ) includes the FU size information.

alf_fs_num_minus1 plus 1 denotes the number of filter sets when alf_fu_size_flag is 1.

alf_fs_param( idx ) includes parameters of one filter set. Each FU goes with one filter set.

alf_enable_fu_merge equal to 0 denotes that an FU cannot be merged with its left or upper FU; equal to 1 denotes that an FU can be merged with its left or upper FU.

alf_fs_selection_param ( r, c ) includes the information of filter set selection for an FU.

alf_chroma_idc equal to 0, 1, 2, and 3 denotes that both chroma components are not filtered, only Cr is filtered, only Cb is filtered, and both chroma components are filtered, respectively. All the chroma processing and syntax design of the TMuC are remained the same in this proposal.

alf_cu_control_flag equal 0 denotes that the CU-level filter on/off control is disabled; equal to 1 denotes that the CU-level filter on/off control is enabled. The rest syntax elements related to the CU-level on/off control are the same as those in TMuC.

alf_fu_size_flag equal to 0 denotes that the FU size is the same as the picture size; equal to 1 denotes that the FU size is smaller than the picture size and can be derived from information that will be sent subsequently.

alf_change_fu_size equal to 0 denotes that the FU size is the same as the default FU size transmitted in the SPS; equal to 1 denotes that the FU size is derived from information that will be sent subsequently.

alf_fu_width_in_lcu_minus1 plus 1 denotes the FU width in LCUs for the current picture when alf_fu_size_flag is 1 and alf_change_fu_size is 1 and alf_fu_size_syntax_mode is 0.

alf_fu_height_in_lcu_minus1 plus 1 denotes the FU height in LCUs for the current picture when alf_fu_size_flag is 1 and alf_change_fu_size is 1 and alf_fu_size_syntax_mode is 0.

alf_fu_size_ratio_minus2 plus 2 denotes the ratio of the current picture FU size to the default FU size in both horizontal and vertical directions when alf_fu_size_flag is 1 and alf_change_fu_size is 1 and alf_fu_size_syntax_mode is 1.

alf_method denotes the filter method used in the FS. When alf_fu_size_flag is 0, it means the FU size is equal to the picture size and there is only one FU in the picture, candidate methods will include SF (alf_method=0), PA (alf_method=1), and AO (alf_method=2). The OFF method will not be used in the FS because it is redundant with alf_flag equal to 1. When alf_fu_size_flag is 1 (i.e. the FU size is smaller than the picture size) and alf_pa_fu_size_flag is 1, candidate methods will include SF (alf_method=0), AO (alf_method=1), and OFF (alf_method=2). When alf_fu_size_flag is 1 (i.e. the FU size is smaller than the picture size) and alf_pa_fu_size_flag is 0, candidate methods will include SF (alf_method=0), PA (alf_method=1), AO (alf_method=2), and OFF (alf_method=3).

alf_sf_shape equal to 0 denotes the filter shape is square; equal to 1 denotes the filter shape is diamond.

alf_sf_size_flag equal to 0 denotes the filter size is 3x3 when the shape is square and denotes the filter size is 7x7 when the shape is diamond; equal to 1 denotes the filter size is 5x5 when the shape is square and denotes the filter size is 9x9 when the shape is diamond.

alf_sf_td_filter equal to 0 denotes that filter coefficients will be sent subsequently; equal to 1 denotes that time-delayed filtering is selected and no filter coefficient will be sent in the current FS.

alf_sf_td_filter_idx denotes the filter buffer index used to derive filter coefficients for time-delayed filtering.

alf_sf_td_pred equal to 0 denotes that temporal prediction is not used for non-central filter coefficients; equal to 1 denotes that temporal prediction is applied for non-central filter coefficients.

alf_sf_td_pred_idx denotes the filter buffer index used to derive filter coefficients for temporal prediction of non-central filter coefficients.

alf_sf_coeff denotes a filter coefficient or a prediction error of filter coefficient for the SF method. The central filter coefficient is always predicted by using the sum of non-central filter coefficients. The non-central filter coefficients can be temporally predicted or not predicted.

alf_pa_shape equal to 0 denotes the filter shape is square; equal to 1 denotes the filter shape is diamond.

alf_pa_size_flag equal to 0 denotes the filter size is 3x3 when the shape is square and denotes the filter size is 7x7 when the shape is diamond; equal to 1 denotes the filter size is 5x5 when the shape is square and denotes the filter size is 9x9 when the shape is diamond.

alf_pa_group equal to 0 denotes that the current group of the PA method is merged with the previous group; equal to 1 denotes that the current group of the PA method is not merged.

alf_pa_group_off equal to 0 denotes that filtering is applied for the current group of the PA method; equal to 1 denotes that filtering is disabled for the current group of the PA method.

alf_pa_coeff denotes a filter coefficient or a prediction error of filter coefficient for the PA method. The central filter coefficient is predicted by using the sum of non-central filter coefficients. The non-central filter coefficients are not predicted.

alf_ao_type equal to 0, 1, 2, 3, 4, 5, 6, and 7 denotes 1-D 0-degree pattern EO, 1-D 90-degree pattern EO, 1-D 135-degree pattern EO, 1-D 45-degree pattern EO, 2-D cross pattern EO, 2-D diagonal cross pattern EO, central bands BO, and side bands BO, respectively.

alf_ao_offset denotes an offset for the AO method.

alf_merge_flag equal to 0 denotes that the current FU is not merged; equal to 1 denotes that the current FU is merged to the left or the upper FU.

alf_fu_filter_set_idx denotes the filter set index of the current FU when the current FU is not merged. The first FU of the picture always has the filter set index equal to 0 and does not need to send the index.

alf_merge_up_flag equal to 0 denotes that the current FU is merged with the left FU; equal to 1 denotes that the current FU is merged with the upper FU.

When multiple slices are used in one picture and each slice has its own alf_param( ), the AlfFuNumW and AlfFuNumH in alf_param( ) have to be sent and related syntax elements in alf_fu_param( r, c ) have to be modified accordingly. In the current TMuC, each picture has only one slice, so the AlfFuNumW and AlfFuNumH can be derived on the decoder side.

During ALF reconstruction process, when an FS uses the SF method with a new filter, the AlfTdBufCnt[AlfFilterType] and the filter buffer should be updated accordingly after filter coefficients are reconstructed. When the AlfTdBufCnt[AlfFilterType] reaches (alf_td_buf_size_minus1+1), it means the filter buffer is full, and the FIFO scheme is adopted to update the filter buffer for now. Please note that new filters in non-reference pictures cannot be stored into the filter buffer.

## Fast distortion estimation (encoder-only issue)

To perform the rate-distortion optimization (RDO) during the FS selection of one FU, filtering distortion and filter information bitrate have to be calculated repetitively. The filtering distortion is the mean squared error between the original picture and the filtered picture. Therefore, in a conventional way, in order to obtain filtering distortions, the picture has to be filtered for many times. However, the filtering process leads to the picture buffer access which may cause an extra heavy burden on hardware implementation. Therefore, a method of fast distortion estimation was developed in [3] to avoid real filtering, which is the key to significantly reduce the number of encoding passes.

## Encoding pass (encoder-only issue)

There is only one encoding pass in our encoder. Here, one pass means one time access of picture buffers. During encoding and deblocking LCUs, autocorrelation and crosscorrelation can be calculated from original pixels and deblocked pixels without any additional picture buffer access and stored. Next, the number of filter sets, the parameters of each FS including adaptive filter coefficients or adaptive offsets, the size of FUs, and the selection of FS in each FU are derived by RDO with the fast distortion estimation by reusing autocorrelation and crosscorrelation. To this step, there is no filtering, and only on/off flags for ALF control blocks are not yet decided. Then, each pixel is filtered according to the RDO decision, which requires reading the deblocked picture for one time. If the deblocked picture is stored in an on-chip buffer, this external memory access can be saved. Meanwhile, the on/off control is performed, which requires reading the original picture for one time. If the original picture is stored in an on-chip buffer, this external memory access can be saved. Finally, filtered pixels and deblocked pixels are outputted for regions where filtering is enabled and for regions where filtering is disabled, respectively, which requires writing the external picture buffer one time. To sum up, at most two reading operations and one writing operation per pixel are required, and they can be done in parallel to achieve single-pass encoding.

# Simulation results

In JCTVC-C500 [9], six configurations are defined as high efficiency all intra (HE-AI), low complexity all intra (LC-AI), high efficiency random access (HE-RA), low complexity random access (LC-RA), high efficiency low delay (HE-LD), and low complexity low delay (LC-LD). We conducted experiments for all configurations and the experimental data are shown in Table 2, Table 3, and Table 4.

Table 2  Experimental data for HE-AI and LC-AI

| MQT_ALF versus JCTVC-C500 |  | High Efficiency All Intra |  |  | Low Complexity All Intra |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % |
| Class A 2560x1600 | Traffic | 0.1 | 0.0 | 0.0 | -5.9 | -0.9 | -1.5 |
|  | PeopleOnStreet | 0.0 | 0.0 | 0.0 | -7.1 | -4.1 | -2.9 |
| Class B 1080p | Kimono | 0.3 | -0.1 | -0.1 | -3.9 | -2.0 | -1.0 |
|  | ParkScene | -0.2 | -0.1 | -0.1 | -4.4 | -4.5 | -1.6 |
|  | Cactus | 0.2 | -0.1 | -0.1 | -3.7 | -2.4 | -4.4 |
|  | BasketballDrive | 0.1 | -0.2 | -0.2 | -2.4 | -2.9 | -3.1 |
|  | BQTerrace | 0.1 | 0.0 | 0.0 | -2.2 | -3.0 | -1.9 |
| Class C WVGA | BasketballDrill | 0.2 | 0.1 | 0.1 | -3.7 | -2.3 | -4.1 |
|  | BQMall | 0.2 | -0.2 | -0.2 | -2.4 | -2.9 | -3.9 |
|  | PartyScene | 0.0 | -0.1 | -0.1 | -0.6 | -1.5 | -2.3 |
|  | RaceHorses | 0.2 | -0.1 | -0.2 | -3.5 | -5.2 | -6.5 |
| Class D WQVGA | BasketballPass | 0.0 | -0.4 | -0.4 | -1.5 | -1.8 | -2.2 |
|  | BQSquare | -0.5 | -0.1 | -0.1 | -1.0 | -1.5 | -2.3 |
|  | BlowingBubbles | 0.0 | -0.2 | -0.2 | -0.6 | -3.0 | -3.5 |
|  | RaceHorses | 0.3 | -0.3 | -0.3 | -3.8 | -5.5 | -6.2 |
| Class E 720p | Vidyo1 | 0.4 | -0.3 | -0.3 | -5.5 | -5.9 | -6.8 |
|  | Vidyo3 | 0.3 | -0.2 | -0.2 | -5.7 | -4.0 | -3.9 |
|  | Vidyo4 | 0.3 | -0.2 | -0.3 | -4.4 | -7.1 | -7.3 |
| Summary | Class A | 0.1 | 0.0 | 0.0 | -6.5 | -2.5 | -2.2 |
|  | Class B | 0.1 | -0.1 | -0.1 | -3.3 | -3.0 | -2.4 |
|  | Class C | 0.2 | -0.1 | -0.1 | -2.6 | -3.0 | -4.2 |
|  | Class D | -0.1 | -0.2 | -0.2 | -1.7 | -2.9 | -3.5 |
|  | Class E | 0.3 | -0.3 | -0.3 | -5.2 | -5.6 | -6.0 |
|  | All | 0.1 | -0.1 | -0.1 | -3.5 | -3.4 | -3.6 |
|  | Encoding Time % | 100 |  |  | 154 |  |  |
|  | Decoding Time % | 85 |  |  | 176 |  |  |

Table 3  Experimental data for HE-RA and LC-RA

| MQT_ALF versus JCTVC-C500 |  | High Efficiency Random Access |  |  | Low Complexity Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % |
| Class A 2560x1600 | Traffic | -0.1 | 0.6 | 0.3 | -3.3 | -2.7 | -2.8 |
|  | PeopleOnStreet | 0.0 | 0.1 | 0.3 | -5.2 | -1.2 | -0.3 |
| Class B 1080p | Kimono | 0.2 | 0.0 | -0.2 | -2.7 | -2.3 | -0.1 |
|  | ParkScene | -0.3 | 0.3 | 0.0 | -2.9 | -6.2 | -2.7 |
|  | Cactus | -0.3 | -0.2 | -0.5 | -4.6 | -0.4 | -0.2 |
|  | BasketballDrive | -0.1 | -0.1 | -0.1 | -4.4 | -3.3 | -3.3 |
|  | BQTerrace | -0.8 | 0.3 | -0.1 | -10.6 | -16.8 | -10.8 |
| Class C WVGA | BasketballDrill | -1.5 | -0.4 | -0.2 | -4.8 | -2.4 | -6.9 |
|  | BQMall | -0.6 | -0.2 | -0.3 | -3.2 | -0.3 | -1.4 |
|  | PartyScene | -0.4 | -0.2 | -0.2 | -9.2 | -5.2 | -5.5 |
|  | RaceHorses | 0.0 | -0.2 | -0.1 | -2.1 | -3.7 | -5.6 |
| Class D WQVGA | BasketballPass | -0.2 | -0.6 | -0.8 | -1.1 | -1.2 | -1.8 |
|  | BQSquare | -1.6 | 0.0 | -0.2 | -19.4 | -5.5 | -5.6 |
|  | BlowingBubbles | -0.4 | -0.5 | -0.4 | -4.2 | -3.3 | -3.4 |
|  | RaceHorses | -0.3 | -0.5 | -0.4 | -1.6 | -4.6 | -5.1 |
| Class E 720p | Vidyo1 |  |  |  |  |  |  |
|  | Vidyo3 |  |  |  |  |  |  |
|  | Vidyo4 |  |  |  |  |  |  |
| Summary | Class A | 0.0 | 0.3 | 0.3 | -4.2 | -2.0 | -1.6 |
|  | Class B | -0.3 | 0.1 | -0.2 | -5.0 | -5.8 | -3.4 |
|  | Class C | -0.6 | -0.2 | -0.2 | -4.8 | -2.9 | -4.8 |
|  | Class D | -0.6 | -0.4 | -0.4 | -6.6 | -3.6 | -3.9 |
|  | Class E |  |  |  |  |  |  |
|  | All | -0.4 | -0.1 | -0.2 | -5.3 | -3.9 | -3.7 |
|  | Encoding Time % | 98 |  |  | 109 |  |  |
|  | Decoding Time % | 85 |  |  | 169 |  |  |

Table 4  Experimental data for HE-LD and LC-LD

| MQT_ALF versus JCTVC-C500 |  | High Efficiency Low Delay |  |  | Low Complexity Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % |
| Class A 2560x1600 | Traffic |  |  |  |  |  |  |
|  | PeopleOnStreet |  |  |  |  |  |  |
| Class B 1080p | Kimono | -0.4 | -0.4 | -0.2 | -2.4 | -3.1 | -0.8 |
|  | ParkScene | -2.1 | -0.4 | 0.0 | -3.7 | -3.9 | -2.6 |
|  | Cactus | -2.1 | -0.6 | -0.6 | -4.5 | 3.3 | 2.4 |
|  | BasketballDrive | -0.6 | -0.3 | 0.0 | -5.7 | -5.1 | -3.5 |
|  | BQTerrace | -1.3 | 0.0 | -0.4 | -11.6 | -8.2 | -9.7 |
| Class C WVGA | BasketballDrill | -3.0 | -0.5 | -0.7 | -12.5 | -5.8 | -9.9 |
|  | BQMall | -2.3 | -0.2 | -0.4 | -5.8 | -1.9 | -3.1 |
|  | PartyScene | -1.6 | -0.1 | -0.5 | -6.0 | -3.3 | -3.9 |
|  | RaceHorses | -0.5 | -0.5 | -0.5 | -2.7 | -6.4 | -7.1 |
| Class D WQVGA | BasketballPass | -0.5 | -0.3 | -0.7 | -2.0 | -1.5 | -1.8 |
|  | BQSquare | -2.6 | 0.2 | 2.4 | -13.5 | -9.7 | -12.6 |
|  | BlowingBubbles | -1.1 | -0.4 | -0.7 | -3.7 | -1.9 | -2.7 |
|  | RaceHorses | -1.1 | -1.1 | -0.6 | -2.9 | -6.6 | -5.8 |
| Class E 720p | Vidyo1 | -2.7 | -0.8 | -1.2 | -3.8 | -1.0 | 1.2 |
|  | Vidyo3 | -5.6 | -1.0 | 0.0 | -11.6 | -0.6 | 2.0 |
|  | Vidyo4 | -2.1 | -1.9 | -2.0 | -3.7 | -1.5 | 1.8 |
| Summary | Class A |  |  |  |  |  |  |
|  | Class B | -1.3 | -0.4 | -0.3 | -5.6 | -3.4 | -2.8 |
|  | Class C | -1.9 | -0.3 | -0.5 | -6.7 | -4.4 | -6.0 |
|  | Class D | -1.3 | -0.4 | 0.1 | -5.6 | -4.9 | -5.7 |
|  | Class E | -3.5 | -1.2 | -1.1 | -6.4 | -1.0 | 1.7 |
|  | All | -1.9 | -0.5 | -0.4 | -6.0 | -3.6 | -3.5 |
|  | Encoding Time % | 98 |  |  | 108 |  |  |
|  | Decoding Time % | 83 |  |  | 175 |  |  |

For high efficiency configurations, the single-pass MQT_ALF is compared with the 16-pass QC_ALF. It can be seen that for HE-AI, HE-RA, and HE-LD, the BD-rates are 0.1%, -0.4%, and -1.9%, respectively, the encoding times are 100%, 98%, and 98%, respectively, and the decoding times are 85%, 85%, and 83%, respectively. Generally speaking, the proposed MQT_ALF can achieve better compression performance with lower complexity than the QC_ALF in TMuC.

For low complexity configurations, the single-pass MQT_ALF is compared with QC_ALF-off. It can be seen that for LC-AI, LC-RA, and LC-LD, the BD-rates are -3.5%, -5.3%, and -6.0%, respectively, the encoding times are 154%, 109%, and 108%, respectively, and the decoding times are 176%, 169%, and 175%, respectively. For now the computation loading of MQT_ALF is still too high for low complexity applications. Further study on complexity reduction with good coding efficiency trade-off is required.

# Conclusions

MediaTek, Qualcomm, and Toshiba successfully developed an improved ALF solution, namely MQT_ALF. In comparison with the ALF adopted in the current HM, the encoding pass is reduced from 16 to only 1, which has very significant benefits for hardware implementations, e.g., lower I/O bandwidth and latency, and lower power consumption. In the high-efficiency configurations, the bitrate reductions are 0.4% and 1.9% for random access and low delay, respectively, the encoding complexity is 2% lower in terms of processing time, and the decoding complexity is 16% lower in terms of processing time. It is proposed to adopt the MQT_ALF into HM and enable it for high efficiency configurations. Further complexity reduction will be investigated to see whether MQT_ALF can become suitable for low complexity configurations.

# Patent rights declaration(s)

MediaTek Inc., Qualcomm, and TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Tomoo Yamakage, Takeshi Chujoh, and Takashi Watanabe, “TE10 Subtest 2: Reduction of Number of Encoding Passes for Quadtree-based Adaptive Loop Filter (QALF),” Document of Joint Collaborative Team on Video Coding, JCTVC-C082, Oct. 2010.

- In Suk Chong, Wei-Jung Chien, and Marta Karczewicz, “Adaptive Loop Filter with Low Encoder Complexity,” Document of Joint Collaborative Team on Video Coding, JCTVC-C113, Oct. 2010.

- Chia-Yang Tsai, Chih-Ming Fu, Ching-Yeh Chen, Yu-Wen Huang, and Shawmin Lei, “TE10 Subtest2: Coding Unit Synchronous Picture Quadtree-based Adaptive Loop Filter,” Document of Joint Collaborative Team on Video Coding, JCTVC-C143, Oct. 2010.

- “Test Model under Consideration,” Document of Joint Collaborative Team on Video Coding, JCTVC-B205, Jul. 2010.

- Frank Bossen, “Common Test Conditions and Software Reference Configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-B300, Jul. 2010.

- Tomoo Yamakage, Takeshi Chujo, Shawmin Lei, Keiichi Chono, Marta Karczewicz ,Andrew Segall, Tomoyuki Yamamoto, Byeungwoo Jeon, Jeongyeon Lim, and Jizheng Xu, “Tool Experiment 10: In-Loop Filtering,” Document of Joint Collaborative Team on Video Coding, JCTVC-B310, Jul. 2010.

- TK Tan, Gary J. Sullivan, and Jens-Rainer Ohm, “Summary of HEVC Working Draft 1 and HEVC Test Model (HM),” Document of Joint Collaborative Team on Video Coding, JCTVC-C405, Oct. 2010.

- Tomoo Yamakage, Takeshi Chujoh, Yu-Wen Huang, Keiichi Chono, In Suk Chong, Tomoyuki Yamamoto, Jeongyeon Lim, Byeungwoo Jeon, Jizheng Xu, and Matthias Narroschke, “Description of CE8: In-Loop Filtering,” Document of Joint Collaborative Team on Video Coding, JCTVC-C508, Oct. 2010.

- Frank Bossen, “Common Test Conditions and Software Reference Configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-C500, Oct. 2010.

- In Suk Chong, Wei-Jung Chien, Naren Malayath, and Marta Karczewicz, “Encoder Complexity Analysis and Performance Report on Adaptive Loop Filter,” Document of Joint Collaborative Team on Video Coding, JCTVC-B095, Jul. 2010.

- Ken McCann, Woo-Jin Han, and Il-Koo Kim, “Samsung’s Response to the Call for Proposals on Video Compression Technology,” Document of Joint Collaborative Team on Video Coding, JCTVC-A124, Apr. 2010.

- Yu-Wen Huang, Chih-Ming Fu, Ching-Yeh Chen, Chia-Yang Tsai, Yongying Gao, JichengAn, Kai Zhang, and Shawmin Lei, “In-Loop Adaptive Restoration,” Document of Joint Collaborative Team on Video Coding, JCTVC-B077, Jul. 2010.

- Chih-Min Fu, Ching-Yeh Chen, Yu-Wen Huang, and Shawmin Lei, “TE10 Subtest3: Quadtree-based Adaptive Offset,” Document of Joint Collaborative Team on Video Coding, JCTVC-C147, Oct. 2010.

- Chih-Min Fu, Ching-Yeh Chen, Yu-Wen Huang, and Shawmin Lei, “CE8 Subset3: Picture Quadtree Adaptive Offset (PQAO),” Document of Joint Collaborative Team on Video Coding, JCTVC-D122, Jan. 2011.