| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D121 |
| --- | --- |

| Title: | CE1: Cross-verification report of Samsung’s Proposal by JVC KENWOOD |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Motoharu Ueda Satoru Sakazume JVC KENWOOD Holdings, Inc. 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-9945 ueda.motoharu@jk-holdings.com sakazume.satoru@jk-holdings.com |
| Source: | JVC KENWOOD Holdings, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-verification results of CE1 proposal JCTVC-D329 by Samsung.
The Bi-Directional Optical Flow (BIO) technique leads pixel-wise motion vector without decoder side motion search and refine the motion compensation quality using optical flow approach.
The cross-verification results show that BIO technique provides 1.9% gain at random access high efficiency case and 4.8% gain at random access low complexity case. The results are matched with the results of the proposal JCTVC-D329.

# Introduction

BIO [1] is the technique that the high accuracy motion vectors are derived by using the calculation based on optical flow concept from the detected motion vectors without any additional information in the decoder side. In very short time difference, the conditions are assumed that the motion of the objects in the picture is constant, and the pixel value is smoothly connected with the surrounding pixel. The model based on the assumption like Figure 1 is applied in this technique.

Figure 1  The assumption of the steady motion vectors

The predictive pixels based on optical flow (POpticalFlow) are calculated by using the function (1)

POpticalFlowi,j=(P0i,j+P1i,j+vxi,jIx0i,j−Ix1i,j++vyi,jIy0i,j−Iy1i,j+1)>>1  ---- (1)

Where, I(l)x[i,j], I(l)y[i,j] are luminance gradients on l=0,1 on references, vx[i,j] and vy[i,j] are pixel-wise motion vector.

For calculation of POpticalFlow, the luminance gradients and pixel-wise motion vector are needed. The luminance gradient values are calculated with the filtering process with the filter coefficients that are derived from the DCT-IF concept. The vx and vy are derived by using the condition (2)

i',j∈ΩΔ2i',j=min.  ---- (2)

Where,

i',j'∈Ω:i−L≤i'≤i+L,j−L≤j'≤j+L  ---- (3)

Δi,j=P0i,j−P1i,j+vxi,jIx0i,j−Ix1i,j+vyi,jIy0i,j−Iy1i,j  ---- (4)

# Experiments

## Test conditions

In CE1 activity [2], the common test conditions [3] are used for experiments of performance and complexity toward HM. In the proposal JCTVC-D329, it was reported results at random access case, because there is no effect to the results (same results as HM) at the low delay case.

## Experimental environment

The experimental environment is shown in Table 1.

Table 1 Experimental environment

| CPU | Intel Core i7 870 2.93GHz |
| --- | --- |
| Memory | 8GB (DDR3) |
| OS | Windows XP SP2 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | Win32(encoder) , Win32/x64(decoder) |

## Simulation Results

### Performance comparison

Table 2 shows the performance comparison results at random access.

Table 2 Performance comparison results at random access

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.7 | -0.8 | -0.7 | -2.1 | -1.0 | -1.0 |
| Class B | -1.6 | -0.7 | -0.8 | -2.7 | -0.8 | -0.7 |
| Class C | -1.6 | -1.0 | -1.0 | -4.5 | -1.8 | -1.7 |
| Class D | -2.8 | -1.5 | -1.4 | -9.1 | -4.0 | -3.8 |
| Class E |  |  |  |  |  |  |
| All | -1.9 | -1.0 | -1.0 | -4.8 | -1.9 | -1.8 |

The results show that the BIO technique provides 1.9% BD-rate [4] gain at random access high efficiency case and 4.8% at random access low complexity case against the HM (TMuC0.9 anchor).

These results are matched with the results of the proposal JCTVC-D329.

### Complexity comparison

For the complexity comparison of decoder, two results of decoding time are used for comparison. One is decoding time with writing out decoded yuv (with –o option), and the other is decoding time without writing out decoded yuv (without –o option).

In addition, the results of decoding time using x64 (64bit executable) are added to observe the influence of different platform (32bit/64bit).

Table 3 shows the complexity comparison results at random access.

Table 3 Complexity comparison results at random access

|  | Condition | Random access High | Random access LoCo |
| --- | --- | --- | --- |
| Enc Time | Win32 Executable | 118% | 130% |
| Dec Time | Win32 Executable with "-o" option | 286% | 435% |
|  | Win32 Executable without "-o" option | 299% | 536% |
|  | x64 Executable with "-o" option | 212% | 279% |
|  | x64 Executable without "-o" option | 223% | 386% |

It seems that the complexity increase of Win32 (32bit compiler) executable decoder on BIO is much than x64 (64bit compiler) executable decoder.

The decoder complexity comparison results of x64 executable are close to results of JCTVC-D329.

## Source code verification

We have verified the source code distributed by Samsung about the macro BIO.

In the case that BIO is defined as 1, BIO technique is performed for Bi-predictive interpolation (without extrapolation) including skip/direct/merge case.

The BIO routine is implemented to motion compensation part on both encoder and decoder into HM software (TMuC0.9).

# Conclusions

This contribution reports the cross-verification results of CE1 proposal JCTVC-D329 by Samsung.

BIO technique can improve coding efficiency under random access condition with other relevant tools, and the performance comparison results are matched with the results of the proposal JCTVC-D329.

# References

[1] “CE1: Samsung’s test for bi-directional optical flow”,

JCTVC-D329, Deagu, January 2011.

[2] “Core Experiment 1: ”,

JCTVC-C501, Guangzhou, October 2010.

[3] “Common test conditions and software reference configurations”,

JCTVC-C500, Guangzhou, October 2010.

[4] G. Bjøntegaard, “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. and Victor Company of Japan, Limited may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).