| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D122 |
| --- | --- |

| Title: | CE8 Subset3: Picture Quadtree Adaptive Offset |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Ming Fu Ching-Yeh Chen Yu-Wen Huang Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Emails: | chihming.fu@mediatek.com chingyeh.chen@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution describes MediaTek’s proposal on adaptive offset (AO). The proposed picture quadtree adaptive offset (PQAO) uses local adaptation to increase coding efficiency. Each picture can be divided into multi-level quadtree partitions, and each leaf partition can be enhanced by band offset (BO) or edge offset (EO). Both BO and EO classify pixels of a partition into groups, and one offset is derived for each group. BO and EO have much lower complexity than adaptive loop filter (ALF) in TMuC0.9 because of much simpler pixel classification operations and only offset adjustment for pixels. When compared with the JCTVC-C500 anchor, PQAO achieves 1.5% and 2.2% bitrate reductions for HE-RA and HE-LD, respectively. For HE, the encoding time is increased by about 1%, and the decoding time is increased by about 2%. Moreover, it is observed that most of the coding gain from PQAO is not overlapped with ALF.

- Introduction

The basic concept of AO is to categorize pixels into different categories, and the pixel value offset between reconstructed pixels and original pixels of each category is calculated for compensation. PQAO is one of MediaTek’s adaptive restoration tools [1][2][3][4] that can reduce mean square error between reconstructed pixels and original pixels by adding an offset on reconstructed pixels. PQAO and adaptive loop filtering (ALF) are now processed sequentially as shown in Figure 1. In this proposal, simplified AO methods are proposed. Besides prior AO methods, they can be selected in each quadtree partition to increase coding efficiency and decrease decoding time. Moreover, the number of decoding passes is reduced from 2, for the AO in JCTVC-A124 [5], to 1, for the PQAO in this proposal. This is because in JCTVC A124 each pixel has to be processed by extreme correction (EXC) and then by band correction (BDC) while in PQAO each pixel will be processed by only one AO method. Special attention is also paid to reduce the number of encoding passes from 3 for our previous work to 1 in this proposal. Although there are many AO methods to be tested in the rate-distortion optimization process, the encoding time is three times faster than our previous proposal [1] by using the technique of distortion estimation. The rest of this contribution is organized as follows. PQAO algorithm will be described in Section 2, simulation results will be discussed in Section 3, and finally Section 4 gives conclusions.

Figure 1  Encoder block diagram with PQAO.

# Algorithm description

## Overview

PQAO divides a picture into multi-level quad-tree partitions, and each partition can select one AO method to compensate the pixel value. In this proposal, there are eight AO candidate methods for each partition to pick the best one according to rate-distortion criterion. In order to let decoders process AO LCU by LCU, the picture quadtree partition boundaries are aligned with LCU boundaries to ensure that pixels in the same LCU will share the same AO method.

A top-down splitting strategy is used for determining picture quadtree structures, as shown in Figure 2. Each partition can select one of eight AO methods including two groups of band offset (BO) and six different patterns of edge offset (EO), as shown in Figure 3. Besides the proposed AO methods, doing nothing (OFF) is also an option for each partition. For all partitions, the collection of statistical information for various AO methods can be performed during encoding LCUs (right after deblocking), which does not require any picture buffer access. The best quadtree partitioning and corresponding AO methods are decided based on a modified RDO process which uses a simple equation to get estimated distortions of AO methods. Only the final results after adding offsets are required to access the external memory (write the reference picture buffer). Hence, single-pass PQAO is achieved.

Figure 2  Example of a 3-level quadtree structure

Figure 3  Example of decision result for each quadtree partition

## Band offset

BO classifies all pixels of a partition into multiple bands where each band contains pixels of the same intensity interval and has a derived offset. The intensity range is equally divided into 32 intervals between 0 and the maximum intensity value (e.g. 255 for 8-bit pixels), and each interval has an offset. We divide these 32 bands into two groups and select one of these two groups for offset compensation. As shown in Figure 4, the central 16 bands and the rest 16 bands are denoted as the first group and the second group, respectively, and only one group of offsets is sent. It is very likely that one partition only have pixel values in a portion of the full range, and the proposed grouping method can adapt to the local property of pixels in one partition and increase the possibility of nonzero offsets while avoid sending many zero offsets. Furthermore, we also detect if the original input signal satisfies the “broadcast legal criterion” or not. If it does (i.e. pixel values all within [16, 235] for luma and [16, 240] for chroma), we do not have to send offsets of the outer 4 bands in the second group. The broadcast legal information is a one-bit syntax, which can also be used for controlled clipping. Because the categorization criterion of BO only needs to read the first five significant bits of pixels, the complexity of the proposed BO method is very low.

Figure 4  Two BO groups.

## Edge offset

EO classifies all pixels of a partition into multiple categories by comparing with neighboring pixels and compensates the average offset in each category. The proposed EO patterns are shown in Figure 5. We have four 1-D patterns and two 2-D patterns to classify pixels. Since a 1-D pattern only uses two neighboring pixels, the number of operations is reduced in comparison with that of a 2-D pattern. The classification rules of 1-D and 2-D patterns are shown in Figure 6. We set the offset coefficients with 1-bit bit-depth increase for better coding efficiency when high efficiency configurations, which use 4-bit internal bit-depth increase, are considered. No bit-depth increase is used when low complexity configurations are considered.

(a) 1-D 0-degree (b) 1-D 90-degree (c) 1-D 135-degree (d) 1-D 45-degree (e) 2-D cross (f) 2-D diagonal

Figure 5  EO pixel classification patterns

|  |  |  | Category | Condition |
| --- | --- | --- | --- | --- |
|  |  |  | 1 | C < 4 neighbors |
| Category | Condition |  | 2 | C < 3 neighbors && C = 4th neighbor |
| 1 | c < 2 neighboring pixels |  | 3 | C < 3 neighbors && C > 4th neighbor |
| 2 | c < 1 neighbor && c == 1 neighbor |  | 4 | C > 3 neighbors && C < 4th neighbor |
| 3 | c > 1 neighbor && c == 1 neighbor |  | 5 | C > 3 neighbors && C = 4th neighbor |
| 4 | c > 2 neighbors |  | 6 | C > 4 neighbors |
| 0 | Nono of the above |  | 0 | None of the above |

- Classification rule for 1-D patterns           (b) Classification rule for 2-D patterns

Figure 6  EO categories

## Syntax description

PQAO syntax design is shown in Table 1. General PQAO parameters are transmitted in the sequence parameter set (SPS) while rest PQAO parameters are sent in the slice header. In the current HM, only one slice per picture is supported, and PQAO parameters of a picture are derived after deblocking. In the future when multiple slices per picture are supported, the locations of PQAO parameters in the bitstream may have to be changed accordingly.

Table 1  Syntax elements for PQAO

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| pqao_used_flag | 1 | u(1) |
| PqaoOffsetNum[ 0 ] = 4 // 1-D 0-degree pattern EO |  |  |
| PqaoOffsetNum[ 1 ] = 4 // 1-D 90-degree pattern EO |  |  |
| PqaoOffsetNum[ 2 ] = 4 // 1-D 135-degree pattern EO |  |  |
| PqaoOffsetNum[ 3 ] = 4 // 1-D 45-degree pattern EO |  |  |
| PqaoOffsetNum[ 4 ] = 6 // 2-D cross pattern EO |  |  |
| PqaoOffsetNum[ 5 ] = 6 // 2-D diagonal cross pattern EO |  |  |
| PqaoOffsetNum[ 6 ] = 16 // central bands BO |  |  |
| PqaoOffsetNum[ 7 ] = 16 // side bands BO |  |  |
| MaxSplitLevel = 4 // maximum split level |  |  |
| … |  |  |
| } |  |  |

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| pqao_param( ) | 2 |  |
| } |  |  |

| pqao_param( ) { | C | Descriptor |
| --- | --- | --- |
| pqao_band_restriction_flag | 2 | u(1)¦ae(v) |
| pqao_flag | 2 | u(1)¦ae(v) |
| if( pqao_flag ) { |  |  |
| pqao_split_param( 0, 0, 0 ) |  |  |
| pqao_offset_param( 0, 0, 0 ) |  |  |
| } |  |  |
| } |  |  |

| pqao_split_param( x, y, Depth ) { | C | Descriptor |
| --- | --- | --- |
| if( Depth < MaxSplitLevel ) |  |  |
| split_flag[ Depth ][ y ][ x ] = pqao_split_flag | 2 | u(1)¦ae(v) |
| else |  |  |
| split_flag[ Depth ][ y ][ x ] = 0 |  |  |
| if( split_flag[ Depth ][ y ][ x ] ) { |  |  |
| pqao_split_param( x + 0, y + 0, Depth + 1 ) |  |  |
| pqao_split_param( x + 1, y + 0, Depth + 1 ) |  |  |
| pqao_split_param( x + 0, y + 1, Depth + 1 ) |  |  |
| pqao_split_param( x + 1, y + 1, Depth + 1 ) |  |  |
| } |  |  |
| } |  |  |

| pqao_offset_param ( x, y, Depth ) { | C | Descriptor |
| --- | --- | --- |
| if( split_flag[ Depth ][ y ][ x ] ) { |  |  |
| pqao_offset_param ( x + 0, y + 0, Depth + 1 ) |  |  |
| pqao_offset_param ( x + 1, y + 0, Depth + 1 ) |  |  |
| pqao_offset_param ( x + 0, y + 1, Depth + 1 ) |  |  |
| pqao_offset_param ( x + 1, y + 1, Depth + 1 ) |  |  |
| } else { |  |  |
| type_idx[ Depth ][ y ][ x ] = pqao_type_idx | 2 | ue(v)¦ae(v) |
| if( pqao_type_idx != 0 ) { |  |  |
| for( i = 0; i < PqaoOffsetNum[ pqao_type_idx ]; i++ ) |  |  |
| offset[ Depth ][ y ][ x ][ i ] = pqao_offset | 2 | se(v)¦ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |

pqao_used_flag equal to 0 denotes that PQAO is disabled for the current sequence; equal to 1 denotes that PQAO is enabled for current sequence.

pqao_band_restriction_flag equal to 0 denotes that the range of luma pixel values remains unchanged; equal to 1 denotes that the luma pixel values are restricted by the range of broadcast legal signal.

pqao_flag equal to 0 denotes that PQAO is disabled for the entire picture; equal to 1 denotes that PQAO is enabled for at least a portion of the current picture.

pqao_split_flag equal to 0 denotes that current picture partition is a leaf; equal to 1 denotes that the current picture partition is further spitted into four children partitions.

pqao_type_idx indicates the offset type for the current picture partition as shown in Table 2.

Table 2  The offset type indicated by pqao_type_idx.

| pqao_type_idx | offset type |
| --- | --- |
| 0 | off |
| 1 | 1-D 0-degree pattern EO |
| 2 | 1-D 90-degree pattern EO |
| 3 | 1-D 135-degree pattern EO |
| 4 | 1-D 45-degree pattern EO |
| 5 | 2-D cross pattern EO |
| 6 | 2-D diagonal cross pattern EO |
| 7 | central bands BO |
| 8 | side bands BO |

pqao_offset denotes an offset of the selected PQAO type for current picture partition.

# Simulation results

Simulation results of all six test configurations in JCTVC-C500 [6] are shown in Table 3, Table 4, and Table 5. The average luma bitrate reductions achieved by PQAO are 0.8%, 1.5% and 2.2% for high efficiency all intra (HE-AI), high efficiency random access (HE-RA), and high efficiency low delay (HE-LD), respectively. The encoding times for HE-AI, HE-RA, and HE-LD are increased by 3%, 1% and 1%, respectively. The decoding times for HE-AI, HE-RA, and HE-LD are increased by 2%, 2% and 3%, respectively. The average luma bitrate reductions achieved by PQAO are 1,1%, 1.3% and 2.7% for low complexity all intra (LC-AI), low complexity random access (LC-RA), and low complexity low delay (LC-LD), respectively. The encoding times for LC-AI, LC-RA, and LC-LD are increased by 7%, 2% and 1%, respectively. The decoding times for LC-AI, LC-RA, and LC-LD are increased by 7%, 7% and 9%, respectively.

Table 3  Experimental data for HE-AI and LC-AI

| PQAO versus JCTVC-C500 |  | High Efficiency All Intra |  |  | Low Complexity All Intra |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % |
| Class A 2560x1600 | Traffic | -0.9 | 0.0 | 0.0 | -1.3 | 0.1 | 0.1 |
|  | PeopleOnStreet | -1.8 | 0.1 | 0.1 | -2.5 | 0.1 | 0.1 |
| Class B 1080p | Kimono | -0.2 | 0.1 | 0.1 | -0.5 | 0.2 | 0.2 |
|  | ParkScene | -0.8 | 0.1 | 0.1 | -0.8 | 0.1 | 0.1 |
|  | Cactus | -0.5 | 0.1 | 0.2 | -0.8 | 0.3 | 0.3 |
|  | BasketballDrive | -0.6 | 0.1 | 0.1 | -0.9 | 0.3 | 0.3 |
|  | BQTerrace | -0.8 | 0.1 | 0.1 | -1.0 | 0.2 | 0.2 |
| Class C WVGA | BasketballDrill | -1.5 | 0.1 | 0.1 | -2.0 | 0.5 | 0.5 |
|  | BQMall | -0.8 | 0.0 | 0.0 | -0.9 | 0.2 | 0.2 |
|  | PartyScene | -0.6 | 0.0 | 0.0 | -0.4 | 0.1 | 0.1 |
|  | RaceHorses | -0.6 | 0.1 | 0.1 | -0.8 | 0.1 | 0.1 |
| Class D WQVGA | BasketballPass | -0.6 | 0.0 | 0.0 | -0.8 | 0.2 | 0.2 |
|  | BQSquare | -1.5 | 0.1 | 0.1 | -1.2 | 0.1 | 0.1 |
|  | BlowingBubbles | -0.4 | 0.0 | 0.0 | -0.4 | 0.1 | 0.1 |
|  | RaceHorses | -0.7 | 0.0 | 0.1 | -0.9 | 0.1 | 0.2 |
| Class E 720p | Vidyo1 | -1.0 | 0.0 | 0.0 | -1.7 | 0.2 | 0.2 |
|  | Vidyo3 | -1.0 | 0.4 | 0.4 | -1.3 | 0.6 | 0.6 |
|  | Vidyo4 | -0.8 | 0.1 | 0.1 | -1.4 | 0.2 | 0.2 |
| Summary | Class A | -1.3 | 0.1 | 0.1 | -1.9 | 0.1 | 0.1 |
|  | Class B | -0.6 | 0.1 | 0.1 | -0.8 | 0.2 | 0.2 |
|  | Class C | -0.9 | 0.0 | 0.0 | -1.0 | 0.2 | 0.2 |
|  | Class D | -0.8 | 0.0 | 0.0 | -0.8 | 0.1 | 0.1 |
|  | Class E | -0.9 | 0.2 | 0.2 | -1.5 | 0.3 | 0.3 |
|  | All | -0.8 | 0.1 | 0.1 | -1.1 | 0.2 | 0.2 |
|  | Encoding Time % | 103 |  |  | 107 |  |  |
|  | Decoding Time % | 102 |  |  | 107 |  |  |

Table 4  Experimental data for HE-RA and LC-RA

| PQAO versus JCTVC-C500 |  | High Efficiency Random Access |  |  | Low Complexity Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % |
| Class A 2560x1600 | Traffic | -1.1 | 0.0 | 0.1 | -0.4 | 0.0 | 0.0 |
|  | PeopleOnStreet | -2.1 | 0.0 | 0.3 | -2.3 | -0.1 | -0.1 |
| Class B 1080p | Kimono | -0.2 | 0.0 | 0.0 | -0.4 | 0.1 | 0.1 |
|  | ParkScene | -0.8 | 0.2 | 0.2 | -0.9 | 0.0 | 0.1 |
|  | Cactus | -1.2 | 0.2 | 0.0 | -1.8 | 0.8 | 0.1 |
|  | BasketballDrive | -1.1 | 0.0 | 0.0 | -1.5 | 0.2 | 0.0 |
|  | BQTerrace | -4.2 | 0.1 | 0.9 | -4.2 | -0.2 | -0.4 |
| Class C WVGA | BasketballDrill | -2.0 | -0.1 | 0.0 | -2.0 | 0.1 | -0.1 |
|  | BQMall | -1.4 | -0.1 | 0.0 | -0.9 | 0.6 | 0.3 |
|  | PartyScene | -0.9 | 0.0 | 0.0 | -1.1 | 0.3 | 0.3 |
|  | RaceHorses | -0.9 | 0.1 | 0.2 | -0.9 | -0.1 | 0.0 |
| Class D WQVGA | BasketballPass | -0.5 | 0.0 | -0.1 | -0.5 | -0.1 | 0.1 |
|  | BQSquare | -4.2 | 0.8 | 0.9 | -2.2 | 1.0 | 1.5 |
|  | BlowingBubbles | -0.7 | -0.1 | 0.1 | -0.2 | 0.2 | 0.4 |
|  | RaceHorses | -0.8 | 0.3 | 0.0 | -0.5 | 0.1 | 0.0 |
| Class E 720p | Vidyo1 |  |  |  |  |  |  |
|  | Vidyo3 |  |  |  |  |  |  |
|  | Vidyo4 |  |  |  |  |  |  |
| Summary | Class A | -1.6 | 0.0 | 0.2 | -1.3 | -0.1 | 0.0 |
|  | Class B | -1.5 | 0.1 | 0.2 | -1.7 | 0.2 | 0.0 |
|  | Class C | -1.3 | 0.0 | 0.0 | -1.2 | 0.2 | 0.1 |
|  | Class D | -1.5 | 0.2 | 0.2 | -0.8 | 0.3 | 0.5 |
|  | Class E |  |  |  |  |  |  |
|  | All | -1.5 | 0.1 | 0.2 | -1.3 | 0.2 | 0.2 |
|  | Encoding Time % | 101 |  |  | 102 |  |  |
|  | Decoding Time % | 102 |  |  | 109 |  |  |

Table 5  Experimental data for HE-LD and LC-LD

| PQAO versus JCTVC-C500 |  | High Efficiency Low Delay |  |  | Low Complexity Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % |
| Class A 2560x1600 | Traffic |  |  |  |  |  |  |
|  | PeopleOnStreet |  |  |  |  |  |  |
| Class B 1080p | Kimono | -0.3 | 0.1 | 0.2 | -0.9 | 0.5 | 0.6 |
|  | ParkScene | -1.1 | 0.1 | 0.4 | -1.7 | 0.2 | -0.1 |
|  | Cactus | -2.1 | 0.3 | 0.5 | -3.5 | 1.0 | 0.9 |
|  | BasketballDrive | -1.4 | 0.0 | 0.2 | -2.3 | 0.4 | 0.6 |
|  | BQTerrace | -3.5 | 0.7 | 0.0 | -5.8 | 0.9 | 1.2 |
| Class C WVGA | BasketballDrill | -4.1 | -0.2 | -0.1 | -5.7 | 0.7 | 0.7 |
|  | BQMall | -2.9 | 0.6 | 0.3 | -2.6 | 0.3 | 0.6 |
|  | PartyScene | -2.5 | -0.1 | -0.2 | -1.3 | 0.1 | 0.0 |
|  | RaceHorses | -1.0 | 0.0 | -0.1 | -0.9 | 0.2 | 0.2 |
| Class D WQVGA | BasketballPass | -0.9 | 0.1 | 0.2 | -1.0 | 0.9 | 0.6 |
|  | BQSquare | -5.6 | -1.1 | 1.3 | -2.9 | -2.2 | -1.7 |
|  | BlowingBubbles | -1.5 | -0.4 | -0.2 | -0.8 | 0.4 | 0.2 |
|  | RaceHorses | -1.2 | -0.3 | 0.2 | -0.9 | 0.1 | 0.2 |
| Class E 720p | Vidyo1 | -2.3 | 1.3 | 0.2 | -2.3 | 0.9 | 1.5 |
|  | Vidyo3 | -4.6 | 0.2 | 1.1 | -7.3 | 2.9 | 2.0 |
|  | Vidyo4 | -1.2 | -0.5 | 0.8 | -2.7 | 1.5 | 0.9 |
| Summary | Class A |  |  |  |  |  |  |
|  | Class B | -1.7 | 0.2 | 0.2 | -2.8 | 0.6 | 0.6 |
|  | Class C | -2.6 | 0.1 | 0.0 | -2.6 | 0.3 | 0.4 |
|  | Class D | -2.3 | -0.4 | 0.4 | -1.4 | -0.2 | -0.2 |
|  | Class E | -2.7 | 0.4 | 0.7 | -4.1 | 1.8 | 1.5 |
|  | All | -2.2 | 0.1 | 0.3 | -2.7 | 0.6 | 0.5 |
|  | Encoding Time % | 101 |  |  | 101 |  |  |
|  | Decoding Time % | 103 |  |  | 108 |  |  |

# Conclusions

MediaTek developed a picture quadtree adaptive offset, namely PQAO. In the high efficiency configurations, the bitrate reductions are 1.5% and 2.2% for random access and low delay, respectively, the encoding complexity increase is 1% in terms of processing time, and the decoding complexity increase is about 2.2% in terms of processing time. It is proposed to adopt the PQAO into HM and enable it for high efficiency configurations. Further complexity reduction will be investigated to see if PQAO can become suitable for low complexity configurations.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Chih-Ming Fu, Ching-Yeh Chen, Yu-Wen Huang, and Shawmin Lei, “TE10 Subtest 3: Quadtree-based Adaptive Offset,” Document of Joint Collaborative Team on Video Coding, JCTVC-C147, Oct. 2010.

- Yu-Wen Huang, Chih-Ming Fu, Ching-Yeh Chen, Chia-Yang Tsai, Yongying Gao, JichengAn, Kai Zhang, and Shawmin Lei, “In-Loop Adaptive Restoration,” Document of Joint Collaborative Team on Video Coding, JCTVC-B077, July, 2010.

- Yu-Wen Huang, Chih-Ming Fu, Yu-Pao Tsai, Jian-Liang Lin, Yulin Chang, Jyh-Huei Guo, Ching-Yeh Chen, Shawmin Lei, Xun Guo, Yongying Gao, Kai Zhang, and Jicheng An, “A Technical Description of MediaTek’s Proposal to the JCT-VC CfP,” Document of Joint Collaborative Team on Video Coding, JCTVC-A109, Apr. 2010.

- Yu-Wen Huang, Chih-Ming Fu, Xun Guo, and Shawmin Lei, “Improved Quadtree-based Adaptive Loop Filter,” ITU-T SG16 Q.6 Document, VCEG-AL24, Jul. 2009.

- Ken McCann, Woo-Jin Han, and Il-Koo Kim, “Samsung’s Response to the Call for Proposals on Video Compression Technology,” Document of Joint Collaborative Team on Video Coding, JCTVC-A124, Apr. 2010.

- Frank Bossen, “Common Test Conditions and Software Reference Configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-C500, Oct. 2010.