| hmmJoint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D123 |
| --- | --- |

| Title: | CE8 Subset3: Controlled Clipping |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang Chih-Ming Fu Yu-Wen Huang Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Emails: | yulin.chang@mediatek.com chihming.fu@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution describes MediaTek’s proposal on controlled clipping (CC). The main concept of CC is to signal the minimum and maximum of original pixel values for predicted or reconstructed pixels to be clipped within the range between the minimum and the maximum. In this way, the distortion between the original video and the decoded video can be reduced. We applied CC at four different positions of the coding path, i.e. post-prediction, post-reconstruction, post-deblocking, and post-adaptive loop filtering (ALF). Moreover, CC also can be applied as a post processing. The in-loop CC can be applied at each of the four positions, while the post-loop CC is applied at the output of the decoder. The output pixels of the post-loop CC are not stored in the reference picture buffer but only used for display. Different in-loop/post-loop CC options are tested, and we found that the post-loop CC (but with normative CC syntax in slice header) can achieve the best coding efficiency. Comparing with the anchor in JCTVC-C500, the normative post-loop CC can achieve 0.6% and 0.4% bitrate reductions for high efficiency random access (HE-RA) configuration and high efficiency low delay (HE-LD) configuration, respectively, when the CC information is sent in the slice header. When the CC information is sent in another NAL packet as a non-normative method, the bitrate reductions are dropped to 0.4% and 0.2% for HE-RA and HE-LD, respectively, due to the increasing overhead. There is almost no increase in encoding or decoding time.

- Introduction

One differentiating property of the high efficiency video coding (HEVC) is that the system adapts to the statistics of the input signal. This adaptation is evident in the motion interpolation design, which incorporates sets of filters to accommodate different signal characteristics, and in ALF design that allows an encoder to design and transmit a loop filter to the decoder. The additional adaptation provides increased coding efficiency for many sequences. Unfortunately, it also results in dynamic range expansion. For image sequences that are transmitted at full range (e.g. [0, 255] for 8-bit pixels), this dynamic range expansion is not an issue. The decoding process currently restricts intermediate values to the full range, and so any dynamic range expansion is handled implicitly. A problem occurs when the range of input data is smaller than the full range. One very common scenario pointed out in JCTVC-B113 [1] is that the image data are stored with “broadcast legal values”. In this case, the luma signal is traditionally in the range of [16, 235], and the chroma signal is traditionally in the range of [16, 240]. As the histogram of BQTerrace sequence shows in Figure 1(a), the original luma signal is in the range [16, 235]. After coding with the HEVC TMuC system, the reconstructed pixel values are no longer in this range and exceed the range of the input values. The histogram of the reconstructed video is shown in Figure 1(b), where the luma signal now exceeds the known input range.

| - Original Sequence | - Decoded Sequence (Encoded by low delay configuration, QP=22) |
| --- | --- |

Figure 1  Histogram for BQSquare 416x240.

In this contribution, we propose to transmit the known range of the luma and chroma values. The encoder and decoder both use these range values during clipping operations. We call this operation “controlled clipping” (CC), and it was previously presented in JCTVC-B077 [2] and JCTVC-C146 [3] and is included in the subset3 of core experiment 8 (CE8), JCTVC-C508 [4]

# Algorithm description

The concept of CC is to send the range (minimum and maximum) of original pixels to decoders. If a pixel is out of the range after reconstruction, it will be clipped to the minimum or the maximum. The minimum and maximum can be predicted before transmission. In this work, in-loop CC and post-loop CC are both discussed.

## In-loop controlled clipping

In-loop CC can be applied at four positions, as shown in Figure 2. The first position is post-prediction. The prediction reference buffers of both intra prediction and inter prediction are used to store the predicted values and generate residues. After prediction, CC can be applied to reduce residues. The second position is post-reconstruction. Before reconstruction, residues are transformed, quantized, inverse quantized, and inversed transformed. These operations expand the dynamic range of residues, and they also change the dynamic range of the reconstructed pixel values. If the range is known in advance, the dynamic range of reconstructed pixel values can be restricted by CC. Then the distortion can be reduced while the reconstructed pixel value exceeds the restricted range. The third position and the fourth position are post-deblocking and post-ALF. Deblocking and ALF change reconstructed pixel values by filtering. CC can also restrict the pixel values in the correct range to reduce distortion.

Figure 2  Encoder block diagram for in-loop CC.

## Post-loop controlled clipping

Post-loop CC is only applied at the output of decoder decoded YUV file as shown in Figure 3. When ALF is enabled in high efficiency configurations, ALF results are stored in the reference picture buffer without the CC process and are inputted to CC for display. When ALF is disabled in low complexity configurations, deblocking results are stored in the reference picture buffer without the CC process and are inputted to CC for display.

Figure 3  Encoder block diagram for post-loop CC.

## Syntax description

The minimum and maximum values of CC can be defined in the picture parameter set (PPS) or in the slice header. When the PPS-level adaptation is used, the minimum and maximum values can be derived from PPS. When the slice-level adaptation is used, the minimum and maximum values conveyed in PPS are used for predicting those values of an intra slice, and the minimum and maximum values of the first list 0 reference picture are used for predicting those values of an inter slice. Details can be seen in Table 1.

Table 1  Modified syntax elements for CC.

| pic_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| controlled_clipping_flag | 1 | u(1) |
| if( controlled_clipping_flag ) |  |  |
| { |  |  |
| controlled_clipping_broadcast_legal_flag | 1 | u(1) |
| if( !controlled_clipping_broadcast_legal_flag ) |  |  |
| { |  |  |
| controlled_clipping_minY | 1 | u(v) |
| controlled_clipping_maxY | 1 | u(v) |
| controlled_clipping_minCr | 1 | u(v) |
| controlled_clipping_maxCr | 1 | u(v) |
| controlled_clipping_sameC_data_flag | 1 | u(1) |
| if( !controlled_clipping_sameC_data_flag ) |  |  |
| { |  |  |
| controlled_clipping_minCb | 1 | u(v) |
| controlled_clipping_maxCb | 1 | u(v) |
| } |  |  |
| } |  |  |
| controlled_clipping_slice_controlY_flag | 1 | u(1) |
| controlled_clipping_slice_controlC_flag | 1 | u(1) |
| } |  |  |
| … |  |  |
| } |  |  |

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| controlled_clipping_use_slice_delta_flag | 1 | u(1) |
| if( controlled_clipping_use_slice_delta_flag ) { |  |  |
| if( controlled_clipping_slice_controlY_flag ) { |  |  |
| controlled_clipping_predicted_minY_delta | 2 | se(v) |
| controlled_clipping_predicted_maxY_delta | 2 | se(v) |
| } |  |  |
| if( controlled_clipping_slice_controlC_flag ) { |  |  |
| controlled_clipping_predicted_minCr_delta | 2 | se(v) |
| controlled_clipping_predicted_maxCr_delta | 2 | se(v) |
| controlled_clipping_predicted_minCb_delta | 2 | se(v) |
| controlled_clipping_predicted_maxCb_delta | 2 | se(v) |
| } |  |  |
| } |  |  |
| … |  |  |
| } |  |  |

controlled_clipping_flag equal to 1 denotes that CC is enabled; equal to 0 denotes that CC is disabled.

controlled_clipping_broadcast_legal_flag equal to 1 denotes that predefined minimum and maximum pixel values are used for CC; equal to 0 denotes that transmitted minimum and maximum pixel values are used for CC.

controlled_clipping_minY defines the minimum pixel value for Y channel. When controlled_clipping_broadcast_legal_flag is 1, controlled_clipping_minY shall be (16 << bit_depth_luma_minus8). When controlled_clipping_flag is 0, controlled_clipping_minY shall be 0.

controlled_clipping_maxY defines the maximum pixel value for Y channel. When controlled_clipping_broadcast_legal_flag is 1, controlled_clipping_maxY shall be (235 << bit_depth_luma_minus8). When controlled_clipping_flag is 0, controlled_clipping_maxY shall be (255 << bit_depth_luma_minus8).

controlled_clipping_minCr defines the minimum pixel value for Cr channel. When controlled_clipping_broadcast_legal_flag is 1, controlled_clipping_minCr shall be (16 << bit_depth_chroma_minus8). When controlled_clipping_flag is 0, controlled_clipping_minCr shall be 0.

controlled_clipping_maxCr defines the maximum pixel value for Cr channel. When controlled_clipping_broadcast_legal_flag is 1, controlled_clipping_maxCr shall be (240 << bit_depth_chroma_minus8). When controlled_clipping_flag is 0, controlled_clipping_maxCr shall be (255 << bit_depth_chroma_minus8).

controlled_clipping_sameC_data_flag equal to 1 denotes that CC parameters in PPS are the same for both chroma channels.

controlled_clipping_minCb defines the minimum pixel value for Cb channel. When controlled_clipping_broadcast_legal_flag is 1, controlled_clipping_minCb shall be (16 << bit_depth_chroma_minus8). When controlled_clipping_sameC_data_flag is 1, controlled_clipping_minCb shall be controlled_clipping_minCr. When controlled_clipping_flag is 0, controlled_clipping_minCb shall be 0.

controlled_clipping_maxCb defines the maximum pixel value for Cb channel. When controlled_clipping_broadcast_legal_flag is 1, controlled_clipping_maxCb shall be (240 << bit_depth_chroma_minus8). When controlled_clipping_sameC_data_flag is 1, controlled_clipping_maxCb shall be controlled_clipping_maxCr. When controlled_clipping_flag is 0, controlled_clipping_maxCb shall be (255 << bit_depth_chroma_minus8).

controlled_clipping_slice_controlY_flag equal to 1 denotes that slice-level CC is enabled for luma; equal to 0 denotes that PPS-level CC is enabled for luma.

controlled_clipping_slice_controlC_flag equal to 1 denotes that slice-level CC is enabled for chroma; equal to 0 denotes that PPS-level CC is enabled for chroma.

controlled_clipping_use_slice_delta_flag is used to denote the use of slice delta in the current slice. If luma or chroma slice delta is used, controlled_clipping_use_slice_delta_flag is 1. Otherwise, controlled_clipping_use_slice_delta_flag is 0.

controlled_clipping_predicted_minY_delta is used to derive controlled_clipping_minY_slice as follows.

controlled_clipping_minY_slice = controlled_clipping_minY_predictor + controlled_clipping_predicted_minY_delta

controlled_clipping_minY_slice denotes the minimum luma value for the current slice. In the current slice, luma pixel values smaller than controlled_clipping_minY_slice shall be replaced by controlled_clipping_minY_slice. If controlled_clipping_slice_controlY_flag is 0, controlled_clipping_predicted_minY_delta will also be 0. Then controlled_clipping_minY_slice is equal to controlled_clipping_minY_predictor. If the current slice is intra, controlled_clipping_minY_predictor is assigned as controlled_clipping_minY. Otherwise, controlled_clipping_minY_predictor will be the minimum luma value in the first list 0 reference picture.

controlled_clipping_predicted_maxY_delta is used to derive controlled_clipping_maxY_slice as follows.

controlled_clipping_maxY_slice = controlled_clipping_maxY_predictor + controlled_clipping_predicted_maxY_delta

controlled_clipping_maxY_slice denotes the maximum luma value for the current slice. In the current slice, luma pixel values larger than controlled_clipping_maxY_slice shall be replaced by controlled_clipping_maxY_slice. If controlled_clipping_slice_controlY_flag is 0, controlled_clipping_predicted_maxY_delta will also be 0. Then controlled_clipping_maxY_slice is equal to controlled_clipping_maxY_predictor. If the current slice is intra, controlled_clipping_maxY_predictor is assigned as controlled_clipping_maxY. Otherwise, controlled_clipping_maxY_predictor will be the maximum luma value in the first list 0 reference picture.

controlled_clipping_predicted_minCr_delta is used to derive controlled_clipping_minCr_slice as follows.

controlled_clipping_minCr_slice = controlled_clipping_minCr_predictor + controlled_clipping_predicted_minCr_delta

controlled_clipping_minCr_slice denotes the minimum Cr value for the current slice. In the current slice, channel Cr pixel values smaller than controlled_clipping_minCr_slice shall be replaced by controlled_clipping_minCr_slice. If controlled_clipping_slice_controlC_flag is 0, controlled_clipping_predicted_minCr_delta will also be 0. Then controlled_clipping_minCr_slice is equal to controlled_clipping_minCr_predictor. If the current slice is intra, controlled_clipping_minCr_predictor is assigned as controlled_clipping_minCr. Otherwise, controlled_clipping_minCr_predictor will be the minimum Cr value in the first list 0 reference picture.

controlled_clipping_predicted_maxCr_delta is used to derive controlled_clipping_maxCr_slice as follows.

controlled_clipping_maxCr_slice = controlled_clipping_maxCr_predictor + controlled_clipping_predicted_maxCr_delta

controlled_clipping_maxCr_slice denotes the maximum Cr value for the current slice. In the current slice, channel Cr pixel values larger than controlled_clipping_maxCr_slice shall be replaced by controlled_clipping_maxCr_slice. If controlled_clipping_slice_controlC_flag is 0, controlled_clipping_predicted_maxCr_delta will also be 0. Then controlled_clipping_maxCr_slice is equal to controlled_clipping_maxCr_predictor. If the current slice is intra, controlled_clipping_maxCr_predictor is assigned as controlled_clipping_maxCr. Otherwise, controlled_clipping_maxCr_predictor will be the maximum Cr value in the first list 0 reference picture.

controlled_clipping_predicted_minCb_delta is used to derive controlled_clipping_minCb_slice as follows.

controlled_clipping_minCb_slice = controlled_clipping_minCb_predictor + controlled_clipping_predicted_minCb_delta

controlled_clipping_minCb_slice denotes the minimum Cb value for the current slice. In the current slice, channel Cb pixel values smaller than controlled_clipping_minCb_slice shall be replaced by controlled_clipping_minCb_slice. If controlled_clipping_slice_controlC_flag is 0, controlled_clipping_predicted_minCb_delta will also be 0. Then controlled_clipping_minCb_slice is equal to controlled_clipping_minCb_predictor. If the current slice is intra, controlled_clipping_minCb_predictor is assigned as controlled_clipping_minCb. Otherwise, controlled_clipping_minCb_predictor will be the minimum Cb value in the first list 0 reference picture.

controlled_clipping_predicted_maxCb_delta is used to derive controlled_clipping_maxCb_slice as follows.

controlled_clipping_maxCb_slice = controlled_clipping_maxCb_predictor + controlled_clipping_predicted_maxCb_delta

controlled_clipping_maxCb_slice denotes the maximum Cb value for the current slice. In the current slice, channel Cb pixel values larger than controlled_clipping_maxCb_slice shall be replaced by controlled_clipping_maxCb_slice. If controlled_clipping_slice_controlC_flag is 0, controlled_clipping_predicted_maxCb_delta will also be 0. Then controlled_clipping_maxCb_slice is equal to controlled_clipping_maxCb_predictor. If the current slice is intra, controlled_clipping_maxCb_predictor is assigned as controlled_clipping_maxCb. Otherwise, controlled_clipping_maxCb_predictor will be the maximum Cb value in the first list 0 reference picture.

# Simulation results

In JCTVC-C500 [5], six configurations are defined as high efficiency all intra (HE-AI), low complexity all intra (LC-AI), high efficiency random access (HE-RA), low complexity random access (LC-RA), high efficiency low delay (HE-LD), and low complexity low delay (LC-LD). We conducted experiments for all configurations. In our tests, controlled_clipping_flag, controlled_clipping_broadcast_legal_flag, and controlled_clipping_slice_controlY_flag are enabled, and controlled_clipping_slice_controlC_flag is disabled.

Table 2, Table 3, and Table 4 show results for post-loop CC in details for AI, RA, and LD configurations, respectively. It can be seen that for HE-AI, HE-RA, HE-LD, LC-AI, LC-RA, and LC-LD, the BD-rates of the normative post-loop CC are -0.3%, -0.6%, -0.4%, -0.2%, -0.3%, and -0.2%, respectively, the average encoding times are 101%, 100%, 100%, 100%, 100%, and 100%, respectively, and the average decoding times are 98%, 99%, 98%, 101%, 101%, and 100%, respectively. The post-loop CC achieves small BD-rate improvements, but almost no complexity increase is observed.

Table 2  Results of post-loop CC for HE-AI and LC-AI.

| CC versus JCTVC-C500 |  | High Efficiency All Intra |  |  | Low Complexity All Intra |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % |
| Class A 2560x1600 | Traffic | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
|  | PeopleOnStreet | -1.8 | 0.0 | 0.0 | -1.4 | 0.0 | 0.0 |
| Class B 1080p | Kimono | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | ParkScene | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | Cactus | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | BasketballDrive | -0.2 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
|  | BQTerrace | -0.6 | 0.0 | 0.0 | -0.5 | 0.0 | 0.0 |
| Class C WVGA | BasketballDrill | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
|  | BQMall | -0.2 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
|  | PartyScene | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
|  | RaceHorses | -0.2 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
| Class D WQVGA | BasketballPass | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
|  | BQSquare | -0.9 | 0.0 | 0.0 | -0.9 | 0.0 | 0.0 |
|  | BlowingBubbles | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
|  | RaceHorses | -0.3 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
| Class E 720p | Vidyo1 | -0.2 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
|  | Vidyo3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | Vidyo4 | -0.3 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
| Summary | Class A | -1.0 | 0.0 | 0.0 | -0.8 | 0.0 | 0.0 |
|  | Class B | -0.2 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
|  | Class C | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
|  | Class D | -0.3 | 0.0 | 0.0 | -0.3 | 0.0 | 0.0 |
|  | Class E | -0.2 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
|  | All | -0.3 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
|  | Encoding Time % | 101 |  |  | 100 |  |  |
|  | Decoding Time % | 98 |  |  | 101 |  |  |

Table 3  Results of post-loop CC for HE-RA and LC-RA.

| CC versus JCTVC-C500 |  | High Efficiency Random Access |  |  | Low Complexity Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % |
| Class A 2560x1600 | Traffic | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
|  | PeopleOnStreet | -1.7 | 0.0 | 0.0 | -1.0 | 0.0 | 0.0 |
| Class B 1080p | Kimono | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | ParkScene | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | Cactus | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | BasketballDrive | -0.4 | 0.0 | 0.0 | -0.3 | 0.0 | 0.0 |
|  | BQTerrace | -1.5 | 0.0 | 0.0 | -0.8 | 0.0 | 0.0 |
| Class C WVGA | BasketballDrill | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | BQMall | -0.2 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
|  | PartyScene | -0.3 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
|  | RaceHorses | -0.2 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
| Class D WQVGA | BasketballPass | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | BQSquare | -3.9 | 0.0 | 0.0 | -1.3 | 0.0 | 0.0 |
|  | BlowingBubbles | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
|  | RaceHorses | -0.2 | 0.1 | 0.1 | -0.1 | 0.1 | 0.1 |
| Class E 720p | Vidyo1 |  |  |  |  |  |  |
|  | Vidyo3 |  |  |  |  |  |  |
|  | Vidyo4 |  |  |  |  |  |  |
| Summary | Class A | -0.9 | 0.0 | 0.0 | -0.5 | 0.0 | 0.0 |
|  | Class B | -0.4 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
|  | Class C | -0.2 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
|  | Class D | -1.1 | 0.0 | 0.0 | -0.4 | 0.0 | 0.0 |
|  | Class E |  |  |  |  |  |  |
|  | All | -0.6 | 0.0 | 0.0 | -0.3 | 0.0 | 0.0 |
|  | Encoding Time % | 100 |  |  | 100 |  |  |
|  | Decoding Time % | 99 |  |  | 101 |  |  |

Table 4  Results of post-loop CC for HE-LD and LC-LD.

| CC versus JCTVC-C500 |  | High Efficiency Low Delay |  |  | Low Complexity Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % |
| Class A 2560x1600 | Traffic |  |  |  |  |  |  |
|  | PeopleOnStreet |  |  |  |  |  |  |
| Class B 1080p | Kimono | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | ParkScene | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | Cactus | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | BasketballDrive | -0.3 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
|  | BQTerrace | -1.1 | 0.0 | 0.0 | -0.6 | 0.0 | 0.0 |
| Class C WVGA | BasketballDrill | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | BQMall | -0.3 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
|  | PartyScene | -0.2 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
|  | RaceHorses | -0.3 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
| Class D WQVGA | BasketballPass | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
|  | BQSquare | -2.9 | 0.0 | 0.0 | -1.2 | 0.0 | 0.0 |
|  | BlowingBubbles | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | RaceHorses | -0.4 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
| Class E 720p | Vidyo1 | -0.4 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
|  | Vidyo3 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | Vidyo4 | -0.5 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
| Summary | Class A |  |  |  |  |  |  |
|  | Class B | -0.3 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
|  | Class C | -0.2 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
|  | Class D | -0.9 | 0.0 | 0.0 | -0.4 | 0.0 | 0.0 |
|  | Class E | -0.3 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
|  | All | -0.4 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
|  | Encoding Time % | 100 |  |  | 100 |  |  |
|  | Decoding Time % | 98 |  |  | 100 |  |  |

In the post-loop CC mentioned above, the side information of each slice is in the slice header, which is also called normative post-loop CC in the following experiments. The post-loop CC can also be performed in a non-normative way. The non-normative post-loop CC transmits CC information in another NAL packet. Table 5 shows the average luma BD-rate comparison for in-loop 3-position CC (post-reconstruction, post-deblocking, and post-ALF.), in-loop 4-postion CC (post-prediction, post-reconstruction, post-deblocking, and post-ALF), normative post-loop CC, and non-normative post-loop CC.

Table 5  Average luma BD-rates comparison for in-loop 3-position CC, 4-position CC, normative post-loop CC, and non-normative post-loop CC.

| CC versus JCTVC-C500 | HE-AI | HE-RA | HE-LD | LC-AI | LC-RA | LC-LD |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-Rate Y % | BD-Rate Y % | BD-Rate Y % | BD-Rate Y % | BD-Rate Y % | BD-Rate Y % |
| In-loop 3-position CC | -0.4 | -0.6 | -0.4 | -0.3 | -0.2 | -0.2 |
| In-loop 4-position CC | -0.4 | -0.6 | -0.4 | -0.3 | -0.2 | -0.2 |
| Normative post-loop CC | -0.3 | -0.6 | -0.4 | -0.2 | -0.3 | -0.2 |
| Non-normative post-loop CC | -0.3 | -0.4 | -0.2 | -0.2 | -0.1 | 0.0 |

Table 6 and Table 7 show the encoding and decoding time comparisons for the experiments in Table 5. Although the multi-position in-loop CC shows a little bit higher coding gain than the post-loop CC, the complexity increase is up to 14% and cannot justify the little increase of compression performance.

Table 6  Comparison of encoding times for in-loop 3-position CC, in-loop 4-position CC, normative post-loop CC, and non-normative post-loop CC.

| CC versus JCTVC-C500 | HE-AI | HE-RA | HE-LD | LC-AI | LC-RA | LC-LD |
| --- | --- | --- | --- | --- | --- | --- |
|  | Enc Time[%] | Enc Time[%] | Enc Time[%] | Enc Time[%] | Enc Time[%] | Enc Time[%] |
| In-loop 3-position CC | 101 | 100 | 100 | 101 | 100 | 100 |
| In-loop 4-position CC | 105 | 101 | 102 | 114 | 103 | 104 |
| Normative post-loop CC | 101 | 100 | 100 | 100 | 100 | 100 |
| Non-normative post-loop CC | 101 | 100 | 100 | 100 | 100 | 100 |

Table 7  Comparison of decoding times for in-loop 3-position CC, in-loop 4-position CC, normative post-loop CC, and non-normative post-loop CC.

| CC versus JCTVC-C500 | HE-AI | HE-RA | HE-LD | LC-AI | LC-RA | LC-LD |
| --- | --- | --- | --- | --- | --- | --- |
|  | Dec Time[%] | Dec Time[%] | Dec Time[%] | Dec Time[%] | Dec Time[%] | Dec Time[%] |
| In-loop 3-position CC | 102 | 104 | 103 | 102 | 102 | 105 |
| In-loop 4-position CC | 103 | 107 | 111 | 106 | 112 | 111 |
| Normative post-loop CC | 98 | 99 | 98 | 101 | 101 | 100 |
| Non-normative post-loop CC | 98 | 99 | 98 | 101 | 101 | 100 |

We found that the BD-rates of non-normative post-loop CC are worse than those of normative post-loop CC due to the NAL packet header overhead. The loss of coding efficiency is about 0.2% in terms of BD-rate. The bitrate reductions are reduced from 0.2%-0.6% for the normative post-loop CC to 0.0%-0.4% for the non-normative post-loop CC. Both The complexity of non-normative post-loop CC and that of normative post-loop CC are essentially the same as that of anchor.

It is confirmed that the normative post-loop CC with side information in the slice header can provide the best trade-off between coding efficiency and complexity. Moreover, a decoder can choose to perform or bypass CC before display, which is a more flexible design.

# Conclusions

The coding tools in the current TMuC may result in dynamic range expansion of reconstructed pictures. This dynamic range excursion can be easily remedied by simple controlled clipping (CC). After testing many possible in-loop and post-loop CC options, we found that the post-loop CC (with normative CC syntax in slice header) achieves the best trade-off between coding gain and complexity. The bitrate reductions are 0.6% and 0.4% for HE-RA and HE-LD, respectively. Almost no encoding or decoding time increase is observed.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Andrew Segall and Yeping Su, “Codeword Restriction for Improved Coding Efficiency,” Document of Joint Collaborative Team on Video Coding, JCTVC-B113, Jul. 2010.

- Yu-Wen Huang, Chih-Ming Fu, Ching-Yeh Chen, Chia-Yang Tsai, Yongying Gao, Jicheng An, Kai Zhang, and Shawmin Lei, “In-Loop Adaptive Restoration,” Document of Joint Collaborative Team on Video Coding, JCTVC-B077, Jul. 2010.

- Yu-Lin Chang, Chih-Ming Fu, Andrew Segall, Yeping Su, Yu-Wen Huang, and Shawmin Lei, “TE10 Subtest3: Controlled Clipping,” Document of Joint Collaborative Team on Video Coding, JCTVC-C146, Oct. 2010.

- Tomoo Yamakage et al, “Description of CE8: In-loop filtering,” Document of Joint Collaborative Team on Video Coding, JCTVC-C508, Oct. 2010.

- Frank Bossen, “Common Test Conditions and Software Reference Configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-C500, Oct. 2010.