| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D125 |
| --- | --- |

| Title: | Improved Advanced Motion Vector Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin Yu-Pao Tsai Yu-Wen Huang Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Emails: | jl.lin@mediatek.com yupao.tsai@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution describes MediaTek’s proposal on improving the advanced motion vector prediction (AMVP) in the test model under consideration (TMuC). Four improvements of AMVP are developed. First, in general cases, six spatial motion vector prediction (MVP) candidates and four temporal MVP candidates are used in the MVP candidate set for motion vector competition (MVC); in low complexity configurations, when the picture order count (POC) difference between the current picture and the co-located picture is only 1 and the co-located MV exists, the MVP candidates are reduced to one spatial MVP candidate and four temporal MVP candidates. Second, the order of the co-located temporal MVP is automatically adjusted according to the POC difference. Next, when each spatial MVP candidate is derived, a predefined priority is used to select one motion vector (MV) from list 0 and list 1 MVs of the spatial neighboring block, and the chosen MV is scaled to the same reference picture of the current prediction unit (PU) as the spatial MVP candidate for MVC. Last, when each temporal MVP candidate is derived, a predefined priority is used to select one MV from list 0 and list 1 MVs of two co-located blocks, and the chosen MV is scaled to the same reference picture of the current PU as the temporal MVP candidate for MVC. Our improved AMVP, namely IAMVP, can achieve noticeable coding efficiency gain with roughly the same encoding or decoding complexity. When compared with the anchor in JCTVC-C500, IAMVP can achieve 2.1%, 2.0%, 1.5%, and 1.0% bitrate reductions for high efficiency random access (HE-RA), low complexity random access (LC-RA), high efficiency low delay (HE-LD), and low complexity low delay (LC-LD), respectively. For HE-RA, LC-RA, HE-LD, and LC-LD, the average encoding times are 98%, 99%, 95%, and 97%, and the average decoding times are 100%, 104%, 101%, and 104%, respectively.

- Introduction

In JCTVC-A124 [1], AMVP is proposed to encode the motion vector predictor by utilizing a competition-based scheme from a given candidate set of motion vector predictors. The candidate set is composed of three spatial MVP candidates, median of the three spatial MVP candidates, and one temporal MVP candidate. The temporal MVP candidate is derived from the co-located block of the first list 0 or list 1 reference picture with a scaling factor based on relative temporal distances where the selection between list 0 and list 1 is fixed for the entire slice and indicated in the slice header. The candidate set of motion vector predictors is reduced by removing duplicated MVP candidates and unnecessary MVP candidates according to the transmitted motion vector difference (MVD). Since the order of MVP candidates is important, JCTVC-B081 [2] and JCTVC-C291 [3] proposed to change the order of MVP candidates in the candidate set and to signal the order at slice level to adapt the behavior to sequence characteristics.

In this proposal, we present a set of surrounding-based MVP candidates to improve the AMVP. Priority-based schemes are also proposed for deriving spatial and temporal MVP candidates. The order of temporal MVP candidates is adaptive according to temporal distances. The rest of this input document is organized as follows. The improved AMVP, namely IAMVP, is described in Section 2, and simulation results are shown in Section 3. Finally, Section 4 gives conclusions.

# Algorithm description

In this proposal, a set of surrounding-based MVP candidates is proposed to increase the coding efficiency of AMVP by introducing three temporal MVP candidates from blocks on the below and right sides and three spatial MVP candidates from neighboring corners of the current PU. For low complexity configurations, when the picture order count (POC) difference between the current picture and the co-located picture is only 1, only one spatial MVP candidate and four temporal MVP candidates are included in the candidate set of motion vector predictors. A priority-based scheme is proposed to derive each spatial MVP candidate by selecting one MV from MVs of the two reference lists in a spatially neighboring block with a scaling factor according to relative temporal distances. Another priority-based scheme is also applied to derive each temporal MVP candidate by selecting one MV from list 0 and list 1 MVs of two co-located blocks with a scaling factor according to their relative temporal distances.

The candidate set of motion vector predictors is reduced by removing duplicated MVP candidates. Based on the transmitted MVD, unnecessary MVP candidates can be further removed according to the rule in AMVP. The order of the co-located temporal MVP candidate is adapted based on the distance between the current picture and the reference picture. Details of each part are described in the following subsections.

## Improvement of the MVP candidate set

In the AMVP of TMuC, three spatial MVP candidates, a median MVP candidate derived from the three spatial MVP candidates, and one temporal MVP candidate are included in the candidate set. The five MVP candidates are denoted as

- M (median of A’, B’, and C’)

- A’ (the first available MV from the blocks on the left side)

- B’ (the first available MV from the blocks on the above side)

- C’ (the first available MV from the three neighboring corner blocks, C, E, and D)

- T (the MV from the co-located block)

The four spatial MVP candidates are all chosen from the neighboring blocks on the above and left sides. Even the temporal MVP candidate is chosen from the above left corner relative to the current PU. In order to introduce MVP candidates from the below and right sides, three temporal MVP candidates from the below, right, and below right neighboring corner blocks (G, F, and H) of a co-located picture are added in the candidate set as shown in Figure 1.

Figure 1  The candidate set of the IAMVP.

Moreover, the three MVP candidates from neighboring corner blocks (C, E, and D) are included in the candidate set of motion vector predictors. The final surrounding-based candidate set of motion vector predictors and the default order are shown as the following:

- M (median of A’, B’, and C’ where C’ denotes the first available MV from C, E, and D)

- A’ (the first available MV from the blocks on the left side)

- B’ (the first available MV from the blocks on the above side)

- C (the MV from the above right neighboring corner)

- E (the MV from the below left neighboring corner)

- D (the MV from the above left neighboring corner)

- T (the MV from the co-located block)

- F (the MV from the right neighboring block in a co-located picture)

- G (the MV from the below neighboring block in a co-located picture)

- H (the MV from the below right neighboring block in a co-located picture)

The median can be swapped with either A’, B’, or C’ according to the CU partition size, which is the same as the current TMuC. For low complexity configurations, when the POC difference between the current picture and the co-located picture is only 1 and the co-located MV exists, only the first spatial MVP candidate is included in the candidate set of motion vector predictors. The candidate set of motion vector predictors and the default order for low complexity configurations are shown as the following:

- M (median of A’, B’, and C’ where C’ denotes the first available MV from C, E, and D)

- T (the MV from the co-located block)

- F (the MV from the right neighboring block in a co-located picture)

- G (the MV from the below neighboring block in a co-located picture)

- H (the MV from the below right neighboring block in a co-located picture )

The median can be replaced by either A’, B’, or C’ according to the CU partition size, which is the same as the current TMuC.

## Improvement of MVP candidate order

The order in the candidate set shown is the previous subsection is the default order. When the reference picture is close to the current picture, the temporal MVP candidates may become more correlated to the MV of current PU than the spatial MVP candidates. Therefore, in this proposal, when the picture order count (POC) difference between the current picture and the reference picture is one, the temporal MVP candidate from the co-located block (T) is set as the first one in the order. The remaining MVP candidates follow the relative positions in the default order.

## Improvement of spatial MVP candidates

A priority-based scheme is applied for deriving each spatial MVP candidate. In the AMVP of TMuC, only the MV with the same reference list and the same reference picture index is considered as an available spatial MVP candidate. In the proposed priority-based scheme, a spatial MVP candidate can be derived from different list and different reference picture. Figure 2 shows an example of a B-slice with single reference picture per list (picture j for list 0 and picture l for list 1). The current list is list 0, and the current reference picture is picture j. The dotted blue arrow denotes the list 0 MV of the neighboring block, and the dotted red arrow denotes the list 1 MV of the neighboring block. The numbers denote the priorities of the two MVs. When the list 0 MV is available, it is used as the spatial MVP candidate. Otherwise, when the list 1 MV is available, it is scaled to the current reference picture as the solid red arrow and then used as the spatial MVP candidate.

Figure 2  Example of deriving a spatial MVP candidate for a B-slice with single reference picture per list.

Figure 3 shows another example of a B-slice with two reference pictures per list (pictures j and i for list 0, pictures m and j for list 1). The current list is list 0, and the current reference picture is j. The dotted purple arrow, the dotted red arrow, the dotted green arrow, and the dotted blue arrow denote the possible list 0 MV for picture j, the possible list 0 MV for picture i, the possible list 1 MV for picture m, and the possible list 1 MV for picture j, respectively. The solid arrows denote the scaled MVs of the four possible MVs to the current reference picture, j. The scaling factors are based on relative temporal distances. The numbers denote the priorities of scaled MVs for deriving the spatial MVP candidate. The general rule is stated as follows. There is one (list 0) or two MVs (list 0 and list 1) in a neighboring block. The MV of the neighboring block with the reference picture same as the current reference picture is used as the spatial MVP candidate. When the two MVs of the neighboring block both have the reference picture same as the current reference picture, the one with the list same as the current list is used as the spatial MVP candidate. When the two MVs of the neighboring block both have reference pictures different from the current reference picture, the one with the list same as the current list is scaled and then used as the spatial MVP candidate.

Figure 3  Example of deriving a spatial MVP candidate for a B-slice with multiple reference pictures per list.

## Improvement of temporal MVP candidates

A priority-based scheme is also used to derive temporal MVP candidates. Since a B-slice has two co-located blocks (one in the first list 0 reference picture and the other in the first list 1 reference picture) and each co-located block has up to two lists, each temporal MVP candidate can be derived from four possible MVs as shown in Figure 4. The left side of Figure 4 shows the case when the current list is list 0 and the current reference picture is picture j, while the right side of Figure 4 shows the case when the current list is list 1 and the current reference picture is picture l. In these two examples, the current picture is picture k, the list 0 co-located picture is picture j, the list 1 co-located picture is picture l, the dotted purple arrow denotes the list 0 MV of the list 0 co-located block, the dotted green arrow denotes the list 1 MV of the list 0 co-located block, the dotted red arrow denotes the list 0 MV of the list 1 co-located block, the dotted blue arrow denotes the list 1 MV of the list 1 co-located block, and solid arrows denote scaled MVs of the four co-located MVs to the same reference picture as the current reference picture. The numbers denote the priorities of the four scaled MVs, and the first available one will be used as the temporal MVP candidate. Please note that not only the co-located MVP candidate but also the other three temporal MVP candidates apply this rule for derivation.

Figure 4  Examples of deriving temporal MVP candidates.

# Simulation results

HE-RA, LC-RA, HE-LD, and LC-LD are tested according to JCTVC-C500 [4]. The results are shown in Table 1 and Table 2.

Table 1  Experimental data for HE-RA and LC-RA.

| IAMVP versus JCTVC-C500 |  | High Efficiency Random Access |  |  | Low Complexity Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % |
| Class A 2560x1600 | Traffic | -2.9 | -2.3 | -2.2 | -2.3 | -1.9 | -2.0 |
|  | PeopleOnStreet | -3.2 | -3.6 | -3.2 | -3.5 | -3.7 | -3.7 |
| Class B 1080p | Kimono | -1.5 | -1.7 | -1.6 | -1.6 | -1.7 | -1.5 |
|  | ParkScene | -1.5 | -1.5 | -1.4 | -1.4 | -1.1 | -1.1 |
|  | Cactus | -2.0 | -1.7 | -2.1 | -2.0 | -1.5 | -1.5 |
|  | BasketballDrive | -1.9 | -1.9 | -2.1 | -1.9 | -1.5 | -1.8 |
|  | BQTerrace | -2.0 | -1.1 | 0.0 | -1.8 | -1.7 | -1.8 |
| Class C WVGA | BasketballDrill | -2.1 | -2.7 | -2.7 | -2.0 | -2.5 | -2.8 |
|  | BQMall | -2.6 | -2.4 | -2.5 | -2.6 | -1.7 | -1.8 |
|  | PartyScene | -2.2 | -1.6 | -1.8 | -1.8 | -1.4 | -1.5 |
|  | RaceHorses | -1.3 | -2.1 | -2.2 | -1.3 | -1.8 | -1.9 |
| Class D WQVGA | BasketballPass | -1.8 | -1.7 | -2.3 | -2.1 | -2.2 | -2.2 |
|  | BQSquare | -3.0 | -1.4 | -1.4 | -1.3 | -1.2 | -1.1 |
|  | BlowingBubbles | -1.8 | -1.4 | -1.2 | -1.6 | -1.1 | -0.9 |
|  | RaceHorses | -1.9 | -2.3 | -2.4 | -2.1 | -2.5 | -2.4 |
| Class E 720p | Vidyo1 |  |  |  |  |  |  |
|  | Vidyo3 |  |  |  |  |  |  |
|  | Vidyo4 |  |  |  |  |  |  |
| Summary | Class A | -3.1 | -2.9 | -2.7 | -2.9 | -2.8 | -2.8 |
|  | Class B | -1.8 | -1.6 | -1.4 | -1.7 | -1.5 | -1.5 |
|  | Class C | -2.0 | -2.2 | -2.3 | -1.9 | -1.8 | -2.0 |
|  | Class D | -2.1 | -1.7 | -1.8 | -1.8 | -1.7 | -1.7 |
|  | Class E |  |  |  |  |  |  |
|  | All | -2.1 | -2.0 | -1.9 | -2.0 | -1.8 | -1.9 |
|  | Encoding Time % | 98 |  |  | 99 |  |  |
|  | Decoding Time % | 100 |  |  | 104 |  |  |

Table 2  Experimental data for HE-LD and LC-LD.

| IAMVP versus JCTVC-C500 |  | High Efficiency Low Delay |  |  | Low Complexity Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % |
| Class A 2560x1600 | Traffic |  |  |  |  |  |  |
|  | PeopleOnStreet |  |  |  |  |  |  |
| Class B 1080p | Kimono | -0.9 | -1.0 | -1.0 | -0.4 | -0.6 | -0.5 |
|  | ParkScene | -1.3 | -0.8 | -0.7 | -0.7 | -0.7 | -0.4 |
|  | Cactus | -2.2 | -1.1 | -1.2 | -1.9 | -1.6 | -1.4 |
|  | BasketballDrive | -1.5 | -1.3 | -1.2 | -0.5 | -0.8 | -0.5 |
|  | BQTerrace | -1.4 | -0.7 | 0.0 | -1.2 | -1.0 | -1.3 |
| Class C WVGA | BasketballDrill | -1.7 | -2.6 | -2.2 | -1.3 | -1.8 | -1.4 |
|  | BQMall | -2.2 | -1.9 | -1.7 | -1.8 | -1.7 | -1.7 |
|  | PartyScene | -1.3 | -0.9 | -0.9 | -1.3 | -0.7 | -0.8 |
|  | RaceHorses | -1.1 | -1.0 | -1.0 | -0.1 | -0.4 | -0.4 |
| Class D WQVGA | BasketballPass | -1.2 | -1.2 | -1.4 | -1.0 | -1.2 | -1.1 |
|  | BQSquare | -1.0 | -3.4 | 0.4 | -1.8 | -1.1 | -0.5 |
|  | BlowingBubbles | -1.7 | -1.8 | -1.4 | -1.9 | -1.0 | -1.2 |
|  | RaceHorses | -1.3 | -1.3 | -0.5 | -1.0 | -1.0 | -0.9 |
| Class E 720p | Vidyo1 | -2.8 | -0.8 | -2.2 | -1.5 | -1.3 | -0.7 |
|  | Vidyo3 | -1.0 | 2.7 | 0.1 | -0.3 | 0.4 | -0.8 |
|  | Vidyo4 | -1.1 | 1.9 | 0.7 | 0.1 | 0.2 | -0.1 |
| Summary | Class A |  |  |  |  |  |  |
|  | Class B | -1.5 | -1.0 | -0.8 | -0.9 | -1.0 | -0.8 |
|  | Class C | -1.6 | -1.6 | -1.5 | -1.1 | -1.2 | -1.1 |
|  | Class D | -1.3 | -1.9 | -0.7 | -1.4 | -1.1 | -0.9 |
|  | Class E | -1.6 | 1.3 | -0.4 | -0.6 | -0.3 | -0.5 |
|  | All | -1.5 | -1.0 | -0.9 | -1.0 | -0.9 | -0.8 |
|  | Encoding Time % | 95 |  |  | 97 |  |  |
|  | Decoding Time % | 101 |  |  | 104 |  |  |

For RA configurations, the BD-rate improvements for HE-RA and LC-RA are 2.1% and 2.0%, respectively, the encoding times are 98% and 99%, respectively, and the decoding times are 100% and 104%, respectively. For LD configurations, the BD-rate improvements for HE-LD and LC-LD are 1.5% and 1.0%, respectively, the encoding times are 95% and 97%, respectively, and the decoding times are 101%, and 104%, respectively.

# Conclusions

MediaTek developed an improved AMVP, namely IAMVP. In comparison with the AMVP adopted in the current HM, the bitrate reductions are about 2.0% and 1.3% for random access and low delay, respectively, with roughly the same encoding or decoding complexity. It is proposed to adopt the IAMVP into HM for a wider study by other experts to see if further improvements can be achieved.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Ken McCann, Woo-Jin Han, Il-Koo Kim, Jung-Hye Min, Elena Alshina, Alexander Alshin, Tammy Lee, Jianle Chen, Vadim Seregin, Sunil Lee, Yoon-Mi Hong, Min-Su Cheon, Nikolay Shlyakhov, “Samsung’s Response to the Call for Proposals on Video Compression Technology,” JCTVC-A124, April, 2010.

- Kazushi Sato and Shinagawa, “On Motion Vector Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-B081, Jul. 2010.

- Joel Jung, Gordon Clare, Stephane Pateux, “TE11: Report on Experiment 3.3.b: ‘Temporally Oriented’ Set of Predictors for MV-Competition,” Document of Joint Collaborative Team on Video Coding, JCTVC-C291, Oct. 2010.

- Frank Bossen, “Common Test Conditions and Software Reference Configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-C500, Oct. 2010.