| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D128 |
| --- | --- |

| Title: | Slice Boundary Processing and Picture Layer Raw Byte Sequence Payload |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Yang Tsai Chih-Wei Hsu Yu-Wen Huang Ching-Yeh Chen Chih-Ming Fu Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Emails: | chiayang.tsai@mediatek.com cw.hsu@mediatek.com yuwen.huang@mediatek.com chingyeh.chen@mediatek.com chihming.fu@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this proposal, three issues related to slices are presented. First, slice-independent deblocking filter (DF) and slice-independent adaptive loop filter (ALF) are proposed. In slice-independent DF and ALF, DF is not performed across slice boundaries, and a padding method is used for ALF to replace any originally required pixels out of the current slice. With the modified DF and ALF processes, each slice can be independently decoded without using any data from other slices. Second, a slice boundary filter (SBF), which is very similar to DF but only processes slice boundaries, is proposed to remove possible artifacts across slice boundaries that may be caused by slice-independent DF and ALF. Third, a picture layer raw byte sequence payload (RBSP) is proposed as an option for sharing common information among slice headers in a picture. Syntax elements that can be moved from the slice header to the picture layer RBSP when they are the same for all slices include slice type, slice quantization parameter, entropy coding mode, interpolation filter type, number of reference pictures, ALF coefficients, …etc. Simulation results show that the provided software, as proofs of the concepts, can successfully support all the new features without any mismatch between the encoder and the decoder.

- Introduction

According to the current high efficiency video coding (HEVC) working draft (WD), one picture can be divided into multiple slices. Each slice can be independently parsed and reconstructed. However, DF and ALF processes of a slice must wait for all prior slices in raster scan order due to the data dependency of DF and ALF between slices. This causes a serious bottleneck for parallel processing, and independent decoding of each slice cannot be really achieved. To achieve slice-independent DF and ALF, DF and ALF have to be performed not across slice boundaries.

Slice-independent DF and ALF may produce slice boundary artifacts. Therefore, a slice boundary filter (SBF) is proposed to reduce the boundary artifacts. The SBF can be in-loop or post-loop. Conceptually, the in-loop SBF can provide better coding efficiency, while the post-loop SBF can provide higher flexibility on the decoder side.

In the current HEVC WD, each slice header has an ALF parameter set, which means the slice-level ALF is adopted. This suffers coding efficiency loss in comparison with the picture-level ALF, due to the increased amount of ALF information, especially when the number of slices in a picture is large. In order to simultaneously support the picture-level ALF and multiple slices per picture with reduced side information and without dependency between slices, ALF coefficients are moved to the proposed picture layer RBSP, while ALF CU control information (i.e. on/off flags) is distributed in each slice header. The picture layer RBSP can also include other syntax elements that are redundant in different slices in a picture.

The rest of this contribution is organized as follows. In Section 2, the slice independent DF and ALF, slice boundary filter (SBF), and picture layer RBSP are proposed. The experimental results are shown in Section 3. Conclusions are given in Section 4.

# Proposed techniques

## Slice-independent DF

The proposed slice-independent DF does not perform deblocking process across slice boundaries. For block boundaries inside the slice, the deblocking process specified in test model under consideration version 0.9 (TMuC0.9) is performed. The slice-independent DF is beneficial for parallel processing of multiple slices in one picture.

## Slice-independent ALF

According to the HEVC WD, each slice header has an ALF parameter set. TMuC0.9 does not support multiple slices per picture, and ALF parameters are trained by using the entire picture. In order to support slices with the ALF syntax design in the HEVC WD, we modified TMuC0.9 as follows. Filter coefficients for each slice are derived from the slice region instead of the entire picture. The derived ALF parameter set is encoded in the slice header. In order to support slice-independent ALF as well, a slice boundary repetitive extension padding process is used. In the padding process, the vertical boundaries are first repetitively extended and then the horizontal boundaries. Note that this boundary padding process for ALF is conceptually the same as the boundary padding process for the unrestricted motion compensation in HM. In [1], slice boundaries are aligned with leaf CU boundaries instead of largest coding unit (LCU) boundaries. Therefore, the padding process for the first and last LCUs of a slice is leaf CU-based while the padding process for rest LCUs are LCU-based. The following pseudo codes explain the padding process of one slice.

N = number of LCUs in slice; for ( lcu = 0 ; lcu < N; lcu++ ) { if ( lcu ==0 ¦¦ lcu == N-1) { for all leaf CUs in the current LCU { Padding vertical edges of the current CU Padding horizontal edges of the current CU } } else { Padding vertical edges of the current LCU Padding horizontal edges of the current LCU } }

In TMuC0.9, the ALF CU on/off control information is in the slice header. The syntax element, num_of_alf_cu_control_flags, denotes “the number of ALF CU on/off flags” minus “the number of LCUs in the picture”. If the number of slices in one picture is large, sometimes the prediction may be not efficient. Therefore, we use a new flag, predict_num_of_alf_cu_control_flags, to indicate whether the number of ALF CU on/off flags in a slice shall be predicted or not.

## Slice boundary filter

In our slice implementation, DF and ALF processes of a slice do not involve any pixels from other slices. However, slice boundaries may suffer boundary artifacts. We propose an SBF to reduce the boundary artifacts. The concept is very similar to DF. The only difference between DF and SBF is that DF is applied inside each slice while SBF is applied only at slice boundaries. The processing order of SBF is vertical edges first and then horizontal edges. Besides, the SBF can be in-loop or post-loop. Figure 1 and Figure 2 show the encoder block diagram and the decoder block diagram, respectively. Please note that the results of slice-independent DF followed by SBF are not the same as those of the conventional DF due to the different processing order for slice boundaries. However, their results are expected to be very similar in terms of BD-rate and subjective quality. Using slice-independent DF and ALF followed by SBF still can parallelize multiple slices for most of the time, and only the SBF is not slice-independent.

Figure 1  Encoder block diagram with SBF.

Figure 2  Decoder block diagram with SBF.

## Picture layer RBSP

In the current HEVC WD, each slice header has an ALF parameter set, which implies that ALF coefficients can be trained separately for each slice. When the number of slices in a picture is large (i.e. slices become small), the ALF information may cost too many bits for each slice, and the coding efficiency will degrade. Hence, we introduce the concept of picture layer RBSP. In order to share one set of ALF coefficients for multiple slices in a picture, the ALF coefficients can be moved into a picture layer RBSP. As for the ALF CU on/off control information, it is slice-dependent and thus still kept in each slice header. Another purpose of the picture layer RBSP is to remove the redundancy among different slice headers of one picture. For syntax elements that have same values in multiple slice headers (e.g. slice type, slice quantization parameter, entropy coding mode, interpolation filter type, number of reference pictures, …etc), they can be moved to the picture layer RBSP with only one exception of the picture order count (POC) information. The POC information should appear in the picture layer RBSP and each slice header for proper decoding.

In our slice implementation, even if the picture layer RBSP is used, DF and ALF are still slice-independent operations. The ALF coefficients are derived from the picture layer RBSP, the ALF CU on/off flags are derived from the slice header, and the filtering processes are not cross slice boundaries. Therefore, the picture layer RBSP still provides the flexibility for parallel decoding.

## Syntax description

Our syntax design is shown in Table 1. Modifications are highlighted with yellow backgrounds.

Table 1  Syntax design of the proposed slice boundary processing and picture layer RBSP.

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| slice_boundary_filter_flag | 1 | u(1) |
| if( slice_boundary_filter_flag ) |  |  |
| slice_boundary_filter_in_loop_flag | 1 | u(1) |
| use_pic_layer_rbsp_flag | 1 | u(1) |
| } |  |  |

| pic_layer_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| pic_order_cnt_lsb | 2 | u(v) |
| slice_header_subset( ) |  |  |
| alf_param( ) |  |  |
| } |  |  |

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| pic_order_cnt_lsb | 2 | u(v) |
| if( !use_pic_layer_rbsp_flag ){ |  |  |
| slice_header_subset( ) |  |  |
| alf_param( ) |  |  |
| } |  |  |
| alf_cu_control_param( ) |  |  |
| } |  |  |

| slice_header_subset( ) { | C | Descriptor |
| --- | --- | --- |
| ... // all syntax elements in the old slice header of TMuC0.9 except POC |  |  |
| } |  |  |

| alf_param ( ) { | C | Descriptor |
| --- | --- | --- |
| … // filter shape, filter size, filter coefficients, … |  |  |
| alf_cu_control_flag | 2 | u(1)¦ae(v) |
| if( alf_cu_control_flag ) { |  |  |
| alf_cu_control_max_depth | 2 | ue(v)¦ae(v) |
| num_of_alf_cu_control_flags | 2 | u(v)¦ae(v) |
| for( i=0; i < num_of_alf_cu_control_flags; i++ ) |  |  |
| alf_onoff_flag | 2 | u(1)¦ae(v) |
| } |  |  |
| } |  |  |

| alf_cu_control_param ( ) { | C | Descriptor |
| --- | --- | --- |
| alf_cu_control_flag | 2 | u(1)¦ae(v) |
| if( alf_cu_control_flag) { |  |  |
| alf_cu_control_max_depth | 2 | ue(v)¦ae(v) |
| predict_num_of_alf_cu_control_flags | 2 | u(1)¦ae(v) |
| if( predict_num_of_alf_cu_control_flags) |  |  |
| delta_num_of_alf_cu_control_flags | 2 | se(v)¦ae(v) |
| Else |  |  |
| num_of_alf_cu_control_flags | 2 | ue(v)¦ae(v) |
| for( i = 0; i < num_of_alf_cu_control_flags; i++ ) |  |  |
| alf_onoff_flag | 2 | u(1)¦ae(v) |
| } |  |  |
| } |  |  |

slice_boundary_filter_flag equal to 0 denotes that SBF is not applied for slice boundaries after DF and ALF; equal to 1 denotes that SBF is applied for slice boundaries after DF and ALF. Before the SBF process, all slices of one picture can be decoded independently.

slice_boundary_filter_in_loop_flag equal to 0 denotes that samples after SBF are stored in the reference picture buffer; equal to 1 denotes that samples after SBF are not stored in the reference picture buffer but only used for display.

use_pic_layer_rbsp_flag equal to 0 denotes that each slice has its own slice_header_subset(), alf_param(), and alf_cu_control_param() in the slice header; equal to 1 denotes that all slices of one picture share the same slice_header_subset() and alf_param() in the picture layer RBSP. Please note that regardless of this use_pic_layer_rbsp_flag, DF and ALF are always slice independent, i.e. not across slice boundaries.

pic_order_cnt_lsb is used to derive the POC.

alf_cu_control_flag equal to 0 denotes that the luma component of the slice has no CU on/off control information; equal to 1 denotes that the luma component of the slice has CU on/off control information, that is, only those CU regions with “on” flags of the luma component will be filtered.

alf_cu_control_max_depth specifies the maximum split depth from the LCU for deciding the CU control region size in ALF.

predict_num_of_alf_cu_control_flags equal to 0 denotes that the number of CU on/off flags shall not be predicted; equal to 1 denotes that the number of CU on/off flags shall be predicted by the maximum number of CUs with the split depth, alf_cu_control_max_depth, in one picture.

num_of_alf_cu_control_flags specifies the number of CU on/off flags.

alf_onoff_flag equal to 0 denotes that the CU region shall not be filtered; equal to 1 denotes that the CU region shall be filtered.

# Simulation results

In JCTVC-C500 [2], six configurations are defined as high efficiency all intra (HE-AI), low complexity all intra (LC-AI), high efficiency random access (HE-RA), low complexity random access (LC-RA), high efficiency low delay (HE-LD), and low complexity low delay (LC-LD). We conducted experiments for our proposed slices according to the above configurations. Our software is based on TMuC0.9. In order to achieve fully independent or parallel decoding of multiple slices in each picture, we modified the DF and the ALF in TMuC0.9. After our modification, DF and ALF processes for each slice do not involve any pixels from other slices. Our software can support leaf CU aligned slices [1]. However, in this proposal, we focus on the slice-independent operations and the picture layer RBSP. Hence, we use LCU aligned slices for experiments.

## Results for slice-independent DF and ALF

In this experiment, we evaluate the performance of slice-independent DF and ALF. The slice settings for the proposed software are

1) Two slices per picture,

2) LCU aligned slices,

3) Slice-independent DF and ALF: DF and ALF processes are not across slice boundaries,

4) One ALF parameter set per picture (achieved by using picture layer RBSP),

5) SBF off.

The results are shown in Table 2. The BD-rate increases of the proposed software are 0.6%, 0.6%, 1.4%, 1.2%, 1.8%, and 1.1% for HE-AI, LC-AI, HE-RA, LC-RA, HE-LD, and LC-LD, respectively. The slice AHG also has its own slice implementation. However, the AHG software does not have the functionality of slice-independent DF and ALF. We also run the AHG software with the similar settings. The slice settings for the AHG software are

1) Two slices per picture,

2) LCU aligned slices,

3) DF and ALF processes are across slice boundaries,

4) One ALF parameter set per picture (achieved by sending an ALF parameter set only in the slice     header of the first slice).

The results are shown in Table 3. The BD-rate increases of the AHG software are 0.5%, 0.6%, 1.2%, 1.1%, 1.3% and 1.0% for HE-AI, LC-AI, HE-RA, LC-RA, HE-LD, and LC-LD, respectively. The compression performance of the proposed software is similar to that of the AHG software for low complexity configurations. However, for high efficiency configurations, the proposed software suffers additional 0.1%-0.5% BD-rate increase. Please recall that ALF is enabled in high efficiency configurations and disabled in low complexity configurations, and DB is enabled in both configurations. Generally speaking, the slice-independent DF causes minor coding efficiency loss, the slice-independent ALF causes higher coding efficiency loss, and the slice-independent DF and ALF can provide the benefits of parallel decoding. This is a trade-off between parallel processing and coding efficiency.

Table 2  Results of the proposed software; settings: “two slices per picture”, “LCU aligned slices”, “slice-independent DF and ALF”, “one ALF parameter set per picture (achieved by using picture layer RBSP)”, and “SBF off”; anchor: JCTVC-C500.

|  | HE-AI |  |  | LC-AI |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.1 | 0.2 | 0.2 | 0.2 | 0.2 |
| Class B | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 |
| Class C | 0.7 | 0.7 | 0.6 | 0.7 | 0.8 | 0.8 |
| Class D | 1.0 | 1.2 | 1.2 | 1.1 | 1.2 | 1.3 |
| Class E | 1.0 | 0.8 | 0.8 | 0.9 | 1.0 | 1.2 |
| All | 0.6 | 0.6 | 0.6 | 0.6 | 0.7 | 0.8 |

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.5 | 0.6 | 0.3 | 0.2 | 0.4 |
| Class B | 0.7 | 0.8 | 0.8 | 0.5 | 0.4 | 0.4 |
| Class C | 1.5 | 1.6 | 1.6 | 1.2 | 1.2 | 1.0 |
| Class D | 2.9 | 3.1 | 3.5 | 2.4 | 1.7 | 2.0 |
| Class E |  |  |  |  |  |  |
| All | 1.4 | 1.6 | 1.7 | 1.2 | 0.9 | 1.0 |

|  | HE-LD |  |  | LC-LD |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.7 | 1.1 | 1.0 | 0.5 | 0.2 | -0.1 |
| Class C | 1.2 | 1.6 | 1.8 | 0.9 | 0.3 | 0.3 |
| Class D | 2.7 | 1.9 | 2.8 | 1.5 | 0.4 | 0.4 |
| Class E | 3.0 | 2.2 | 1.6 | 1.6 | 1.8 | 2.4 |
| All | 1.8 | 1.6 | 1.7 | 1.1 | 0.6 | 0.6 |

Table 3  Results of the AHG software; settings: “two slices per picture”, “LCU aligned slices”, “DF and ALF across slice boundaries”, and “one ALF parameter set per picture (achieved by sending an ALF parameter set only in the slice header of the first slice)”; anchor: JCTVC-C500.

|  | HE-AI |  |  | LC-AI |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.0 | 0.1 | 0.2 | 0.1 | 0.1 |
| Class B | 0.2 | 0.2 | 0.1 | 0.3 | 0.2 | 0.2 |
| Class C | 0.6 | 0.5 | 0.4 | 0.7 | 0.7 | 0.6 |
| Class D | 0.9 | 0.8 | 0.8 | 1.0 | 0.9 | 0.9 |
| Class E | 0.8 | 0.4 | 0.5 | 0.8 | 0.8 | 1.0 |
| All | 0.5 | 0.4 | 0.4 | 0.6 | 0.5 | 0.6 |

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.5 | 0.4 | 0.3 | 0.2 | 0.3 |
| Class B | 0.6 | 0.8 | 0.5 | 0.5 | 0.4 | 0.4 |
| Class C | 1.2 | 1.4 | 1.3 | 1.1 | 1.0 | 0.9 |
| Class D | 2.3 | 2.5 | 2.6 | 2.1 | 1.5 | 1.7 |
| Class E |  |  |  |  |  |  |
| All | 1.2 | 1.3 | 1.3 | 1.1 | 0.8 | 0.9 |

|  | HE-LD |  |  | LC-LD |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.5 | 1.1 | 0.6 | 0.4 | 0.2 | -0.1 |
| Class C | 1.0 | 1.4 | 1.4 | 0.8 | 0.1 | 0.2 |
| Class D | 2.2 | 1.8 | 2.0 | 1.4 | 0.4 | 0.1 |
| Class E | 2.1 | 1.6 | 1.1 | 1.5 | 1.4 | 2.0 |
| All | 1.3 | 1.4 | 1.3 | 1.0 | 0.5 | 0.4 |

## Results for slice boundary filter

In this experiment, we evaluate the BD-rate performance of the SBF. In the proposed slice implementation, both in-loop and post-loop SBF schemes are supported. The first case is in-loop SBF. The slice settings for the proposed software are

1) Two slices per picture,

2) LCU aligned slices,

3) Slice-independent DF and ALF: DF and ALF processes are not across slice boundaries,

4) One ALF parameter set per picture (achieved by using picture layer RBSP),

5) In-loop SBF is enabled.

With the above slice settings, Table 4 shows the results of in-loop SBF. The BD-rate increases of the in-loop SBF are 0.6%, 0.6%, 1.4%, 1.2%, 1.7%, and 1.1% for HE-AI, LC-AI, HE-RA, LC-RA, HE-LD, and LC-LD, respectively. The second case is post-loop SBF. The slice settings for the proposed software are

1) Two slices per picture,

2) LCU aligned slices,

3) Slice-independent DF and ALF: DF and ALF processes are not across slice boundaries,

4) One ALF parameter set per picture (achieved by using picture layer RBSP),

5) Post-loop SBF is enabled.

With the above slice settings, Table 5 shows the results of post-loop SBF. The BD-rate increases of the in-loop SBF are 0.6%, 0.6%, 1.4%, 1.2%, 1.7%, and 1.0% for HE-AI, LC-AI, HE-RA, LC-RA, HE-LD, and LC-LD, respectively.

Table 4  Results of the proposed software; settings: “two slices per picture”, “LCU aligned slices”, “slice-independent DF and ALF”, “one ALF parameter set per picture (achieved by using picture layer RBSP)”, and “in-loop SBF”; anchor: JCTVC-C500.

|  | HE-AI |  |  | LC-AI |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.1 | 0.1 | 0.2 | 0.1 | 0.1 |
| Class B | 0.2 | 0.2 | 0.2 | 0.3 | 0.2 | 0.2 |
| Class C | 0.6 | 0.5 | 0.4 | 0.7 | 0.7 | 0.6 |
| Class D | 1.0 | 0.9 | 0.9 | 1.1 | 0.9 | 0.9 |
| Class E | 0.9 | 0.5 | 0.6 | 0.8 | 0.8 | 1.0 |
| All | 0.6 | 0.5 | 0.5 | 0.6 | 0.5 | 0.6 |

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.5 | 0.5 | 0.3 | 0.3 | 0.2 |
| Class B | 0.7 | 0.7 | 0.7 | 0.5 | 0.4 | 0.4 |
| Class C | 1.4 | 1.5 | 1.6 | 1.2 | 1.1 | 1.0 |
| Class D | 2.8 | 2.7 | 3.3 | 2.3 | 1.7 | 2.0 |
| Class E |  |  |  |  |  |  |
| All | 1.4 | 1.4 | 1.6 | 1.2 | 0.9 | 0.9 |

|  | HE-LD |  |  | LC-LD |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.7 | 1.2 | 0.8 | 0.5 | 0.2 | 0.1 |
| Class C | 1.2 | 1.6 | 1.7 | 0.9 | 0.2 | 0.3 |
| Class D | 2.6 | 2.1 | 2.7 | 1.6 | 0.7 | 0.4 |
| Class E | 2.9 | 2.6 | 1.9 | 1.7 | 1.7 | 2.0 |
| All | 1.7 | 1.8 | 1.7 | 1.1 | 0.6 | 0.6 |

Table 5  Results of the proposed software; settings: “two slices per picture”, “LCU aligned slices”, “slice-independent DF and ALF”, “one ALF parameter set per picture (achieved by using picture layer RBSP)”, and “post-loop SBF”; anchor: JCTVC-C500.

|  | HE-AI |  |  | LC-AI |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.1 | 0.1 | 0.2 | 0.1 | 0.1 |
| Class B | 0.2 | 0.2 | 0.2 | 0.3 | 0.2 | 0.2 |
| Class C | 0.6 | 0.5 | 0.4 | 0.7 | 0.7 | 0.6 |
| Class D | 1.0 | 0.9 | 0.9 | 1.1 | 0.9 | 0.9 |
| Class E | 0.9 | 0.5 | 0.6 | 0.8 | 0.8 | 1.0 |
| All | 0.6 | 0.5 | 0.5 | 0.6 | 0.5 | 0.6 |

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.5 | 0.5 | 0.3 | 0.2 | 0.3 |
| Class B | 0.7 | 0.8 | 0.7 | 0.5 | 0.4 | 0.4 |
| Class C | 1.4 | 1.6 | 1.6 | 1.2 | 1.2 | 0.9 |
| Class D | 2.8 | 3.0 | 3.4 | 2.4 | 1.6 | 2.0 |
| Class E |  |  |  |  |  |  |
| All | 1.4 | 1.6 | 1.7 | 1.2 | 0.9 | 0.9 |

|  | HE-LD |  |  | LC-LD |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.7 | 1.1 | 0.9 | 0.5 | 0.2 | -0.1 |
| Class C | 1.2 | 1.6 | 1.8 | 0.8 | 0.3 | 0.3 |
| Class D | 2.7 | 1.9 | 2.8 | 1.5 | 0.4 | 0.4 |
| Class E | 2.9 | 2.2 | 1.6 | 1.6 | 1.8 | 2.4 |
| All | 1.7 | 1.6 | 1.7 | 1.0 | 0.6 | 0.6 |

Compared with the experimental results of no SBF in Table 2, either in-loop or post-loop SBF affects luma component BD-rate slightly and has 0.1%-0.2% BD-rate improvement on chroma components.

Although the BD-rate gains are minor, subjective quality improvement by SBF is more significant. Here we show two example images; both are 2 slices per picture setting, QP = 37, and HE-AI configurations, but one is SBF off and the other is in-loop SBF. Figure 3 and Figure 4 show the 7th frames of the results for SBF off and in-loop SBF, respectively. In Figure 3, obviously, a horizontal slice boundary across the entire frame in the middle can be seen. This slice boundary causes severe subjective quality loss. However, in Figure 4, the in-loop SBF significantly improves the subjective quality at the slice boundary.

Conceptually, the in-loop SBF can provide better coding efficiency, while the post-loop SBF can provide higher flexibility on the decoder side where a decoder can choose to support if the sequential processing of the SBF can be tolerated after parallel processes of parsing, reconstruction, DF, and ALF for all slices.

Figure 3  Example image with SBF off. The 7th frame of BlowingBubble with QP= 37, HE-AI, and 2 slices per picture configurations.

Figure 4  Example image with in-loop SBF. The 7th frame of BlowingBubble with QP= 37, HE-AI, and 2 slices per picture configurations.

## Results for picture layer RBSP

In this experiment, we evaluate the performance of the proposed picture layer RBSP. Picture layer RBSP is proposed to reduce the overheads of slice headers. In the above experiments, the number of slices per picture is 2. Now we increase the number of slices per picture to 4. Then we compare the BD-rate for two cases: picture layer RBSP is disabled and enabled. First, the picture layer RBSP is disabled. The proposed slice settings are

1) Four slices per picture,

2) LCU aligned slices,

3) Slice-independent DF and ALF: DF and ALF processes are not across slice boundaries,

4) One ALF parameter set per slice (not using picture layer RBSP),

5) SBF off.

The results are shown in Table 6. The BD-rate increases of the proposed software are 2.0%, 1.7%, 3.9%, 3.1%, 4.0%, and 2.6% for HE-AI, LC-AI, HE-RA, LC-RA, HE-LD, and LC-LD, respectively. Second, the picture layer RBSP is enabled. The proposed slice settings are

1) Four slices per picture,

2) LCU aligned slices,

3) Slice-independent DF and ALF: DF and ALF processes are not across slice boundaries,

4) One ALF parameter set per picture (using picture layer RBSP),

5) SBF off.

The results are shown in Table 7. The BD-rate increases of the proposed software are 1.6%, 1.7%, 3.5%, 2.9%, 3.9%, and 2.4% for HE-AI, LC-AI, HE-RA, LC-RA, HE-LD, and LC-LD, respectively. As expected, using picture layer RBSP can reduce redundancy between multiple slice headers and have a little BD-rate improvement. The advantage of the picture layer RBSP is expected to increase as the number of slices in a picture increases.

Table 6  Results of the proposed software; settings: “four slices per picture”, “LCU aligned slices”, “slice-independent DF and ALF”, and “one ALF parameter set per slice (not using picture layer RBSP)”; anchor: JCTVC-C500.

|  | HE-AI |  |  | LC-AI |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.4 | 0.3 | 0.3 | 0.5 | 0.5 | 0.5 |
| Class B | 0.8 | 0.7 | 0.5 | 0.9 | 0.8 | 0.9 |
| Class C | 2.2 | 1.7 | 1.8 | 1.9 | 2.1 | 2.2 |
| Class D | 3.2 | 4.2 | 4.4 | 2.7 | 3.1 | 3.3 |
| Class E | 3.1 | 2.8 | 2.0 | 2.2 | 3.0 | 3.3 |
| All | 2.0 | 2.0 | 1.9 | 1.7 | 1.9 | 2.1 |

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.9 | 1.2 | 1.0 | 0.7 | 0.7 | 0.7 |
| Class B | 1.8 | 2.3 | 1.4 | 1.6 | 1.2 | 1.2 |
| Class C | 4.0 | 3.7 | 4.4 | 3.2 | 2.9 | 2.9 |
| Class D | 7.8 | 11.0 | 12.1 | 6.0 | 4.5 | 5.1 |
| Class E |  |  |  |  |  |  |
| All | 3.9 | 4.8 | 5.0 | 3.1 | 2.5 | 2.6 |

|  | HE-LD |  |  | LC-LD |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 1.3 | 2.7 | 1.4 | 1.3 | 0.6 | 0.3 |
| Class C | 3.1 | 2.2 | 2.2 | 2.1 | 1.1 | 1.0 |
| Class D | 6.5 | 7.8 | 9.0 | 3.8 | 1.9 | 1.7 |
| Class E | 6.1 | 3.6 | 2.7 | 3.7 | 3.9 | 3.9 |
| All | 4.0 | 4.0 | 3.8 | 2.6 | 1.7 | 1.5 |

Table 7  Results of the proposed software; settings: “four slices per picture”, “LCU aligned slices”, “slice-independent DF and ALF”, and “one ALF parameter set per picture (using picture layer RBSP)”; anchor: JCTVC-C500.

|  | HE-AI |  |  | LC-AI |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.4 | 0.4 | 0.4 | 0.5 | 0.5 | 0.5 |
| Class B | 0.7 | 0.7 | 0.8 | 0.9 | 0.8 | 0.9 |
| Class C | 1.8 | 1.8 | 1.9 | 1.9 | 2.1 | 2.2 |
| Class D | 2.6 | 3.1 | 3.3 | 2.7 | 3.1 | 3.3 |
| Class E | 2.4 | 2.6 | 2.1 | 2.2 | 3.0 | 3.3 |
| All | 1.6 | 1.8 | 1.7 | 1.7 | 1.9 | 2.1 |

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.7 | 1.1 | 1.2 | 0.7 | 0.7 | 0.7 |
| Class B | 1.9 | 2.2 | 2.0 | 1.5 | 1.2 | 1.2 |
| Class C | 3.6 | 4.2 | 4.2 | 3.1 | 2.7 | 2.7 |
| Class D | 6.6 | 7.3 | 7.7 | 5.6 | 4.1 | 4.7 |
| Class E |  |  |  |  |  |  |
| All | 3.5 | 3.9 | 4.0 | 2.9 | 2.3 | 2.5 |

|  | HE-LD |  |  | LC-LD |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 1.7 | 2.7 | 1.9 | 1.2 | 0.6 | 0.3 |
| Class C | 3.0 | 3.7 | 3.7 | 2.0 | 1.0 | 0.9 |
| Class D | 5.6 | 5.4 | 5.7 | 3.5 | 1.6 | 1.4 |
| Class E | 6.5 | 5.2 | 4.7 | 3.3 | 3.4 | 3.4 |
| All | 3.9 | 4.1 | 3.8 | 2.4 | 1.5 | 1.3 |

# Conclusions

MediaTek proposed slice-independent deblocking filter (DF), slice-independent adaptive loop filter (ALF), slice boundary filter (SBF), and picture layer raw byte sequence payload (RBSP). The slice-independent DF and ALF can better support parallel encoding and decoding of multiple slices in a picture, the SBF can reduce artifacts across slice boundaries and significantly improve the subjective quality when slice-independent DF and ALF are used, and the picture layer RBSP can provide an option for sharing common information among slice headers in a picture. It is proposed to adopt the three tools as options of slice settings in HEVC.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Chih-Wei Hsu, Chia-Yang Tsai, Yu-Wen Huang, Ching-Yeh Chen, Chih-Ming Fu, and Shawmin Lei, “Leaf CU Aligned Slices,” Document of Joint Collaborative Team on Video Coding, JCTVC-D127, Jan. 2011.

- Frank Bossen, “Common Test Conditions and Software Reference Configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-C500, Oct. 2010.