| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D132 WG11 Number: m18884 |
| --- | --- |

| Title: | CE9: verification of experiment 3.1.e |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | J. Jung, G. Clare Orange Labs 38-40 rue du Général Leclerc 9279 ISSY LES MOULINEAUX | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange-ftgroup.com gordon.clare@orange-ftgroup.com |
| Source: | Orange Labs |  |  |

_____________________________

# Introduction

This contribution is the verification of the contribution JCTVC-Dxxx by HHI. The software has been verified and the experimental results are confirmed.

# Experiments

Experiment 3.1.e proposed by HHI in the contribution JCTVC-Dxxx consists in applying the following modifications:

- Skip and Direct modes are disabled.

- “Modified PU based 2” is used for the Merge mode. “Modified PU based 2” consists enabling the Left, Above, Corner, Median and Collocated predictors at the PU level

The software has been verified and does not raise any issue.  It should be noted that with the extension of Merge behaviour from merging only A and B neighbours to the merging of multiple neighbours, the encoding of the effective merge “predictor index” has changed.  The difference between the simple encoding of the original skip predictor index (using two contexts to code the index bits) and the more sophisticated encoding of the merge predictor index (using a selection of up to four contexts, that depend on predictor presence, to code the index bits) may also contribute to the overall gains or losses seen in merge/skip/direct comparisons.

Another thing to notice is that the experiment 3.1.d is with the macro HHI_MRG_CBF_2Nx2N enabled, meaning that when the Merge mode is selected, a new syntax element mrg_cbf can signal that no residue is transmitted. This can be seen as a way to reproduce the Skip mode effect.

The cross check of the results have been done for all classes and configurations sequences. Results obtained by HHI and Orange Labs are reported in the following table. A perfect match is achieved.

|  | Random Access Y BD-rate |  | Random Access LoCo Y BD-rate |  | Low Delay Y BD-rate |  | Low Delay LoCo Y BD-rate |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HHI | Orange Labs | HHI | Orange Labs | HHI | Orange Labs | HHI | Orange Labs |
| ClassA | -0,9 | -0,9 | -2,0 | -2,0 |  |  |  |  |
| ClassB | -0,8 | -0,8 | -1,4 | -1,4 | 0,0 | 0,0 | -0,5 | -0,5 |
| ClassC | -0,7 | -0,7 | -1,8 | -1,8 | -0,5 | -0,5 | -1,5 | -1,5 |
| ClassD | -1,1 | -1,1 | -2,0 | -2,0 | -0,7 | -0,7 | -1,9 | -1,9 |
| ClassE |  |  |  |  | -0,5 | -0,5 | 0,6 | 0,6 |
| AVG | -0,9 | -0,9 | -1,8 | -1,8 | -0,4 | -0,4 | -0,9 | -0,9 |

# Conclusion

This verification contribution confirms the results obtained by HHI for the experiment 3.1.e.