| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D135 |
| --- | --- |

| Title: | Crosscheck of Samsung and Qualcomm's Partial Frequency Transform in JCTVC-D257 by MediaTek |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang Ching-Yeh Chen Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Emails: | peter.chuang@mediatek.com chingyeh.chen@mediatek.com yuwen.huang@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to cross-check the implementation of JCTVC-D257 [1] on partial frequency transform proposed by Samsung and Qualcomm. The verification task has been done successfully, and the results match exactly those provided by Samsung and Qualcomm.

# Verification

Samsung provided the TMuC source code of their proposed codec described in JCTVC-D257. The verification is carried out by setting the following parameters in the provided code:

#define PARTIAL_TRANSF_32x32

#define PARTIAL_TRANSF_32x32_SIZE     16

Six configurations (intra HE, random access HE, low delay HE, intra LC, random access LC, low delay LC) were run for the partial frequency transform with the provided code. The running of the tests includes encoding, decoding, and collecting the run time of encoding and decoding processes.

# Results

The BD-rates from our experimental results match exactly those provided by Samsung and Qualcomm. The encoding time and decoding time are also close to those provided by Samsung and Qualcomm. The results are listed in JCTVC-D135_r1.xls.

# Conclusion

The results presented by Samsung and Qualcomm in JCTVC-D257 are confirmed.

# References

- J. Sole, R. Joshi, M. Karczewicz, Y.-M. Hong, M.-S. Cheon, and I.-K. Kim, “Low complexity 32×32 transform by partial frequency transform,” Document of Joint Collaborative Team on Video Coding, JCTVC-D257, Jan. 2011.