| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D138 |
| --- | --- |

| Title: | Rounding control for Chroma interpolation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li, Jizheng Xu, Gary J. Sullivan, Feng Wu and Houqiang Li | Email: | { jzxu, garysull, fengwu}@microsoft.com |
| Source: | University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

In the current HEVC design, bilinear interpolation filter is used for chroma components, which leads to many half integer cases when interpolating the chroma pixels. This contribution presents a chroma rounding control scheme to improve the coding performance. For random access low complexity case, average 0.1% bits saving on U and V is obtained; for low delay low complexity case, average 2.0% bits saving on U and 2.7% bits saving on V is achieved. The proposed chroma rounding control scheme improves the RD performance with no appreciable impact on complexity at both encoder and decoder.

# Introduction

In the current HEVC, TMuC 0.9, bilinear interpolation filter is adopted on chroma. So there are many half integer cases when interpolating the chroma pixels. Rounding up or rounding down is a problem here. If it is always rounding to one direction, the system error will be accumulated and leading to worse coding efficiency. So rounding control on chroma, which is very low complexity, is necessary to improve the coding efficiency. For low delay case, which the system error will be accumulated up to 600 frames, the proposed method will save 2.0% bits on U and 2.7% bits on V on average. And as the chroma prediction is improved, 0.1% bits saving on Y is also obtained.

# Solution

The proposed chroma rounding control scheme can deal with half, 1/4, and 1/8 integer interpolation in the current TMuC 0.9.

Figure 1 interpolation of chroma

As shown in figure 1, A, B, C, and D is integer pixels. As only 1/8 precision motion compensation is used, so the distance between A and B can be set to 8. And ab, ac, bd, and cd is half pixel in one direction, and center is the center position of A, B, C, and D. P(x,y) is an arbitrary position among A, B, C, and D, where 0<=x<=8, and 0<=y<=8, x and y is integer. And f is rounding factor, f can be 0 or 1. For half integer value, 0 means rounding up and 1 means rounding down.

When the target pixel is at the integer position, no interpolation is needed, and no rounding is needed.

When the target pixel is at integer position in one direction, and not at integer position in the other direction, such as ab, ac, bd, and cd, for example ab, the target pixel value is ( A + B + 1 – f ) >> 1.

When the target pixel is at center position, the target pixel value is ( A + B + C + D + 2 – f ) >> 2.

The idea can also be extended to the case when the target pixel is at arbitrary position, such as P. The target pixel value is ( (8-x)(8-y)A + x(8-y)B + (8-x)yC + xyD + 32 – f ) >> 6.

The f of each slice is fixed and sent to the decoder. In our implementation, f is decided at the encoder by the following rules:

If it is Intra slice, no chroma interpolation is needed. So f can be either 0 or 1. And this flag does not need to be sent to the decoder.

If the GOP size is 1, when the POC&1 == 1, f is 0; otherwise f is 1. It is the different rounding direction with bi-prediction rounding control.

If the GOP size is not 1, when the current will be used as reference frame, f is 1; otherwise f is 0. It is the same rounding direction with bi-prediction rounding control.

The syntax changes are shown in table 1.

Table 1 syntax changes in slice header

| if( slice_type != I ) |  |  |
| --- | --- | --- |
| chroma_rounding_control_flag | 1 | u(1) |

Besides one flag is sent in the slice header, when bilinear interpolation filter is used, the interpolation process is changed as described above.

- Experimental results

The proposed algorithm was implemented on TMuC 0.9. As it is designed for bilinear interpolation, random access low complexity case and low delay low complexity case was tested. The anchor is the TMuC 0.9 default anchor. The summary result is shown in Table 2. The detail result can be found in JCTVC-D138.xls.

Table 2 Summary result of the proposed chroma rounding control algorithm

|  | Random access low complexity |  |  | Low delay low complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.1 | -0.1 |  |  |  |
| Class B | 0.0 | -0.1 | 0.0 | -0.1 | -2.3 | -3.5 |
| Class C | 0.0 | -0.1 | -0.1 | -0.1 | -1.4 | -1.4 |
| Class D | 0.0 | 0.0 | -0.1 | -0.1 | -1.8 | -2.3 |
| Class E |  |  |  | -0.1 | -2.5 | -3.7 |
| All | 0.0 | -0.1 | -0.1 | -0.1 | -2.0 | -2.7 |
| Enc Time | 100% |  |  | 100% |  |  |
| Dec Time | 100% |  |  | 98% |  |  |

It can be seen that the proposed chroma rounding control algorithm can save bits on chroma component, and will not increase the complexity when improving the RD performance.

# References

[1] T. K. Tan, G. J. Sullivan and J. R. Ohm, “Summary of HEVC working draft 1 and HEVC test model (HM),” JCTVC-C405, JCT-VC Meeting, Guangzhou, Oct. 2010.

# Patent rights declaration(s)

University of Science and Technology of China, and Microsoft Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).