| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D139 |
| --- | --- |

| Title: | Constrained temporal motion vector prediction for error resilience |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Feng Wu Houqiang Li | Email: | {jzxu, fengwu}@microsoft.com |
| Source: | University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

In the current AMVP design, up to 5 MV predictors, including one predictor from the temporal co-located block, are used. The temporal motion vector predictor brings coding gains, but it is very unfriendly to errors. A small transmission error can mostly likely lead to a failure of even entropy decoding of all the following inter frames. This document presents the testing results of the coding performance brought by temporal motion vector predictors and proposes a scheme to make a tradeoff between coding efficiency and error resilience by disabling temporal motion vector predictor for some frames.

# Introduction

The AMVP used in the current TMuC 0.9Error: Reference source not found, which provides remarkable coding gains compared to H.264/AVC. In AMVP, one of the MV predictor is from the temporal co-located block. If the reference frame is lost, the entropy decoding will fail due to the wrong value of the max number of the MV predictors. Moreover, for all the following frames, even the entropy decoding cannot be performed until receiving one IDR frame. This kind of error propagation is not expected. In the AVC design, the loss of one slice will not influence the entropy decoding process of the other slices. Thus, when one reference frame is missing, one can still get the correct motion vectors for other frames. The decoded information can be used to do the error concealment. If the entropy decoding also fails, the decoder can do nothing besides frame copy. In another word, all the following bits are useless when one error happens.

A tradeoff method between the coding efficiency and the error resilience is introduced in this proposal. The proposed method is much friendlier to the packet loss error than the current TMuC 0.9 with acceptable RD performance loss.

- Solution

In the current TMuC 0.9, temporal MV predictor is used. When one frame is lost, the MV predictor to the next frame is wrong, and the number of the MV predictors derived in the decoding process will be most likely wrong. But the decoder needs that number to decide how many symbols to read. So if one frame is lost, the entropy decoding will fail for the following slices, until the next Intra slice. It is not expected that one packet loss to cause the other slices failure on entropy decoding. If the entropy decoding can be done successfully, the decoder can do any kind of error concealment using the entropy decoding result as it wants. But when the entropy decoding fails, the decoder can do nothing but the frame copy until the next Intra slice.

The most straightforward method to solve the parsing problem is to parse the bit-stream without the information of the max value. But when the max value is not used, the loss will be too much. The experiment result will be shown in the next section. And also we found, only coding the syntax element of MVPIdx without the information of the max value cannot solve the problem. In the merge mode, not only CU based merging, but also PU based merging, will use the motion information to decide the number of merge candidate too. If this syntax element is also coded without the information of the possible number of candidate, the loss will be much greater. So we give up this solution.

In the proposed method, one flag to indicate whether the current slice uses temporal MVP or not, is sent in the slice header. If we insert some slices, in which the temporal MVP is not adopted, periodically, the error propagation will be truncated at these slices. Thus, the entropy decoding process will not fail after that slice. It is much friendlier than the current design. The syntax changes in the slice header are shown in table 1. And also, one flag to indicate whether the temporal MVP is used for the whole sequence can be sent in SPS or PPS.

Table 1 syntax changes in slice header

| if( slice_type != I ) |  |  |
| --- | --- | --- |
| temporal_mvp_flag | 1 | u(1) |

But according to our test, only inserting frames not using TMVP cannot solve the problem of the failure on the entropy decoding. Let us take the low delay case as the example. Such as frame POC 8 is the frame without TMVP, if the frame POC 7 lost, the frame POC 8 can be successfully entropy decoding. But for the frame POC 9, as it uses 2 reference frames, the information in the frame POC 7, which is not available or incorrect, is important for the entropy decoding of frame POC 9. The entropy decoding will fail too. So in our design, two consecutive frames without TMVP should be inserted together to truncate the error propagation. The number of frames without TMVP should be inserted depends on the number of reference frame. In the default test condition, inserting two consecutive frames without TMVP can solve the problem.

Another issue should be noticed is that the TMVP will be scaled according to the POC. If the POC of the reference frame and the POC of the reference frame of the collocated block in the reference frame can be derived correctly according to the information such as frame number when packet loss exists, it is OK for the following entropy decoding. But when the POC deriving process cannot be done correctly, the scale process must be avoided for the frames which reference the frame without TMVP. The detailed results will be shown in the next section.

In conclusion, by disabling some frames’ temporal motion vector prediction can improve the error resilience of the current design. However, to let it work, corresponding encoder decision or even decoder modification should also be needed. Otherwise, if one wants to keep a simple design and can tolerate the loss caused by not using temporal motion vector prediction, disabling temporal motion vector predictor is also an alternative solution. In such case, temporal_mvp_flag can be sent in SPS.

- Experimental results

Test 1: Anchor is the default TMuC 0.9 anchor, test is TMuC 0.9 but disabling the temporal MV predictor by #define AMVP_NEIGH_COL 0. The summary result is shown in table 2. The detail result can be found in JCTVC-D139_1.xls.

Table 2 summary result of disabling temporal MVP (Y BD-rate)

|  | RA_HE | RA_LC | LD_HE | LD_LC |
| --- | --- | --- | --- | --- |
| Class A | 2.5 | 2.7 |  |  |
| Class B | 1.8 | 1.4 | 1.8 | 3.0 |
| Class C | 1.6 | 1.7 | 2.3 | 3.3 |
| Class D | 2.0 | 1.9 | 2.2 | 2.8 |
| Class E |  |  | 2.5 | 3.0 |
| All | 1.9 | 1.8 | 2.2 | 3.0 |

It is clear that temporal MVP will bring coding efficiency improvement to TMuC 0.9. Disabling temporal MVP is not suitable.

Test 2: Anchor is the default TMuC 0.9 anchor, test is the proposed method that two consecutive frames in 8 frames without TMVP for low delay case; and inserting one frame without TMVP in every 8 frames for random access case. The summary result is shown in table 3. The detail result can be found in JCTVC-D139_2_r1.xls.

Table 3 summary result of proposed method (Y BD-rate)

|  | RA_HE | RA_LC | LD_HE | LD_LC |
| --- | --- | --- | --- | --- |
| Class A | 0.2 | 0.2 |  |  |
| Class B | 0.2 | 0.2 | 0.4 | 0.7 |
| Class C | 0.2 | 0.2 | 0.5 | 0.8 |
| Class D | 0.6 | 0.6 | 0.5 | 0.7 |
| Class E |  |  | 0.5 | 0.6 |
| All | 0.3 | 0.3 | 0.5 | 0.7 |

It is clear that the proposed method will lose a little coding efficiency, but provide the recovery point of the entropy decoding every 8 frames. And compared with disabling the temporal MVP in all frames, the coding efficiency is better.

Test 3: Anchor is the default TMuC 0.9 anchor, test is the proposed method that two consecutive frames in 8 frames without TMVP for low delay case, and also the scaling process of the TMVP is disabled for the frames which reference the frame without TMVP. Only low delay case was tested. The summary result is shown in table 4.

Table 4 summary result of the method of disabling scaling process (Y BD-rate)

|  | LD_HE | LD_LC |
| --- | --- | --- |
| Class A |  |  |
| Class B | 0.4 | 0.7 |
| Class C | 0.6 | 0.9 |
| Class D | 0.6 | 0.7 |
| Class E | 0.6 | 0.8 |
| All | 0.6 | 0.8 |

It can be seen that the results for low delay case in table 3 is a little better than those in table 4. And the results in table 3 need the decoder to derive the correct reference frame POC of the frames without TMVP.

Test 4: Anchor is the default TMuC 0.9 anchor; test is coding MVPIdx without the information of the max value. The summary result is shown in table 5. And the detail result can be found in JCTVC-D139_3.xls.

Table 5 summary result of coding MVPIdx without the known max value (Y BD-rate)

|  | RA_HE | RA_LC | LD_HE | LD_LC |
| --- | --- | --- | --- | --- |
| Class A | 1.6 | 2.3 |  |  |
| Class B | 1.1 | 2.1 | 0.8 | 2.6 |
| Class C | 1.2 | 1.9 | 1.1 | 2.3 |
| Class D | 1.4 | 2.1 | 1.3 | 2.3 |
| Class E |  |  | 2.6 | 7.3 |
| All | 1.3 | 2.1 | 1.3 | 3.3 |

# Patent rights declaration(s)

University of Science and Technology of China, and Microsoft Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).