| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D140 |
| --- | --- |

| Title: | Adaptive coding order for skip and split flags in LCEC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Feng Wu Houqiang Li | Email: | {jzxu, fengwu}@microsoft.com |
| Source: | University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

In the current HEVC design, the quad-tree based hierarchical representation is used to handle variable CU sizes. When the current CU size is larger than smallest CU, one flag, called split flag, is used to indicate whether the current CU is split into sub-CUs or not. For the CU that will not be further split, one flag, called skip flag, is used to indicate whether the current CU is skip mode or not. This contribution presents an adaptive algorithm to adjust the coding order of the two flags mentioned above for LCEC coding. For RA_LC case and LD_LC cases, average 0.4% and 0.6% bit-saving are achieved with no appreciable impact on complexity at both encoder and decoder side. For class E, average bit-saving is 2.3%.

# Introduction

The quad-tree structure is widely used in the current TMuC 0.9, which provides remarkable coding gains compared to H.264/AVC. In the current design, when the current CU size is larger than the smallest CU, split flag is needed to be sent first. And in inter coder frame, if a CU which will not be further split, skip flag is needed to be sent. So there are three combinations of these two flag, a CU is further split (1); a CU is not split and it is skip mode (01); a CU is not split and it is not skip mode (00). But for some CUs, the second case is more likely to happen. For these CUs, encoding skip flag first will cost fewer bits. On the contrary, encoding split flag first is more reasonable. An adaptive algorithm to decide which flag to be sent first is proposed in this contribution. With the proposed algorithm, up to 7% (low delay low complexity case, Vidyo1 and Vidyo4, QP 37) bits saving are achieved. The average gain is 0.4% bits saving for random access low complexity case and 0.6% bits saving for low delay low complexity case. No loss is observed. The complexity is almost the same as anchor.

- Solution

An adaptive algorithm to solve this problem was proposed in this document. When there are more split case than skip case, send split flag first; otherwise, send skip flag first. The bits need to be sent is shown in Table 1.

Table 1 The flags need to be sent

|  | Split flag first | Skip flag first |
| --- | --- | --- |
| Further split case | 1 | 01 |
| Not further split and skip case | 01 | 1 |
| Not further split and not skip case | 00 | 00 |

It can be seen that there is no difference on the third case. So only the first and the second case need to be considered. The proposed algorithm is based on the flags of the surrounding LCUs. The number of the first two cases is counted. If there are more first case than the second, the split flag is sent first; otherwise, the skip flag is sent first. Three LCUs (left, above, and above left) are used to count the number of the cases in the proposed adaptive algorithm. The syntax changing is shown in Table 2.

Table 2 The modified syntax

| coding_unit( x0, y0, currCodingUnitSize ) { | C | Descriptor |
| --- | --- | --- |
| if( x0+currCodingUnitSize < PicWidthInSamplesL && y0+currCodingUnitSize < PicHeightInSamplesL && slice_type != I && skip_first_flag){ |  |  |
| skip_flag | 2 | u(1) ¦ ae(v) |
| if( skip_flag ) { |  |  |
| if( mv_competition_flag ) { |  |  |
| if( inter_pred_idc != Pred_L1 && NumMVPCand( L0 ) > 1 ) |  |  |
| mvp_idx_l0 | 2 | ue(v) ¦ ae(v) |
| if( inter_pred_idc != Pred_L0 && NumMVPCand( L1 ) > 1 ) |  |  |
| mvp_idx_l1 | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| return; |  |  |
| } |  |  |
| } |  |  |
| if( x0+currCodingUnitSize < PicWidthInSamplesL && y0+currCodingUnitSize < PicHeightInSamplesL && |  |  |
| currCodingUnitSize > MinCodingUnitSize ) |  |  |
| split_coding_unit_flag | 2 | u(1) ¦ ae(v) |
| ………… |  |  |

| prediction_unit( x0, y0, currPredUnitSize ) { | C | Descriptor |
| --- | --- | --- |
| if( slice_type != I && !skip_first_flag ) |  |  |
| skip_flag |  |  |
| if( skip_flag && !skip_first_flag) { |  |  |
| if( mv_competition_flag ) { |  |  |
| if( inter_pred_idc != Pred_L1 && NumMVPCand( L0 ) > 1 ) |  |  |
| mvp_idx_l0 | 2 | ue(v) ¦ ae(v) |
| if( inter_pred_idc != Pred_L0 && NumMVPCand( L1 ) > 1 ) |  |  |
| mvp_idx_l1 | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| ………… |  |  |
| } |  |  |

The skip_first_flag in the above table is obtained from the surrounding LCUs by counting the number of the first two cases mentioned in table 1.

- Experimental results

The proposed algorithm was implemented on TMuC 0.9. As it is designed for LCEC coding, random access low complexity case and low delay low complexity case was tested. The anchor is the TMuC 0.9 default anchor. The summary result is shown in Table 3. And the detail result can be found in JCTVC-D140_1.xls.

Table 3 Summary result of the proposed adaptive algorithm

|  | Random access low complexity |  |  | Low delay low complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | -0.3 | -0.3 |  |  |  |
| Class B | -0.6 | -0.6 | -0.6 | -0.3 | -0.3 | -0.3 |
| Class C | -0.2 | -0.2 | -0.2 | 0.0 | 0.0 | 0.0 |
| Class D | -0.3 | -0.3 | -0.3 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  | -2.5 | -2.4 | -2.4 |
| All | -0.4 | -0.4 | -0.4 | -0.6 | -0.6 | -0.6 |
| Enc Time | 99% |  |  | 100% |  |  |
| Dec Time | 99% |  |  | 98% |  |  |

Also, there may be other solutions of this problem.

One is to count all these cases in one slice to decide which flag is to be sent first. And which to be sent first is signaled in slice header. Actually, this solution will get better result. But all the data in one slice needs to be stored before entropy coding, which needs large buffer.

Another method is always to code skip flag first to make the skip mode cost fewer bits.

All these results are shown below in Table 4 and 5. The detail results can be found in JCTVC-D140_2.xls and JCTVC-D140_3.xls

Table 4 Results of signal in slice header

|  | Random access low complexity |  |  | Low delay low complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | -0.3 | -0.3 |  |  |  |
| Class B | -0.6 | -0.6 | -0.5 | -0.2 | -0.2 | -0.2 |
| Class C | -0.2 | -0.2 | -0.2 | 0.0 | 0.0 | 0.0 |
| Class D | -0.3 | -0.3 | -0.3 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  | -2.3 | -2.3 | -2.2 |
| All | -0.4 | -0.4 | -0.3 | -0.5 | -0.5 | -0.5 |
| Enc Time | 100% |  |  | 100% |  |  |
| Dec Time |  |  |  |  |  |  |

Table 5 Results of always coding skip flag first

|  | Random access low complexity |  |  | Low delay low complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.1 |  |  |  |
| Class B | -0.4 | -0.3 | -0.3 | 0.1 | 0.1 | 0.1 |
| Class C | 0.2 | 0.2 | 0.2 | 0.6 | 0.6 | 0.6 |
| Class D | 0.1 | 0.1 | 0.1 | 0.6 | 0.6 | 0.6 |
| Class E |  |  |  | -2.3 | -2.3 | -2.2 |
| All | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 | -0.1 |
| Enc Time | 100% |  |  | 100% |  |  |
| Dec Time |  |  |  |  |  |  |

It can be seen from the above tables that adaptive coding the skip and the split flag will bring bit saving.  Signaling the coding order in slice header will get similar gain but need to buffer the bit-stream which will need large buffer. For some sequences, always coding skip flag first will bring bit saving, but for others it will also bring loss.

As discussed above, coding the skip and split flag adaptively will improve the RD performance. For random access low complexity case, average 0.4% bits saving is obtained. For low delay low complexity case, average 0.6% bits saving is obtained. Also the complexity does not increase when improving the RD performance.

# References

[1] JCT-VC, “Summary of HEVC working draft 1 and HEVC test model (HM)”, JCTVC-C405, JCT-VC Meeting, Guangzhou, Oct. 2010.

# Patent rights declaration(s)

University of Science and Technology of China, and Microsoft Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).