| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D141 |
| --- | --- |

| Title: | Improvement and extension of inter prediction direction and reference frame index combined coding in LCEC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Feng Wu Houqiang Li | Email: | {jzxu, fengwu}@microsoft.com |
| Source: | University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

In the current TMuC 0.9, inter prediction direction and reference frame index in combined coding for LCEC to improve the RD performance. But this combined coding can only be used when the number of reference frames is not larger than 2. This contribution presents improvement on the combined coding and also extends the combined coding to any number of references. For the improvement of the current combined coding, average 0.3% bits saving are achieved for both RA_LC and LD_LC cases. When using 3 references, extending the combined coding to arbitrary number of reference frames leads to average 0.5% and 0.4% bits saving for RA_LC and LD_LC cases. The proposed algorithm does not impact the complexity at both encoder and decoder side.

- Introduction

In the current TMuC 0.9[1], inter prediction direction and reference frame index in combined coding for LCEC to improve the RD performance. But this combined coding can only be used when reference frames is no more than 2. In the call for proposal of HEVC[2], 4 reference frames are used. So changing the current combined coding method to accommodate with more than 2 references frames is necessary. In this document, some method to improve the RD performance of 2 references was proposed. Also, the combined coding of inter prediction direction and reference frame index is extended to arbitrary number of reference frames. With the improvement on the combined coding, average 0.3% bits can be saved for both random access low complexity case and low delay low complexity case. For 3 reference frames case, average 0.5% and 0.4% bits can be saved for random access low complexity case and low delay low complexity case. The complexity is the same as the anchor.

- Solution

In the current TMuC 0.9, one look up table is used to combined code the inter prediction direction and reference frame index. There are 8 entries in the table, from 0 to 7. The meaning of the value in the lookup table is shown in table 1.

Table 1 the meaning of the value in lookup table

| Value | Meaning |
| --- | --- |
| 0 | List 0 prediction using reference frame 0 |
| 1 | List 0 prediction using reference frame 1 |
| 2 | List 1 prediction using reference frame 0 |
| 3 | List 1 prediction using reference frame 1 |
| 4 | Bi-prediction using reference frame (0, 0) |
| 5 | Bi-prediction using reference frame (0, 1) |
| 6 | Bi-prediction using reference frame (1, 0) |
| 7 | Bi-prediction using reference frame (1, 1) |

The initial lookup table is {0,2,1,4,3,6,5,7}, based on the probability. And when list 1 prediction is not allowed (when list 1 is identical to list 0, list 1 prediction is not allowed), the initial lookup table is {0,1,4,6,5,7,2,3}, moving the unused value (2 and 3) to the end of the lookup table. Four changes is adopted in the proposal:

- When there is up to one frame in each list, the value 1 is never used, so it should be moved to back of lookup table. So the lookup table changed to be {0,2,4,1,3,6,5,7}, and {0,4,1,2,3,6,5,7}.

- When one CU is direct mode (always using the reference frame with the index 0, can only be list 0 direct mode, list 1 direct mode and bi-direct mode). It will cost no more than 2 bits to encode the direction of the direct mode. So when one CU is direct mode, there is no need to use the lookup table to encode the direction. But when there is one reference in each list, the lookup table will be used to adaptive the coding order of 3 modes.

- In the current coding scheme, the max index of all the probabilities in not used. The max index in the current coding scheme is always 7 (count from 0, there are eight entries in the default table, so the max index is 7). The max index is helpful to improve the coding efficiency.

When the max reference frame number is 1, and list 1 prediction is allowed, the max index should be 2.

When the max reference frame number is 1, and list 1 prediction is not allowed, the max index should be 1.

When the max reference frame number is 2, and list 1 prediction is allowed, the max index should be 7.

When the max reference frame number is 2, and list 1 prediction is not allowed, the max index should be 5.

- One bug fix related to xWriteUnarySymbol with version 420.

The detail results of these four improvements can be found in the next section.

Also, we should deal with the case when the reference frame is more than 2. When at least one list has more than 2 reference frames, the current coding scheme will not use the lookup table. But using the lookup table will be helpful to improve the coding efficiency. So we extend the lookup table from 8 entries to 9 entries, where the first eight entries have the same meaning with the current coding scheme, and the ninth entry means at least one reference frame index is equal or larger than 2. When the decoder receives the exception (the last entry), it will decode the inter prediction direction and the reference frame index separately. Also adaptive algorithm is adopted to choose using the lookup table or encoding those syntax elements separately. The proposed adaptive algorithm is that if all the surrounding (left, above and above left) PUs do not use the reference frame with the index equal or greater than 2, the current CU will use the lookup table with 9 entries. And if any of the surrounding PUs uses the reference frame with the index equal or greater than 2, the current CU will encode those syntax elements separately. And if it uses the lookup table and it is an exception, the combinations in the lookup table can be excluded, which will be helpful to encode these syntax elements more efficient. And also when using the lookup table, the max index will be helpful.

When the max reference frame number is greater than 2, and list 1 prediction is allowed, the max index should be 8.

When the max reference frame number is greater than 2, and list 1 prediction is not allowed, the max index should be 6.

- Experimental results

The proposed algorithm was implemented on TMuC 0.9. As it is designed for LCEC coding, random access low complexity case and low delay low complexity case was tested.

Table 2 shows the result of the four improvements on the default configuration. The anchor is the TMuC 0.9 default anchor. And the detail result can be found in JCTVC-D141_1.xls.

Table 2 Summary result of the four improvements on LCEC combined coding

|  | Random access low complexity |  |  | Low delay low complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | -0.2 | -0.2 |  |  |  |
| Class B | -0.3 | -0.1 | -0.2 | -0.3 | -0.2 | -0.2 |
| Class C | -0.3 | -0.1 | -0.2 | -0.2 | -0.2 | -0.2 |
| Class D | -0.4 | -0.4 | -0.2 | -0.3 | 0.1 | -0.3 |
| Class E |  |  |  | -0.2 | 0.0 | -0.2 |
| All | -0.3 | -0.2 | -0.2 | -0.3 | -0.1 | -0.2 |
| Enc Time | 99% |  |  | 100% |  |  |
| Dec Time | 97% |  |  | 99% |  |  |

Table 3 shows the result of 3 reference frames. The anchor also uses 3 reference frames but no combined coding. And the detail result can be found in JCTVC-D141_2.xls.

Table 3 Summary result of LCEC combined coding when reference frame more than 2

|  | Random access low complexity |  |  | Low delay low complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.5 | -0.3 | -0.2 |  |  |  |
| Class B | -0.4 | -0.2 | -0.3 | -0.4 | -0.3 | -0.3 |
| Class C | -0.5 | -0.3 | -0.4 | -0.4 | -0.4 | -0.2 |
| Class D | -0.4 | -0.3 | -0.3 | -0.4 | -0.2 | -0.2 |
| Class E |  |  |  | -0.3 | -0.6 | -0.2 |
| All | -0.5 | -0.3 | -0.3 | -0.4 | -0.3 | -0.2 |
| Enc Time | 100% |  |  | 100% |  |  |
| Dec Time | 102% |  |  | 100% |  |  |

Table 4 shows the coding efficiency improvement of the combined coding in TMuC 0.9. The anchor is the TMuC 0.9 default anchor. The test is disabling the combined coding of inter prediction direction and reference frame index. In the test, the fix related to xWriteUnarySymbol with version 420 is implemented. This fix will not impact the default anchor, because the combined coding is used. And the other 3 changes are not included in the test. The detail result can be found in JCTVC-D141_3.xls.

Table 4 Summary result of LCEC combined coding

|  | Random access low complexity |  |  | Low delay low complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.1 | 0.2 |  |  |  |
| Class B | 0.0 | 0.1 | 0.0 | 0.1 | -0.1 | -0.1 |
| Class C | 0.0 | 0.1 | 0.1 | 0.3 | 0.2 | 0.3 |
| Class D | -0.1 | 0.0 | 0.1 | 0.2 | 0.4 | 0.4 |
| Class E |  |  |  | 0.2 | 0.4 | 0.3 |
| All | 0.0 | 0.1 | 0.1 | 0.2 | 0.2 | 0.2 |
| Enc Time | 100% |  |  | 100% |  |  |

It can be seen that both the two methods mentioned in this proposal can improve the RD performance and do no impact the complexity. Also, the coding gain (0.5% and 0.4%) obtained from the improvement on more than 2 reference frames case is similar to the gain (0.0% and 0.2% without the proposed four changes, and about 0.3% and 0.5% with the proposed four changes) obtained from the combined coding method designed for no more than 2 reference frames.

# References

- JCT-VC, “Summary of HEVC working draft 1 and HEVC test model (HM)”, JCTVC-C405, JCT-VC Meeting, Guangzhou, Oct. 2010.

- VCEG, “Joint Call for Proposals on Video Compression Technology”, VCEG-AM91, VCEG Meeting, Kyoto, Jan. 2010.

# Patent rights declaration(s)

University of Science and Technology of China, and Microsoft Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).