| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D142 |
| --- | --- |

| Title: | A unified design of RQT cbf coding in LCEC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Feng Wu Houqiang Li | Email: | {jzxu, fengwu}@microsoft.com |
| Source: | University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

In the current TMuC 0.9, RQT is adopted to represent the residue. RQT is more flexible than a fixed number of transform levels. When representing the residue in RQT, besides the residue coefficients, the quad-tree structure and the cbf related to the quad-tree structure should be signaled to the decoder. When LCEC is used, the combined coding of the cbf is adopted in the current TMuC to improve the coding efficiency. But in the current design, when max transform depth > 2, disabling the combined coding will get better result. This contribution presents a uniform design for both max RQT depth = 2 and depth > 2, which can achieve the best performance for both cases. Compared with the default anchor, the proposed method will improve the coding efficiency with no appreciable impact on complexity at both encoder and decoder side.

- Introduction

The RQT is used to represent the residue in the current design of TMuC 0.9, which is more flexible than the fixed depth transform. When representing the residue in RQT, besides the residue coefficients, the quad-tree structure and the cbf related to the quad-tree structure should be signaled to the decoder. When LCEC is adopted, the combined coding of the cbf is adopted in the current TMuC to improve the coding efficiency. But in the current design, when max transform depth > 2, disabling the combined coding will get better result. Two different methods to represent the cbf in the current design makes inconvenience. In this proposal, a uniform design to represent the cbf was proposed to deal with the different max RQT depth. When the max depth is 2, as the default anchor, the proposed design will bring more than 1% bits saving on U and V components. When the max depth is 3, the proposed design will also bring more than 1% bits saving on all components. More detailed result can be found in section 3. The proposed method also will not bring appreciable impact on complexity.

- Solution

When max RQT depth = 2, one block has only two states, split or not, and all the sub-blocks have the same transform depth. So the current combined coding design of TMuC 0.9 works well. But when the max RQT depth > 2, as shown in figure 1, the depth of some blocks (0 and 3) are 1(count from 0), and some (1 and 2) are 2. If the current combined coding is used in this block, some information is not need. Such as in depth 1, the cbfs of block 1 and 2 are not needed to be signaled, as they are further split. But the current design of combined coding must combine 4 cbfs, so it is a kind of waste. That is the reason why not using combined coding when max depth > 2 will improve the coding efficiency than the combined coding.

Figure 1 different block different transform depth

In this proposal, a uniform design of combined coding cbf was proposed. The proposed design can deal with both the RQT max depth = 2 and max depth > 2 cases. Generally speaking, the changes of signaling cbf including:

- Only code the cbf of the block will not be further split.

- For intra coded blocks, do not code RQT_CBF_ROOT, because the intra coded block will most likely have residue. For inter coded block, the RQT_CBF_ROOT is sent.

The proposed design will be introduced in details.

For intra coded CUs, the quad-tree is coded first using the current TMuC 0.9 method. If the block is not split, the YUV cbf is combined coded with adaptive method. If the block is split, at each layer, only the cbf of the block not further split is coded. If only one block is not split in the current layer, the YUV cbf of this block is combined coded. If there are two blocks not split in the current layer, the two cbfs of Y is combined coded; the two cbfs of U is combined coded; and the two cbfs of V is combined coded. If there are three blocks not split in the current layer, the three cbfs of Y is combined coded; the three cbfs of U is combined coded; and the three cbfs of V is combined coded. If all the four blocks are not split, the four cbfs of Y is combined coded with adaptive method; UV cbf root is combined coded with adaptive method. If U or V root is not 0, the four chroma cbfs are combined coded with adaptive method.

For inter coded CUs, the YUV root is sent using combined coding with adaptive method. If at least one cbf is not 0, the quad-tree is coded using the current TMuC 0.9 method. At each layer, U and V cbf is sent if the upper layer’s cbf is 1 without combined coding. Also the method in  to deal with the last cbf is adopted. For U and V components, the cbf is signaled using the same method as the current design when the combined coding is not used. If there is one, two or three blocks not split in the current layer, the Y’s cbf is sent without combined coding. If all the four blocks are not split, the four cbfs of Y are combined coded with adaptive method.

- Experimental results

Three sets of test result is shown here.

Test 1: anchor is the default anchor of TMuC 0.9, test is the proposed design of signaling the cbf. The result is shown in table 1. The detail result can be found in JCTVC-D142_1.xls.

Table 1 summary result of the RQT max depth = 2.

|  | All intra LC |  |  | Random access LC |  |  | Low delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0 | -1.0 | -1.0 | 0.0 | -1.1 | -1.2 |  |  |  |
| Class B | -0.1 | -0.9 | -1.0 | 0.0 | -1.2 | -1.3 | -0.1 | -2.3 | -2.6 |
| Class C | 0.0 | -0.8 | -0.9 | 0.0 | -1.0 | -1.0 | -0.1 | -2.2 | -2.0 |
| Class D | -0.1 | -1.0 | -1.0 | -0.1 | -1.4 | -1.0 | -0.2 | -3.3 | -2.9 |
| Class E | -0.1 | -1.6 | -1.3 |  |  |  | -0.1 | -2.4 | -1.5 |
| All | -0.1 | -1.0 | -1.0 | 0.0 | -1.2 | -1.1 | -0.1 | -2.5 | -2.3 |
| Enc T | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec T | 101% |  |  | 103% |  |  | 99% |  |  |

It is clear from table 1 that the proposed uniform design works better than the anchor with no significant impact on complexity.

Test 2: anchor is TMuC 0.9, but changing the max RQT depth to 3 (QuadtreeTUMaxDepthInter: 3, QuadtreeTUMaxDepthIntra: 2), the test is TMuC 0.9, but #define LCEC_CBP_YUV_ROOT 0 and #define QC_BLK_CBP 0, also changing the max RQT depth to 3. Table 2 shows the summary result. The detail result can be found in JCTVC-D142_2.xls.

Table 2 summary result of the RQT max depth = 3, combined coding vs. not combined coding.

|  | All intra LC |  |  | Random access LC |  |  | Low delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -6.8 | -8.6 | -9.9 | -2.1 | -4.1 | -5.1 |  |  |  |
| Class B | -4.5 | -6.6 | -7.9 | -1.4 | -2.5 | -3.1 | -0.4 | 0.4 | 0.6 |
| Class C | -3.6 | -6.2 | -6.8 | -1.0 | -1.8 | -2.1 | -0.4 | 0.0 | 0.1 |
| Class D | -3.0 | -5.8 | -5.8 | -0.8 | -1.6 | -1.4 | -0.4 | 0.2 | 0.5 |
| Class E | -5.1 | -12.2 | -11.1 |  |  |  | 0.4 | -2.1 | -0.9 |
| All | -4.3 | -7.5 | -7.9 | -1.2 | -2.3 | -2.7 | -0.2 | -0.2 | 0.2 |
| Enc T | 99% |  |  | 100% |  |  | 100% |  |  |
| Dec T |  |  |  |  |  |  |  |  |  |

It is clear that when the RQT depth = 3, disabling combined coding works better than the combined coding. So in the next test, the better result, i.e. disabling combined coding in TMuC0.9, will be used as anchor.

Test 3: anchor is TMuC 0.9, disabling those two macros, changing max RQT depth to 3, test is the proposed uniform design of the combined coding of cbf, changing max RQT depth to 3. The summary result is shown in table 3. The detail result can be found in JCTVC-D142_3.xls

Table 3 summary result of the RQT max depth = 3, not combined coding vs. proposed design.

|  | All intra LC |  |  | Random access LC |  |  | Low delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.8 | -1.1 | -0.8 | -1.2 | -1.3 | -1.1 |  |  |  |
| Class B | -2.0 | -1.2 | -0.9 | -1.3 | -2.4 | -2.1 | -1.1 | -3.7 | -4.1 |
| Class C | -1.5 | -1.0 | -1.0 | -1.0 | -1.9 | -1.6 | -0.9 | -2.8 | -3.0 |
| Class D | -1.3 | -0.9 | -0.9 | -0.9 | -1.9 | -2.0 | -0.9 | -3.5 | -3.9 |
| Class E | -3.2 | -1.3 | -1.7 |  |  |  | -1.6 | -2.4 | -2.1 |
| All | -1.9 | -1.1 | -1.0 | -1.1 | -2.0 | -1.8 | -1.1 | -3.2 | -3.4 |
| Enc T | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec T | 102% |  |  | 100% |  |  | 100% |  |  |

It is clear that the proposed uniform design of the combined coding of cbf will bring more than 1% bits saving compared with the better scheme of the current design in TMuC 0.9.

From table 1 and table 3, it is clear that the proposed uniform design of the combined coding works better than the current design in TMuC 0.9, for both max depth = 2 and max depth > 2.

Test 4 anchor is the RQT with the proposed algorithm, max depth=default (inter:2, intra:1); test is the RQT with the proposed algorithm, max depth=default+1. The summary result is shown in table 4. The detail result can be found in JCTVC-D142_4.xls.

Table 4 summary result of the RQT max depth = 2 with max depth=3.

|  | All intra LC |  |  | Random access LC |  |  | Low delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.4 | 0.9 | 0.9 | 0.0 | 0.8 | 0.8 |  |  |  |
| Class B | 0.0 | 0.7 | 0.7 | -0.4 | 0.9 | 1.0 | -0.7 | 0.6 | 0.7 |
| Class C | -0.1 | 0.3 | 0.4 | -0.5 | 0.4 | 0.6 | -0.8 | 0.4 | 0.4 |
| Class D | -0.1 | 0.3 | 0.2 | -0.5 | 0.5 | 0.2 | -1.1 | 0.5 | 0.1 |
| Class E | 0.3 | 1.7 | 1.5 |  |  |  | -0.1 | 2.6 | 1.7 |
| All | 0.1 | 0.7 | 0.7 | -0.4 | 0.6 | 0.7 | -0.7 | 0.9 | 0.7 |
| Enc T | 111% |  |  | 112% |  |  | 110% |  |  |
| Dec T | 102% |  |  | 99% |  |  | 100% |  |  |

It can be seen that, for inter configuration, increasing the max depth of RQT will bring bit saving for Y.

# References

[1] “Summary of HEVC working draft 1 and HEVC test model (HM)”, JCTVC-C405, JCT-VC Meeting, Guangzhou, Oct. 2010.

[2] “Redundancy reduction in Cbf and merge coding”, JCTVC-C277, JCT-VC Meeting, Guangzhou, Oct. 2010.

# Patent rights declaration(s)

University of Science and Technology of China, and Microsoft Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).