| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D144 |
| --- | --- |

| Title: | CE8, Subset 1: Cross-verification report for NEC's proposal (JCTVC-D043) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Zhongbo Shi Jizheng Xu | Email: | jzxu@microsoft.com |
| Source: | University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

This document reports cross-verification results for NEC’s proposal JCTVC-D043, including objective results and visual quality observations.

# Introduction

In JCTVC-D043, NEC proposes to use joint deblocking and debanding filter to improve the decoded quality of HEVC coding system. We got the source code from NEC and independently compiled and ran the code with the following running environment.

| Platform | CPU | Memory | Compiler |
| --- | --- | --- | --- |
| Windows Server 2008 R2 64 bits | 2xIntel Xeon L5420@2.5Ghz | 16GB | VS2010 |

There are two testing conditions. One is to evaluate the coding performance of the proposed scheme compared with TMuC0.9 anchors. The other is to change the ALF setting in the default configurations and evaluate the performance.

The objective results we got match NEC’s. The encoding/decoding times compared with the anchors’ are also similar to what NEC reported. No mismatch between encoder and decoder is found.

# Results

## Objective results

### Comparison under default configurations

The summaries of the results are shown in the following tables. The detail results can be found in the file JCTVC-D144_1.xls.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.2 | 0.0 | 0.0 |
| Class B | 0.1 | 0.0 | 0.0 | 0.6 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.2 | -0.1 | -0.1 | 1.6 | 0.0 | 0.0 |
| All | 0.1 | 0.0 | 0.0 | 0.5 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 103% |  |  | 105% |  |  |

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
| Class B | 0.0 | 0.0 | -0.1 | 0.2 | 0.1 | 0.1 |
| Class C | 0.0 | -0.1 | 0.0 | 0.0 | 0.1 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | -0.2 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 103% |  |  | 106% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.1 |
| Class C | 0.1 | -0.1 | -0.1 | 0.1 | -0.1 | 0.0 |
| Class D | 0.1 | -0.3 | 0.3 | 0.0 | 0.2 | 0.0 |
| Class E | 0.1 | -0.2 | 0.2 | 0.3 | 0.4 | -0.2 |
| All | 0.1 | -0.1 | 0.1 | 0.1 | 0.1 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 105% |  |  |

### Comparison under additional configurations

In this test, for HE coding, ALF is turned off for both the anchor and the NEC’s software; for LC coding, ALF is turned off for both the anchor and the NEC’s software.

The summaries of the results are shown in the following tables. The detail results can be found in the file JCTVC-D144_2.xls.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.6 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Class C | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 1.6 | 0.0 | 0.0 | 0.2 | -0.1 | -0.1 |
| All | 0.5 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.0 | -0.1 | 0.0 | 0.0 | -0.1 |
| Class B | 0.2 | 0.1 | 0.0 | 0.0 | 0.0 | -0.2 |
| Class C | 0.1 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.1 | 0.2 | 0.1 | 0.0 | 0.0 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.1 | 0.1 | 0.1 | 0.0 | 0.0 | -0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.2 | -0.2 | 0.0 | 0.0 | 0.2 |
| Class C | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | -0.1 | -0.3 | 0.0 | -0.1 | -0.1 |
| Class E | 0.2 | -0.1 | 0.3 | 0.2 | 0.2 | -0.3 |
| All | 0.1 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

## Visual quality observations

We have compared the visual quality of NEC’s decoded videos with the anchors with a DELL 24inch monitor. We have observed that for the sky area in Kimono sequences, NEC’s scheme can improve the visual quality by reducing the banding artifacts when Qp=22, 27 and 32, for all_intra and random_access setting and under both the default configurations and the additional configurations. The improvement is more obvious for all_intra cases than random_access cases. For low_delay case, we also observe slight improvement when Qp=22 under the default configurations. No obvious visual degradation has been observed for NEC’s scheme.