| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D145 |
| --- | --- |

| Title: | CE6: Cross-verification report for Samsung and LG's proposal (JCTVC-D350) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xiulian Lan Jizheng Xu | Email: | jzxu@microsoft.com |
| Source: | University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

This document reports cross-verification results for Samsung’s proposal JCTVC-D350.

# Introduction

In JCTVC-D350, Samsung and LG propose to use linear prediction from luma component to chroma components. We got the source code from Samsung and independently compiled and ran the code with the following running environment.

| Platform | CPU | Memory | Compiler |
| --- | --- | --- | --- |
| Windows Server 2008 R2 64 bits | 2xIntel Xeon L5420@2.5Ghz | 16GB | VS2010 |

We have conducted two tests and compared them with TMuC0.9 anchors. one is to use linear prediction from luma to chroma as one of the five intra prediction modes for chroma, called LM_5modes. The other one is to rearrange the order of chroma intra prediction modes, called code-word-reassignment.

The objective results we got match Samsung and LG’s. The encoding/decoding times compared with the anchors’ are also similar to what they reported. No mismatch between encoder and decoder is found.

# Experimental results

For the LM_5modes test, the results are shown in the following tables. The detail results can be found in the file JCTVC-D145_1.xls.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.4 | -7.4 | -3.6 | -1.1 | -7.7 | -3.1 |
| Class B | -1.5 | -7.8 | -4.2 | -1.3 | -8.1 | -3.8 |
| Class C | -1.9 | -7.7 | -8.2 | -1.9 | -7.1 | -7.7 |
| Class D | -1.3 | -5.5 | -5.3 | -1.3 | -5.1 | -4.7 |
| Class E | -1.1 | -4.6 | -5.2 | -0.5 | -3.4 | -4.6 |
| All | -1.5 | -6.7 | -5.5 | -1.3 | -6.4 | -4.9 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6 | -7.2 | -3.2 | -0.5 | -7.1 | -2.1 |
| Class B | -0.7 | -8.3 | -4.3 | -0.6 | -6.0 | -1.7 |
| Class C | -0.9 | -6.2 | -6.3 | -0.8 | -5.1 | -5.1 |
| Class D | -0.6 | -4.1 | -3.7 | -0.6 | -3.1 | -2.9 |
| Class E |  |  |  |  |  |  |
| All | -0.7 | -6.5 | -4.5 | -0.6 | -5.1 | -3.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | -2.4 | -1.3 | -0.2 | -2.0 | -1.3 |
| Class C | -0.3 | -2.4 | -2.7 | -0.3 | -2.2 | -1.8 |
| Class D | -0.2 | -2.2 | -1.3 | -0.2 | -1.0 | -0.8 |
| Class E | -0.2 | -2.4 | -3.3 | 0.1 | -2.7 | -3.7 |
| All | -0.2 | -2.3 | -2.0 | -0.1 | -1.9 | -1.7 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

For the code-word-reassignment test, the results are shown in the following tables. The detail results can be found in the file JCTVC-D145_2.xls.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.8 | 0.1 | 0.3 | -0.6 | -0.3 | -0.1 |
| Class B | -0.7 | 0.1 | 0.1 | -0.6 | -0.5 | -0.4 |
| Class C | -0.7 | 0.1 | 0.2 | -0.7 | -0.5 | -0.4 |
| Class D | -0.5 | 0.2 | 0.3 | -0.5 | -0.3 | -0.3 |
| Class E | -1.0 | -0.6 | -0.3 | -0.5 | -0.1 | -0.2 |
| All | -0.7 | 0.0 | 0.1 | -0.6 | -0.4 | -0.3 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | -0.1 | 0.4 | -0.3 | -0.5 | -0.2 |
| Class B | -0.3 | 0.0 | 0.1 | -0.2 | -0.4 | -0.3 |
| Class C | -0.3 | 0.1 | 0.1 | -0.3 | -0.3 | -0.2 |
| Class D | -0.2 | -0.1 | 0.4 | -0.2 | -0.2 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | -0.3 | 0.0 | 0.2 | -0.2 | -0.3 | -0.2 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | -0.1 | -0.2 | -0.1 | -0.1 | -0.1 |
| Class C | -0.2 | 0.0 | 0.0 | -0.1 | -0.4 | -0.2 |
| Class D | -0.1 | -0.9 | 0.1 | -0.1 | 0.4 | -0.2 |
| Class E | 0.0 | -0.5 | -0.4 | -0.1 | 0.3 | -0.1 |
| All | -0.1 | -0.3 | -0.1 | -0.1 | 0.0 | -0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |