| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D146 |
| --- | --- |

| Title: | CE11: Cross-verification report for Samsung’s proposal (JCTVC-D360) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jizheng Xu | Email: | jzxu@microsoft.com |
| Source: | Microsoft |  |  |

_____________________________

# Abstract

This document reports cross-verification results for Samsung’s proposal JCTVC-D350.

# Introduction

In JCTVC-D360, Samsung proposes a low-complexity adaptive coefficient scanning scheme. We got the source code from Samsung and independently compiled and ran the code with the following running environment.

| Platform | CPU | Memory | Compiler |
| --- | --- | --- | --- |
| Windows Server 2008 R2 64 bits | 2xIntel Xeon L5420@2.5Ghz | 16GB | VS2010 |

We have compared Samsung’s scheme with TMuC0.9 anchor.

The objective results we got match Samsung’s. The encoding/decoding times compared with the anchors’ are also similar to what Samsung reported. No mismatch between encoder and decoder is found.

# Experimental results

Summaries are shown in the following tables. Detail results can be found in the file JCTVC-D146.xls.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.5 | -0.8 | -0.6 | 0.8 | 0.4 | 0.4 |
| Class B | -0.7 | -1.1 | -1.1 | 0.0 | -0.3 | -0.4 |
| Class C | -1.3 | -1.3 | -1.3 | -0.5 | -0.7 | -0.7 |
| Class D | -1.2 | -1.4 | -1.5 | -0.3 | -0.5 | -0.5 |
| Class E | -1.5 | -1.0 | -1.1 | 0.0 | -0.3 | -0.3 |
| All | -1.0 | -1.2 | -1.2 | -0.1 | -0.3 | -0.4 |
| Enc Time[%] | 102% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | -0.4 | -0.2 | 0.7 | 0.3 | 0.1 |
| Class B | -0.6 | -0.8 | -0.9 | -0.5 | -0.7 | -0.7 |
| Class C | -0.7 | -1.0 | -0.9 | -0.4 | -0.7 | -0.7 |
| Class D | -0.6 | -0.7 | -0.6 | -0.1 | -0.6 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | -0.6 | -0.8 | -0.7 | -0.2 | -0.5 | -0.4 |
| Enc Time[%] | 113% |  |  | 113% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.8 | -0.8 | -1.0 | -1.5 | -2.1 | -2.8 |
| Class C | -0.6 | -1.0 | -0.8 | -1.0 | -2.1 | -2.0 |
| Class D | -0.6 | -1.3 | -1.0 | -0.7 | -2.3 | -2.2 |
| Class E | -0.3 | -0.4 | -0.3 | -0.9 | -1.6 | -1.9 |
| All | -0.6 | -0.9 | -0.8 | -1.1 | -2.1 | -2.3 |
| Enc Time[%] | 111% |  |  | 105% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |