| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D149 WG11 Number: m18901 |
| --- | --- |

| Title: | CE9: Summary report for CE9 on motion vector coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Core Experiment Report |  |  |
| Author(s) or Contact(s): | J. Jung 38-40 rue du Général Leclerc 9279 ISSY LES MOULINEAUX B. Bross Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange-ftgroup.com +49 30 31002 622 benjamin.bross@hhi.fraunhofer.de |
| Source: | CE coordinators |  |  |

___________________________

# Introduction

This document summarizes the activities in the Core Experiment CE9 on motion vector coding. A total of 16 companies or universities have registered to the CE.  A few emails were exchanged mainly to coordinate the cross-check activity and exchange the modified versions of the HM0.9 software. Also it can be noticed that for some topics the contribution was cancelled by the proponent (lack of time running the experiment, or insufficient results).

Three main topics have been addressed:

- Study of the configuration of the Skip, Direct and Merge modes,

- Adaptation and modification of the set of predictors,

- Temporal motion vector memory compression.

# Documents List

|  | Tester 1 | Tester 2 (cross-check) |
| --- | --- | --- |
| 3.1.b | HHI - JCTVC-D314 | Motorola - JCTVC-D248 |
| 3.1.c | HHI - JCTVC-D314 | Qualcomm - JCTVC-D355 |
| 3.1.d | HHI - JCTVC-D314 | Orange Labs - JCTVC-D130 |
| 3.1.e | HHI - JCTVC-D314 | Orange Labs - JCTVC-D132 |
| 3.1.f | HHI - JCTVC-D314 | NTT DOCOMO - JCTVC-D233 |
| 3.1.g | Mediatek - JCTVC-D133 | Intel - JCTVC-D172 |
| 3.1.h | JVC Kenwood - JCTVC-D118 | Mitsubishi - JCTVC-D097 |
| 3.1.i | HHI - JCTVC-D314 | NTT DOCOMO - JCTVC-D233 |
| 3.1.j | HHI - JCTVC-D314 | SKKU - JCTVC-D330 |
| 3.1.k | HHI - JCTVC-D314 | NTT DOCOMO - JCTVC-D233 |
| 3.1.l | HHI - JCTVC-D314 | NTT DOCOMO - JCTVC-D233 |
| 3.1.m | NTT DOCOMO - JCTVC-D233 | HHI - JCTVC-D314 |
| 3.1.n | NTT DOCOMO - JCTVC-D233 | Orange Labs - JCTVC-D134 |
| 3.1.o | NTT DOCOMO - JCTVC-D233 | Samsung - JCTVC-D410 |
| 3.1.p | NTT DOCOMO - JCTVC-D233 | ETRI & Kyunghee University - JCTVC-D277 |
| 3.1.q | HHI - JCTVC-D314 | NTT DOCOMO - JCTVC-D233 |
| 3.1.r | HHI - JCTVC-D314 | NTT DOCOMO - JCTVC-D233 |
| 3.1.s | Samsung – JCTVC-D411 | Sony – JCTVC-D413 |
| 3.1.t | Samsung – JCTVC-D411 | Sony – JCTVC-D413 |
| 3.1.u | NTT DOCOMO – JCTVC-D233 | HHI - JCTVC-D316 |
| 3.2.c | Sony - JCTVC-D301 | Canon - JCTVC-D117 |
| 3.2.d | NTT DOCOMO - JCTVC-D231 | Sharp - JCTVC-D067 |
| 3.3.b | Sharp - JCTVC-D072 | NTT DOCOMO - JCTVC-D232 |

# Experiments

## Variations on Skip, Direct and Merge modes

Two types of Merge modes are tested: CU Merge (Merge at the coding unit level) and PU Merge (Merge at the prediction unit level). For both modes, 4 options are possible:

| Merge | left | above | collocated | corner** |
| --- | --- | --- | --- | --- |
| PU/(CU) based | 1 | 1 | 0 | 0 |
| PU/(CU) based 1 | 1 | 1 | 0 | 1 |
| PU/(CU) based 2 | 1 | 1 | 1 | 1 |
| PU/(CU) based 3 | 1 | 1 | 1 | 1 |

The PU Merge signalling has been modified with a new syntax element mrg_cbf  for 2Nx2N inter blocks. A mrg_cbf syntax element equal to 1 signalizes that the current block is merged (merge flag equal to 1) and that no residual data is coded for the current block. A mrg_cbf  syntax element equal to 0 signalizes that the merge and cbf flags have to be coded after mrg_cbf. The PU/CU based 2 and PU/CU based 3 respectively enable or disable this feature.

For the Skip and the Direct modes, 4 options are proposed:

| Skip or Direct | (0,0) * | Left | above | median | corner | collocated |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| AMVP | 1 | 1 | 1 | 1 | 1 | 1 |
| AMVP 1 | 1 | 0 | 0 | 0 | 1 | 1 |
| AMVP 2 | 0 | 0 | 0 | 0 | 1 | 1 |
| AMVP 3 | 0 | 0 | 0 | 0 | 0 | 1 |

16 combinations of these configurations have been tested and compared. They can be split into two categories: CU Merge on the one hand, PU Merge on the other hand. For each category there are 3 sub categories:

- Mix of Skip, Direct and Merge with different predictor configurations

- Disable the Direct mode with different predictor configurations for Skip and Merge

- Disable the Skip and Direct modes with different predictor configurations for Merge

The basic idea of most of the experiments is to move predictors from one mode to the other. All the predictors are always used, but their distribution among the different modes (Skip, Merge) is changed. This classification of the experiments is shown in the figure below, with the corresponding gain compared to the reference.

|  | Skip | Direct | Merge | Software / Configuration | Tester 1 |
| --- | --- | --- | --- | --- | --- |
| 3.1.a Reference | AMVP based | AMVP based | CU based*** | HM1.0 | - |
| 3.1.b (JCTVC-C109) | Modified AMVP 1 | Modified AMVP 1 | PU based | HM1.0 modified by HHI (available Nov. 5th) | HHI |
| 3.1.c | Modified AMVP 2 | Modified AMVP 2 | PU based | HM1.0 modified by HHI (available Nov. 5th) | HHI |
| 3.1.d | Modified AMVP 3 | Modified AMVP 3 | Modified PU based 1 | HM1.0 modified by HHI (available Nov. 5th) | HHI |
| 3.1.e | 0 | 0 | Modified PU based 2 | HM1.0 modified by HHI (available Nov. 5th) | HHI |
| 3.1.f | 0 | 0 | Modified PU based 3 | HM1.0 modified by HHI (available Nov. 5th) | HHI |
| 3.1.g | AMVP based | AMVP based | PU based | HM1.0 modified by HHI (available Nov. 5th) | MediaTek |
| 3.1.h | Modified AMVP 1 | Modified AMVP 1 | CU based | HM1.0 modified by HHI (available Nov. 15th) | JVC Kenwood |
| 3.1.i | Modified AMVP 2 | Modified AMVP 2 | CU based | HM1.0 modified by HHI (available Nov. 15th) | HHI |
| 3.1.j | Modified AMVP 3 | Modified AMVP 3 | Modified CU based 1 | HM1.0 modified by HHI (available Nov. 15th) | HHI |
| 3.1.k | 0 | 0 | Modified CU based 2 | HM1.0 modified by HHI (available Nov. 15th) | HHI |
| 3.1.l | 0 | 0 | Modified CU based 3 | HM1.0 modified by HHI (available Nov. 15th) | HHI |
| 3.1.m | AMVP based | 0 | CU based*** | HHI’s version of HM1.0 modified by DOCOMO (available Nov. 9th) | NTT DOCOMO |
| 3.1.n | Modified AMVP 1 | 0 | PU based | HHI’s version of HM1.0 modified by DOCOMO (available Nov. 9th) | NTT DOCOMO |
| 3.1.o | AMVP based | 0 | PU based | HHI’s version of HM1.0 modified by DOCOMO (available Nov. 9th) | NTT DOCOMO |
| 3.1.p | Modified AMVP 1 | 0 | CU based*** | HHI’s version of HM1.0 modified by DOCOMO (available Nov. 9th) | NTT DOCOMO |
| 3.1.q | Modified AMVP 2 | 0 | CU based*** | HHI’s version of HM1.0 modified by DOCOMO (available Nov. 9th) | HHI |
| 3.1.r | Modified AMVP 2 | 0 | PU based | HHI’s version of HM1.0 modified by DOCOMO (available Nov. 9th) | HHI |
| 3.1.s | AMVP based | 0 | Modified PU based 2 | HHI’s version of HM1.0 modified by DOCOMO (available Jan. 13th) | Samsung |
| 3.1.t | AMVP based | 0 | Modified PU based 3 | HHI’s version of HM1.0 modified by DOCOMO (available Jan. 13th) | Samsung |

Without drawing conclusions in favor of any particular method, it can be noticed that in general using PU Merge instead of CU Merge, on the one hand, and disabling the Direct at the CU level combined with increasing the set of predictors (including a temporal) for the PU Merge on the other hand, seem to be efficient choices. Additionaly, a contribution with a competing set of predictors for the Skip, and a PU based Merge reports the highest bitrate saving.

## Adaptation and modification of the set of predictors

The contributions JCTVC-B081 and JCTVC-C166 by Sony report that dynamic code number assignment for each of the motion predictor candidate is useful to adapt the set of prediction to the characteristic of the sequence. A similar trend is reported in JCTVC-C291 (Orange Labs), where coding efficiency varying on a time-basis has been shown. Another variation of the adaptation was also reported in JCTVC-B094.

|  | Adaptation | Software / Configuration | Tester 1 | Tester 2 |
| --- | --- | --- | --- | --- |
| 3.2.b | With Adaptation based on JCTVC-C291 | Withdrawn |  |  |
| 3.2.c | Adaptation based on JCTVC-C166 |  | Sony | Canon |
| 3.2.d | Modification of the set based on JCTVC-B094 |  | NTT DOCOMO | Sharp |

## 3.2.c experiment proposes a dynamic code_number assignment for PMV candidates. The experiment has been performed with Merge mode off.

3.2.d experiment includes the three following tests:

- Removing median predictor from the candidates of motion vector predictors.

- Disabling the process that prunes the list of motion vector predictors according to MVD.

- Combination of these 2 simplifications with modification of selection for left/top predictor.

The experiment reports that item 3 achieves no loss to current AMVP but some functionality benefits.

## Temporal motion vector memory compression

The contribution JCTVC-C257 proposes a method to reduce the complexity of motion vector competition.  The focus is on reducing the size of the motion vector buffer needed for storing temporal collocated vectors.  The size of the motion vector buffer is significant, considering the granularity of motion representation and that there are two vectors per motion block for B_SLICE.  To reduce the size of that buffer and the associated memory access bandwidth, it is proposed to use a block-based median filter to store motion vectors with a larger granularity.

In this experiment, the coding efficiency impact of the method will be tested. It will also be compared with a method using a simple decimation operation.

|  | Adaptation | Software / Configuration |
| --- | --- | --- |
| 3.3.b | With memory compression based on JCTVC-C257 |  |

In the last meeting, it was proposed to store the motion vector data from previous frames at lower resolution.  This data is used in the motion vector competition tool in the HM design, and the goal was to reduce the resolution by a factor of four in order to reduce the storage requirements.  The technology is tested in CE9, and it is reported that 4x memory size reduction results in 0.4% bit-rate increase for random access and 0.6-0.7% bit-rate increase for low delay test configurations.

# Conclusion

Several experiments have been performed with different configurations of the software. Trying to summarize, the following observations can be made:

- For the Inter mode, it is reported that a configuration without the median and without the pruning process can achieve no loss.

- For the Skip, Direct, Merge modes:

  - in general, having a configuration with PU Merge instead of CU Merge seems to be an efficient choice,

  - in general, disabling the Direct at the CU level combined with increasing the set of predictors (including a temporal) for the PU Merge seems to be an efficient choice.

  - a configuration with a competing set of predictors for the Skip, and a PU based Merge reports the highest bitrate savings. For motion vector competition a four time memory size reduction is proposed with reported bit rate increase of 0.5% on average,