| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D150 |
| --- | --- |

| Title: | Response to CE3: Region-based adaptive interpolation filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shohei Matsuo Yukihiro Bandoh Seishi Takamura Hirohisa Jozawa 1-1, Hikari-no-oka, Yokosuka-shi, Kanagawa 239-0847, JAPAN | Tel: Email: | +81-46-859-2070 matsuo.shohei@lab.ntt.co.jp |
| Source: | NTT (Nippon Telegraph and Telephone) Corporation |  |  |

_____________________________

# Abstract

This document reports the enhancement of Adaptive Interpolation filter (AIF) that adjusts filter coefficients based on the characteristic of the input video signal. Conventional AIF optimizes the filter coefficients on a frame-by-frame basis. When an original image has uniform texture or movement, conventional AIF is adequate. However, when the image has multiple movements or each region of the image has different texture, the coding efficiency could improve by using region-by-region interpolation filter. The proposal was implemented in TMUC 0.9 to evaluate its performance. With the anchor of TMUC 0.9, the average BD-rate gain for the P-frame of the proposal compared to TMuC 0.9 was 0.3 - 1.3 % for the HEVC CfP sequences. The maximum gain was about 7.1 % for the sequence “BQSquare”.

# Introduction

Interpolation filter that supports motion estimation with sub-pixel accuracy is one of important coding tools for improving efficiency of inter frame coding. As interpolation filter, TMuC 0.9 adopts two kinds of filters to interpolate the pixels at sub-pel position. The coefficient values of the filters are constant regardless of the characteristics of the input video sequences. To improve the performance of interpolation, adaptive interpolation filters (AIF) were proposed, for example, 2D-AIF [1], SAIF [2]. Each AIF derives the optimal filter coefficients for each frame. However, single set of filter coefficients may not be the optimal option, when the image has plural regions each of which has different characteristics (motion, texture, etc.). In this contribution, a region-based adaptive approach is introduced into AIF. This approach is called Region-based AIF (RBAIF) [3]. RBAIF generates multiple sets of filter coefficients that are optimized for each region.

# Region-Based AIF (RBAIF)

Fig. 1: Conventional AIF (left) and Region-based AIF (right).

RBAIF designs the optimal multiple sets of filter coefficients for each region in a frame. For example, let us consider a frame which consists of two regions as shown in Fig.1. In this case, RBAIF generates two sets of filter coefficients, while AIF provides single set of filter coefficients on a frame-by-frame basis.

The current RBAIF support the segmentation modes which Tab.1 shows. Based on criteria shown in Tab.1, every pixel in each frame is classified into one of multiple regions. As shown in this table, RBAIF also includes DCT_IF and AIF. The parameters W and H for mode 5 and 6 represent the width and height of the frame, respectively. The parameters  and  for mode 2 and 3 represent the threshold values of motion vectors. The thresholds are determined based on the histogram of motion vectors. In mode 2 and 3, the minimum unit of this classification is 4x4 block. Among candidates of modes, a RBAIF encoder selects the best segmentation mode that minimizes the RD cost. Overhead information added by RBAIF is indices that identifies selected mode and filter coefficient for all regions.

Note that the proposed scheme can support arbitrary number of regions. The proposal is currently an extension of SAIF in terms of filter coefficient coding. Of course, the concept of the proposal can be also coupled with other AIF methods.

Tab. 1: Definition of segmentation modes

# Experimental results

The proposal has been implemented in TMuC 0.9. It is noted that the proposed method is only applied to luminance signal. The configurations of RBAIF are subject to the following conditions:

- Filter type: separable filter

- Tap length: 12 taps for HE

- algorithm to generate filter coefficients of each region: that of SAIF [2]

The other configuration is subject to the common test condition [4] of high coding efficiency anchor.

The objective performance is shown in Tab. 2. The gains (BD-rate and BD-PSNR [5]) with the anchor of TMUC 0.9 are calculated. The average gain of BD-rate was about 0.3 % for random access configuration and 1.3 % for low delay configuration. The maximum gain was about 7.1 % for the sequence “BQSquare” in low delay case.

Tab. 2: RD-performance compared to TMuC0.9

|  | Random access |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | -0.1 | 0.0 |
| Class B | -0.7 | -0.6 | -0.7 |
| Class C | -0.3 | -0.4 | -0.4 |
| Class D | 0.0 | 0.2 | 0.4 |
| Class E |  |  |  |
| All | -0.3 | -0.2 | -0.2 |
| Enc Time[%] | 332% |  |  |
| Dec Time[%] | 99% |  |  |

|  | Low delay |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0.9 | -0.5 | -0.7 |
| Class C | -1.4 | -0.5 | -0.6 |
| Class D | -2.2 | -0.9 | -0.1 |
| Class E | -0.8 | 2.1 | 3.1 |
| All | -1.3 | -0.1 | 0.2 |
| Enc Time[%] | 332% |  |  |
| Dec Time[%] | 106% |  |  |

# Conclusion

In this contribution, a region-based AIF technique was proposed. Average gain of coding efficiency against TMuC 0.9 was about 0.3 % for RA and 1.3 % for LD, respectively. The proposal can be applied to other AIF methods such as 2D-AIF and EAIF that have already been employed in KTA software. More effective region-dividing methods will be investigated.

It is recommended to incorporate the proposal into HEVC.

# References

[1] Y. Vatis, B. Edler, D. T. Nguyen and J. Osterman., “Two-dimensional non-separable adaptive wiener interpolation filter for H.264/AVC,” ITU-T SG16 Q.6 Document, VCEG-Z17r1, 26th VCEG Meeting, Busan, Korea, April 2005.

[2] S. Wittmann and T. Wedi., “Separable adaptive interpolation filter for video coding,” in Proc. International Conference on Image Processing (ICIP) 2008, San Diego, California, USA, October 2008.

[3] S. Matsuo, Y. Bandoh, S. Takamura and H. Jozawa., “Region-based adaptive interpolation filter,” JCTVC-B051, 2nd. JCT-VC Meeting, Geneva, CH, 21-28 July, 2010.

[4] F. Bossen, “Common TM Test Conditions,” JCTVC-C500, 3rd. JCT-VC Meeting, Guangzhou, CN, 7-15 Oct, 2010.

[5] G. Bjontegaard, “Calculation of average PSNR differences between RD-curves,” ITU-T SG16 Q.6 Document, VCEG-M33, 13th VCEG Meeting, Austin, Texas, USA, April 2001.

# Patent rights declaration(s)

NTT Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).