| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D151 WG11 Number: m18903 |
| --- | --- |

| Title: | Mode Dependent 2-step Transform for Intra Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Youji Shibahara Takahiro Nishi | Tel: Email: | +81-50-3587-2044 shibahara.youji@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This contribution proposes a unification of MDDT and ROT, in which mode dependent 1-step transform is used for 4x4 transform block and mode dependent 2-step transforms are used for larger ones. The same 4x4 mode dependent KLT as MDDT is used for 4x4 transform block. The first one of the 2-step transform is either AVC transform or Chen-DCT in the TMuC0.9-hm, and the second one is 4x4 KLT which is partially applied only for the lower frequency components. Application of the second transform is depending on the intra prediction mode of the corresponding prediction unit. An experimental result shows that the proposed method provides 1.3% and 0.6% gain for Intra HE and Random Access HE respectively over the TMuC0.9-hm anchors. The encoding time (and decoding time) is 103% (101%) for Intra HE and no major difference observed for Random Access HE.

- Introduction

The KLT which is designed for intra prediction residual is an effective technology in terms of coding efficiency. On the other hand, however, its complexity is higher than DCT’s in terms of number of operation and encoder/decoder software runtime. This contribution presents an approach for the unification of MDDT and ROT, which encoding/decoding runtimes are comparable with those of TMuC0.9-hm software.

# Solutions and experimental results

We use 2-step transform to reduce the number of multiply. For 16x16 and 32x32 transform blocks, the first transform is Chen-DCT in the TMuC0.9-hm and the second transform is 4x4 KLT which is applied only for the lower frequency components. For 8x8 transform block, the first transform is AVC 8x8 transform in the TMuC0.9-hm and the second transform is mode dependent 4x4 KLT. The 4x4 KLT is applied only for lower frequency components and also the norm adjusting for low frequency component is done between the first and the second transform operations. For 4x4 transform block, we use the same mode dependent KLT as MDDT. This is because, as shown in Table 1 and 2, the number of operation per pixel (written as bold number) increases significantly for larger blocks, but the complexity of 4x4 transform block is considered to be not critical. Besides this, for 4x4 transform block, a factorization method to reduce the complexity to half has been proposed in JCTVC-B24 [1].

The number of operation of this proposal is shown in Table 2 with the comparison with TMuC0.9-hm. And the number of operation for each transform size and for each step of 2-step transform are shown in Appendix Table 5-8.

|  |  | TMuC 0.9 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | AVC 4T | AVC 8T | Chen 16T | Chen 32T |
| Ops. / Line | Addition (+) | 8 | 32 | 74 | 194 |
|  | Multiplication (*) | 0 | 0 | 44 | 116 |
|  | Shift (>>, <<) | 2 | 10 | 26 | 66 |
|  | Total | 10 | 42 | 144 | 376 |
| Ops. / Block | Addition (+) | 64 | 512 | 2368 | 12416 |
|  | Multiplication (*) | 0 | 0 | 1408 | 7424 |
|  | Shift (>>, <<) | 16 | 160 | 832 | 4224 |
|  | Total | 80 | 672 | 4608 | 24064 |
| Ops. / Pixel | Addition (+) | 4.00 | 8.00 | 9.25 | 12.13 |
|  | Multiplication (*) | 0 | 0 | 5.50 | 7.25 |
|  | Shift (>>, <<) | 1.00 | 2.50 | 3.25 | 4.13 |
|  | Total | 5.00 | 10.50 | 18.00 | 23.50 |

Table 1. Number of operations of transforms in TMuC0.9

|  |  | Proposal |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 1 step MDDT 4T | AVC 8T + KLT 4T | Chen 16T + KLT 4T | Chen 32T + KLT 4T |
| Ops. / Block | Addition (+) | 96 | 624 | 2,488 | 12,536 |
|  | Multiplication (*) | 128 | 144 | 1,536 | 7,552 |
|  | Shift (>>, <<) | 32 | 208 | 864 | 4,256 |
|  | Total | 256 | 976 | 4,888 | 24,344 |
| Ops. / Pixel | Addition (+) | 6.00 | 9.75 | 9.72 | 12.24 |
|  | Multiplication (*) | 8.00 | 2.25 | 6.00 | 7.38 |
|  | Shift (>>, <<) | 2.00 | 3.25 | 3.38 | 4.16 |
|  | Total | 16.00 | 15.25 | 19.09 | 23.77 |
| Difference of Ops./ Pixel against TMuC 0.9 | Addition (+) | 2.00 | 1.75 | 0.47 | 0.12 |
|  | Multiplication (*) | 8.00 | 2.25 | 0.50 | 0.13 |
|  | Shift (>>, <<) | 1.00 | 0.75 | 0.13 | 0.03 |
|  | Total | 11.00 | 4.75 | 1.09 | 0.27 |

Table 2. Number of operations of transforms in this proposal

We designed only one KLT matrix for each transform block sizes except for 4x4 transform block, for which MDDT is used. In order to get KLT matrices, we derived covariance matrices for the output of the first transform by the equation M2=TM1TT, where M1 is the covariance matrix for 1-step transform presented in JCTVC-B24[1] and JCTVC-C108[2] and T is the transform matrix of the first transform, i.e. DCT. Since the second transform is applied only for lower frequency components and the transform block size is 4x4, the upper left part of M2ij (0≤i,j<4) was considered for derivation of transform matrix.

As an example, the covariance matrix for 1-step 8x8 transform presented in [1][2] is:

M18×8=1111111112222222123333331234444412345555123456661234567712345678

The covariance matrix for second transform is:

M28×8=1.000−0.357−0.087−0.037−0.020−0.011−0.006−0.003−0.3570.258000000−0.08700.06700000−0.037000.0320000−0.0200000.020000−0.01100000.01400−0.006000000.0110−0.0030000000.010

The transform matrix of second transform with 4x4 size for 8x8 transform block is:

T28×8=1128118−47−10−439109−53−152339109−50182839117

Then, for larger transform blocks including 8x8, we made the slope of M1 more gentle since such blocks would be usually used for spatially uniform region. Specifically, we used 50%, 25% and 25% gradient relative to the above covariance matrix M1 for 8x8, 16x16 and 32x32 respectively. For 8x8 transform block, we used the following covariance matrix and second transform matrix:

M18×8=1111111111.51.51.51.51.51.51.511.522222211.522.52.52.52.52.511.522.5333311.522.533.53.53.511.522.533.54411.522.533.544.5

T28×8=1128122−37−8−334119−33−101628122−2491421125

We used the same assignment of transform mode to each intra prediction mode as proposed in JCTVC-C39[5]. The assignment is shown in Table 3.

Table 3. Assignment of the second transform

| “Logical” Mode | Column Transform | Row Transform |
| --- | --- | --- |
| 0 | KLT | NO OPERATION |
| 1 | NO OPERATION | KLT |
| 2 | NO OPERATION | NO OPERATION |
| 3 | KLT | KLT |
| 4 | KLT | KLT |
| 5 | KLT | NO OPERATION |
| 6 | KLT | NO OPERATION |
| 7 | KLT | KLT |
| 8 | NO OPERATION | KLT |
| 9 | NO OPERATION | KLT |
| 10 | KLT | KLT |
| 11 | KLT | KLT |
| 12 | KLT | NO OPERATION |
| 13 | KLT | NO OPERATION |
| 14 | KLT | KLT |
| 15 | KLT | KLT |
| 16 | NO OPERATION | KLT |
| 17 | NO OPERATION | KLT |
| 18 | KLT | KLT |
| 19 | KLT | KLT |
| 20 | KLT | KLT |
| 21 | KLT | KLT |
| 22 | KLT | NO OPERATION |
| 23 | KLT | NO OPERATION |
| 24 | KLT | NO OPERATION |
| 25 | KLT | NO OPERATION |
| 26 | KLT | KLT |
| 27 | KLT | KLT |
| 28 | KLT | KLT |
| 29 | KLT | KLT |
| 30 | NO OPERATION | KLT |
| 31 | NO OPERATION | KLT |
| 32 | NO OPERATION | KLT |
| 33 | NO OPERATION | KLT |

Table 4 shows RD performance and execution time of the software relative to TMuC0.9-hm. The coding gain is 1.3% and 0.6% for Intra HE and Random Access HE respectively. The encoding time (and decoding time) is 103% (101%) for Intra HE, but the significant increase was not observed for Random Access HE.

Table 4. The RD performance of proposed method (anchor: TMuC0.9)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y psnr | U psnr | V psnr | Y psnr | U psnr | V psnr |
| Class A | -1.7 | -1.7 | -1.6 | -2.1 | -1.0 | -0.7 |
| Class B | -1.0 | -0.9 | -0.8 | -1.5 | -0.7 | -0.6 |
| Class C | -1.3 | -1.0 | -1.1 | -1.7 | -0.8 | -0.8 |
| Class D | -1.3 | -1.1 | -1.0 | -1.7 | -0.7 | -0.7 |
| Class E | -1.7 | -1.8 | -1.8 | -2.4 | -1.0 | -1.2 |
| All | -1.3 | -1.2 | -1.2 | -1.8 | -0.8 | -0.8 |
| Enc Time[%] | 103% |  |  | 106% |  |  |
| Dec Time[%] | 101% |  |  | 108% |  |  |

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y psnr | U psnr | V psnr | Y psnr | U psnr | V psnr |
| Class A | -0.8 | -0.3 | -0.1 | -0.8 | -0.1 | -0.1 |
| Class B | -0.5 | -0.2 | -0.3 | -0.6 | -0.2 | -0.2 |
| Class C | -0.7 | -0.5 | -0.4 | -0.6 | -0.3 | -0.2 |
| Class D | -0.6 | -0.2 | 0.1 | -0.5 | -0.1 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | -0.6 | -0.3 | -0.2 | -0.6 | -0.2 | -0.1 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

# Conclusion

In this contribution, low-complexity unification of MDDT and ROT is proposed. An experimental result shows that the proposed method provides 1.3% and 0.6% gain for Intra HE and Random Access HE without significant increase of software execution time in comparison with TMuC0.9-hm. We recommend that this low-complexity unification of MDDT and ROT be studied in a CE on transform itself.

# References

- C. Yeo, Y. Tan, Z. Li, S. Rahardja, “Mode-Dependent Fast Separable KLT for Block-based Intra Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-B024, Geneva, CH, July 2010.

- A. Saxena, F. Fernandes, “Jointly optimal intra prediction and adaptive primary transform”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C108/m18131, Guangzhou, CN, October 2010

- C. Yeo, Y. Tan, Z. Li, S. Rahardja, “Choice of transforms in MDDT for Unified Intra Prediction”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C039/m18060, Guangzhou, CN, October 2010

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

Table 5. Number of operations of transform 4x4 in this proposal

|  |  | MDDT T4 | Ratio (diff) against TMuC0.9 |
| --- | --- | --- | --- |
| Ops . / Line | Addition (+) | 12 |  |
|  | Multiplication (*) | 16 |  |
|  | Shift (>>) | 4 |  |
|  | Total | 32 |  |
|  | Number of col / row in block | 8 |  |
| Ops. / Block | Addition (+) | 96 | 150.0% |
|  | Multiplication (*) | 128 | (+128) |
|  | Shift (>>) | 32 | (+16) |
|  | Total | 256 | 320.0% |
| Ops. / Pixel | Addition (+) | 6.0 | 150.0% |
|  | Multiplication (*) | 8.0 | (+8) |
|  | Shift (>>) | 2.0 | (+1) |
|  | Total | 16.0 | 320.0% |

Table 6. Number of operations of transform 8x8 in this proposal

|  |  | first AVC 8T | norm adjusting | second KLT 4T | Total | Ratio (diff) against TMuC0.9 |
| --- | --- | --- | --- | --- | --- | --- |
| Ops . / Line | Addition (+) | 32 |  | 12 |  |  |
|  | Multiplication (*) | 0 |  | 16 |  |  |
|  | Shift (>>) | 10 |  | 4 |  |  |
|  | Total | 42 |  | 32 |  |  |
|  | Number of col / row in block | 16 |  | 8 |  |  |
| Ops. / Block | Addition (+) | 512 | 16 | 96 | 624 | 121.9 % |
|  | Multiplication (*) | 0 | 16 | 128 | 144 | (+144) |
|  | Shift (>>) | 160 | 16 | 32 | 208 | 130.0 % |
|  | Total | 672 | 48 | 256 | 976 | 145.2 % |
| Ops. / Pixel | Addition (+) | 8.00 | 0.25 | 1.50 | 9.75 | 121.9 % |
|  | Multiplication (*) | 0.00 | 0.25 | 2.00 | 2.25 | (+2.25) |
|  | Shift (>>) | 2.50 | 0.25 | 0.50 | 3.25 | 130.0 % |
|  | Total | 10.50 | 0.75 | 4.00 | 15.25 | 145.2 % |

Table 7. Number of operations of transform 16x16 in this proposal

|  |  | first Chen 16T | second KLT 4T | Total | Ratio against TMuC 0.9 |
| --- | --- | --- | --- | --- | --- |
| Ops . / Line | Addition (+) | 74 | 15 |  |  |
|  | Multiplication (*) | 44 | 16 |  |  |
|  | Shift (>>) | 26 | 4 |  |  |
|  | Total | 144 | 35 |  |  |
|  | Number of col / row in block | 32 | 8 |  |  |
| Ops. / Block | Addition (+) | 2368 | 120 | 2488 | 105.1 % |
|  | Multiplication (*) | 1408 | 128 | 1536 | 109.1 % |
|  | Shift (>>) | 832 | 32 | 864 | 103.8 % |
|  | Total | 4608 | 280 | 4888 | 106.1 % |
| Ops. / Pixel | Addition (+) | 9.25 | 0.47 | 9.72 | 105.1 % |
|  | Multiplication (*) | 5.50 | 0.50 | 6.00 | 109.1 % |
|  | Shift (>>) | 3.25 | 0.13 | 3.38 | 103.8 % |
|  | Total | 18.00 | 1.09 | 19.09 | 106.1 % |

Table 8. Number of operations of transform 32x32 in this proposal

|  |  | first Chen 32T | second KLT 4T | Total | Ratio against TMuC 0.9 |
| --- | --- | --- | --- | --- | --- |
| Ops . / Line | Addition (+) | 194 | 15 |  |  |
|  | Multiplication (*) | 116 | 16 |  |  |
|  | Shift (>>) | 66 | 4 |  |  |
|  | Total | 376 | 35 |  |  |
|  | Number of col / row in block | 64 | 8 |  |  |
| Ops. / Block | Addition (+) | 12416 | 120 | 12536 | 101.0 % |
|  | Multiplication (*) | 7424 | 128 | 7552 | 101.7 % |
|  | Shift (>>) | 4224 | 32 | 4256 | 100.8 % |
|  | Total | 24064 | 280 | 24344 | 101.2 % |
| Ops. / Pixel | Addition (+) | 12.13 | 0.12 | 12.24 | 101.0 % |
|  | Multiplication (*) | 7.25 | 0.13 | 7.38 | 101.7 % |
|  | Shift (>>) | 4.13 | 0.03 | 4.16 | 100.8 % |
|  | Total | 23.50 | 0.27 | 23.77 | 101.2 % |