| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D152r1 WG11 Number: m18904 |
| --- | --- |

| Title: | Adaptive scaling for bit depth compression on IBDI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2288 takeshi.chujoh@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

In this contribution, a bit depth compression on IBDI and a definition for standardization are proposed.   This contribution shows a solution to improve coding efficiency by increasing internal process of video codec while minimizing reference frame memory access bandwidth.  There are two points.  Firstly, an adaptive scaling method with fixed length format is proposed and secondly, a definition by distortion for memory compression is introduced.  In addition, a solution for fixed rounding problem that was pointed at previous meeting is shown.  As experimental results, the loss bitrate is average of 0.71% for bit depth compression on IBDI.

# Introduction

The mandates of memory compression Adhoc group mandates are as follows.

- - Study motion compensation memory access bandwidth of HM design

- - Study coding efficiency loss caused by fixed rounding of extended bit depth values for reference picture storage.

- - Study the visual quality impact of reference picture memory compression

- - Study use cases for memory compression techniques

- - Study the appropriate scope of standardization for memory compression

- - Report on conclusions reached

In this contribution, considering above items, we propose an adaptive scaling method for depth compression on IBDI from 12-bit to 8-bit.

# Adaptive scaling for bit depth compression

IBDI (Internal Bit Depth Increase) is a coding tool that increases the bit depth of input picture at encoder side and decreases the bit depth of output picture to input bit depth.  By increasing internal bit depth, the accuracy of all internal processes is increased.  This intends to reduce the rounding error of intra-fame prediction, spatial transform, in-loop filtering in order to improve coding efficiency.

For example, by increasing from 8-bit to 12-bit, DPB (Decode Picture Buffer) will be required 1.5 times of memory capacity and 1.5 times of memory bandwidth compared to storing the 8-bit depth.  This is one of the most important problems about complexity.  Therefore, JCTVC-A117 [2], JCTVC-B044 [3] and JCTVC-C075r1 [4] introduced an adaptive scaling for bit depth compression.  The scaling algorithm in the case of 4-bit increase is as follows:

- obtain the minimum pixel value min and the maximum pixel value max of 4x4 block,

- calculate an 6-bit minimum pixel value M (= min >> 6),

- calculate a dynamic range R between max and (M<<6),

- decide the scaling value S so that R becomes less than (128 << S),

- if S is equal to 4, then if pixel_value[0] is less than 8, then P is set to 1 else calculate sixteen storing pixel values P (= (pixel_value[i]+8)>>4). If S is not equal to 4, then calculate sixteen storing pixel values P (= (pixel_value[i] - (M<<6))>>S) and store S, M, and P.

Although stored format on DPB is non-normative, for example, first 1 byte by 4x4 block indicates fixed scaling or adaptive scaling and if the value of first 1 byte is not equal to 0, sixteen 8-bit values P are described and if the value is equal to 0, a following 2-bit value S, an 6-bit minimum pixel value M and sixteen 7-bit value P are described.  As a result, it is possible to describe 128-bit format by 4x4 blocks. Figure 2 shows an example of fixed length format.

The de-scaling algorithm is as follows:

- read first 1 byte P[0] by 4x4 block.

- if P[0] is not equal to 0, then calculate sixteen de-scaling values D[i] (=P[i] << 4), else calculate sixteen de-scaling values D[i] (= (P[i] << S) + (M<<6) + ((S!=4 || S!=0)? (1<<(S-1)):0) ).

Figure 1 explains the relationship of above values.

Figure 1  Bit depth compression process

| adaptive_scaling_block( ) { |  |
| --- | --- |
| P[0] | u(8) |
| if (P[0]!=0) { |  |
| for (i=1;i<16;i++) { |  |
| P[i] | u(8) |
| } |  |
| } else { |  |
| S | u(2) |
| M | u(6) |
| for (i=0; i<16; i++) { |  |
| P[i] | u(7) |
| } |  |
| } |  |
| } |  |

Figure 2  An example of fixed length format (128-bit by 4x4 block)

Actually, this algorithm is a loss less compression on 8-bit depth level except value 0.  For example, according to BT.709, values 0 and 255 don’t appear.  Then, practically there is no problem.  This means that fixed scaling pixel value matches pixel value, which is adaptive de-scaled firstly and is fixed scaled secondary.  Therefore, adaptive scaling can get the results surely better than fixed scaling.

# Specification for standardization

Memory compression or bit depth compression is a closed process into coding loop.  Therefore, if the part of loss less coding or bit stream format on frame memory is not specified by the standardization, any mismatch is not caused on both encoder and decoder.  The technical essential point is the distortion of memory compression.  At this time, we propose a definition of adaptive scaring by controlling bit depth.  Lower bits of each pixel are padded zero by following algorithm.  If the bit depth compression is not needed on encoder side or decoder side, the following method is more effective and the same results as the adaptive scaling can be obtained.

(1) obtain the minimum pixel value min and the maximum pixel value max of 4x4 block

(2) decide the scaling value S from difference between max and min.

for (S=0; max – (min & ~0x3f)>=(128<<S) && S<4; S++);

(3) control bit depth of pixels according to S.

if (S==4) {

D[0] = (pixel_value[0]<8)? 16: (pixel_value[0]+8) & ~0xf;

for (i=1;i<16;i++) D[i] = (pixel_value[i]+8) & ~0xf;

} else if (S==0) {

for (i=0;i<16;i++) D[i] = pixel_value[i];

} else {

for (i=0;i<16;i++) D[i] = (pixel_value[i] & (~0<<S)) + (1<<(S-1));

}

# Fixed rounding problem

At Guangzhou meeting, we reported that there were some performance degradations of fixed rounding method on TMuC software.  This is also one of mandates of Adhoc group.  As a result of investigation, the reason is a difference of definition of square error.

TMuC software uses following equation,

D=i((org[i]<<4)−ref[i])×(org[i]<<4)−ref[i]))>>4					(1)

Where, org[i] is input signal and ref[i] is reference frame memory signal.

KTA software uses following equation,

D=i(org[i]−(ref[i]+8)>>4)×(org[i]−(ref[i]+8)>>4)		(2)

Table 1 shows loss BD-rate of Eq.1 and Eq.2.  Anchor is IBDI with Eq.1.  IBDI with fixed rounding and Eq.1 is significant worse than IBDI off.  This is a problem.  Although in the case of IBDI, Eq. 1 is a little better than Eq. 2; however, in the case of IBDI with fixed rounding, equation 2 is better than equation 1.  When some frame memory compression is introduced, Eq.2 should be used.

Table 1  Loss BD-rate (%) of Eq.1 and Eq.2

|  | IBDI with Eq. 2 |  | IBDI with fixed rounding and Eq.1 |  | IBDI with fixed rounding and Eq. 2 |  | IBDI off |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access | Low delay | Random access | Low delay | Random access | Low delay | Random access | Low delay |
| Class A | 0.1 | N/A | 1.3 | N/A | 0.9 | N/A | 1.5 | N/A |
| Class B | 0.2 | 0.4 | 1.6 | 4.1 | 1.1 | 2.2 | 2.1 | 3.5 |
| Class C | 0.2 | 0.2 | 1.0 | 2.5 | 0.8 | 1.5 | 1.3 | 2.4 |
| Class D | 0.1 | 0.2 | 0.6 | 1.6 | 0.5 | 1.1 | 1.0 | 1.6 |
| Class E | N/A | 0.4 | N/A | 27.1 | N/A | 8.5 | N/A | 11.4 |
| Total | 0.2 | 0.3 | 1.2 | 7.4 | 0.8 | 2.9 | 1.5 | 4.2 |

# Experimental results

The simulation was conducted for all sequences of WQVGA, WVGA, 720p, 1080p and cropped 4kx2k based on the recommended simulation common conditions, JCTVC-C500 [5].  The version of TMuC software is 0.90 plus to memory Adhoc group modification.  The OS is Windows XP SP2 64-bit, the CPU is Xeon® W3565, 3.2GHz and the compiler is Visual C++ 2008.  Table 2 indicates the results of loss coding efficiency of adaptive scaling and fixed scaling compared to high efficiency anchors.  Since there values show loss results from anchor condition, the smaller values mean better results.  While fixed scaling losses 1.89% on average, adaptive scaling only losses 0.71% on average.

scaling and fixed scaling

Table 3  Loss BD-rate (%) of adaptive scaling and fixed scaling

|  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Adaptive scaling | Fixed rounding | IBDI off | Adaptive scaling | Fixed rounding | IBDI off |
| S01 | Traffic | 0.37 | 1.49 | 2.51 | N/A | N/A | N/A |
| S02 | PeopleOnStreet | 0.12 | 0.37 | 0.43 | N/A | N/A | N/A |
| S03 | Kimono | 0.24 | 0.90 | 1.92 | 0.38 | 1.51 | 2.20 |
| S04 | ParkScene | 0.35 | 1.02 | 1.75 | 0.78 | 1.99 | 2.65 |
| S05 | Cactus | 0.42 | 1.20 | 1.74 | 1.40 | 3.06 | 4.18 |
| S06 | BasketballDrive | 0.40 | 0.99 | 2.17 | 0.56 | 1.87 | 3.27 |
| S07 | BQTerrace | 0.54 | 1.50 | 3.03 | 1.20 | 2.72 | 5.40 |
| S08 | BasketballDrill | 0.31 | 1.10 | 1.46 | 0.97 | 2.34 | 3.01 |
| S09 | BQMall | 0.36 | 1.04 | 1.82 | 0.65 | 2.21 | 3.71 |
| S10 | PartyScene | 0.22 | 0.49 | 1.14 | 0.30 | 0.84 | 1.80 |
| S11 | RaceHorses | 0.21 | 0.42 | 0.71 | 0.20 | 0.65 | 1.09 |
| S12 | BasketballPass | 0.22 | 0.49 | 0.73 | 0.34 | 0.89 | 1.15 |
| S13 | BQSquare | 0.28 | 0.65 | 1.57 | 0.61 | 1.34 | 2.40 |
| S14 | BlowingBubbles | 0.29 | 0.55 | 0.98 | 0.35 | 1.24 | 1.89 |
| S15 | RaceHorses | 0.14 | 0.38 | 0.55 | 0.15 | 0.85 | 1.01 |
| S16 | Vidyo1 | N/A | N/A | N/A | 2.53 | 8.42 | 10.96 |
| S17 | Vidyo3 | N/A | N/A | N/A | 3.90 | 8.33 | 11.58 |
| S18 | Vidyo4 | N/A | N/A | N/A | 3.55 | 8.68 | 11.61 |
| Class A |  | 0.25 | 0.93 | 1.47 | N/A | N/A | N/A |
| Class B |  | 0.39 | 1.12 | 2.13 | 0.86 | 2.23 | 3.54 |
| Class C |  | 0.28 | 0.76 | 1.28 | 0.53 | 1.51 | 2.40 |
| Class D |  | 0.23 | 0.52 | 0.96 | 0.36 | 1.08 | 1.61 |
| Class E |  | N/A | N/A | N/A | 3.33 | 8.48 | 11.38 |
| Total |  | 0.30 | 0.84 | 1.50 | 1.22 | 2.93 | 4.25 |

The complexity is measured by the encoding and decoding time.  The measurement values are output of encoder and decoder and the decoder does not output decoded image.  Table 3 indicates the results of increase rate of average encoding and decoding times. The results of adaptive scaling and fixed scaling are not significantly different.  As a result of distortion definition, both encoding and decoding time is a little reduced.

Table 4  Increase rate (%) of average encoding and decoding times

| Proponent | Adaptive scaling with fixed length format |  | Adaptive scaling by distortion definition |  | Fixed rounding |  |
| --- | --- | --- | --- | --- | --- | --- |
| Condition | Random access | Low delay | Random access | Low delay | Random access | Low delay |
| Encoding Time | 0.46% | 0.41% | 0.34% | 0.18% | 1.11% | 1.19% |
| Decoding Time | 7.98% | 7.88% | 7.10% | 4.07% | 8.11% | 8.24% |

Table 4 and 5 indicate average compression rate of MC read memory bandwidth on decoder side.  The module of measurement that is provided by Adhoc group of memory compression is used.  In this method, both luma and chroma block size is 4x4 and the compression ratio is defined 0.667.

on high efficiency anchor

anchor

Table 7  Compression rate (%) of MC read memory bandwidth on high efficiency anchor

|  | 8bit/ 8bit | 32bit/ 64bit | 32bit/ 128bit | 64bit/ 128bit | 64bit/ 256bit | 64bit/ 512bit | 64bit/ 256bit FIFO | 64bit/ 512bit FIFO |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | -12.0% | -21.2% | -25.1% | -30.0% | -40.0% | -50.3% | -34.9% | -34.8% |
| Class B | -13.9% | -23.0% | -27.2% | -31.0% | -38.4% | -46.4% | -34.4% | -34.8% |
| Class C | -11.5% | -21.6% | -25.5% | -30.8% | -39.6% | -49.7% | -37.9% | -38.6% |
| Class D | -6.6% | -18.2% | -22.3% | -29.0% | -39.3% | -50.7% | -39.7% | -40.3% |
| Class E | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| All | -11.1% | -21.1% | -25.1% | -30.3% | -39.2% | -48.9% | -36.8% | -37.3% |

Table 8  Compression rate (%) of MC read memory bandwidth on low complexity anchor

|  | 8bit/ 8bit | 32bit/ 64bit | 32bit/ 128bit | 64bit/ 128bit | 64bit/ 256bit | 64bit/ 512bit | 64bit/ 256bit FIFO | 64bit/ 512bit FIFO |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Class B | -9.7% | -20.0% | -23.9% | -29.1% | -37.8% | -47.0% | -37.1% | -37.7% |
| Class C | -7.8% | -19.0% | -22.8% | -29.5% | -39.6% | -50.2% | -41.4% | -42.5% |
| Class D | -1.7% | -15.0% | -19.2% | -27.6% | -40.4% | -51.5% | -41.5% | -42.5% |
| Class E | -23.3% | -27.6% | -30.8% | -32.5% | -40.4% | -52.4% | -38.2% | -40.6% |
| All | -9.8% | -19.9% | -23.7% | -29.5% | -39.4% | -49.9% | -39.5% | -40.7% |

In addition, as subjective quality checking of decoded sequences, the subjective quality difference caused by adaptive scaling is visually negligible.

# Conclusion

In this contribution, the experimental results of adaptive scaling for bit depth compression on IBDI were shown.  As experimental results, the bitrate loss by adaptive scaling was average of 0.71% and that value is definitely smaller than the value of fixed rounding.  Since additional complexity is negligible, it is suggested that the adaptive scaling should be introduce into the bit depth compression on IBDI.

# Reference

[1]	T. Chujoh, “Tool Experiment 2 on IBDI and memory compression,” JCT-VC Document, JCTVC-A302r2, Geneva, July 2010.

[2]	T. Chujoh, A. Tanizawa and T. Yamakage, “Description of video coding technology proposal by TOSHIBA,” JCT-VC Document, JCTVC-A117, Dresden, April 2010.

[3]	T. Chujoh, T. Shiodera and T. Yamakage, “TE2: Adaptive scaling for bit depth compression on IBDI,” JCT-VC Document, JCTVC-B044, Geneva, July 2010.

[4]	T. Chujoh and T. Yamakage, “TE2: Adaptive scaling for bit depth compression on IBDI,” JCT-VC Document, JCTVC-C075r1, Guangzhou, October 2010.

[5]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-C500, Guangzhou, October 2010.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).