| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D154 WG11 Number: m18906 |
| --- | --- |

| Title: | CE3: Non-uniform Tap Length Filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Kazuyo Kanou Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2288 takeshi.chujoh@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

In this contribution, experimental results of non-uniform tap length filters are reported.  This is one of proposals of core experiment 3 on interpolation for MC (Luma).  The purposes of this core experiment are to further investigate interpolation filtering for motion compensation (Luma) technology. A unified solution that is 12/8-tap DCT-IF for high efficiency anchor and 8/4-tap DCT-IF for low complexity anchor is proposed.  Our simulation results show variation 1 is an average of 0.22% and variation 2 is an average of 0.45% bitrate reduction for high efficiency anchor and an average of 4.19% bitrate reduction for low complexity anchor.  The complexity of 12/8-tap DCT-IF and 8/4-tap DCT are reduced from them of 12-tap DCT-IF and 8-tap DCT-IF, respectively.

# Introduction

The purpose of CE3 [1] is to improve the performance of inter prediction with enhanced motion compensation filters.  In this contribution, according to common condition, JCTVC-C503 [1], the experimental results of non-uniform tap length filtering are reported

# Non-uniform Tap Length Filtering (NTLF)

An interpolation that has non-uniform tap length filters for computational complexity reduction is tested.  For example, in case of inner fractional position, asymmetrical M-tap (M<N) is used and symmetrical N-tap filter is used and in case of border fractional position, both asymmetrical and symmetrical N-tap filter is used.  Since the number of multiplication of symmetrical filter is smaller than that of asymmetrical filter at the same tap-length generally, the worst number of multiplication is reduced.

- 12/8-tap DCT-IF for high efficiency anchor

Variation 1: 12-tap and 8-tap DCT-IF on TMuC software

Variation 2: new 12-tap DCT-IF provided by Samsung and 8-tap DCT-IF on TMuC software

- 8/4-tap DCT-IF for low complexity anchor

8-tap and 4-tap DCT-IF on TMuC software

Figure 1  Non-uniform tap length for each sub-pixel

# Experimental results

The simulation was conducted for all sequences of WQVGA, WVGA, 720p, 1080p and cropped 4kx2k based on the recommended simulation common conditions, JCTVC-B500.  Table 1 indicates the average results of coding efficiency compared to each anchor.  According to JCTVC-500, the interpolation filter of high efficiency anchors is 12-tap DCT-IF and the interpolation filter of low complexity anchors is 6-tap DIF (Directional Interpolation Filter).

Table 1	Average results of luminance BD-rate (%)

|  |  | Low complexity | High efficiency |  |
| --- | --- | --- | --- | --- |
|  |  | 8/4-tap | 12/8-tap (1) | 12/8-tap (2) |
| Random access | Class A | 0.1 | -0.1 | 0.0 |
|  | Class B | -1.1 | -0.2 | -0.3 |
|  | Class C | -4.0 | 0.1 | 0.1 |
|  | Class D | -8.8 | 0.2 | 0.2 |
|  | Class E | N/A | N/A | N/A |
|  | All | -3.7 | 0.0 | 0.0 |
|  | Enc Time[%] | 108% | 100% | 102% |
|  | Dec Time[%] | 112% | 94% | 98% |
| Low delay | Class A | N/A | N/A | N/A |
|  | Class B | -2.8 | -0.6 | -0.7 |
|  | Class C | -4.5 | -0.3 | -0.6 |
|  | Class D | -8.6 | -0.2 | -1.4 |
|  | Class E | -2.8 | -0.8 | -0.7 |
|  | All | -4.6 | -0.5 | -0.9 |
|  | Enc Time[%] | 107% | 100% | 103% |
|  | Dec Time[%] | 109% | 94% | 97% |

# Conclusions

In this contribution, the simulation results of non-uniform tap length filtering have been shown.  As experimental results, the coding efficiencies for two anchors have been improved and the total complexities of the solution have been balanced.

We would like to recommend that this solution should be adopted on HM.

# References

[1] 	T. Chujoh, “CE3: Interpolation for MC (Luma),” Joint Collaborative Team on Video Coding, JCTVC-C305r5, Guangzhou, Oct. 2010.

[2] 	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-C500, Guangzhou, October 2010.

[3] 	T. Chujoh, K. Kano and T. Yamakage, “Description of video coding technology proposal by TOSHIBA,” Joint Collaborative Team on Video Coding, JCTVC-C78, Guanzhou, Oct. 2010.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).