| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KOR, 20-28 January, 2011 | Document: JCTVC-D155r1 WG11 Number: m18907 |
| --- | --- |

| Title: | Summary of CE3: interpolation for MC (Luma) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh | Tel: Email: | +81-44-549-2288 takeshi.chujoh@toshiba.co.jp |
| Source: | CE coordinator |  |  |

_____________________________

# Abstract

This contribution is a summary of CE3: interpolation for MC (Luma).  Eleven companies have been registered in CE3 and seven proposed tools have been evaluated on the common condition with cross-verification after all.  Detailed results will be reported by proponents.

# Introduction

In this core experiment, interpolation filters for motion compensation of luminance pixels have been discussed.  There are following two purposes;

- Improvement of coding efficiency by interpolation filter

- Reduction of complexity and reference frame memory access bandwidth generally

# Document list

- Summary document

  - JCTVC-D155 [CE coordinator] Summary of CE3 : interpolation for MC (Luma)

- Proponent's documents

  - JCTVC-D056 [HHI] CE3: Luma Interpolation using MOMS

  - JCTVC-D090 [Sony] CE3 : Results on Bi/Single MC interpolation filter

  - JCTVC-D150 [NTT] Response to CE3: Region-based adaptive interpolation filter

  - JCTVC-D154 [TOSHIBA] CE3: Non-uniform tap length filtering

  - JCTVC-D323 [Nokia and Qualcomm] CE3: Results for luma interpolation filter tests by Nokia and Qualcomm

  - JCTVC-D344 [Samsung] CE3: Experimental results of DCTIF by Samsung

  - JCTVC-D376 [Qualcomm] CE3: Qualcomm’s proposal for luma interpolation filters

- Cross-checking documents

  - JCTVC-D058 [TI] CE3: Verification of Nokia adaptive DCT-IF/DIF filters

  - JCTVC-D059 [TI] CE3: Verification of Samsung new 6-tap/8-tap DCT-IF filter

  - JCTVC-D076 [Qualcomm] CE3: Verification results of Nokia’s Proposal(JCTVC-D323)

  - JCTVC-D091 [Sony] CE3: Verification of Samsung new 8-tap DCT-IF filter

  - JCTVC-D158 [TOSHIBA] CE3: Verification of Sony's Bi/Single MC interpolation filter (JCTVC-D090)

  - JCTVC-D159 [TOSHIBA] CE3: Verification of Samsung's new 12-tap DCT-IF (JCTVC-D344)

  - JCTVC-D160 [TOSHIBA] CE3: Verification of Qualcomm's 12/8-tap interpolation filter (JCTVC-D376)

  - JCTVC-D179 [KDDI] CE3:Cross-check for NTT's proposal on Region-Based Adaptive Interpolation Filter (JCTVC-D150)

  - JCTVC-D288 [MediaTek] CE3: Crosscheck of Qualcomm’s one-pass SIFO filtering for low complexity coding by MediaTek

  - JCTVC-D294 [MediaTek] CE3 : Crosscheck of Qualcomm’s 12/8 tap interpolation filter by MediaTek

  - JCTVC-D327 [Nokia] CE3: Cross verification of MOMS interpolation filter

  - JCTVC-D341 [Samsung] CE3: Cross-verification of Qualcomm’s experimental results for Luma interpolation using SIFO6 by Samsung

  - JCTVC-D343 [Samsung] CE3: Cross-verification of Toshiba's experimental results for NTLF with DCTIF 12/8 by Samsung

  - JCTVC-D372 [HHI] CE3: Cross check of Toshiba's luma interpolation filter JCTVC-D154

  - JCTVC-D414 [LG] CE3 : Cross-check report of Toshiba’s Non-uniform tap length filter

- Discussion documents

  - JCTVC-D322 [Nokia] CE3: On complexity calculation of low complexity anchor

  - JCTVC-D161 [TOSHIBA] CE3: Verification of Nokia's complexity calculation of low complexity anchor (JCTVC-D322)

# Proposed Tools

Seven tools have been proposed.

Table 1  Proposals

| Proponent | Method |
| --- | --- |
| HHI | MOMS of 4-tap and 6-tap for HE. |
| Nokia | Switching DIF |
| NTT | Region based adaptive interpolation filter |
| Qualcomm | Switching interpolation filter s, 6-tap for LC and 8-tap for HE 12/8-tap interpolation filter. |
| Samsung | New DCTIFs, 6, 8, 12-tap and encoder side restriction. |
| Sony | Bi/Single filters switching in FIF, 6-tap for LC, 8-tap and 12-tap for HE. |
| TOSHIBA | Non-uniform tap length filtering, 8/4-tap DCTIF for LC and 12/8-tap DCTIF for HE. |

# Cross-verification

Proponents provided their source code exchange to all participants.  Cross-checkers run the software independently and compared their results with proponent’s results.

Table 2  Cross checkers

| Proponent | Cross-checker |
| --- | --- |
| HHI | Nokia |
| Nokia | Qualcomm TI |
| NTT | KDDI |
| Qualcomm | MediaTek Nokia Samsung TOSHIBA |
| Samsung | Sony TI TOSHIBA |
| Sony | TOSHIBA |
| TOSHIBA | HHI LG Samsung |

# Experimental results

All experiments for all sequences based on the common conditions, JCTVC-B302r2 [1] have been conducted.  On this common condition, the following evaluation criteria are defined.

1.	Measure impact on bitrate/PSNR using provided data. Use 4-point BD-PSNR and BD-Rate [9].

2.	Complexity (encoding/decoding times, including more detailed analysis on software and hardware implementation complexity generally).

3.	Subjective quality (If sequences coded using a proposal exhibit subjective improvements or degradations, proponents shall describe these subjective differences)

Note: regarding item 1 and 2, as a discussion of participants, a common Excel sheet has been provided.

## Coding efficiency

See attached Excel file.

## Complexity analyses

See attached Excel file.

## Subjective quality (informal comments)

There were the following comments:

# Conclusion

In this core experiment, seven proposals have been tested and their performances have been verified.

# Reference

[1] T. Chujoh, “CE3: Interpolation for MC (Luma),” Joint Collaborative Team on Video Coding, JCTVC-C503r5, Guangzhou, Oct. 2010.