| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, Korea, 20-28 January, 2011 | Document: JCTVC-D163 WG11 Number: m18915 |
| --- | --- |

| Title: | CE 8 subtest1: Improved deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | *Jicheng An, *Xun Guo, *Qian Huang, #Yu-Wen Huang and #Shawmin Lei *North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 #No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Contact: Xun Guo +86-10-82800818 ext. 83347 xun.guo@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution describes MediaTek’s proposal of improved deblocking filter (IDF) which is based on TEN deblocking filter (DF) [1] in HM1.0 and our previous proposal in Guangzhou meeting [2]. There are three major improvements in IDF: mode dependent deblocking filter (MDDF), modified filtering conditions and thresholds (MFCT) and modified filter operation (MFO). In MDDF, we use intra mode information to define different boundary types (BT), which would be used to adjust filter strength and directions. In MFCT, we modified the conditions which determine filtering an edge or not, and the conditions which determine using a weak filter or a strong filter. We also modified the corresponding thresholds. And In MWF, we modified the filtering operations for luma weak filter and chroma filter. Compared with HM1.0, IDF outperforms the anchor in all the six test configurations. Average Bitrate reductions of 1.3%, 1.5%, 1.0%, 0.8%, 1.6% and 1.6% can be observed for intra, intra LoCo, random access, random access LoCo, low delay and low delay LoCo, respectively. Though the visual quality differences between IDF and HM1.0 are small in most cases, some improvements sill can be observed, especially along the boundaries of moving objects. The coding time of IDF is almost the same with HM1.0. About 1.1% increase is observed for both encoder and decoder.

# Introduction

In TEN DF in HM1.0, the filtering process is invoked for luma and chroma components separately and based on 8x8 block. Only the edges on the 8x8 block boundaries are filtered. The filtering is in the order of largest coding unit (LCU) for the whole frame. In a LCU, the filtering is in the order of coding unit (CU). For each 8-pixel boundary, the filtering decision is first made to determine whether the edge should be filtered or not according to some filtering conditions. Only two lines of pixels, i.e. the third and the sixth lines, among the eight lines are used to make this decision. If the boundaryis determined to be filtered, filter selection is then made for each pixel line to determine whether a strong filter or a weak filter should be used.

In order to make the deblocking filter more efficient, we propose to make three major modifications: MDDF, MFCT and MFO. In MDDF, each boundary of prediction unit (PU) or transform unit (TU) is classified into 5 boundary types (BT) according to the mode information across the boundary. For different BT values, the filter selection conditions are different. And the filtering directions of luma weak filter also would be different. In MFCT, we modified the conditions for both filtering decision and filter selection. The threshold values for those conditions are also modified accordingly. In MFO, we added directional filter for the boundary between two intra blocks for luma weak filter, and modified the filtering operations and clipping values for both luma weak filter and chroma filter.

The rest of this contribution is organized as follows. The proposed IDF algorithm is described in Section Error: Reference source not found. Experimental results are provided in Section Error: Reference source not found. Section Error: Reference source not found gives a conclusion.

# Algorithm description

The proposed IDF filter is based on 8x8 block as TEN DF. Fig. 1 shows an example of a vertical edge to be filtered between two blocks A and B. The deblocking filter operates on each 8x8 block in a CU no matter the shapes of A and B. In this example, A and B can be two 8x8 prediction units (PU) or transform units (TU). The major modifications of IDF compared to TEN DF are described as the following sub-sections.

Fig. 1. An example of a vertical boundary for deblocking

## Mode dependent deblocking filter (MDDF)

To every boundary between the two PUs or TUs, A and B, a BT value is assigned with an integer from 0 to 4. The assignment rules are as follows.

- If A and B are both inter coded, BT = 4.

- Else if one of A and B is intra coded, BT = 2.

- Else (i.e., A and B are both intra coded) BT is deduced according to their intra prediction modes.

  - If ModeA = 2 or ModeB =2, BT = 2.

  - Else if (ModeA = ModeB && ModeA = 1 && IsHorizontalEdge) || (ModeA = ModeB && ModeA = 0 && IsVerticalEdge), BT = 0

  - Else if ModeA = ModeB, BT = 3

  - Else BT = 1

Where intra prediction mode “2” represents the DC mode; IsHorizontalEdge represents the boundary is horizontal; and IsVerticalEdge represents the boundary is vertical. ModeA and ModeB are the intra prediction modes of A and B, which have been quantized from 34 to 9. The quantization is according to a mapping table shown in Table 1.

When BT is 3, the intra prediction of A and B are the same. In this case, the filtering direction across the boundary between A and B is according to their intra prediction direction. BT values are also used in filter selection.

Table 1. Intra Mode Mapping Value

| IntraMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mapping | 0 | 1 | 2 | 4 | 5 | 7 | 3 | 6 | 8 |
| IntraMode | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| Mapping | 3 | 5 | 0 | 0 | 7 | 6 | 1 | 1 | 8 |
| IntraMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| Mapping | 4 | 5 | 5 | 0 | 0 | 7 | 7 | 3 | 4 |
| IntraMode | 27 | 28 | 29 | 30 | 31 | 32 | 33 |  |  |
| Mapping | 6 | 6 | 1 | 1 | 8 | 8 | 3 |  |  |

## Modified filtering conditions and thresholds (MFCT)

The filtering conditions in TEN DF include two parts: conditions for filtering decision and conditions for filter selection. The conditions for filtering decision are used to determine whether a boundary should be filtered or not. If a boundary is determined to be filtered, the conditions for filter selection are used to determine whether a strong filter or a weak filter should be used. We modified both parts and the corresponding thresholds as described in the following sub-sections.

### Modifications for filtering decision

We modified the pixel value inequality for filtering decision as follows. In the original inequality, all the pixels in one line shares the same threshold values. We separate the inequality into three inequalities according to pixel positions and different threshold values are used for these three inequalities. The modified conditions are as follows:

- For the third line and the sixth line, i.e. i=2 and 5, respectively, the three conditions are checked:

  - |p0i – q0i| <  α[A or B is intra]

  - |p1i – p0i| <  β[A or B is intra]

  - |q1i – q0i| <  β[A or B is intra]

- The 8-pixel boundary is to be filtered if all six conditions (3 for each 3rd and 6th line) are true. Otherwise, no deblocking filter is needed for this boundary.

The threshold variable α[0], α[1], β[0], and β[1] are specified in Table 2 with dependency on QP.

Table 2. Proposed threshold variables α[0], α[1], β[0], β[1], and clipping variable C0

| QP | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| α [0] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 4 |
| α [1] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 4 |
| β [0] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 2 |
| β [1] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 2 |
| C0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| QP | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| α [0] | 5 | 6 | 7 | 8 | 9 | 10 | 12 | 13 | 15 | 17 | 20 | 22 | 25 | 28 | 32 | 36 | 40 | 45 |
| α [1] | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 55 | 60 | 65 | 70 | 75 | 80 | 85 | 90 |
| β [0] | 2 | 3 | 3 | 3 | 3 | 4 | 4 | 4 | 6 | 6 | 7 | 7 | 8 | 8 | 9 | 9 | 10 | 10 |
| β [1] | 2 | 3 | 3 | 5 | 9 | 13 | 17 | 21 | 25 | 29 | 30 | 32 | 34 | 36 | 38 | 40 | 42 | 44 |
| C0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 3 | 3 | 4 | 4 | 4 | 5 |
| QP | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |  |  |
| α [0] | 50 | 56 | 63 | 71 | 80 | 90 | 101 | 113 | 127 | 144 | 162 | 182 | 203 | 226 | 255 | 255 |  |  |
| α [1] | 95 | 100 | 105 | 110 | 115 | 120 | 125 | 133 | 145 | 160 | 175 | 190 | 215 | 230 | 250 | 255 |  |  |
| β [0] | 11 | 11 | 12 | 12 | 13 | 13 | 14 | 14 | 15 | 15 | 16 | 16 | 17 | 17 | 18 | 18 |  |  |
| β [1] | 47 | 50 | 53 | 56 | 60 | 64 | 68 | 72 | 76 | 80 | 84 | 86 | 88 | 90 | 91 | 92 |  |  |
| C0 | 6 | 7 | 8 | 8 | 9 | 10 | 11 | 12 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |  |  |

### Modifications for filter selection

If the 8-pixel boundary between A and B has been determined to be filtered, one of the two filters (weak or strong filter) need to select for each line. We modified the filter selection conditions as follows with consideration of BT value.

- BT!=4

- |p0i – q0i| < α’

- |q1i -q0i | + |q2i -q0i | + |q3i -q0i | + |p1i -p0i | + |p2i -p0i | + |p3i -p0i | < ((β[0]-4)<<2)

For each line, the strong filter is used if all the above three conditions are true; otherwise, the weak filter is used. The α’ is shown in Table 3, where α[1] in the table is given in Table 2.

Table 3. Values of α’, C0', p, and q as a function of BT

| BT | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- |
| α' | (α[1]>>2)-(α[1]>>4) | (α[1]>>2)-(α[1]>>4) | (α[1]>>2)-(α[1]>>4) | α[1]>>3 | α[1]>>4 |
| C0' | C0 | C0 | C0 | (C0+1)>>1 | min(1,C0) |
| p |  | 0p |  | 0 | 0 |
| q |  | 0q |  | 0 | 0 |

## Modified filter operation (MFO)

### Luma weak filter

The filtered values, p0’, q0’, p1’, and q1’, are calculated as follows:

- p0’  =  Clip0-255 (p0 + 0p)

- q0’  =  Clip0-255 (q0 - 0q)

- p1’  =  |p2-p0| < β[0]-4 ? Clip0-255 (p1+1p): p1

- q1’  =  |q2-q0| < β[0]-4 ? Clip0-255 (q1-1q): q1

The delta values are computed according to different BT values. When BT is less than 3, the filtering directions are still vertical or horizontal that is orthogonal to the boundary. In this case, we use the following equations:

0 =0p =0q = Clip(-C0', C0', (12(q0-p0) + 3(q1-p1) +16)>>5),

where C0' can be found in Table 3 and its related C0 can be found in Table 2.

When BT is 3, the filtering directions are along the intra prediction directions. In this case, we use the following equations:

0p = Clip(-C0', C0', (12(VirtualPix(p0, 1)-p0) + 3(VirtualPix(p0, 2)- VirtualPix(p0, -1)) +16)>>5)

0q = Clip(-C0', C0', (12(q0- VirtualPix(q0, -1)) + 3(VirtualPix(q0, 1)- VirtualPix(q0, -2)) +16)>>5)

where the C0', 1p and 1q  are specified in Table 3 depending on BT.

VirtualPix(pix,offset) can be computed as follows:

Assume that pix has a coordinate (x, y), AbsOffset = abs(offset), and SignOffset = sign(offset).

- If the boundary is vertical,

VirtualPix(pix,offset)=(RecY[y+SignOffset*pos0][x+offset]+RecY[y+SignOffset*pos1][x+offset ]) >> 1, where pos0 and pos1 can be fetched from Table 4.

- Else (i.e., the boundary is horizontal),

VirtualPix(pix,offset)=(RecY[y+offset][x+SignOffset*pos0]+RecY[y+offset][x+SignOffset*pos1]) >> 1, where pos0 and pos1 are fetched from Table 5.

Table 4. Filter Position (Vertical)

| Offset | 0 |  | 1 |  | 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Pos Mode | 0 | 1 | 0 | 1 | 0 | 1 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| 3 | 0 | 0 | -1 | -1 | -2 | -2 |
| 4 | 0 | 0 | 1 | 1 | 2 | 2 |
| 5 | 0 | 0 | 2 | 2 | 4 | 4 |
| 6 | 0 | 0 | 0 | 1 | 1 | 1 |
| 7 | 0 | 0 | -2 | -2 | -4 | -4 |
| 8 | 0 | 0 | 0 | -1 | -1 | -1 |

Table 5. Filter Position (Horizontal)

| Offset | 0 |  | 1 |  | 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Pos Mode | 0 | 1 | 0 | 1 | 0 | 1 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| 3 | 0 | 0 | -1 | -1 | -2 | -2 |
| 4 | 0 | 0 | 1 | 1 | 2 | 2 |
| 5 | 0 | 0 | 0 | 1 | 1 | 1 |
| 6 | 0 | 0 | 2 | 2 | 4 | 4 |
| 7 | 0 | 0 | 0 | -1 | -1 | -1 |
| 8 | 0 | 0 | -2 | -2 | -4 | -4 |

### Chroma filter

The chroma filter is only applied for the intra boundaries, i.e. the PUs on both sides of the edges are intra coded. We modified the filter coefficients as follows:

 = Clip(-C0, C0, (12(q0 - p0) + 3(p1 - q1) + 16)  >> 5)

p0 = Clip0-255(p0 + )

q0 = Clip0-255(q0 - )

# Experimental Results

## Objective quality

In order to verify the performance of our proposed IDF, we have run simulations according to the test conditions of JCTVC-C500 [3]. Table 6 shows the experimental results of IDF. It can be seen that IDF can achieve additional gain compared with HM1.0. For high efficiency configures, BD-rate reductions of 1.3%, 1.0% and 1.6% for intra, random access and low delay can be observed respectively. For low complexity configurations, BD-rate reductions of 1.5%, 0.8% and 1.6% for these three cases can be observed respectively.

Table 6. Experimental results for IDF compared with anchor of HM1.0

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.2 | -0.7 | -0.6 | -2.1 | -0.5 | -0.4 |
| Class B | -1.4 | -0.9 | -0.6 | -1.5 | -0.9 | -0.7 |
| Class C | -1.1 | -1.1 | -0.9 | -1.3 | -0.9 | -0.7 |
| Class D | -1.0 | -1.0 | -0.9 | -1.2 | -0.7 | -0.5 |
| Class E | -1.5 | 0.2 | 0.1 | -1.7 | 0.5 | 0.5 |
| All | -1.3 | -0.7 | -0.6 | -1.5 | -0.6 | -0.4 |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.7 | -0.4 | -0.4 | -1.2 | -0.8 | -0.7 |
| Class B | -1.1 | -0.7 | -0.9 | -0.9 | -1.0 | -1.0 |
| Class C | -0.9 | -0.8 | -0.8 | -0.7 | -1.1 | -1.1 |
| Class D | -0.7 | -0.5 | -0.4 | -0.5 | -1.1 | -0.7 |
| Class E |  |  |  |  |  |  |
| All | -1.0 | -0.6 | -0.6 | -0.8 | -1.0 | -0.9 |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.5 | -0.3 | -0.2 | -1.6 | -0.3 | -0.3 |
| Class C | -1.3 | 0.0 | -0.1 | -1.3 | -0.4 | -0.1 |
| Class D | -0.9 | -0.6 | 0.1 | -1.2 | 0.0 | -0.1 |
| Class E | -2.9 | -0.6 | -0.1 | -2.3 | -0.1 | -0.5 |
| All | -1.6 | -0.4 | -0.1 | -1.6 | -0.2 | -0.2 |

## Subjective quality

Fig. 2. Visual quality of Kimono1 high efficiency random access with QP27 (Left: IDF, Right: TEN)

The subjective quality differences between IDF and TEN in HM1.0 are very small in many cases. But we can still observe some visible improvements by using IDF. Fig. 2 shows the visual quality of IDF compared with TEN in HM1.0. This is the 23th frame in Kimono1 for high efficiency random access with QP27. The left image is the result of IDF and the right image is the result of TEN in HM1.0.

From the images, we can see a big difference in the eye of the woman. The eye should be opened according to the original image.	 The TEN result was distorted as if the eye were closed.

## Encoding time and decoding time

The encoding time, decoding time and deblocking filter time of IDF compared to HM1.0 for all the 6 configurations are shown in table 7. From the table, we can see that the average encoding time increase and the average decoding time increase are both 1.1%. And the average time increase for deblocking filter is only about 7%.

Table 7. Running time of IDF compared with HM1.0

|  | Intra | Intra LoCo |
| --- | --- | --- |
| Enc Time[%] | 99% | 99% |
| Dec Time[%] | 100% | 104% |
| DF Time[%] | 115% | 126% |
|  | Random access | Random access LoCo |
| Enc Time[%] | 104% | 103% |
| Dec Time[%] | 100% | 102% |
| DF Time[%] | 91% | 111% |
|  | Low delay | Lowdelay LoCo |
| Enc Time[%] | 100% | 102% |
| Dec Time[%] | 100% | 101% |
| DF Time[%] | 90% | 108% |

# Conclusion

This contribution described MediaTek’s IDF proposal. Compared with HM1.0, IDF outperforms the anchor in all the six test configurations. Average BD-rate reductions of 1.3%, 1.5%, 1.0%, 0.8%, 1.6% and 1.6% can be observed for intra, intra LoCo, random access, random access LoCo, low delay and low delay LoCo, respectively. The coding time of IDF is almost the same with HM1.0. About 1.1% increase is observed for both encoding time and decoding time.

# References

- K. Ugur, K. R. Andersson and A. Fulseth, “Video coding technology proposal by Tandberg, Nokia and Ericsson,” JCTVC-A119, Dresden, April 2010.

- J. An, K. Zhang, Y. Gao, X. Guo, Y. Huang and S. Lei, “TE10 Subtest1: Improved Deblocking Filter, ” Document of Joint Collaborative Team on Video Coding, JCT-VC-C142, Guangzhou, Oct. 2010.

- Doc. JCTVC-C500, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, China, October 2010.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).