| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D164 WG11 Number: m18916 |
| --- | --- |

| Title: | Temporal MV predictor modification for MV-Comp, Skip, Direct and Merge schemes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Jung, G. Clare Orange Labs 38-40 rue du Général Leclerc 9279 ISSY LES MOULINEAUX | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange-ftgroup.com gordon.clare@orange-ftgroup.com |
| Source: | Orange Labs |  |  |

_____________________________

# Introduction

The current version of the HM includes temporal motion vector prediction through the Inter, the Skip and the Direct modes. The temporal predictor is mapping the origin of the block to a collocated frame. In this contribution, 2 different temporal predictors are proposed. The first one is based on the computation of the median of the motion vectors of the collocated block, while the second one maps the center of the blocks to the collocated frame. For the latter, an average gain for each configuration with negligible complexity increase and no syntax change is reported.

# Improved temporal predictors

The standard set of predictors used for the Inter, the Skip and the Direct modes is the following:

- M (median of A, B, X)

- A (1st left neighbor found)

- B (1st top neighbor found)

- X (a corner, 1st of C, E, D neighbors)

- T (a temporal, found by mapping the origin of the block to a collocated frame)

Two modified temporal predictors are proposed to improve this set.

## Median of collocated MVs

### Description

Two modifications are proposed to improve the set of predictors. First, the temporal predictor is moved to the first position in the set, as suggested in JCTVC-C291. Second, the temporal predictor is modified. It is found by mapping the current partition being treated to a collocated frame and examining all vectors there, instead of mapping just the origin of the block.

The corresponding set of predictors becomes:

- Tmed (a temporal, found by mapping the current partition to the collocated frame, and performing a median of all vectors present)

- M (median of A, B, X)

- A (1st left neighbor found)

- B (1st top neighbor found)

- X (a corner, 1st of C, E, D neighbors)

Each 4x4 block in the collocated frame that intersects the mapped partition is examined in order to collect its motion information.  Each 4x4 block may contain 0, 1 or 2 motion vectors.  If two vectors are present, the vector that is chosen is one that:

- ‘Crosses’ the current frame.

- If both vectors cross the current frame, or both don’t cross, the shortest (in terms of reference distance) is taken.

Each of these selected vectors is scaled to account for different reference distances and collected into a set.  The vectors in this set are then sorted in Y, then X and the vector at the center of this ordered set is taken to be the ‘median’ predictor.

### Experimental results

Reference: HM0.9, with AMVP based Skip and Direct, and CU Merge. Table 1 shows a gain for each configuration, from 0.9% for RAHE to 0.1% for LDLC. In particular, about 1.5% gain is achieved on classA sequences.

|  | Random Access | Random Access - LC | Low Delay | Low Delay - LC |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | Y BD-rate | Y BD-rate | Y BD-rate |
| ClassA | -1,6 | -1,4 |  |  |
| ClassB | -0,7 | -0,5 | -0,2 | -0,1 |
| ClassC | -0,8 | -0,5 | -0,2 | -0,1 |
| ClassD | -0,7 | -0,5 | -0,2 | -0,2 |
| ClassE |  |  | -0,9 | 0,0 |
| AVG | -0,9 | -0,6 | -0,3 | -0,1 |

Table 1: average gain of ‘Tmed’. Reference: CU merge non robust, HM0.9 default configuration, positive values indicate a loss.

Complexity: the fabrication of this ‘Tmed’ predictor is more complex than the original temporal predictor.  The complexity arrives from:

- The collection of scaled vectors from all intersecting 4x4 blocks in the collocated frame, rather than using just a single vector.

- The sorting of this set, in order to find a ‘median’ representative value.

## Centered temporal predictor

### Description

Two modifications are proposed to improve the set of predictors. First, the temporal predictor is moved to the first position in the set, as suggested in JCTVC-C291. Second, the temporal predictor is modified. It is found by mapping the center of the block to a collocated frame, instead of the origin of the block.

The corresponding set of predictors becomes:

- Tctr (a temporal, found by mapping the center of the block to a collocated frame)

- M (median of A, B, X)

- A (1st left neighbor found)

- B (1st top neighbor found)

- X (a corner, 1st of C, E, D neighbors)

Figure 1: Location of the center for 3 kind of partitioning of a 32x32 CU.

- The Figure 1 represents three 32x32 CUs, with three different partitionings, i.e 32x32, 32x16, 16x16. The red dot represents the 4x4 block considered as the center of the considered partition. The Tctr temporal motion vector predictor is derived from the motion vector of the corresponding 4x4 block in the collocated frame.  This collocated 4x4 block contains a copy of the motion information of the enclosing collocated partition.

- In addition, once the collocated motion information has been found, there may be more than one vector present if the collocated partition is bi-predicted.  In that case, the collocated vector chosen is one that ‘crosses’ the current frame. If both vectors cross the current frame, or both don’t cross, the shortest (in terms of reference distance) is taken.

This selected vector is then appropriately scaled to take into account different reference distances between the collocated frame (and its reference) and the current frame (and its reference).  This scaled vector then becomes the collocated predictor.

There is negligible complexity difference between the construction of this collocated predictor and the original.

### Experimental results

The Table 2 shows the gains obtained by using the proposed set of predictors. Table 2 shows a gain from 0.8% for RAHE to 0% for LDLC. In particular, about 1.4% gain is achieved on classA sequences for RAHE.

A sequence by sequence analysis of the results shows in Figure 2 that for each configuration, the gains are consistent. Losses are observed for Vidyo sequences for LD configurations. For some sequences, gains larger than 1% are observed. We observe that the gains provided by Tctr are really close to the gains provided by Tmed.

|  | Random Access | Random Access - LC | Low Delay | Low Delay – LC |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | Y BD-rate | Y BD-rate | Y BD-rate |
| ClassA | -1,4 | -1,1 |  |  |
| ClassB | -0,7 | -0,4 | -0,1 | 0,0 |
| ClassC | -0,6 | -0,4 | -0,1 | 0,0 |
| ClassD | -0,6 | -0,5 | -0,1 | -0,1 |
| ClassE |  |  | -0,6 | 0,3 |
| AVG | -0,8 | -0,5 | -0,2 | 0,0 |

Table 2: gains using Tctr centered temporal predictor. Reference: CU merge non robust, HM0.9 default configuration, positive values indicate a loss.

Figure 2: sequence by sequence analysis of the gain using the centered temporal predictor.

### Experimental results with CE9 3.1.e configuration

Table 3 reminds the gains obtained by the configuration 3.1.e against the current HM0.9 reference. Table 3 shows that the centered temporal predictor further improves this configuration. This gains provided by this predictor are not specific to one or another configuration. It helps in any configuration.

|  | Random Access | Random Access - LC | Low Delay | Low Delay - LC |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | Y BD-rate | Y BD-rate | Y BD-rate |
| ClassA | -0,9 | -2,0 | - | - |
| ClassB | -0,8 | -1,4 | 0,0 | -0,5 |
| ClassC | -0,7 | -1,8 | -0,5 | -1,5 |
| ClassD | -1,1 | -2,0 | -0,7 | -1,9 |
| ClassE | - | - | -0,5 | 0,6 |
| AVG | -0,9 | -1,8 | -0,4 | -0,9 |

Table 3: gains obtained using configuration 3.1.e. Reference: CU merge non robust, HM0.9 default configuration, positive values indicate a loss.

|  | Random Access | Random Access - LC | Low Delay | Low Delay - LC |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | Y BD-rate | Y BD-rate | Y BD-rate |
| ClassA | -1,8 | -2,8 | - | - |
| ClassB | -1,1 | -1,7 | -0,1 | -0,6 |
| ClassC | -1,1 | -2,2 | -0,6 | -1,5 |
| ClassD | -1,5 | -2,3 | -0,7 | -2,0 |
| ClassE | - | - | -0,4 | 0,6 |
| AVG | -1,3 | -2,2 | -0,4 | -1,0 |

Table 4: gains obtained using configuration 3.1.e with a centered temporal predictor. Reference: CU merge non robust, HM0.9 default configuration, positive values indicate a loss.

# Syntax and Semantics

No syntax change.

# Conclusion

Two modified temporal predictors have been proposed. The first one, Tmed, shows a gain for each configuration, from 0.9% for RAHE to 0.1% for LDLC. The second one, Tctr, shows nearly similar gains, yet with no impact on the complexity compared to the current temporal predictor. Tctr has been tested with the 3.1.e configuration from CE9, and improves the gain of this configuration. It is consequently not dedicated to a specific configuration but intended to help any configuration that has a temporal predictor. In addition, using Tctr does not change the syntax of the standard draft.

For all these reasons, we recommend adopting Tctr has the new temporal predictor, whatever the mode (Inter, Skip, Direct, or Merge).

# Patent rights declaration(s)

Orange Labs may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).