| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, Korea, 20-28 January, 2011 | Document: JCTVC-D165 WG11 Number: m18917 |
| --- | --- |

| Title: | CU level directional merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | *Jicheng An, *Xun Guo, #Yu-Wen Huang and #Shawmin Lei *North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 #No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Contact: Shawmin Lei +886-3-567-0766 ext. 25555 shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, a directional merge method is proposed to improve the CU-level merge mode [1] in HM1.0. The syntaxes of two merge methods are exactly the same. We only change their semantics to utilize the motion information from more neighboring blocks to increase the chance of sharing motion parameters from its neighboring blocks. Compared to the anchor of HM1.0, average bitrate reductions of 0.2%, 0.3%, 0.4 and 0.5% can be observed for random access, random access LoCo, low delay and low delay LoCo, respectively. If compared to HM1.0 with merge mode off, the proposed method can achieve bitrate reductions of 0.8%, 0.2%, 1.2 and 1.6% can be observed for random access, random access LoCo, low delay and low delay LoCo, respectively. There is essentially no increase in both encoding time and decoding time for the proposed method.

# Introduction

In HM1.0, the CU-level merge mode enables the current coding unit (CU) to share the same motion parameters from its upper block or left block. The upper block is the upper neighbor of the upper-left sample in the current CU, and the left block is the left neighbor of the upper-left sample in the current CU, as shown in Fig. 1. The motion parameters of the two blocks, A and B, are both tried by the current CU. The one with less rate-distortion (RD) cost is chosen to be used by the current CU. For each CU, a MergeFlag is transmitted to indicate whether merge mode is used or not. If the merge mode is used (MergeFlag=1), one additional flag, MergeIndex, is further transmitted to indicate the motion parameters of the upper block or the left block is used by the current CU.

Fig. 1, The left neighbor A and the upper neighbor B of the current CU C.

# Algorithm description

Since in some cases, the pixels within one CU may not belong to the same object, we propose to utilize the motion information from more neighboring blocks to increase the chance of sharing motion parameters. Furthermore, analogous to directional intra prediction, we propose directional merge method which uses only two major directions: horizontal direction and vertical direction in this proposal. The syntaxes of the directional merge methods are exactly the same as those of the CU-level merge mode in HM1.0. We only change their semantics Fig. 2 shows the prediction process of directional merge method. In the figure, grey blocks are the smallest unit (SU, 4x4 block in HM1.0) in the current CU; the white blocks labeled with Ai and Bi are the neighboring SUs. The directional merge method consists of two merge modes: horizontal merge and vertical merge. For horizontal merge (MergeIndex=Left), the SUs in the same row in the current CU share the motion parameters of their left neighbors as shown in Fig. 2(a). For example, the SUs in the same row with A0 will use the motion parameters of A0. For vertical merge (MergeIndex=Upper), the SUs in the same column in the current CU share the motion parameters of their upper neighbors as shown in Fig. 2(b). For example, the SUs in the same column with B0 will share the motion parameters of B0.

(a)                              (b)

Fig. 2 proposed directional merge method. (a) horizontal merge, (b) vertical merge.

The mode selection process is very similar to the current merge mode, in which the direction with less RD cost is chosen as the final direction. For each CU, the MergeFlag is transmitted to indicate whether the directional merge mode is used or not when one of the neighboring SUs has motion parameters. If the directional merge mode is used (MergeFlag=1), one additional flag (MergeIndex) is transmitted to indicate which direction is used when the two directions are both available. If none of the neighboring SUs in one direction has motion parameter (intra coded or not exist), this direction is unavailable for merge.

# Compression performance discussion

In order to verify the performance of the proposed algorithm, we have run simulations according to the common test conditions defined in JCTVC-C500 [2]. Table 1 shows the experimental results of directional merge compared with anchor of HM1.0. From the results, we can see that the proposal method can achieve coding gain in all cases for both Luma and Chroma. And an average bitrate reduction of 0.3% can be observed. The encoding time and decoding time are almost the same with HM1.0. Table 2 shows the experimental results of directional merge compared with HM1.0 with CU-level merge off. This result can give an entire performance evaluation for CU-level directional merge. The results of Table 1 have been verified by HHI.

Table 1. Experimental results of directional merge

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | -0.4 | -0.3 | -0.3 | -0.5 | -0.4 |
| Class B | -0.2 | -0.3 | -0.5 | -0.3 | -0.4 | -0.5 |
| Class C | -0.2 | -0.4 | -0.4 | -0.3 | -0.4 | -0.5 |
| Class D | -0.2 | -0.2 | -0.2 | -0.2 | -0.4 | -0.3 |
| Class E |  |  |  |  |  |  |
| All | -0.2 | -0.3 | -0.3 | -0.3 | -0.4 | -0.4 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | -0.4 | -0.6 | -0.4 | -0.5 | -0.5 |
| Class C | -0.3 | -0.3 | -0.4 | -0.2 | -0.5 | -0.3 |
| Class D | -0.3 | -0.7 | -0.5 | -0.3 | -0.4 | -0.3 |
| Class E | -0.8 | -1.0 | -1.3 | -1.2 | -1.2 | -1.4 |
| All | -0.4 | -0.6 | -0.7 | -0.5 | -0.6 | -0.6 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 2. Experimental results of directional merge compared with HM1.0 with CU-level merge off

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.9 | -1.0 | -1.0 | 0.0 | -0.3 | -0.4 |
| Class B | -1.1 | -0.9 | -1.0 | -0.5 | -0.3 | -0.1 |
| Class C | -0.7 | -0.8 | -0.8 | -0.2 | -0.4 | -0.4 |
| Class D | -0.5 | -0.8 | -0.7 | 0.0 | -0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | -0.8 | -0.9 | -0.8 | -0.2 | -0.3 | -0.2 |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.2 | -1.1 | -1.0 | -1.7 | -1.1 | -0.9 |
| Class C | -1.0 | -0.8 | -0.9 | -1.2 | -0.8 | -0.8 |
| Class D | -1.1 | -0.5 | -1.0 | -1.6 | -0.8 | -0.8 |
| Class E | -1.4 | -1.4 | -2.3 | -2.2 | -1.5 | -1.7 |
| All | -1.2 | -1.0 | -1.2 | -1.6 | -1.0 | -1.0 |
| Enc Time[%] | 105% |  |  | 102% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |

# Conclusion

This contribution described a directional merge method, which can improve the coding performance of the current CU-level merge mode in HM1.0. In this method, the correlation and consistency of the motion between neighboring blocks are further explored to improve the motion prediction efficiency. Compared to the anchor of HM1.0, average bitrate reductions of 0.2%, 0.3%, 0.4 and 0.5% can be observed for random access, random access LoCo, low delay and low delay LoCo, respectively. There is essentially no increase of encoding time and decoding time for the proposed method.

# References

- Martin Winken, Sebastian Boße, Benjamin Bross and et al., “Description of video coding technology proposal by Fraunhofer HHI” JCTVC-A116, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Dresden, DE, 15-23 April, 2010.

- Doc. JCTVC-C500, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, China, October 2010.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).