| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, Korea, 20-28 January, 2011 | Document: JCTVC-D166 WG11 Number: m18918 |
| --- | --- |

| Title: | Improved Intra Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | *Mei Guo, *Xun Guo, and #Shawmin Lei *North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 #No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Contact: Xun Guo +86-10-82800818 ext. 83347 xun.guo@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes an improved intra mode coding method, which includes two major parts: improved intra mode prediction (IIMP) and context dependent intra mode coding (CDIMC). In IIMP, the most probable mode of luma block is adaptively selected from the modes of two neighboring blocks instead of directly using the one with smaller mode index in HM1.0 [1]. The prediction modes of chroma are also re-ordered, so that the mode which is equal to the corresponding luma mode has the shortest codeword. In CDIMC, the coding tree for the current block is adaptively selected from a set of pre-defined coding trees according to the modes of its neighboring blocks. Experimental results show that the proposed method can achieve an average bit-rate reduction of 1.6% and 2.0% for high efficiency all-intra (HE-AI) and low complexity all-intra (LC-AI), respectively. The encoding time increases are 1% and 3% for HE-AI and LC-AI, respectively. The decoding time increases are also 1% and 3% for HE-AI and LC-AI, respectively.

# Introduction

For luma mode coding in HM1.0, a predictive coding method is used to improve the coding efficiency. The smaller one of the intra modes from the left prediction unit (PU) and the upper PU is selected as the most probable mode of the current PU. One PU-level flag, MostProbableMode, is transmitted to indicate whether the current mode is equal to the most probable mode or not. If not (MostProbableMode=0), the current mode is then transmitted using a fixed length coding method. As for chroma, totally five prediction modes are used including four fixed modes and the best mode of corresponding luma PU. A variable length coding table is used to code these modes. In this table, the mode “0” (i.e. vertical mode) has the shortest codeword and the best luma mode has the longest codeword.

In this proposal, in order to fully utilize the correlations among neighboring PUs and the correlations between luma and chroma, we use IIMP to improve mode prediction for both luma and chroma. For luma, the most probable mode is adaptively selected. For chroma, the best mode from luma is assigned the highest probability and has the shortest codeword. Furthermore, in order to code the luma modes more efficiently, we use CDIMC to allow switching among different coding trees according to the modes of neighbors The rest of this contribution is organized as follows. Section 2 describes the proposed method. Section3 provides experimental results. Finally, Section 4 draws a conclusion.

# Algorithm description

## Improved intra mode prediction (IIMP)

The proposed IIMP includes two major improvements: improved most probable mode for luma and mode re-order for chroma, which are described as the following subsections.

### Improved most probable mode

Fig. 1 Illustration of neighboring PUs to the left and to the top of the current PU

When the intra mode of current PU (ModeC) is being coded, the mode of left PU (ModeA) and the mode of upper PU (ModeB), as depicted in Fig. 1, are used to generate the most probable mode of ModeC. Fig. 2 Illustrates the coding flow. Besides the current MostProbableMode, we added an additional flag, MostProbableIndex, after MostProbableMode to indicate which one of ModeA and ModeB is chosen as the most probable mode. When ModeA or ModeB is equal to ModeC, MostProbableMode is 1; otherwise, MostProbableMode is 0. When MostProbableMode is 1, MostProbableIndex is then transmitted. MostProbableIndex can be 0 or 1, which represents the smaller one and the larger one of ModeA and ModeB, respectively.

Fig. 2 Coding flow of IIMP for luma

When ModeA or ModeB is unavailable, ModeA or ModeB is set as DC mode. When ModeA and ModeB are both unavailable, or ModeA is identical to ModeB, the most probable mode is set as DC or ModeA (Mode B), respectively, and then is used to do intra mode prediction. In this case, MostProbableIndex needs not to be transmitted.

### Mode re-order for chroma

When the chroma mode of current PU is coded, the intra mode of luma component (ModeL) is considered as the most probable mode and is assigned to the shortest codeword in the proposed IIMP. In HM1.0 [1], if the index of ModeL is less than 4, there are only four candidate modes involved in the mode decision process of chroma component, namely mode “0”, “1”, “2”, and “3”. If the index of ModeL is equal to or larger than 4, ModeL appears as an additional candidate mode corresponding to the mode index “4”. Besides, no matter whether there are four or five candidate modes available, the mode “3” is always coded with “1110”. Obviously, it is unnecessary when there are only four candidates.

In the proposed IIMP, we first propose to grant the shortest codeword “0” to the mode which is identical to ModeL. To be specific, if ModeL is larger than or equal to 4, mode “4” always owns the shortest codeword; otherwise, the one that appears the same as ModeL has the shortest codeword. We also remove that unnecessary bit “0” from “1110” when only four options are accessible. The codewords for chroma mode used in IIMP are listed in Table 1.

Table 1. Codewords for chroma mode coding (LM:luma mode,CM:chroma mode)

| CM LM | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 10 | 10 | 10 | 10 |
| 1 | 10 | 0 | 111 | 111 | 110 |
| 2 | 111 | 111 | 0 | 110 | 1111 |
| 3 | 110 | 110 | 110 | 0 | 1110 |
| 4 | ---------- | ---------- | --------- | --------- | 0 |

## Context dependent intra mode coding (CDIMC)

We have proposed a Context Dependent Mode Representation (CDMR) method to code the Intra_16x16, Intra_8x8, and Intra_4x4 prediction modes in JCTVC-A109 [3]. In this contribution, we propose to apply that method to the intra mode coding of 4x4, 8x8, 16x16 and 32x32 PUs.

We define the intra modes of the neighboring PUs to the left and to the top of the current PU (A and B) as the ‘context’. This is based on the observation that the conditional distribution given the modes of A and B is usually uneven. Fig. 3 gives an example for 8x8 PUs with the neighboring modes ‘0’ and ‘2’. As depicted in Fig. 1, the context can be denoted as (MA, MB). If PU A or B is unavailable, MA or MB is set to ‘2’, i.e. DC mode. All the blocks (PU) are classified into two categories according to PU size. One includes 4x4 PUs, while the other one contains PUs with other sizes. For each category, two tables, denoted as , and , and a parameter, denoted as  , are maintained for each individual context (MA, MB). The length of bothand  is equal to the number of valid prediction modes in the context (MA, MB). MT records the modes in a descending order of occurrence frequency, PT records the position of each mode in MT and C selects the coding trees. In other words, mode MT[i] takes the ith largest occurrence frequency and the position of MT[i], namely i,, is recorded as PT[MT[i]]. When mode MT[i] is transmitted, i is the codeword. There are 400 candidate coding trees T0, T1,…, T399 , and Tk is used to code the word i where k = . For a 4x4 PU, we take context (1, 0) as an example. The selected coding tree is shown in Fig. 4, in which the solid line indicates one coded bit “0” and the dotted line indicates one coded bit “1”. For example, mode “16” is coded with “011”. It can be observed from Fig. 4 that given context (1, 0), the mode “1” “0” and some intra modes having directions close to them, such as “16”,”15”,”11”, and ”12”, have much larger probabilities of occurrence and thus shorter codewords, compared to others.

Fig. 3. The conditional distribution of intra modes for 8x8 PUs given two neighboring modes “0” and “2”.

Fig.4. The coding tree of context (1, 0) for 4x4 PU. (solid line: one coded bit “0”; dotted line: one coded bit “1”).

## Combination of IIMP and CDIMC

In this contribution, we also proposed to combine IIMP and CDIMC to achieve more promising coding performance as illustrated in Fig.5. At first, IIMP is utilized to predict the intra modes of current block. If the intra mode of luma is different from either neighboring mode, CDIMC is then applied to compress the intra mode with adaptive coding trees, as given in Table 2. Thus, CDIMC produces the coding trees only based on the probability distribution of intra modes except ModeA and ModeB. In the combination of IIMP and CDIMC, there only 284 coding trees are needed.

Fig.5. Illustration of the combination of IIMP and CDIMC

Table 2: Coding method in the combination of IIMP and CDIMC.

| Intra Mode Indices | Codewords |
| --- | --- |
| Min(ModeA, ModeB) | 1 0 |
| Max(ModeA, ModeB) | 1 1 |
| else | 0 + CDIMC codeword |

# Simulation results

Our software is based on TMuC 0.9. Our simulations follow the recommended test condition in JCTVC-C500 [2]. Table 3~Table 5 show the coding performance and running time of IIMP, CDIMC, and the combination of IIMP and CDIMC, respectively. The bit-rate reduction of luma and chrome components, in terms of BD-Rate for all intra (high efficiency and low complexity) is used to evaluate the coding performance. From the tables, we can see that both of the two algorithms have coding gain for luma and chroma. When combined together, an additional coding gain can be achieved. As illustrated in these tables, there are only minor increases of encoding time and decoding.

Table 3: Coding performance and complexity of IIMP.

Table 4: Coding performance and complexity of CDIMC.

Table 5: Coding performance and complexity of the combination of IIMP and CDIMC

# Conclusion

This contribution proposed an improved intra mode coding method, in which the correlation among neighboring intra modes and the correlation between luma and chroma are further exploited. Experimental results show that the proposed method can achieve the average bit-rate reduction of 2.0% and 1.6% for intra low complexity and high efficiency, respectively. The increases of encoding time and decoding time are quite small.

# References

- T. Wiegand, W.-J. Han, J.-R. Ohm, G. J. Sullivan (editors), “High Efficiency Video Coding (HEVC) text specification Working Draft 1”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C403, 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

- F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C500, 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

- Y. Huang, C. Fu. Y. Pao, J. Lin, Y. Chang, J. Guo, C. Chen, S. Lei, X. Guo, Y. Gao, K. Zhang, and A. Ji, “A Technical Description of MediaTek’s Proposal to the JCT-VC CfP”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 1st Meeting: Dresden, DE, 15-23 April, 2010.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).