| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D167 WG11 Number: m18919 |
| --- | --- |

| Title: | CE1: Report of self derivation of motion estimation in TMuC 0.9 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 lidong.xu@intel.com |
|  | Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171657 wenhao.zhang@intel.com |
|  | Hong Jiang 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 916-356-0457 hong.h.jiang@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution provides performance result and description for CE1 topic of Self Derivation of Motion Estimation (SDME) techniques. Two presented versions of SDME from Guangzhou meeting, rounded-candidate based SDME without motion vector (MV) refinement and rounded-candidate based SDME with MV refinement, are ported onto TMuC 0.9 reference software.  The version of rounded-candidate based SDME with MV refinement has demonstrated that an overall 2.8% BD bitrate improvement for the test of random access (high efficiency) category under the common test conditions with 35% increase in encoding time and 21% increase in decoding time increase. The version of rounded-candidate based SDME without MV refinement has demonstrated an overall 1.5% BD bitrate improvement for the test of random access (high efficiency) category with 24% increase in encoding time and 5% increase in decoding time. (An improved software version based on 9-candidate motion vector pairs of rounded-candidate based SDME without MV refinement has demonstrated an overall 1.7% BD bitrate improvement for the test of random access (high efficiency) category with 23% increase in encoding time and 4% increase in decoding time.) A description about the normative syntax change about SDME in the form of HEVC specification is included.

# Introduction

Various decoder-side motion vector derivation (DMVD) techniques have been proposed to improve the coding efficiency of hybrid based video encoder [1-6]. In the response to the HEVC CFP, a mirror-based self derivation of motion estimation (SDME) technique was presented to improve the coding efficiency of the bi-prediction modes of B pictures [7]. The fast techniques of candidate-based motion vector prediction were proposed to improve the mirror-based SDME in [8-9]. This contribution reports the improvement of the rounded-candidates based SDME technique in TMuC [9]. The following of this contribution has Section 2 to provide an introduction of the proposed SDME versions, and Section 3 to describe the normative syntax change in the form of TMuC specification. The performance data of the coding efficiency and coding complexity of the proposed scheme are contained in Section 4. Section 5 concludes this contribution. Section 6 contains additional results.

# Proposed Techniques

The candidate-based SDME is proposed to improve the coding efficiency of B picture for the next generation of video coding standard [7-8]. Figure 1 shows the block diagram of a typical hybrid-based video encoder with SDME module being added. The SDME module performs motion search only on reference pictures, and the produced motion vector could be applied to current target prediction unit (PU) for motion compensation when the best RDO performance is achieved with SDME mode during the phase of coding mode selection. On the video decoder side, the decoder performs the identical SDME operation to produce the motion compensated predictions for the target PU when the said target PU is encoded in SDME mode.

Figure 1 - Block diagram of hybrid video encoding with SDME being supported

## Rounded-candidate based SDME without MV refinement

The candidate-based SDME module first checks 9 candidate motion vector pairs, and then perform a small range fractional pixel refinement ME around the best candidate motion vector pair [7]. To further reduce the coding complexity, the number of candidates was reduced from 9 to 6 in [9]. The candidates were rounded to integer pixel positions to avoid fractional pixel interpolation in SDME process [9]. The 6 candidates are selected from the MVs of spatial and temporal neighboring PUs, as depicted in Table 1 and Figure 2. (The description of spatial MV candidate predictors, median MV predictor, and the temporal MV predictor can be found in Sections 8.4.2.1.5, 8.2.4.1.6 and 8.4.2.1.7 of [10]) All candidate MVs are scaled (by POC distances) to the nearest forward and backward reference pictures. Each candidate consists of one forward MV and one backward MV. Since the proposed SDME is performed between the two reference pictures, the decoding process does not have dependency on neighbor PU reconstructions.

Figure 2 – Candidate MVs from spatial neighboring PUs

Table 1 - Description of the candidates

| Candidates | Idx | Description |
| --- | --- | --- |
| Base MV | 0 | the zero MVs |
| Spatial neighbor MVs scaled by POC distances | 1 | MVs of left neighbor PU A: a. If it is bi-predicted, use the two MVs b. If it is FW predicted, get the BW MV by mirroring the FW MV c. If it is BW predicted, get the FW MV by mirroring the BW MV d. If it is intra coded, this candidate will be unavailable |
|  | 2 | MVs of top neighbor PU B a. If it is bi-predicted, use the two MVs b. If it is FW predicted, get the BW MV by mirroring the FW MV c. If it is BW predicted, get the FW MV by mirroring the BW MV d. If it is intra coded, this candidate will be unavailable |
|  | 3 | MVs of top-right neighbor PU C (as described in 1) - If PU C is not available, use that of bottom left PU E - If PU E is also not available, use that of top-left PU D - If PU D is also not available, this candidate will be unavailable |
|  | 4 | Median filtered MVs of the above three candidates 1, 2 and 3 |
| Temporal neighbor MVs scaled by POC distances | 5 | MVs of collocated neighbor PU in BW reference a. If it is bi-predicted, use the two MVs b. If it is FW predicted, get the BW MV by mirroring the FW MV c. If it is BW predicted, get the FW MV by mirroring the BW MV d. If it is intra coded, this candidate will be unavailable |

In the candidate rounding process, multiple candidates may be rounded to an identical integer pixel position. The candidate with the minimum SAD will be selected, and its corresponding the non-rounded MV value is used as the output MV pair of rounded-candidates based SDME process. When SDME refinement is disabled, this output MV pair will be directly applied for motion compensation process for rounded candidate-based SDME. For SDME MV refinement mode, this output MV pair will be used as the starting point of MV refining process described below. The simulation data of an improved software based on the 9 candidate motion vector pairs in [8] has been provided in the Table 9 in Section 6.

## Rounded-candidate based SDME with MV refinement

SDME refinement is to perform the fine-grained MV refinement. A two-step 8-point MV refinement process will be applied after the location of the rounded candidates is determined. Consider the output MV pair, which will be used as the starting point of refining process, of rounded-candidates based SDME to be denoted as (mvf_x, mvf_y) and (mvb_x, mvb_y). (mvf_x, mvf_y) is forward MV in quarter pixel precision and (mvb_x, mvb_y) is backward MV in quarter pixel precision. Then the refinement process follows the below steps.

- Refining with half-pixel step size: Check the SADs of two reference blocks pointed by (mvf_x+dx, mvf_y+dy) and (mvb_x-dx, mvb_y-dy), where (dx, dy)=(0,0), (0,2), (2,0), (0,-2), and (-2,0). Denote (dx_m, dy_m) as the position to produce the minimum SAD. The output of MV for the half-pixel refinement  (mvf_x’, mvf_y’) = (mvf_x+dx_m, mvf_y+dy_m) and (mvb_x’, mvb_y’) = (mvb_x-dx_m, mvb_y-dy_m).

- Refining with quarter-pixel step size: Check the SADs of two reference blocks pointed by (mvf_x’+dx’, mvf_y’+dy’) and (mvb_x’-dx’, mvb_y’-dy’), where (dx’, dy’)=(0,1), (1,0), (0,-1), and (-1,0). Denote (dx’_m, dy’_m) as the position to produce the minimum SAD. The output of MV for the quarter-pixel refinement  (mvf_x*, mvf_y*) = (mvf_x’-dx’_m, mvf_y’-dy’_m) and (mvb_x*, mvb_y*) = (mvb_x’-dx’_m, mvb_y’-dy’_m). (mvf_x*, mvf_y*) and (mvb_x*, mvb_y*) are the final MVs used for the motion compensation of SDME coding mode.

### Simplified interpolation filter for SDME refinement

To reduce the complexity of SDME refinement, a simplified interpolation filter is applied instead of the 12-tap filter used in TMuC0.9. Consider “X” in Figure 3 is a fractional pixel position and the values of its nearest four integer pixels are Pa, Pb, Pc, and Pd. Then, the pixel value Px of “X” is calculated by the weighted average of Pa, Pb, Pc, and Pd, and the weighting factors are decided by the position of pixel “X”.

Figure 3 – Simplified fractional pixel interpolation

A look-up table technique is implemented. Consider the pixel position of “X” is (x,y) in quarter pixel precision. An index is first calculated in Equation (1), with which the weighting factor (a, b, c, d) can be accessed from the look-up table shown in Table 2. The output pixel Px can be calculated in Equation (2).

Idx = (y%3) * 4 + (x%3)                               (1)

Px = (a*Pa+b*Pb+c*Pc+d*Pd+8) / 8                          (2)

Table 2 Weighting factors for fractional pixel interpolation

| Idx | (a, b, c, d) |
| --- | --- |
| 0 | (8, 0, 0, 0) |
| 1 | (6, 2, 0, 0) |
| 2 | (4, 4, 0, 0) |
| 3 | (2, 6, 0, 0) |
| 4 | (6, 0, 2, 0) |
| 5 | (6, 1, 1, 0) |
| 6 | (3, 3, 1, 1) |
| 7 | (1, 6, 0, 1) |
| 8 | (4, 0, 4, 0) |
| 9 | (3, 1, 3, 1) |
| 10 | (2, 2, 2, 2) |
| 11 | (1, 3, 1, 3) |
| 12 | (2, 0, 6, 0) |
| 13 | (1, 0, 6, 1) |
| 14 | (1, 1, 3, 3) |
| 15 | (0, 1, 6, 1) |

### Sampled SAD

Sampling technique is applied to reduce the computational complexity when calculating the SAD between two reference blocks, i.e., only the pixels in even columns and even rows are used for SAD calculation. Enabling the sampled SAD requires only ¼ number of fractional pixels to be interpolated, which will greatly reduce the SDME complexity.

## SDME based coding modes

The SDME based motion vector derivation method can be used as a new coding mode in addition to the existing coding modes of the regular ME based coding structure. In bitstream syntax, a SDME flag is added for each PU to signal the occurrence of SDME mode. The SDME mode is applied to the bi-prediction coding modes of all PU sizes except NxN mode in order to save complexity. For 2Nx2N PUs, SDME is applied only to Skip/Direct modes to tradeoff complexity. In this contribution, the SDME is currently only applied for the traditional B pictures. (GPB picture is not applied)

# Required modifications to HEVC text specification

To support the proposed DMVD techniques in HEVC, the following modification to HEVC text specifications [9] are required.

## Syntax

The modifications to syntax tables are marked in yellow.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| bit_depth_luma_increment | ue(v) |
| bit_depth_chroma_increment | ue(v) |
| log2_max_frame_num_minus4 | ue(v) |
| pic_order_cnt_type | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |
| delta_pic_order_always_zero_flag | u(1) |
| offset_for_non_ref_pic | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |
| offset_for_ref_frame[ i ] | se(v) |
| } |  |
| max_num_ref_frames | ue(v) |
| gaps_in_frame_num_value_allowed_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| interpolation_filter_flag | u(1) |
| BiDMVDFlag | u(1) |
| if( BiDMVDFlag ) { |  |
| BiDMVDRefineFlag | u(1) |
| BiDMVDSampledSADFlag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( BiDMVDFlag && slice_type == B && !slice_is_GPB) | u(1) ¦ ae(v) |
| skip_dmvd_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( !skip_dmvd_flag[ x0 ][ y0 ]){ |  |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } else if( PredMode = = MODE_INTRA ) { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( !prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { /* MODE_MERGE, MODE_DIRECT, MODE_INTER */ |  |
| if( merge_flag[ x0 ][ y0 ] && NumMergeCandidates > 1 ) { |  |
| merge_left_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } else if( PredMode = = MODE_DIRECT ) { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( BiDMVDFlag && slice_type == B && !slice_is_GPB && inter_pred_idc[ x0 ][ y0 ]== Pred_Bi) |  |
| direct_dmvd_flag[ x0 ][ y0 ] | ue(1) ¦ ae(v) |
| if( !direct_dmvd_flag[ x0 ][ y0 ]) { |  |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L1 && NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 && NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( BiDMVDFlag && slice_type == B && !slice_is_GPB && inter_pred_idc[ x0 ][ y0 ]== Pred_Bi && PartMode != PART_2Nx2N && PartMode != PART_NxN) |  |
| pu_dmvd_flag[ x0 ][ y0 ] | ue(1) ¦ ae(v) |
| if( !pu_dmvd_flag[ x0 ][ y0 ]) { |  |
| if( inter_pred_idc[ x0 ][ y0 ] ! = Pred_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0 | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1 | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

## Semantics

BiDMVDFlag specifies whether the DMVD method is used in bitstream.

- If 1, DMVD derivation method is enabled. Otherwise, it is disabled.

BiDMVDRefineFlag

- If 1, SDME refinement is enabled to be performed after rounded-candidate checking.

- Otherwise, SDME refinement is disabled.

BiDMVDSampledSADFlag

- If 1, SAD is calculated using down-sampled PU in SDME process.

- Otherwise, SAD is calculated using the full PU in SDME process.

skip_dmvd_flag[x0][y0] specifies the MV derivation method for Skip mode. Default value is 0.

- If 1, DMVD derivation method is used.

- Otherwise HM original derivation method is used.

pu_dmvd_flag[x0][y0] specifies the MV derivation method for the PU if it is predicted in bi-direction mode. It is not applied to 2Nx2N and NxN PUs. Default value is 0.

- If 1, DMVD derivation method is used.

- Otherwise HM original derivation method is used.

direct_dmvd_flag[x0][y0] specifies the MV derivation method for bi-direction predicted direct mode. Default value is 0.

- If 1, DMVD derivation method is used.

- Otherwise HM original derivation method is used.

## Decoding process

The following paragraphs is provided to be added into the decoding process section of [10].

### Derivation process for MV components and ref. indices of DMVD prediction units

DMVD prediction is only applicable to the bi-directional prediction modes of non-GPB B slices.

Input to this process is a prediction partition index puPartIdx.

Outputs of this process are

- luma motion vectors mvL0 and mvL1 and when ChromaArrayType is not equal to 0, the chroma motion vectors mvCL0 and mvCL1,

- reference indices refIdxL0 and refIdxL1.

The variables mvLX and refIdxLX (with X being 0 or 1) are derived as follows.

- The variables refIdxLX are set to the indices of the nearest reference frame in the forward and backward reference buffers, i.e., refIdxL0 = refIdxL1 = 0.

- The variables mvLX are obtained through the SDME process described in Section 2.

# Performance

The encoder is complied in Linux OS and is tested in a compute farm consisting of hundred of CPU cores. The computing cores in the system may not be identical, and information of the log of encoding time ratio may produce slightly different result in each occurrence of the simulation. The decoder was tested on a single workstation with the following configurations: Windows Server 2003 Enterprise x64 Edition, Intel Xeon CPU E5335 @ 2.00GHz, 8GB RAM, and Visual Studio 2008.

## Rounded-candidate based SDME without MV refinement

The proposed SDME techniques were implemented onto TMuC0.9 Reference Software and evaluated under the test conditions defined by the CE1 group [10]. The experiment is investigated on the test case of random access because SDME is for improving the coding efficiency of B picture.

Table 3 and Table 4 show the rounded-candidate based SDME performance compared with the CE1 anchors. In high efficiency case, the average BD bitrate reduction for Luma component is about 1.5% with about 24% increase in encoding time and about 5% increase in decoding time. If YUV output is skipped in decoding tests, the average decoding time increase is about 8%.

Table 3 Rounded-candidates based SDME performance (high efficiency)

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -1.93 | -1.50 | -1.66 |  |  |  |
|  | PeopleOnStreet | -2.80 | -3.64 | -3.40 |  |  |  |
| Class B | Kimono1 | -1.50 | -1.70 | -1.65 |  |  |  |
|  | ParkScene | -1.36 | -1.56 | -1.27 |  |  |  |
|  | Cactus | -1.88 | -1.60 | -1.79 |  |  |  |
|  | BasketballDrive | -1.12 | -1.43 | -1.42 |  |  |  |
|  | BQTerrace | -1.26 | -0.89 | -0.76 |  |  |  |
| Class C | BasketballDrill | -1.27 | -1.79 | -1.72 |  |  |  |
|  | BQMall | -2.22 | -2.39 | -2.39 |  |  |  |
|  | PartyScene | -1.11 | -1.31 | -1.27 |  |  |  |
|  | RaceHorses | -0.66 | -1.04 | -1.20 |  |  |  |
| Class D | BasketballPass | -1.94 | -2.65 | -2.18 |  |  |  |
|  | BQSquare | -0.99 | -0.88 | -0.36 |  |  |  |
|  | BlowingBubbles | -1.38 | -1.42 | -1.44 |  |  |  |
|  | RaceHorses | -1.35 | -1.93 | -2.10 |  |  |  |
| Average of Class A |  | -2.37 | -2.57 | -2.53 |  |  |  |
| Average of Class B |  | -1.42 | -1.44 | -1.38 |  |  |  |
| Average of Class C |  | -1.32 | -1.63 | -1.64 |  |  |  |
| Average of Class D |  | -1.42 | -1.72 | -1.52 |  |  |  |
| Overall Average |  | -1.52 | -1.72 | -1.64 | 124% | 105% | 108% |

In low complexity case, the average BD bitrate reduction for Luma component is about 1.2% with about 30% increase in encoding time and about 17% increase in decoding time. If YUV output is skipped in decoding tests, the average decoding time increase is about 18%.

Table 4 Rounded-candidates based SDME performance (low complexity)

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -1.31 | -1.25 | -1.02 |  |  |  |
|  | PeopleOnStreet | -2.10 | -2.22 | -2.24 |  |  |  |
| Class B | Kimono1 | -1.42 | -1.48 | -1.47 |  |  |  |
|  | ParkScene | -1.14 | -1.12 | -1.05 |  |  |  |
|  | Cactus | -1.46 | -1.18 | -1.49 |  |  |  |
|  | BasketballDrive | -1.17 | -1.31 | -1.38 |  |  |  |
|  | BQTerrace | -0.95 | -0.85 | -0.45 |  |  |  |
| Class C | BasketballDrill | -0.88 | -1.31 | -1.66 |  |  |  |
|  | BQMall | -1.53 | -1.24 | -1.60 |  |  |  |
|  | PartyScene | -0.90 | -1.28 | -1.15 |  |  |  |
|  | RaceHorses | -0.82 | -0.97 | -1.05 |  |  |  |
| Class D | BasketballPass | -1.47 | -1.78 | -1.99 |  |  |  |
|  | BQSquare | -1.09 | -1.27 | -0.30 |  |  |  |
|  | BlowingBubbles | -0.94 | -1.14 | -1.06 |  |  |  |
|  | RaceHorses | -1.07 | -1.39 | -1.43 |  |  |  |
| Average of Class A |  | -1.71 | -1.73 | -1.63 |  |  |  |
| Average of Class B |  | -1.23 | -1.19 | -1.17 |  |  |  |
| Average of Class C |  | -1.03 | -1.20 | -1.36 |  |  |  |
| Average of Class D |  | -1.15 | -1.39 | -1.19 |  |  |  |
| Overall Average |  | -1.22 | -1.32 | -1.29 | 130% | 117% | 118% |

## Rounded-candidate based SDME with MV refinement

Table 5 and Table 6 show the performance of SDME with refinement compared with the CE1 anchors. In high efficiency case, the average BD bitrate reduction for Luma component is about 2.8% with about 35% increase in encoding time and about 21% increase in decoding time. If YUV output is skipped in decoding tests, the average decoding time increase is about 24%.

Table 5 Performance of SMDE with refinement (high efficiency)

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -3.17 | -2.36 | -2.48 |  |  |  |
|  | PeopleOnStreet | -4.11 | -4.93 | -4.73 |  |  |  |
| Class B | Kimono1 | -2.89 | -2.50 | -2.68 |  |  |  |
|  | ParkScene | -2.89 | -2.53 | -2.31 |  |  |  |
|  | Cactus | -3.80 | -3.54 | -3.40 |  |  |  |
|  | BasketballDrive | -2.44 | -2.46 | -2.46 |  |  |  |
|  | BQTerrace | -1.91 | -1.45 | -1.04 |  |  |  |
| Class C | BasketballDrill | -2.08 | -2.44 | -2.33 |  |  |  |
|  | BQMall | -4.06 | -3.75 | -3.80 |  |  |  |
|  | PartyScene | -2.25 | -1.96 | -2.15 |  |  |  |
|  | RaceHorses | -1.49 | -1.75 | -2.21 |  |  |  |
| Class D | BasketballPass | -3.39 | -3.63 | -3.44 |  |  |  |
|  | BQSquare | -2.30 | -1.65 | -1.96 |  |  |  |
|  | BlowingBubbles | -2.89 | -2.23 | -2.00 |  |  |  |
|  | RaceHorses | -2.80 | -3.13 | -3.15 |  |  |  |
| Average of Class A |  | -3.64 | -3.65 | -3.60 |  |  |  |
| Average of Class B |  | -2.78 | -2.49 | -2.38 |  |  |  |
| Average of Class C |  | -2.47 | -2.48 | -2.62 |  |  |  |
| Average of Class D |  | -2.85 | -2.66 | -2.64 |  |  |  |
| Overall Average |  | -2.83 | -2.69 | -2.68 | 135% | 121% | 124% |

In low complexity case, the average BD bitrate reduction for Luma component is about 2.2% with about 35% increase in encoding time and about 39% increase in decoding time. If YUV output is skipped in decoding tests, the average decoding time increase is about 44%.

Table 6 Performance of SMDE with refinement (low complexity)

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -2.39 | -1.55 | -1.51 |  |  |  |
|  | PeopleOnStreet | -3.46 | -3.46 | -3.43 |  |  |  |
| Class B | Kimono1 | -2.81 | -2.11 | -2.08 |  |  |  |
|  | ParkScene | -2.30 | -1.58 | -1.44 |  |  |  |
|  | Cactus | -3.22 | -2.61 | -2.56 |  |  |  |
|  | BasketballDrive | -2.36 | -1.94 | -2.11 |  |  |  |
|  | BQTerrace | -1.69 | -0.51 | -0.40 |  |  |  |
| Class C | BasketballDrill | -1.49 | -1.63 | -1.80 |  |  |  |
|  | BQMall | -2.78 | -2.06 | -2.35 |  |  |  |
|  | PartyScene | -1.25 | -1.06 | -1.18 |  |  |  |
|  | RaceHorses | -1.51 | -1.66 | -2.04 |  |  |  |
| Class D | BasketballPass | -2.64 | -2.68 | -2.53 |  |  |  |
|  | BQSquare | -1.31 | -0.71 | -0.74 |  |  |  |
|  | BlowingBubbles | -1.49 | -0.90 | -0.95 |  |  |  |
|  | RaceHorses | -2.19 | -2.21 | -2.35 |  |  |  |
| Average of Class A |  | -2.92 | -2.51 | -2.47 |  |  |  |
| Average of Class B |  | -2.48 | -1.75 | -1.72 |  |  |  |
| Average of Class C |  | -1.76 | -1.60 | -1.84 |  |  |  |
| Average of Class D |  | -1.91 | -1.63 | -1.64 |  |  |  |
| Overall Average |  | -2.19 | -1.78 | -1.83 | 135% | 139% | 144% |

## Performance with MRG and AMVP

For reference, the SDME with refinement is tested with the different testing settings of MRG and AMVP and the result is listed in Table 7 & Table 8. The data shows that SDME achieved sizeable coding gain (>2.8% & > 2.0%) with all kinds of settings of MRG and AMVP for test category of Random Access (high efficiency & low complexity).

Table 7 BD-rate Y of SDME under different MRG and AMVP settings (high efficiency)

|  | Sequences | MRG = 0 AMVP off | MRG = 0 AMVP on | MRG = 1 AMVP off | MRG = 1 AMVP on |
| --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -6.25 | -3.34 | -5.17 | -3.17 |
|  | PeopleOnStreet | -6.68 | -4.20 | -6.06 | -4.11 |
| Class B | Kimono1 | -6.11 | -3.33 | -4.58 | -2.89 |
|  | ParkScene | -5.04 | -3.21 | -3.80 | -2.89 |
|  | Cactus | -6.77 | -4.03 | -5.99 | -3.80 |
|  | BasketballDrive | -4.76 | -2.48 | -3.99 | -2.44 |
|  | BQTerrace | -3.27 | -1.62 | -2.84 | -1.91 |
| Class C | BasketballDrill | -3.66 | -1.96 | -3.42 | -2.08 |
|  | BQMall | -6.80 | -4.23 | -5.92 | -4.06 |
|  | PartyScene | -3.45 | -2.12 | -3.13 | -2.25 |
|  | RaceHorses | -2.70 | -1.69 | -2.28 | -1.49 |
| Class D | BasketballPass | -5.53 | -3.41 | -4.97 | -3.39 |
|  | BQSquare | -2.67 | -1.75 | -2.53 | -2.30 |
|  | BlowingBubbles | -4.18 | -2.76 | -3.85 | -2.89 |
|  | RaceHorses | -4.74 | -2.95 | -4.16 | -2.80 |
| Average of Class A |  | -6.46 | -3.77 | -5.62 | -3.64 |
| Average of Class B |  | -5.19 | -2.94 | -4.24 | -2.78 |
| Average of Class C |  | -4.15 | -2.50 | -3.69 | -2.47 |
| Average of Class D |  | -4.28 | -2.72 | -3.88 | -2.85 |
| Overall Average |  | -4.84 | -2.87 | -4.18 | -2.83 |

Table 8 BD-rate Y of SDME under different MRG and AMVP settings (low complexity)

|  | Sequences | MRG = 0 AMVP off | MRG = 0 AMVP on | MRG = 1 AMVP off | MRG = 1 AMVP on |
| --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -5.36 | -2.25 | -4.77 | -2.39 |
|  | PeopleOnStreet | -6.47 | -3.45 | -6.00 | -3.46 |
| Class B | Kimono1 | -5.88 | -2.74 | -4.76 | -2.81 |
|  | ParkScene | -4.28 | -2.14 | -3.35 | -2.30 |
|  | Cactus | -5.92 | -3.15 | -5.37 | -3.22 |
|  | BasketballDrive | -4.39 | -2.22 | -3.95 | -2.36 |
|  | BQTerrace | -2.64 | -1.48 | -2.23 | -1.69 |
| Class C | BasketballDrill | -3.40 | -1.45 | -3.22 | -1.49 |
|  | BQMall | -5.59 | -2.76 | -5.09 | -2.78 |
|  | PartyScene | -2.39 | -1.12 | -2.33 | -1.25 |
|  | RaceHorses | -2.75 | -1.31 | -2.45 | -1.51 |
| Class D | BasketballPass | -4.85 | -2.56 | -4.37 | -2.64 |
|  | BQSquare | -1.86 | -1.04 | -1.94 | -1.31 |
|  | BlowingBubbles | -2.71 | -1.33 | -2.49 | -1.49 |
|  | RaceHorses | -4.35 | -2.18 | -4.01 | -2.19 |
| Average of Class A |  | -5.92 | -2.85 | -5.39 | -2.92 |
| Average of Class B |  | -4.62 | -2.35 | -3.93 | -2.48 |
| Average of Class C |  | -3.54 | -1.66 | -3.27 | -1.76 |
| Average of Class D |  | -3.44 | -1.78 | -3.20 | -1.91 |
| Overall Average |  | -4.19 | -2.08 | -3.76 | -2.19 |

# Conclusions

This contribution provides performance result for CE1 task of Self Derivation of Motion Estimation (SDME) techniques. Two presented versions of SDME from Guangzhou meeting, rounded-candidate based SDME without motion vector (MV) refinement and rounded-candidate based SDME with MV refinement, are ported onto TMuC 0.9 reference software.  The version of rounded-candidate based SDME with MV refinement has demonstrated that an overall 2.8% BD bitrate improvement for the test of random access (high efficiency) category under the common test conditions with 35% increase in encoding time and 21% increase in decoding time increase. The version of rounded-candidate based SDME without MV refinement has demonstrated an overall 1.5% BD bitrate improvement with 24% increase in encoding time and 5% increase in decoding time. A description about the normative syntax change about SDME in the form of HEVC specification is included. With the performance result demonstrated sizeable coding efficiency improvement with feasible complexity increase, the proponents suggest JCT to adopt the proposed techniques into HM to be further studied by broader community.

# Additional Results

Table 9 shows the performance data of the rounded-candidate based SDME without MV refinement based on the 9-candidate motion vector pairs on an improved software version. For high efficiency case, the average BD bitrate reduction for Luma component is about 1.7% with about 23% increase in encoding time and about 4% increase in decoding time.

Table 9 Rounded-candidates based SDME performance (high efficiency, 9 candidates)

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -2.21 | -1.75 | -1.72 |  |  |  |
|  | PeopleOnStreet | -3.13 | -3.87 | -3.51 |  |  |  |
| Class B | Kimono1 | -1.70 | -1.87 | -1.84 |  |  |  |
|  | ParkScene | -1.49 | -1.62 | -1.52 |  |  |  |
|  | Cactus | -2.07 | -1.80 | -2.09 |  |  |  |
|  | BasketballDrive | -1.29 | -1.64 | -1.64 |  |  |  |
|  | BQTerrace | -1.52 | -1.35 | -0.83 |  |  |  |
| Class C | BasketballDrill | -1.50 | -2.00 | -1.87 |  |  |  |
|  | BQMall | -2.49 | -2.54 | -2.73 |  |  |  |
|  | PartyScene | -1.21 | -1.36 | -1.45 |  |  |  |
|  | RaceHorses | -0.82 | -1.18 | -1.41 |  |  |  |
| Class D | BasketballPass | -2.06 | -2.93 | -2.40 |  |  |  |
|  | BQSquare | -1.00 | -0.57 | -0.86 |  |  |  |
|  | BlowingBubbles | -1.53 | -1.81 | -1.33 |  |  |  |
|  | RaceHorses | -1.60 | -2.07 | -2.09 |  |  |  |
| Average of Class A |  | -2.67 | -2.81 | -2.61 |  |  |  |
| Average of Class B |  | -1.61 | -1.66 | -1.58 |  |  |  |
| Average of Class C |  | -1.50 | -1.77 | -1.87 |  |  |  |
| Average of Class D |  | -1.55 | -1.84 | -1.67 |  |  |  |
| Overall Average |  | -1.71 | -1.89 | -1.82 | 123% | 104% | -- |

# References

- S. Kamp, M. Evertz, M. Wien, “Decoder side motion vector derivation,” ITU-T Q.6/SG16 33rd Meeting Document VCEG-AG16, October 2007.

- Y.-W. Huang, C.-Y. Chen, C.-W. Hsu, J.-L. Lin, Y.-P. Tsai, J. An, S. Lei, “TE1: Decoder-side motion vector derivation with switchable template matching”, Meeting Document JCTVC-A076, April 2010.

- W.-J. Chien, M. Karczewicz, P. Chen, “TE1: Decoder-side motion vector derivation report from Qualcomm”, Meeting Document JCTVC-A097, April 2010.

- M. Ueda, “TE1.a: Implementation report of Refinement Motion Compensation using DMVD on TMuC”, Meeting Document JCTVC-C138, Oct. 2010.

- M. Yang, S. Lin, D. Wang, J. zhou, and H. Yu, “TE1：Huawei report on DMVD improvements in TMuC”, Meeting Document JCTVC-C097, Oct. 2010.

- Y. Itani, S. Sekiguchi, and T. Murakami, “TE1 report: implicit direct vector derivation”, Meeting Document JCTVC-C124, Oct. 2010.

- Y. Chiu, L. Xu, W. Zhang, H. Jiang, “Description of video coding technology proposal: self derivation of motion estimation and adaptive (Wiener) loop filtering”, Meeting Document JCTVC-A106, April 2010.

- Y. Chiu, L. Xu, W. Zhang, H. Jiang, “TE1: Fast techniques to improve self derivation of motion estimation”, Meeting Document JCTVC-B047, July 2010.

- Y. Chiu, L. Xu, W. Zhang, H. Jiang, “TE1: Report of self derivation of motion estimation improvement in TMuC”, Meeting Document JCTVC-C127, October 2010.

- “WD1: Working Draft 1 of High-Efficiency Video Coding”, Meeting Document JCTVC-C403, Oct. 2010.

- Yi-Jen Chiu, Haoping Yu, Yu-Wen Huang, and Shun-ichi Sekiguchi, “Core Experiment 1: Decoder-Side Motion Vector Derivation”, Meeting Document JCTVC-C501, Oct. 2010.

# Patent rights declaration(s)

(NOTE – Activities in the JCT-VC and contributions to the JCT-VC are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-VC proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. An example of an informal IPR notification statement for a contribution is provided below.)

Intel Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).