| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D171 |
| --- | --- |

| Title: | Improved AIS filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: | madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

Adaptive Intra smoothing currently uses [1 2 1] filtering. This contribution presents results of using an alternate filter [1 1 4 1 1] which emphasizes high frequency and edge information. For Intra only coding, filter [1 1 4 1 1] provides an additional coding efficiency improvement of 0.1-0.2% BD-Rate. The additional coding gain is on top of the 0.3-0.7% gain shown by AIS with filter [1 2 1] for Intra only conditions.

# Introduction

Adaptive Intra smoothing currently uses [1 2 1] filtering. This contribution presents results of using an alternate filter [1 1 4 1 1] which emphasizes high frequency and edge information. For Intra only coding, filter [1 1 4 1 1] provides an additional coding efficiency improvement of 0.1-0.2% BD-Rate. The additional gain is on top of the 0.3-0.7% gain shown by AIS with filter [1 2 1] for Intra only conditions.

Figure below shows the frequency response of the filters [1 2 1] and [1 1 4 1 1].

Filter [1 1 4 1 1] was integrated into TMuC-0.9. Summary of results is provided in table below for Intra only conditions.

|  | Intra [1 2 1] |  |  | Intra [1 1 4 1 1] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6 | -0.6 | -0.8 | -0.8 | -0.6 | -0.8 |
| Class B | -0.6 | -0.3 | -0.4 | -0.8 | -0.5 | -0.6 |
| Class C | -0.2 | 0.3 | 0.3 | -0.3 | 0.2 | 0.2 |
| Class D | -0.1 | 0.3 | 0.4 | -0.2 | 0.3 | 0.4 |
| Class E | -0.1 | -0.1 | 0.0 | -0.5 | -0.3 | -0.2 |
| All | -0.3 | 0.0 | -0.1 | -0.5 | -0.1 | -0.2 |
| Enc Time[%] | 135% |  |  | 143% |  |  |
| Dec Time[%] | 104% |  |  | 119% |  |  |
|  | Intra LoCo [1 2 1] |  |  | Intra LoCo [1 1 4 1 1] |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.7 | -0.1 | -0.4 | -1.8 | -0.5 | -0.7 |
| Class B | -0.8 | 0.2 | -0.1 | -1.1 | -0.1 | -0.4 |
| Class C | -0.3 | 0.7 | 0.6 | -0.3 | 0.5 | 0.4 |
| Class D | -0.4 | 0.6 | 0.6 | -0.4 | 0.5 | 0.5 |
| Class E | -0.9 | -0.2 | 0.4 | -1.0 | -0.4 | 0.1 |
| All | -0.7 | 0.3 | 0.3 | -0.8 | 0.1 | 0.0 |
| Enc Time[%] | 141% |  |  | 162% |  |  |
| Dec Time[%] | 97% |  |  | 117% |  |  |

|  |  | Filter [1 2 1] |  | Filter [1 1 4 1 1] |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Intra | Intra LoCo | Intra | Intra Loco |
| S01 | Traffic | -0.4 | -1.4 | -0.6 | -1.5 |
| S02 | PeopleOnStreet | -0.8 | -2.0 | -0.9 | -2.2 |
| S03 | Kimono | -0.4 | -1.0 | -0.7 | -1.6 |
| S04 | ParkScene | -0.7 | -1.1 | -0.8 | -1.4 |
| S05 | Cactus | -0.4 | -1.0 | -0.7 | -1.2 |
| S06 | BasketballDrive | -0.9 | -0.7 | -1.1 | -0.9 |
| S07 | BQTerrace | -0.5 | -0.4 | -0.5 | -0.2 |
| S08 | BasketballDrill | -0.1 | 0.1 | -0.4 | 0.1 |
| S09 | BQMall | -0.2 | -0.2 | -0.3 | -0.2 |
| S10 | PartyScene | -0.2 | -0.1 | -0.2 | -0.2 |
| S11 | RaceHorses | -0.2 | -0.9 | -0.4 | -1.1 |
| S12 | BasketballPass | -0.2 | -0.4 | -0.4 | -0.5 |
| S13 | BQSquare | 0.1 | 0.0 | 0.1 | 0.1 |
| S14 | BlowingBubbles | -0.2 | -0.5 | -0.3 | -0.5 |
| S15 | RaceHorses | -0.1 | -0.8 | -0.3 | -0.8 |
| S16 | Vidyo1 | -0.1 | -1.1 | -0.5 | -1.5 |
| S17 | Vidyo3 | -0.3 | -0.8 | -0.5 | -0.3 |
| S18 | Vidyo4 | -0.1 | -0.8 | -0.5 | -1.1 |
|  | Average | -0.3 | -0.7 | -0.5 | -0.8 |
|  | Max | 0.1 | 0.1 | 0.1 | 0.1 |
|  | Min | -0.9 | -2.0 | -1.1 | -2.2 |

# Conclusions

We request starting a core experiment on AIS filters.

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).