| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4rd Meeting: Daegu, CN, 20-28 January, 2011 | Document: JCTVC-D173 WG11 Number: m18925 |
| --- | --- |

| Title: | CE2-subset 4: Cross-check report of Technicolor’s proposal on Geometry adaptive block partitioning Simplification from INRIA |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Laurent Guillo (IRISA/CNRS), Ronan Boitard (INRIA) | Tel: Email: | +33-2-99-84-71-00 laurent.guillo@inria.fr ronan.boitard@inria.fr |
| Source: | INRIA. |  |  |

_____________________________

# Abstract

This document reports the results of Core Experiment 2 (CE2)-subset 4 cross-checking. The CE2-subset 4 relates to the simplification of Geometry adaptive Block Partitioning (GEO) at the encoder side. The principle of GEO is not modified and is still conforming to the tool description as provided in [1] and later in [2, 3]. This simplification is proposed by Qualcomm and Technicolor.

Two algorithms are proposed providing different levels of simplification.

- Code verification

Technicolor provided INRIA with two source codes of Simplified GEO (SGEO).

The source codes have been successfully compiled using Linux (Fedora Core 11 64 bits) with gcc 4.4.4.

Tests were run on a cluster grid with the following characteristics:

- CPU: 2 x Intel(R) Xeon(R) E5640 @ 2.67GHz, 4 cores.

- RAM: 12 Go per core.

# Results

Full results have been generated for the Random Access and Low Delay High Efficiency cases, as specified in the CE2 test conditions, [4]. Two levels of simplification of GEO (SGEO_1 and SGEO_2) were submitted by the proponent, leading to different performances. The comparison is made with Anchor TMuC 0.9.

For both SGEO_1 and SGEO_2, a set of simplifications deals with code optimization in the SAD computation of GEO predictions as well as the reduction of the motion vector search range. In SGEO_2, the number of GEO modes is reduced by removing significantly unbalanced partitioning modes (one partition much larger than the other one). In addition early termination in the best mode search applies to the 2NxN, Nx2N and GEO partitioning modes. A decision based on the RDcost of NxN partitioning and of 2Nx2N partitioning is made to test or not the 2NxN, Nx2N and GEO partitioning modes.

The following tables summarize the obtained results:

| SGEO_1(rev78) | Random Access HE |  |  | Low Delay HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -3,9 | -4,3 | -4,3 |  |  |  |
| Class B | -1,9 | -2,7 | -2,6 | -2,0 | -3,1 | -3,3 |
| Class C | -3.4 | -4.5 | -4.5 | -3,0 | -4,0 | -4,3 |
| Class D | -2.8 | -3.3 | -3.2 | -2,8 | -4,2 | -3,6 |
| Class E |  |  |  | -4,9 | -4,8 | -5,4 |
| All | -2,8 | -3,5 | -3,5 | -3,0 | -3,9 | -4,0 |
| Enc. time | 190% |  |  | 190% |  |  |
| Dec. time | 97% |  |  | 101% |  |  |

Table 1: SGEO_1 vs TMuC0.9 default configuration.

| SGEO_2 (rev81) | Random Access HE |  |  | Low Delay HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2,6 | -2,8 | -2,9 |  |  |  |
| Class B | -1,3 | -1,9 | -2,0 | -1,5 | -2,6 | -3,0 |
| Class C | -2.2 | -3.2 | -3.2 | -2,0 | -3,0 | -3,1 |
| Class D | -1.9 | -2.2 | -2.1 | -1,9 | -3,5 | -2,8 |
| Class E |  |  |  | -4,0 | -4,9 | -4,3 |
| All | -1,9 | -2,5 | -2,5 | -2,2 | -3,4 | -3,2 |
| Enc. Time | 130% |  |  | 134% |  |  |
| Dec. time | 98% |  |  | 100% |  |  |

Table 2: SGEO_2 vs TMuC0.9 default configuration.

The results obtained by INRIA perfectly match the results provided by Technicolor.

# Conclusion

Two source code versions for the simplification of the Geometry adaptive Block Partitioning at the encoder side (SGEO) have been provided. The cross-checking of results have been done by Technicolor and INRIA, and perfectly match. The SGEO_1 provides an average gain of around 3% with an average encoding time increased of 90% compared to the TMuC 0.9 encoding time. Regarding SGEO_2, the average gain is around 2% with an average encoding time increased by 30%. The Decoder complexity remains similar to the reference TMuC0.9 version.

# References

- M.Karczewicz, P.Chen, R.Joshi, X.Wang, W-J.Chien, R.Panchal, “Video coding technology proposal by Qualcomm Inc”, Doc. JCTVC-A121, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Dresden, Germany, April 2010.

- P.Chen, L.Guo, M.Karczewicz , (Qualcomm), “Geometry motion partition”, Doc. JCTVC-C301 (m18516), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, China, October 2010.

- E.Francois, D.Thoreau, P.Bordes (Technicolor), L.Guillo (INRIA), “TE3 subtest 2: Report on simplified geometry block partitioning”, Doc. JCTVC-C031 (m18052), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, China, October 2010.

- E. Francois, X. Zheng, P. Chen, “Core Experiment 2: Flexible Motion Partitioning”, Doc. JCTVC-C502 (m18602), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, China, October 2010.