| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D174_r1 WG11 Number: m18926 |
| --- | --- |

| Title: | CE1: Summary Report of Decoder-Side Motion Vector Derivation (DMVD) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu Intel Corp. Haoping Yu Huawei Technologies Co., Ltd. Yu-Wen Huang Mediatek Inc. Shun-ichi Sekiguchi Mitsubishi | Tel: Email: | yi-jen.chiu@intel.com haopingyu@huawei.com yuwen.huang@mediatek.com Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp |
| Source: | [CE Coordinators] Intel Corp., Huawei Technologies Co., Ltd., Mediatek Inc., Mitsubishi Electric Corp. |  |  |

_____________________________

# Abstract

This report is a summary of core experiment 1 (CE1) on Decoder-side Motion Vector Derivation (DMVD). There are four Subsets in CE1, candidate-based reference block prediction DMVD (Subset 1), template matching based DMVD (Subset 2), per-pixel optical-flow based DMVD (Subset 3), refinement techniques based on transmitted MVs (Subset 4) as described in JCTVC-C501. The proposals are evaluated based on the common conditions in JCTVC-C500. The detailed results will be reported by proponents, and each proposal has at least one cross-checker to examine the result.

# Introduction

In this tool experiment, DMVD techniques have been tested for the following functionalities:

- candidate-based reference block prediction DMVD (3 proposals)

- template matching based DMVD (1 proposal)

- per-pixel optical-flow based DMVD (1 proposal)

- refinement techniques based on transmitted MVs (1 proposal)

The CE tools are implemented in the TMuC 0.9 software available on November, 2010, and they was tested and evaluated under the official common test configurations in [1]. (Intra-only tests are not included) Data are requested to compare with the anchor from HEVC committee.

# Document list

- Summary document (this document)

  - JCTVC-D174 [CE coordinators] Summary of Core Experiment 8 on In-Loop filtering

- Proponent's and verification/study documents

Subset 1: candidate-based reference block prediction DMVD

  - JCTVC-D099 [Mitsubishi] CE1 Report of implicit direct vector derivation

    - Cross-check: JCTVC-D168 [from Intel]

  - JCTVC-D295 [Huawei] CE1 Huawei report on TMDMVD and STDM in HM

    - Cross-check: JCTVC- D247 [from Motorola]

  - JCTVC-D167 [Intel] CE1: Report of self derivation of motion estimation in TMuC0.9

    - Cross-check: JCTVC-D098 [from Mitsubishi]

Subset 2: template matching based DMVD

  - JCTVC-D295 [Huawei] CE1 Huawei report on TMDMVD and STDM in HM

    - Cross-check: JCTVC-D169 [from Intel]

Subset 3: per-pixel optical-flow based DMVD

  - JCTVC-D329 [Samsung] CE1 Samsung’s test for bi-directional optical flow

    - Cross-check: JCTVC-D121 [from JVC-Kenwood]

Subset 4: refinement techniques based on transmitted

  - JCTVC-D120 [JVC-Kenwood] CE1: Refinement motion compensation using DMVD with merge extension

    - Cross-check: JCTVC-D340 [from Samsung]

# Cross-verification

Proponents conducted source code exchange for each other. The assignment of cross-checker was listed in [2]. Cross-checkers run the software independently and compared their results with proponent’s results. The comment from cross-checker are provided below in Section 4.

# Experimental results

Based on the common conditions [1], CE1 defined subsets of test conditions and sequences as mandatory tests [2]. And the result of the experiments follows.

## Subset 1: Candidate-based reference block prediction DMVD

### JCTVC-D099 Report of implicit direct vector derivation ( Mitsubishi)

B-skip and B-direct modes using the proposed technique has been implemented into TMuC version0.9 software that was made available to the group. Table 1 shows test conditions used for our experiment in the CE1 document. The anchor streams have been generated with TMuC 0.9 default configurations[3].

Table 1  Test Condition

| Codec software | Implementation of the tool proposed in JCTVC-C124[2] based on TMuC0.9 software |
| --- | --- |
| Test configuration | JCTVC-C500[3] |
| QP setting | QPI=22,37,32,37 |
| Constraint Sets | CS2, CS3, CS4, CS5 |

The BD rate reductions in each condition obtained by the proposed method are listed in Table 2 to 5 respectively. It is observed that the proposed direct mode can achieve consistent coding gain especially for the sequences where spatially detailed texture and stable background scene co-exist, such as PeopleOnStreet or Cactus. In such cases, spatially adjacent motion vector may not work as sufficient estimator for direct vector, but temporal co-located candidate could contribute to improvement of direct vector accuracy.

Table 2 Performance in BD measurements (low delay)

|  | BD-PSNR[dB] |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Kimono | 0.7 | 0.1 | 0.0 |
| ParkScene | 0.1 | 0.4 | -0.4 |
| Cactus | 0.8 | 0.4 | -0.7 |
| BasketballDrive | 1.3 | 1.2 | 1.7 |
| BQTerrace | 0.0 | 0.3 | -2.5 |
| BasketballDrill | 1.1 | 1.1 | 1.9 |
| BQMall | 0.9 | 1.2 | 0.4 |
| PartyScene | 0.4 | 0.4 | 0.2 |
| RaceHorses | 0.2 | 0.4 | 0.6 |
| BasketballPass | 1.1 | 0.7 | 0.9 |
| BQSquare | -0.1 | -1.4 | -0.4 |
| BlowingBubbles | 0.5 | -0.1 | -0.2 |
| RaceHorses | 0.3 | 0.3 | 0.6 |
| Vidyo1 | -0.5 | -3.8 | -5.1 |
| Vidyo3 | 0.5 | -4.8 | -4.0 |
| Vidyo4 | 0.2 | -2.8 | -3.8 |
| Average Class B | 0.6 | 0.5 | -0.4 |
| Average Class C | 0.7 | 0.8 | 0.8 |
| Average Class D | 0.5 | -0.1 | 0.2 |
| Average Class E | 0.1 | -3.8 | -4.3 |
| Average All | 0.5 | -0.4 | -0.7 |

Table 3 Performance in BD measurements (low delay Loco)

|  | BD-PSNR[dB] |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Kimono | 1.2 | 1.2 | 1.4 |
| ParkScene | -0.1 | -0.9 | -0.5 |
| Cactus | 0.6 | 0.6 | -0.2 |
| BasketballDrive | 1.8 | 1.1 | 1.4 |
| BQTerrace | -0.2 | -2.2 | -3.6 |
| BasketballDrill | 0.6 | 1.1 | 1.2 |
| BQMall | 1.4 | -0.3 | 0.5 |
| PartyScene | 1.3 | 0.2 | -0.2 |
| RaceHorses | 0.9 | 0.8 | 0.8 |
| BasketballPass | 1.4 | 0.8 | 0.9 |
| BQSquare | 0.7 | -1.4 | -0.7 |
| BlowingBubbles | 1.2 | -0.3 | -1.0 |
| RaceHorses | 0.9 | 0.4 | 0.7 |
| Vidyo1 | -1.2 | -2.0 | -0.8 |
| Vidyo3 | -0.6 | -0.5 | -0.7 |
| Vidyo4 | -0.5 | 0.2 | 0.5 |
| Average Class B | 0.7 | 0.0 | -0.3 |
| Average Class C | 1.0 | 0.5 | 0.6 |
| Average Class D | 1.0 | -0.1 | 0.0 |
| Average Class E | -0.8 | -0.8 | -0.3 |
| Average All | 0.6 | -0.1 | 0.0 |

Table 4 Performance in BD measurements (random access)

|  | BD-PSNR[dB] |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic | -1.1 | -0.7 | -0.7 |
| PeopleOnStreet | -0.7 | -0.5 | 0.0 |
| Kimono | -0.2 | -0.1 | -0.3 |
| ParkScene | -0.5 | -0.3 | -0.3 |
| Cactus | -1.3 | -0.9 | -1.1 |
| BasketballDrive | 0.2 | 0.3 | 0.6 |
| BQTerrace | -0.3 | -0.3 | -0.3 |
| BasketballDrill | -0.8 | -0.9 | -0.8 |
| BQMall | -0.7 | -0.3 | -0.6 |
| PartyScene | -1.0 | -0.3 | -0.6 |
| RaceHorses | 0.2 | 0.7 | 0.7 |
| BasketballPass | -0.3 | 0.0 | 0.1 |
| BQSquare | -1.0 | -1.0 | -0.5 |
| BlowingBubbles | -1.2 | -0.7 | -0.5 |
| RaceHorses | -0.1 | 0.0 | 0.6 |
| Average Class A | -0.9 | -0.6 | -0.4 |
| Average Class B | -0.4 | -0.2 | -0.3 |
| Average Class C | -0.3 | 0.1 | 0.0 |
| Average Class D | -0.6 | -0.4 | -0.1 |
| Average All | -0.5 | -0.2 | -0.1 |

Table 5 Performance in BD measurements (random access LoCo)

|  | BD-PSNR[dB] |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic | -1.0 | -1.0 | -0.9 |
| PeopleOnStreet | -0.6 | -0.4 | -0.3 |
| Kimono | -0.6 | -0.9 | -0.9 |
| ParkScene | -0.5 | -0.7 | -0.7 |
| Cactus | -1.4 | -0.9 | -1.4 |
| BasketballDrive | 0.6 | 0.3 | 0.4 |
| BQTerrace | -0.3 | -0.7 | -0.7 |
| BasketballDrill | 0.3 | 0.4 | 0.3 |
| BQMall | -0.3 | 0.0 | 0.0 |
| PartyScene | 0.1 | 0.3 | 0.2 |
| RaceHorses | 0.6 | 0.8 | 0.8 |
| BasketballPass | -0.1 | -0.2 | 0.2 |
| BQSquare | 1.5 | 0.9 | 1.7 |
| BlowingBubbles | -0.3 | -0.2 | -0.1 |
| RaceHorses | 0.3 | 0.4 | 0.6 |
| Average Class A | -0.8 | -0.7 | -0.6 |
| Average Class B | -0.4 | -0.6 | -0.7 |
| Average Class C | 0.2 | 0.4 | 0.3 |
| Average Class D | 0.3 | 0.2 | 0.6 |
| Average All | -0.1 | -0.1 | -0.1 |

Table 6 to 9 show encoding and decoding time those are measured using “time” command in Linux platform. They were also compared with that of the anchor streams using the same platform (Fedora x86_64). In average, the increase of decoding time was about 25% relative to the anchor, and encoding time was almost the same as that of the anchor.

The derivation process of direct vector with the proposed technique requires doubled memory access to reference pictures to obtain all prediction block candidates and computation of two SAD calculations per PU at decoder-side, relative to HM-1. Our proposed technique requires derivation of the temporal co-located vector and the spatial neighboring vector, which does not have any additional impact to memory size required for motion vector storage if direct vector derivation process relies on temporal motion vector reference.

Table 6 Encoding and Decoding time (low delay)

| sequence | encoder | decoder |
| --- | --- | --- |
| Kimono | 101.91 | 107.71 |
| ParkScene | 102.18 | 107.01 |
| Cactus | 105.78 | 115.34 |
| BasketballDrive | 102.50 | 103.91 |
| BQTerrace | 101.34 | 104.13 |
| BasketballDrill | 100.42 | 104.22 |
| BQMall | 100.44 | 109.25 |
| PartyScene | 101.26 | 105.06 |
| RaceHorses | 101.81 | 101.42 |
| BasketballPass | 100.46 | 108.89 |
| BQSquare | 101.67 | 116.11 |
| BlowingBubbles | 100.38 | 110.99 |
| RaceHorses | 101.46 | 104.44 |
| Vidyo1 | 99.83 | 111.99 |
| Vidyo3 | 100.25 | 107.57 |
| Vidyo4 | 100.55 | 110.29 |
| Average Class B | 102.74 | 107.62 |
| Average Class C | 100.98 | 104.99 |
| Average Class D | 104.14 | 109.63 |
| Average Class E | 101.92 | 104.02 |
| Average All | 101.39 | 108.02 |

Table 7 Encoding and Decoding time (low delay Loco)

| sequence | encoder | decoder |
| --- | --- | --- |
| Kimono | 100.83 | 113.69 |
| ParkScene | 101.08 | 125.53 |
| Cactus | 100.48 | 119.46 |
| BasketballDrive | 101.50 | 112.67 |
| BQTerrace | 101.07 | 122.94 |
| BasketballDrill | 99.78 | 110.58 |
| BQMall | 100.96 | 119.18 |
| PartyScene | 101.66 | 113.57 |
| RaceHorses | 101.23 | 108.15 |
| BasketballPass | 100.67 | 111.14 |
| BQSquare | 101.56 | 117.76 |
| BlowingBubbles | 100.67 | 115.28 |
| RaceHorses | 101.37 | 111.83 |
| Vidyo1 | 99.67 | 130.68 |
| Vidyo3 | 99.01 | 124.76 |
| Vidyo4 | 99.95 | 130.96 |
| Average Class B | 100.99 | 118.86 |
| Average Class C | 100.91 | 112.87 |
| Average Class D | 100.99 | 116.07 |
| Average Class E | 101.29 | 117.81 |
| Average All | 100.72 | 118.01 |

Table8 Encoding and Decoding time (random access)

| sequence | encoder | decoder |
| --- | --- | --- |
| Traffic | 103.98 | 117.94 |
| PeopleOnStreet | 102.45 | 114.29 |
| Kimono | 106.19 | 128.23 |
| ParkScene | 105.49 | 125.47 |
| Cactus | 102.22 | 123.64 |
| BasketballDrive | 104.93 | 121.90 |
| BQTerrace | 105.16 | 131.08 |
| BasketballDrill | 101.42 | 111.72 |
| BQMall | 103.73 | 131.35 |
| PartyScene | 102.47 | 135.10 |
| RaceHorses | 103.33 | 110.45 |
| BasketballPass | 102.76 | 122.04 |
| BQSquare | 103.41 | 149.31 |
| BlowingBubbles | 104.12 | 147.31 |
| RaceHorses | 103.66 | 117.10 |
| Average Class A | 103.22 | 116.11 |
| Average Class B | 104.80 | 126.06 |
| Average Class C | 102.74 | 122.15 |
| Average Class D | 103.49 | 133.94 |
| Average All | 103.69 | 125.79 |

Table9 Encoding and Decoding time (random access Loco)

| sequence | encoder | decoder |
| --- | --- | --- |
| Traffic | 105.28 | 129.26 |
| PeopleOnStreet | 101.25 | 127.41 |
| Kimono | 104.12 | 135.27 |
| ParkScene | 104.26 | 139.41 |
| Cactus | 102.07 | 128.11 |
| BasketballDrive | 103.73 | 133.90 |
| BQTerrace | 104.07 | 141.65 |
| BasketballDrill | 100.85 | 115.34 |
| BQMall | 103.64 | 136.67 |
| PartyScene | 103.42 | 130.37 |
| RaceHorses | 103.17 | 115.99 |
| BasketballPass | 102.73 | 122.44 |
| BQSquare | 104.92 | 133.02 |
| BlowingBubbles | 104.29 | 136.91 |
| RaceHorses | 103.66 | 122.79 |
| Average Class A | 103.27 | 128.33 |
| Average Class B | 103.65 | 135.67 |
| Average Class C | 102.77 | 124.59 |
| Average Class D | 103.90 | 128.79 |
| Average All | 103.43 | 129.90 |

#### Comments/Remarks from Cross Checkers

JCTVC-D168 from Intel provides the cross-check results of JCTVC-D099 (Mitsubishi). The software package obtained from Mitsubishi was successfully compiled and tested under the test conditions defined in CE1. The encoder was compiled in Linux OS and was tested in a parallel computing farm system. The capability of the computing cores in the computing farm system is not identical, and the measured average encoding time ratio may not be identical per simulation. Therefore, the measurement of the encoding time should be served as one reference point. The decoder was complied with Visual Studio 2008 and tested on a single server with the following configurations: Windows Server 2003 Enterprise x64 Edition, Intel Xeon CPU E5335 @ 2.00GHz, 8GB RAM.

The accompanied spreadsheet of JCTVC-D168 contains the detailed data of the results. The BD rate results perfectly match with the data provided by the proponent. The encoding time and decoding time do not achieve the identical result provided by proponent maybe due to the difference in the computing facility. The cross-check marks that the decoding time and encoding time produced by cross-checker has the tendency to be consistent with the data provided by proponents. It is noted that the decoding time was tested without the writing of YUV output.

### JCTVC-D295 CE1 Huawei report on TMDMVD and STDM in HM

#### Compression Performance Discussion

In this proposal, TM-DMVD is applied to the GPB slices in both low delay and random access cases, whereas STDM is only applied to the B (non-GPB) slices in the random access case. To provide the performance and complexity data in details for each individual tool, the test results of TM-DMVD+STDM and STDM only will be given separately for the random access cases.

| Video Sequence | STDM |  |  |
| --- | --- | --- | --- |
|  | BD-rate Y(%) | BD-rate U(%) | BD-rate V(%) |
| Class A | -2.91 | -2.85 | -2.89 |
| Class B | -1.72 | -1.48 | -1.54 |
| Class C | -1.79 | -1.98 | -2.00 |
| Class D | -2.13 | -1.95 | -2.16 |
| Total average | -2.00 | -1.92 | -2.01 |

Table 10.	STDM RD Performance in random access high efficiency Configuration

As shown in Table 10, the proposed STDM technique can achieve about 2% bitrate saving in the HE configuration.

| Video Sequence | TMDMVD+STDM |  |  |
| --- | --- | --- | --- |
|  | BD-rate Y(%) | BD-rate U(%) | BD-rate V(%) |
| Class A | -3.16 | -3.06 | -2.89 |
| Class B | -1.98 | -1.60 | -1.79 |
| Class C | -2.01 | -2.01 | -2.23 |
| Class D | -2.49 | -2.17 | -2.43 |
| Total average | -2.28 | -2.06 | -2.23 |

Table 11.	TMDMVD+STDM RD Performance in random access high efficiency Configuration

As shown in Table 11 the proposed TMDMVD+STDM technique can achieve about 2.28% bitrate saving in the HE configuration.

| Video Sequence | TMDMVD+STDM |  |  |
| --- | --- | --- | --- |
|  | BD-rate Y(%) | BD-rate U(%) | BD-rate V(%) |
| Class A | -2.24 | -2.17 | -2.23 |
| Class B | -1.59 | -1.49 | -1.67 |
| Class C | -1.49 | -1.64 | -1.86 |
| Class D | -1.96 | -1.97 | -1.91 |
| Total average | -1.75 | -1.75 | -1.86 |

Table 12.	TM-DMVD+STDM RD Performance in random access low complexity Configuration

As shown in Table 12 the proposed TM-DMVD technique can achieve about 1.75% bitrate saving in the LC configuration.

| Video Sequence | STDM |  |  |
| --- | --- | --- | --- |
|  | BD-rate Y(%) | BD-rate U(%) | BD-rate V(%) |
| Class A | -2.16 | -2.11 | -2.08 |
| Class B | -1.47 | -1.24 | -1.37 |
| Class C | -1.36 | -1.40 | -1.59 |
| Class D | -1.86 | -1.78 | -1.75 |
| Total average | -1.64 | -1.54 | -1.63 |

Table 13.	STDM RD Performance in random access low complexity Configuration

As shown in Table 13 the total gain from the proposed STDM technique is about 1.64% bitrate saving in the LC configuration.

#### Complexity Analysis

| Video Sequence | STDM (%) |  |  |
| --- | --- | --- | --- |
|  | Enc-Time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| Class A | 130.53% | 105.78% | 106.00% |
| Class B | 145.18% | 103.05% | 103.23% |
| Class C | 154.53% | 105.86% | 106.33% |
| Class D | 264.48% | 108.86% | 109.23% |
| Total Average | 170.78% | 105.69% | 106.00% |

Table 14.	STDM Complexity Analysis in random access high efficiency Configuration

As shown in Table 14, the decoding time of STDM is about 6% more than that of anchor for the HE configuration.

| Video Sequence | TM-DMVD+STDM (%) |  |  |
| --- | --- | --- | --- |
|  | Enc-Time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| Class A | 160.53% | 111.17% | 111.41% |
| Class B | 196.64% | 108.24% | 108.51% |
| Class C | 176.23% | 111.39% | 111.76% |
| Class D | 232.99% | 114.81% | 115.32% |
| Total Average | 194.48% | 111.20% | 111.55% |

Table 15.	TM-DMVD+STDM Complexity Analysis in random access high efficiency Configuration

As shown in Table 15, the decoding time of TM-DMVD+STDM is about 11% more than that of CE1 anchor in the RA+HE case.

| Video Sequence | STDM (%) |  |  |
| --- | --- | --- | --- |
|  | Enc-Time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| Class A | 157.93% | 115.42% | 118.35% |
| Class B | 166.62% | 114.25% | 116.83% |
| Class C | 182.20% | 116.15% | 118.80% |
| Class D | 193.74% | 122.20% | 124.11% |
| Total Average | 176.38% | 116.99% | 119.47% |

Table 16.	STDM Complexity Analysis in random access low complexity Configuration

As shown in Table 16, the decoding time of STDM for the RA+LC case is about 19% more than that of CE1 anchor.

| Video Sequence | TM-DMVD+STDM (%) |  |  |
| --- | --- | --- | --- |
|  | Enc-Time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| Class A | 170.10% | 121.36% | 124.23% |
| Class B | 206.83% | 119.78% | 122.04% |
| Class C | 216.94% | 122.79% | 124.68% |
| Class D | 221.68% | 128.70% | 131.43% |
| Total Average | 207.90% | 123.12% | 125.49% |

Table 17.	TM-DMVD+STDM Complexity Analysis in random access low complexity Configuration

As shown in Table 17, the decoding time of TM-DMVD+STDM for the RA+LC case is about 25% more than that of CE1 anchor.

#### Comments/Remarks from Cross Checkers

### JCTVC-D247 (from Motorola)

The cross-checker is currently running the simulation and will provide an update when the simulation is complete

### JCTVC-D167 Report of self derivation of motion estimation improvement in TMuC0.9 (from Intel)

Table 18 and Table 19 show the rounded-candidates based SDME performance compared with the CE1 anchors. In high efficiency case, the average BD bitrate reduction for Luma component is about 1.5% with about 24% increase in encoding time and about 5% increase in decoding time. If YUV output is skipped in decoding tests, the average decoding time increase is about 8%.

Table 18 Rounded-candidates based SDME performance (high efficiency)

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -1.93 | -1.50 | -1.66 |  |  |  |
|  | PeopleOnStreet | -2.80 | -3.64 | -3.40 |  |  |  |
| Class B | Kimono1 | -1.50 | -1.70 | -1.65 |  |  |  |
|  | ParkScene | -1.36 | -1.56 | -1.27 |  |  |  |
|  | Cactus | -1.88 | -1.60 | -1.79 |  |  |  |
|  | BasketballDrive | -1.12 | -1.43 | -1.42 |  |  |  |
|  | BQTerrace | -1.26 | -0.89 | -0.76 |  |  |  |
| Class C | BasketballDrill | -1.27 | -1.79 | -1.72 |  |  |  |
|  | BQMall | -2.22 | -2.39 | -2.39 |  |  |  |
|  | PartyScene | -1.11 | -1.31 | -1.27 |  |  |  |
|  | RaceHorses | -0.66 | -1.04 | -1.20 |  |  |  |
| Class D | BasketballPass | -1.94 | -2.65 | -2.18 |  |  |  |
|  | BQSquare | -0.99 | -0.88 | -0.36 |  |  |  |
|  | BlowingBubbles | -138 | -1.42 | -1.44 |  |  |  |
|  | RaceHorses | -1.35 | -1.93 | -2.10 |  |  |  |
| Average of Class A |  | -2.37 | -2.57 | -2.53 |  |  |  |
| Average of Class B |  | -1.42 | -1.44 | -1.38 |  |  |  |
| Average of Class C |  | -1.32 | -1.63 | -1.64 |  |  |  |
| Average of Class D |  | -1.42 | -1.72 | -1.52 |  |  |  |
| Overall Average |  | -1.52 | -1.72 | -1.64 | 124% | 105% | 108% |

In low complexity case, the average BD bitrate reduction for Luma component is about 1.2% with about 30% increase in encoding time and about 17% increase in decoding time. If YUV output is skipped in decoding tests, the average decoding time increase is about 18%.

Table 19 Rounded-candidates based SDME performance (low complexity)

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -1.31 | -1.25 | -1.02 |  |  |  |
|  | PeopleOnStreet | -2.10 | -2.22 | -2.24 |  |  |  |
| Class B | Kimono1 | -1.42 | -1.48 | -1.47 |  |  |  |
|  | ParkScene | -1.14 | -1.12 | -1.05 |  |  |  |
|  | Cactus | -1.46 | -1.18 | -1.49 |  |  |  |
|  | BasketballDrive | -1.17 | -1.31 | -1.38 |  |  |  |
|  | BQTerrace | -0.95 | -0.85 | -0.45 |  |  |  |
| Class C | BasketballDrill | -0.88 | -1.31 | -1.66 |  |  |  |
|  | BQMall | -1.53 | -1.24 | -1.60 |  |  |  |
|  | PartyScene | -0.90 | -1.28 | -1.15 |  |  |  |
|  | RaceHorses | -0.82 | -0.97 | -1.05 |  |  |  |
| Class D | BasketballPass | -1.47 | -1.78 | -1.99 |  |  |  |
|  | BQSquare | -1.09 | -1.27 | -0.30 |  |  |  |
|  | BlowingBubbles | -0.94 | -1.14 | -1.06 |  |  |  |
|  | RaceHorses | -1.07 | -1.39 | -1.43 |  |  |  |
| Average of Class A |  | -1.71 | -1.73 | -1.63 |  |  |  |
| Average of Class B |  | -1.23 | -1.19 | -1.17 |  |  |  |
| Average of Class C |  | -1.03 | -1.20 | -1.36 |  |  |  |
| Average of Class D |  | -1.15 | -1.39 | -1.19 |  |  |  |
| Overall Average |  | -1.22 | -1.32 | -1.29 | 130% | 117% | 118% |

## SDME with refinement

Table 20 and Table 21 show the performance of SDME with refinement compared with the CE1 anchors. In high efficiency case, the average BD bitrate reduction for Luma component is about 2.8% with about 35% increase in encoding time and about 21% increase in decoding time. If YUV output is skipped in decoding tests, the average decoding time increase is about 24%.

Table 20 Performance of SMDE with refinement (high efficiency)

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -3.17 | -2.36 | -2.48 |  |  |  |
|  | PeopleOnStreet | -4.11 | -4.93 | -4.73 |  |  |  |
| Class B | Kimono1 | -2.89 | -2.50 | -2.68 |  |  |  |
|  | ParkScene | -2.89 | -2.53 | -2.31 |  |  |  |
|  | Cactus | -3.80 | -3.54 | -3.40 |  |  |  |
|  | BasketballDrive | -2.44 | -2.46 | -2.46 |  |  |  |
|  | BQTerrace | -1.91 | -1.45 | -1.04 |  |  |  |
| Class C | BasketballDrill | -2.08 | -2.44 | -2.33 |  |  |  |
|  | BQMall | -4.06 | -3.75 | -3.80 |  |  |  |
|  | PartyScene | -2.25 | -1.96 | -2.15 |  |  |  |
|  | RaceHorses | -1.49 | -1.75 | -2.21 |  |  |  |
| Class D | BasketballPass | -3.39 | -3.63 | -3.44 |  |  |  |
|  | BQSquare | -2.30 | -1.65 | -1.96 |  |  |  |
|  | BlowingBubbles | -2.89 | -2.23 | -2.00 |  |  |  |
|  | RaceHorses | -2.80 | -3.13 | -3.15 |  |  |  |
| Average of Class A |  | -3.64 | -3.65 | -3.60 |  |  |  |
| Average of Class B |  | -2.78 | -2.49 | -2.38 |  |  |  |
| Average of Class C |  | -2.47 | -2.48 | -2.62 |  |  |  |
| Average of Class D |  | -2.85 | -2.66 | -2.64 |  |  |  |
| Overall Average |  | -2.83 | -2.69 | -2.68 | 135% | 121% | 124% |

In low complexity case, the average BD bitrate reduction for Luma component is about 2.2% with about 35% increase in encoding time and about 39% increase in decoding time. If YUV output is skipped in decoding tests, the average decoding time increase is about 44%.

Table 21 Performance of SMDE with refinement (low complexity)

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -2.39 | -1.55 | -1.51 |  |  |  |
|  | PeopleOnStreet | -3.46 | -3.46 | -3.43 |  |  |  |
| Class B | Kimono1 | -2.81 | -2.11 | -2.08 |  |  |  |
|  | ParkScene | -2.30 | -1.58 | -1.44 |  |  |  |
|  | Cactus | -3.22 | -2.61 | -2.56 |  |  |  |
|  | BasketballDrive | -2.36 | -1.94 | -2.11 |  |  |  |
|  | BQTerrace | -1.69 | -0.51 | -0.40 |  |  |  |
| Class C | BasketballDrill | -1.49 | -1.63 | -1.80 |  |  |  |
|  | BQMall | -2.78 | -2.06 | -2.35 |  |  |  |
|  | PartyScene | -1.25 | -1.06 | -1.18 |  |  |  |
|  | RaceHorses | -1.51 | -1.66 | -2.04 |  |  |  |
| Class D | BasketballPass | -2.64 | -2.68 | -2.53 |  |  |  |
|  | BQSquare | -1.31 | -0.71 | -0.74 |  |  |  |
|  | BlowingBubbles | -1.49 | -0.90 | -0.95 |  |  |  |
|  | RaceHorses | -2.19 | -2.21 | -2.35 |  |  |  |
| Average of Class A |  | -2.92 | -2.51 | -2.47 |  |  |  |
| Average of Class B |  | -2.48 | -1.75 | -1.72 |  |  |  |
| Average of Class C |  | -1.76 | -1.60 | -1.84 |  |  |  |
| Average of Class D |  | -1.91 | -1.63 | -1.64 |  |  |  |
| Overall Average |  | -2.19 | -1.78 | -1.83 | 135% | 139% | 144% |

The proponent marks that the encoder was compiled in Linux OS and was tested in a parallel computing farm system. The capability of the computing cores in the computing farm system is not identical, and the measured average encoding time ratio may not be identical per simulation. Therefore, the measurement of the encoding time should be served as one reference point. The decoder was complied with Visual Studio 2008 and tested on a single server with the following configurations: Windows Server 2003 Enterprise x64 Edition, Intel Xeon CPU E5335 @ 2.00GHz, 8GB RAM. The information from cross-check seems to indicate less amount of the increase in encoding time is measured

#### Comments/Remarks from Cross Checkers

### JCTVC-D098 (from Mitsubishi)

The cross-check report is at JCTVC-D098

- Mitsubishi performed cross-check on the SDME technique. The results will be provided as JCTVC-D098. The software package provided by Intel was successfully compiled and both encoding and decoding simulations were run onto Linux 64bit platform using workstations equipped with state-of-the-art Intel Xeon processors. BD performance perfectly matched with what Intel provided, and similar tendency could be found on encoding and decoding time. For decoding time, we obtained two types of values: one is the result obtained by running decoder within our PC cluster system, and the other is those obtained by closed local PC system. For the former case, we measured decoding time with “-o” option (with outputting YUV files). The measurement for the latter case was done without –o option. From our result in JCTVC-D098, decoding time does not seem to depend so much on measurement methods except for LC-RA case of “2-step refinement” (around 5% difference).

  - Subset 2: Template matching based DMVD

### Huawei

### Compression Performance Discussion

| Video Sequence | TM-DMVD+STDM |  |  |
| --- | --- | --- | --- |
|  | BD-rate Y(%) | BD-rate U(%) | BD-rate V(%) |
| Class B | -0.88 | -0.98 | -1.48 |
| Class C | -1.03 | -1.13 | -1.34 |
| Class D | -0.92 | -1.55 | -1.13 |
| Class E | -1.66 | -3.35 | -3.45 |
| Total average | 1.08 | -1.6 | -1.7 |

Table 22.	TMDMVD RD Performance in low delay high efficiency Configuration

Compared with CE1 anchor, the average gain for proposed techniques is about 1.08% bitrate saving.

| Video Sequence | TM-DMVD+STDM |  |  |
| --- | --- | --- | --- |
|  | BD-rate Y(%) | BD-rate U(%) | BD-rate V(%) |
| Class B | -0.48 | -0.35 | -0.32 |
| Class C | -0.28 | -0.69 | -0.71 |
| Class D | -0.23 | -0.45 | -0.36 |
| Class E | -0.82 | -0.33 | -0.52 |
| Total average | -0.43 | -0.46 | -0.47 |

Table 23.	TM-DMVD RD Performance in low delay low complexity Configuration

Compared with CE1 anchor, the average gain for proposed techniques is about 0.43% bitrate saving.

### Complexity Analysis

| Video Sequence | TM-DMVD+STDM (%) |  |  |
| --- | --- | --- | --- |
|  | Enc-Time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| Class B | 153.28% | 109.98% | 110.69% |
| Class C | 165.53% | 116.36% | 116.03% |
| Class D | 235.24% | 121.09% | 121.00% |
| Class E | 174.61% | 108.46% | 108.12% |
| Total Average | 178.22% | 113.96% | 114.022% |

Table 24.	TM-DMVD Complexity Analysis in low delay high efficiency Configuration

From Table 24, it can be seen that the decoding time of TM-DMVD is about 14% increasing over that of CE1 anchor.

| Video Sequence | TM-DMVD+STDM (%) |  |  |
| --- | --- | --- | --- |
|  | Enc-Time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| Class B | 151.86% | 120.99% | 121.14% |
| Class C | 163.20% | 124.85% | 126.44% |
| Class D | 231.29% | 127.52% | 132.13% |
| Class E | 197.12% | 112.11% | 113.96% |
| Total Average | 180.38% | 124.15% | 126.07% |

Table 25.	TM-DMVD Complexity Analysis in low delay low complexity Configuration

From Table 25, it can be seen that the decoding time of TM-DMVD is about 26% increasing over that of CE1 anchor.

### Comments/Remarks from Cross Checkers

### From Intel

JCTVC-D169 from Intel provides the cross-check results from Huawei. The software package obtained from proponents was successfully compiled and tested under the test conditions defined in CE1. The encoder was compiled in Linux OS and was tested in a parallel computing farm system. The capability of the computing cores in the computing farm system is not identical, and the measured average encoding time ratio may not be identical per simulation. Therefore, the measurement of the encoding time should be served as one reference point. The decoder was complied with Visual Studio 2008 and tested on a single server with the following configurations: Windows Server 2003 Enterprise x64 Edition, Intel Xeon CPU E5335 @ 2.00GHz, 8GB RAM.

The accompanied spreadsheet of JCTVC-D169 contains the detailed data of the results. The BD rate results perfectly match with the data provided by the proponents. The encoding time and decoding time do not achieve the identical result provided by proponent maybe due to the difference in the computing facility. The cross-check marks that the decoding time and encoding time produced by cross-checker has the tendency to be consistent with the data provided by proponents. It is noted that the decoding time was tested without the writing of YUV output.

## Subset 3: Per-pixel optical-flow based DMVD

JCTVC-D329 [Samsung] CE1 Samsung’s test for bi-directional optical flow

Table 26 and Table 27 show the bi-directional optical flow (BIO) performance compared TMuC 0.9. In random-access high efficiency case, the average BD bit-rate reduction for Luma component is 1.9% with about 12% increase in encoding time and 99% increase in decoding time.

Table 26 BIO performance (high efficiency)

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -1.9 | -0.6 | -0.6 |  |  |  |
|  | PeopleOnStreet | -1.5 | -0.9 | -0.8 |  |  |  |
| Class B | Kimono1 | -1.4 | -0.7 | -0.9 |  |  |  |
|  | ParkScene | -2.1 | -0.5 | -0.8 |  |  |  |
|  | Cactus | -2.0 | -1.2 | -1.0 |  |  |  |
|  | BasketballDrive | -1.0 | -0.3 | -0.3 |  |  |  |
|  | BQTerrace | -1.4 | -0.9 | -0.9 |  |  |  |
| Class C | BasketballDrill | -0.8 | -0.5 | -0.2 |  |  |  |
|  | BQMall | -2.8 | -1.9 | -1.7 |  |  |  |
|  | PartyScene | -1.8 | -1.6 | -1.7 |  |  |  |
|  | RaceHorses | -1.0 | -0.1 | -0.2 |  |  |  |
| Class D | BasketballPass | -2.0 | -0.8 | -0.7 |  |  |  |
|  | BQSquare | -4.2 | -2.5 | -2.3 |  |  |  |
|  | BlowingBubbles | -3.0 | -2.0 | -1.7 |  |  |  |
|  | RaceHorses | -1.9 | -0.6 | -0.8 |  |  |  |
| Average of Class A |  | -1.7 | -0.8 | -0.7 |  |  |  |
| Average of Class B |  | -1.6 | -0.7 | -0.8 |  |  |  |
| Average of Class C |  | -1.6 | -1.0 | -1.0 |  |  |  |
| Average of Class D |  | -2.8 | -1.5 | -1.4 |  |  |  |
| Overall Average |  | -1.9 | -1.0 | -1.0 | 112% | 199% | 273% |

In random-access low complexity case, the average BD bitrate reduction for Luma component is 4.8% with 11% increase in encoding time and about 150% increase in decoding time.

Table 27 BIO performance (low complexity)

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -2.7 | -0.8 | -0.7 |  |  |  |
|  | PeopleOnStreet | -1.5 | -1.2 | -1.2 |  |  |  |
| Class B | Kimono1 | -1.1 | -0.7 | -0.6 |  |  |  |
|  | ParkScene | -3.5 | -0.3 | -0.6 |  |  |  |
|  | Cactus | -3.3 | -1.4 | -0.8 |  |  |  |
|  | BasketballDrive | -1.8 | -0.4 | -0.5 |  |  |  |
|  | BQTerrace | -3.9 | -1.1 | -0.9 |  |  |  |
| Class C | BasketballDrill | -1.3 | -0.3 | -0.3 |  |  |  |
|  | BQMall | -4.3 | -1.7 | -2.0 |  |  |  |
|  | PartyScene | -10.6 | -4.5 | -4.3 |  |  |  |
|  | RaceHorses | -1.7 | -0.5 | -0.4 |  |  |  |
| Class D | BasketballPass | -2.7 | -1.3 | -1.1 |  |  |  |
|  | BQSquare | -22.8 | -10.2 | -10.1 |  |  |  |
|  | BlowingBubbles | -8.1 | -3.9 | -3.7 |  |  |  |
|  | RaceHorses | -2.7 | -0.5 | -0.3 |  |  |  |
| Average of Class A |  | -2.1 | -1.0 | -1.0 |  |  |  |
| Average of Class B |  | -2.7 | -0.8 | -0.7 |  |  |  |
| Average of Class C |  | -4.5 | -1.8 | -1.7 |  |  |  |
| Average of Class D |  | -9.1 | -4.0 | -3.8 |  |  |  |
| Overall Average |  | -4.8 | -1.9 | -1.8 | 111% | 248% | 465% |

### Comments/Remarks from Cross Checkers

### The cross-check result is at JCTVC-D121 from JVC-Kenwood

JCTVC-D121 from JVC KENWOOD provides the cross-check results of Samsung’s proposal.

The software provided by proponent was successfully compiled (using Win32/x64/Linux) and tested using Win32 executables under the CE1 common conditions.

The performance comparison results (BD-bitrate) by the cross-check are matched with the results of the Samsung’s proposal.

For the complexity comparison (encoding time/decoding time increase), the cross-check results are slightly different from the results of Samsung’s proposal. It seems that the difference is caused by the difference of the simulation environment (HW specification) and the complier (Win32/x64/Linux/…).

The decoder complexity comparison results of x64 executable are more close to the results of Samsung’s proposal than the results of Win32 executable on our simulation environment.

## Subset 4: JCTVC-D120 CE1: Refinement techniques based on transmitted (from JVC-Kenwood)

Table 28 Performance comparison results of random access high efficiency

|  | Random access High Efficiency |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y BD-rate | U BD-rate | V BD-rate |
| ClassA | Traffic | -1.4 | -1.0 | -1.3 |
|  | PeopoleOnStreet | -2.3 | -3.1 | -2.7 |
| ClassB | Kimono | -1.4 | -1.1 | -1.4 |
|  | ParkScene | -1.1 | -1.2 | -1.2 |
|  | Cactus | -1.2 | -1.2 | -1.2 |
|  | BasketballDrive | -2.2 | -2.1 | -2.2 |
|  | BQTerrace | -1.3 | -1.2 | -1.1 |
| ClassC | BasketballDrill | -1.9 | -2.2 | -1.7 |
|  | BQMall | -1.6 | -1.5 | -1.5 |
|  | PartyScene | -1.2 | -1.5 | -1.6 |
|  | RaceHorses | -2.0 | -1.9 | -2.3 |
| ClassD | BasketballPass | -2.2 | -2.3 | -1.8 |
|  | BQSquare | -0.5 | -0.3 | 0.3 |
|  | BlowingBubbles | -1.0 | -1.0 | -1.0 |
|  | RaceHorses | -2.3 | -2.1 | -2.4 |
|  | ClassA Average | -1.9 | -2.1 | -2.0 |
|  | ClassB Average | -1.4 | -1.4 | -1.4 |
|  | ClassC Average | -1.7 | -1.8 | -1.7 |
|  | ClassD Average | -1.5 | -1.5 | -1.2 |
|  | All | -1.6 | -1.6 | -1.5 |

Table 29 Performance comparison results of random access low complexity

|  | Random access Low Complexity |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y BD-rate | U BD-rate | V BD-rate |
| ClassA | Traffic | -1.1 | -0.8 | -0.8 |
|  | PeopoleOnStreet | -2.0 | -2.6 | -2.3 |
| ClassB | Kimono | -1.0 | -1.0 | -0.9 |
|  | ParkScene | -0.6 | -0.5 | -0.6 |
|  | Cactus | -1.2 | -1.0 | -1.1 |
|  | BasketballDrive | -1.8 | -1.2 | -1.4 |
|  | BQTerrace | -0.9 | -0.1 | 0.4 |
| ClassC | BasketballDrill | -1.5 | -1.3 | -1.3 |
|  | BQMall | -1.2 | -0.8 | -1.1 |
|  | PartyScene | -0.7 | -0.7 | -0.9 |
|  | RaceHorses | -1.4 | -1.3 | -1.3 |
| ClassD | BasketballPass | -1.8 | -1.6 | -1.1 |
|  | BQSquare | 0.0 | -0.1 | 0.2 |
|  | BlowingBubbles | -0.6 | -0.6 | -0.4 |
|  | RaceHorses | -1.6 | -1.2 | -1.2 |
|  | ClassA Average | -1.6 | -1.7 | -1.6 |
|  | ClassB Average | -1.1 | -0.8 | -0.7 |
|  | ClassC Average | -1.2 | -1.0 | -1.2 |
|  | ClassD Average | -1.0 | -0.9 | -0.6 |
|  | All | -1.2 | -1.0 | -0.9 |

Table 30 Performance comparison results of low delay high efficiency

|  | Low Delay High Efficiency |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y BD-rate | U BD-rate | V BD-rate |
| ClassB | Kimono | 0.0 | -0.2 | -0.2 |
|  | ParkScene | 0.0 | 0.1 | 0.0 |
|  | Cactus | 0.0 | 0.2 | 0.0 |
|  | BasketballDrive | -0.1 | -0.2 | 0.0 |
|  | BQTerrace | -0.2 | 0.4 | -0.2 |
| ClassC | BasketballDrill | 0.1 | -0.3 | 0.5 |
|  | BQMall | 0.0 | 0.4 | 0.5 |
|  | PartyScene | 0.0 | 0.3 | 0.0 |
|  | RaceHorses | -0.2 | -0.3 | -0.2 |
| ClassD | BasketballPass | 0.1 | 0.4 | -0.1 |
|  | BQSquare | 0.5 | -2.0 | 0.9 |
|  | BlowingBubbles | 0.2 | 0.2 | 0.2 |
|  | RaceHorses | -0.2 | -0.2 | 0.4 |
| ClassE | Vidyo1 | -0.1 | 1.4 | 1.6 |
|  | Vidyo2 | 0.4 | 0.4 | 1.1 |
|  | Vidyo3 | 0.1 | 0.6 | 0.9 |
|  | ClassB Average | -0.1 | 0.1 | -0.1 |
|  | ClassC Average | 0.0 | 0.0 | 0.2 |
|  | ClassD Average | 0.1 | -0.4 | 0.4 |
|  | ClassE Average | 0.1 | 0.8 | 1.2 |
|  | All | 0.0 | 0.1 | 0.4 |

Table 31 Performance comparison results of low delay low complexity

|  | Low Delay Low Complexity |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y BD-rate | U BD-rate | V BD-rate |
| ClassB | Kimono | 0.1 | -0.1 | 0.0 |
|  | ParkScene | 0.2 | 0.5 | 0.3 |
|  | Cactus | 0.2 | 0.1 | 0.0 |
|  | BasketballDrive | 0.0 | -0.1 | 0.1 |
|  | BQTerrace | 0.1 | 0.2 | 0.7 |
| ClassC | BasketballDrill | 0.3 | -0.1 | 0.3 |
|  | BQMall | 0.2 | -0.1 | 0.2 |
|  | PartyScene | 0.2 | 0.3 | 0.3 |
|  | RaceHorses | 0.1 | 0.1 | 0.1 |
| ClassD | BasketballPass | 0.2 | 0.4 | 0.3 |
|  | BQSquare | 0.5 | 1.0 | 0.4 |
|  | BlowingBubbles | 0.1 | 0.5 | 0.2 |
|  | RaceHorses | 0.1 | 0.3 | 0.4 |
| ClassE | Vidyo1 | 0.4 | 0.3 | 0.9 |
|  | Vidyo2 | 0.4 | 0.7 | 0.0 |
|  | Vidyo3 | 0.2 | 0.7 | 0.5 |
|  | ClassB Average | 0.1 | 0.1 | 0.2 |
|  | ClassC Average | 0.2 | 0.1 | 0.2 |
|  | ClassD Average | 0.2 | 0.6 | 0.3 |
|  | ClassE Average | 0.4 | 0.5 | 0.5 |
|  | All | 0.2 | 0.3 | 0.3 |

The simulation results show that the proposed technique provides 1.6% BD-rate gain for random access high efficiency case and 1.2% for random access low complexity case compared to the HM (TMuC0.9 anchor).

On the other hand, the results show that the proposed technique does not provide BD-rate gain for low delay case.

### Complexity comparison

For the complexity comparison of decoder, two results of decoding time are used for comparison. One is decoding time with writing out decoded yuv(with –o option), and other is decoding time without writing out decoded yuv (without –o option).

In the case of random access setting, the results of decoding time using x64 (64bit executable) are added to observe the influence of different platform (32bit/64bit).

Table 32 shows the complexity comparison results of random access.

Table 33 shows the complexity comparison results of low delay.

Table 32 Complexity comparison results of random access

|  | Condition | Random access High | Random access LoCo |
| --- | --- | --- | --- |
| Enc Time | Win32 Executable | 152% | 147% |
| Dec Time | Win32 Executable with "-o" option | 139% | 147% |
|  | Win32 Executable without "-o" option | 141% | 163% |
|  | x64 Executable with "-o" option | 137% | 135% |
|  | x64 Executable without "-o" option | 139% | 161% |

Table 33 Complexity comparison results of low delay

|  | Condition | Low Delay High | LowDelay LoCo |
| --- | --- | --- | --- |
| Enc Time | Win32 Executable | 105% | 105% |
| Dec Time | Win32 Executable with "-o" option | 107% | 110% |
|  | Win32 Executable without "-o" option | 108% | 115% |

It shows that this proposal increases by 50% compared with HM in the aspect of encoding/decoding time in the case of random access setting. It seems that the complexity of decoder is not depend on the platform.

### Comments/Remarks from Cross Checkers

Declared by proponent performance is totally matched. Decoder/encoder time ratio has slight difference with our measurement. It can be explained by OS and compiler distinction analysis.

# Conclusion

Based on the observed results, it is suggested to review the result of the proposal with the JCT-VC committee, and to make recommendation to adopt the proposed technologies onto the HEVC Test Model (HM) software for further investigation.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCTVC-C500, Guangzhou, October 2010.

[2]	Yi-Jen Chiu, Haoping Yu, Yu-Wen Huang, Shun-ichi Sekiguchi, “CE1: Decoder-side motion vector derivation,” JCT-VC Document, JCTVC-C501_r1, Guangzhou, October 2010.