| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D178 WG11 number: m18932 |
| --- | --- |

| Title: | Impact of cascaded coding on HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | A. Gabriellini D. Flynn T.J. Davies | Tel: Email: | +44.303 0409619 andrea.gabriellini@bbc.co.uk |
| Source: | BBC Research and Development |  |  |

_____________________________

# Abstract

This contribution explores the impact of cascaded coding on the performance of HEVC coding, where the prior coding is representative of that typically used for high-end capture and playout in broadcast systems. JM 16.2 is used as an anchor reference to evaluate the coding performance of HM 0.9 when fed both processed and unprocessed class B pictures. HM in its default configuration demonstrates very good performance when applied to processed material, with a larger BD-rate gain than for unprocessed pictures. A second test shows the importance of the adaptive loop filter (ALF) in ensuring the positive results highlighted by the first test. The comparison between HM with and without ALF, with both sets of inputs (pristine and processed material) shows how the BD-rate increase for HM with ALF is 2.0% higher for Random Access when the input material has coding artefacts.

# Problem Statement

The development of HEVC has been based on the use of largely pristine sequences, devoid of compression artefacts. Real-life use of any video compression tool, however, requires dealing with content that has already being processed by other codecs, perhaps multiple times, as cascaded coding is known to generate more artefacts. It is therefore desirable to understand the coding performance of the new standard under those conditions.

If HEVC demonstrates good performance with cascaded coding it is also desirable to understand whether there is any feature that is particularly important in ensuring these gains. This contribution considered only one such feature, ALF.

# Setup

Input

Sequences

(HD)

HM 0.9

JM 16.2

HM 0.9

JM 16.2

HDCAM SR

AVC/H.264

I-100

P

B

C

D

A

O

O

Figure 1 – First Test Setup

To investigate the effect of cascaded coding seven HD sequences have been selected, the five test sequences used for the development of HEVC (basketballdrive, bqterrace, cactus, kimono and parkscene) and two sequences supplied by NHK, steam-train and nebuta-festival. The NHK sequences originate in Super-High-Vision format, 7680*4320, and were downconverted to 1920x1080 format for the test.

All seven sequences are fed through a simple production chain consisting of two stages, a Sony HDCAM SR followed by an AVC-I coder. The AVC-I coding was achieved by conversion to HD-SDI format at a nominal frame rate of 25 and bit rate of 100Mb/s; this bit-rate is therefore scaled by the appropriate frame rate for each sequence.

The processed sequences have been visually inspected and no compression artefacts were noticeable. See Table 1 for the PSNR values of the 7 processed sequences.

| Sequence | Y-PSNR | Cb-PSNR | Cr-PSNR |
| --- | --- | --- | --- |
| Nebuta Festival | 37.431 | 39.749 | 39.733 |
| Steam Train | 43.919 | 48.172 | 49.015 |
| Kimono | 44.980 | 46.824 | 47.660 |
| Parkscene | 41.627 | 43.843 | 44.439 |
| Cactus | 39.491 | 42.363 | 44.154 |
| BasketballDrive | 43.394 | 44.173 | 44.951 |
| BQTerrace | 39.957 | 42.719 | 44.295 |

Table 1 - PSNR values of the processed sequences (Point P against O in Figure 1)

Two sets of coding runs using random access high-efficiency (RA-HE) and low-delay high-efficiency (LD-HE) configurations were carried out on both sets of input, original and processed, as shown in Figure 1. Initially, the HM coding parameters were exactly as specified in the common conditions [1] and JM quantisation parameters were chosen in order to match the resulting bitrates for the two coders. However, for the NHK sequences the resulting PSNRs for HM coding were too high for a realistic test, being close to or higher than the PSNR generated by the pre-coding. Therefore QP=22 was removed for these sequences and QP=42 added for a more realistic test.

The first test compared the performance of HM 0.9 against JM 16.2 with both sets of input. In particular, BD-rate (PSNR(A-O), PSNR(B-O)) vs BD-rate (PSNR(C-P), PSNR(D-P)).

The second test did not use JM. It used two versions of HM 0.9 differing in one element only, the presence of ALF.

Both tests ran over the full length of sequences.

# Results

|  |  | Random access |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Original Sequences | -41.5 | -36.8 | -37.6 | -37.0 | -8.0 | -4.1 |
| Processed Sequences | -45.0 | -40.3 | -41.4 | -39.1 | -6.8 | -2.4 |

Table 2 - First Test: HM 0.9 vs JM 16.2

Table 2 shows the results of the first test for the two configurations used, random access high-efficiency and low-delay high-efficiency. Negative values of the BD-rate show the performance improvement of HM 0.9 over JM 16.2. The results show how HM performance improvement over JM increases by 3.5% (RA-HE) and 2.1% (LD-HE) if the input contains coding artefacts.

|  |  | Random access |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Original Sequences | 6.7 | 13.9 | 11.7 | 7.6 | 12.4 | 11.2 |
| Processed Sequences | 8.7 | 12.1 | 10.2 | 7.4 | 11.5 | 9.0 |

Table 3 - Second Test: HM 0.9 no-ALF vs HM 0.9 with-ALF

The second test aims to identify the main feature in HM providing the gains showed in the first test when fed material with coding artefacts.

shows the BD-rate loss HM 0.9 incurs when ALF is switched off. The loss increases when processed material is used in input. This test seems to suggest that ALF is responsible for more than half of the additional RA-HE gain on processed sequences shown in Table 2.

ALF appeared to have negligible impact in the LD-HE case.

# Conclusions

The coding chain used is very benign, but gives a significantly different BD-rate gain. More heavily compressed inputs might be expected to have a larger effect. Although it is not recommended that pre-compressed pictures be added to the standard test set, it will be useful to evaluate performance with some pre-compressed pictures from time to time. A preliminary analysis suggests that ALF is one tool that is particularly useful for processed pictures. Other tools can be tested and identified similarly.

# References

[1] JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C500, JCT-VC Meeting, Guangzhou, October 2010

# Patent rights declaration(s)

The British Broadcasting Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).