| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D180 |
| --- | --- |

| Title: | Hardware Friendly Rotational Transform (Related to CE7) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xing Wen, Oscar C. Au, Jiali Li, Lin Sun, Feng Zou, Chao Pang, Jingjing Dai Department of Electronic and Computer Engineering Hong Kong University of Science and Technology Kowloon, Hong Kong | Tel: Email: | 852-6626 1845 wxxab@ust.hk eeau@ust.hk |
| Source: | Hong Kong University of Science and Technology |  |  |

_____________________________

# Abstract

The 8x8 Rotational Transform (ROT) is a secondary transform after DCT and is implemented in the HEVC Test Model (HM). Although ROT can achieve superior coding efficiency compared to only use DCT, original ROT design cannot be efficiently implement by hardware. In this proposal, we propose a hardware friendly rotational transform (HF-ROT) which can achieve similar coding efficiency gain and also can be easily implemented by hardware. We designed four HF-ROT modes, which can achieve very regular data flow and saving many clock cycles to finish the rotational transform operation. The most important property of HF-ROT is that it takes same clock cycles to computer all parameters in transformed block. With averaged BD-rate of 0.175% this technique can save 53% and 35% reduction in the elemental-adder count for forward and inverse ROT implementations respectively, and can save 51% and 62% reduction in the clock cycles count for forward and inverse ROT implementations respectively.

# Introduction / Problem Statement

## Rotational Transform

The secondary 8x8 Rotational Transform (ROT) [1][2] is applied after DCT, the block diagram is shown in Fig. 1.

Figure 1: ROT is applied after the DCT

The ROT transfomed block mo can be obtained by following equation:

where mi is the input residue block, D is DCT matrix, Rh and Rv are horizontal ROT and vertical ROT matrices respectively.

The matrices Rh and Rv are defined as the following compund Given’s rotation matrix products:

which Rh and Rv can are in following forms:

In TMuC 0.9, there are four sets of rotation angles θ1i, θ2i, θ3i, θ4i, θ5i, θ6i (i = 1 to 4) which equals to four sets of Rh and Rv respectively.

In TMuC, four sets of ROT matrics are stored in following forms:

in which:

- “1->” means mode 1, each mode only store 36 non-zero coefficients in Rh and Rv , and the scanning order is shown above

- g_FWD_ROT_MATRIX_8 is used to store all coefficients for inverse transform, g_INV_ROT_MATRIX_8 stores all coefficients for forward transform.

The forward ROT process can be described as follows:

*	      *

## Hardware implementation

From Section 1.1, we notice that ROT transform matrices do not have similar property as DCT transform matrices. For DCT, many fast algorithms and hardware architectures are designed based on butterfly design. But it is hard to apply same strategy to ROT.

Table 1 shows binary number for all coefficients in forward ROT matrices (four modes with index 0~3). We notice that in each mode, all non-zero coefficients have different length, and different number of “1”s. And Rh is not the transpose of Rv.

Table 1 Binary numbers of all coefficients in Rv and Rh

If we do not adopt any fast algorithm for ROT, we use following simply architecture to give an example.

Figure 3 Example of ROT architecture

In forward transform, since Rv only has three non-zero coefficients in first row. So the first step is to calculate:

m（1,1）= Rv(1,1)*T(1,1) + Rv(1,2)*T(2,1) + Rv(1,3)*T(3,1)

which m (i, j) is the ijth coefficient in temporal result matrix after Rv finish computation. Normally, we would use some architecture like Fig .3 to implement such operations. However, due to the variance of length of ROT coefficients, such direct implementation would result in irregular data-flow and lower the efficiency of hardware. For example, at Mode 2, Rv(1, 1) is “111100111111” which needs 10 times shift operation and 9 times add operation, on the other hand, Rv(1, 3) is “1100100010” which only needs 4 times shift operation and 3 times add operations.

Table 2 Shift and add number for mode 2 (index 1)

If we assume add and shift operation take one clock cycle each, then we could observe that Rv(1, 3) only take 7 clock cycles while Rv(1, 1) needs 19 clock cycles to finish computation. If we use architecture in Fig .3 to compute m(1,1), the multiplier use Rv(1, 3) must wait the multiplier which take Rv(1, 1) for 19-7=12 clock cycles. This would result in irregular data-flow and lower the data throughput for entire system.

# Hardware Friendly Rotational Transform (HF-ROT)

In order to solve the problems we described above, and also achieve similar coding gain compared to original ROT, we propose a hardware friendly rotational transform (HF-ROT).

## HF-ROT Design

In this section, we describe our proposed HF-ROT and the design flow. The proposed method can achieve similar coding gain compared to original ROT, but also can be efficiently implemented by hardware, lower the complexity, and can easily cooperate with DCT architectures. On the other hand, the proposed design flow can be utilized by other matrix operations or transforms which cannot directly adopt butterfly design (for example KLT, MDDT).

Figure 4 Proposed HF-ROT matrices design flow

Fig .4 shows our proposed design flow. The steps are described as follows:

Step 1:  coefficient of original inverse ROT matrices is input to get absolute value, and sign_bit is stored

Step 2:  the absolute value is mapping to 2a, and a is rounded to get integer shift_bits with certain strategy and certain quantization offset. The offset can be different for different coefficients.

Step 3:  the quantization error is input to the loop again and repeat step 1 and 2.

Step 4:  repeat step 1 and 2 for n times and then get the “n-set” inverse HF-ROT matrix

Step 5:  use inverse of “n-set” inverse HF-ROT matrix as new forward HF-ROT matrix

Step 6:  loop step 1 and 2 for m times with new forward HF-ROT matrix and get “m-set” forward HF-ROT matrix

There are three benefits for our proposed HF-ROT compared to original ROT.

- All coefficients inside transform matrices would require same clock cycle to finish multiplication operations, which result in very regular data flow.

- Furthermore, m and n do not need to be identical, and can be adjust to achieve different trade-off between data throughput and coding gain.

- According to our simulation results, the HF-ROT achieves similar coding gain as original ROT.

# Complexity Analysis

In this section, we would analysis the complexity of original ROT and proposed HF-ROT. In typical hardware implementations, multiplication is achieved with adders and shifters. Table 1 and 3 show the binary numbers for forward and inverse ROT matrices respectively.

For each coefficient which has l non-zero bits to multiply DCT coefficient, it would need l times shift operations according to each non-zero bit position, and l-1 times add operations. According to Table 1 and 3, the numbers of add operations are shown in Table 3. The original ROT would take 836 add operations for forward transform and 600 for inverse transform. HF-ROT (m=3, n=3) only takes 386 and 388 add operations for forward and inverse transform respectively, compared to original HF-ROT save 53% and 35% add operations for forward and inverse transform respectively. HF-ROT (m=5, n=3) takes 582 and 388 add operations for forward and inverse transform respectively.

Table 3 number of add operations for ROT and HF-ROT

|  | Forward Transform | Inverse Transform |
| --- | --- | --- |
| ROT | 836 | 600 |
| HF-ROT (m=3 n=3) | 386 | 388 |
| HF-ROT (m=5 n=3) | 582 | 388 |

We assume shift and add operation both take 1 clock cycle. And if we assume they compute all non-zero coefficient at same time for each mode. So the latency to get result coefficients would be decided by the coefficient which has most “1”s (highlighted by red color in Table 4 and 5) among all non-zero coefficients. For example, we can only get all result coefficients when finishing the multiplication of Rv(2, 2) since it has nine “1”s. So it would take 37+37+37+37-4-4 = 140 clock cycles to finish computation for mode 1~4. On the other hand, it only takes 4*(4m-2) clock cycles to finish computation for mode 1~4. When m = 3, it would take 40 clock cycles and save 71% compared to original ROT. When m = 5, it would take 72 clock cycles and save about 50% compared to original ROT.

Table 4 Binary numbers for forward transform coefficients

For inverse transform, it takes 30+24+26+20 = 100 clock cycles to compute results for four modes. And HF-ROT only takes 4*(4n-2) clock cycles. When n = 3, it takes 40 and save 60% compared to original ROT.

Table 5 Binary numbers for inverse transform coefficients

# Simulation Results

We have conducted simulation on TMuC 0.9 using the two intra configurations, i.e., high-efficiency and low-complexity, and the two random access configurations from the common coding configurations [3].

Table 6 shows the BD-Bitrate [6] results for our proposed HF-ROT (m=5, n=3). The anchor is original ROT (TMuC 0.9 with setting “ROT = 1” in configuration file.). And we could find the averaged BD-Bitrate increase is about 0.25% for “all intra” test condition and 0.1% for random access. The decoding time increase is due to different hardware configuration for our simulation PCs. For test performance in detail, please see attached JCTVC-D180.xls.

Table 6 Results comparing HF-ROT and original ROT

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 0.3 | 0.2 | 0.1 | 0.4 | 0.2 | 0.2 |
| Class B | 0.2 | 0.2 | 0.2 | 0.4 | 0.3 | 0.3 |
| Class C | 0.1 | 0.0 | 0.0 | 0.2 | 0.1 | 0.1 |
| Class D | 0.1 | 0.0 | 0.0 | 0.2 | 0.1 | 0.1 |
| Class E | 0.2 | 0.1 | 0.0 | 0.3 | 0.2 | 0.1 |
| All | 0.2 | 0.1 | 0.1 | 0.3 | 0.2 | 0.2 |
| Enc Time[%] | 106% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 118% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 0.1 | 0.1 | 0.1 | 0.1 | 0.0 | 0.2 |
| Class B | 0.1 | 0.3 | 0.1 | 0.1 | 0.0 | 0.1 |
| Class C | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 | -0.1 |
| Class D | 0.0 | -0.3 | -0.1 | 0.1 | 0.0 | 0.2 |
| Class E |  |  |  |  |  |  |
| All | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 |
| Enc Time[%] | 86% |  |  | 100% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |

Table 7 shows the BD-Bitrate [6] results for our proposed HF-ROT. The anchor is TMuC 0.9 with ROT off (TMuC 0.9 with default setting “ROT = 0” in configuration file.). Since we used different computers to run the simulation, so the run times are not precise so we would not show them.

Table 7 Results of HF-ROT vs TMuC 0.9 anchor

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | -2.5 | -0.3 | 0.8 | -3.4 | -0.9 | 0.5 |
| Class B | -1.9 | 0.1 | 0.8 | -2.8 | -0.7 | 0.0 |
| Class C | -2.6 | -0.6 | -0.6 | -3.3 | -1.8 | -1.9 |
| Class D | -2.5 | -0.5 | -0.7 | -3.2 | -1.7 | -2.2 |
| Class E | -2.6 | 3.1 | 1.8 | -3.6 | 3.2 | 1.3 |
| All | -2.4 | 0.3 | 0.3 | -3.2 | -0.5 | -0.7 |
| Enc Time[%] | -- |  |  | -- |  |  |
| Dec Time[%] | -- |  |  | -- |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | -1.1 | 0.1 | 0.9 | -1.3 | -0.3 | 0.5 |
| Class B | -1.0 | 0.2 | 0.6 | -1.2 | -0.4 | 0.0 |
| Class C | -1.3 | -0.6 | -0.5 | -1.3 | -0.9 | -1.1 |
| Class D | -1.1 | -0.4 | -0.3 | -1.0 | -0.6 | -0.7 |
| Class E |  |  |  |  |  |  |
| All | -1.1 | -0.2 | 0.1 | -1.2 | -0.6 | -0.4 |
| Enc Time[%] | -- |  |  | -- |  |  |
| Dec Time[%] | -- |  |  | -- |  |  |

And we also tested the original ROT performance on TMuC 0.9 and also submitted as a contribution [5] (JCTVC-D222), and Table 8 shows the BD-Bitrate results for original ROT on TMuC 0.9.

Table 8  Results comparing ROT on and off on TMuC 0.9

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | -2.8 | -0.5 | 0.7 | -3.8 | -1.0 | 0.2 |
| Class B | -2.1 | -0.1 | 0.6 | -3.2 | -1.0 | -0.3 |
| Class C | -2.8 | -0.6 | -0.6 | -3.5 | -1.9 | -2.0 |
| Class D | -2.7 | -0.5 | -0.7 | -3.3 | -1.8 | -2.3 |
| Class E | -2.8 | 3.0 | 1.8 | -3.9 | 3.0 | 1.1 |
| All | -2.6 | 0.2 | 0.2 | -3.5 | -0.7 | -0.8 |
| Enc Time[%] | -- |  |  | -- |  |  |
| Dec Time[%] | -- |  |  | -- |  |  |

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | -1.2 | 0.0 | 0.8 | -1.5 | -0.3 | 0.4 |
| Class B | -1.1 | -0.1 | 0.5 | -1.3 | -0.4 | -0.2 |
| Class C | -1.3 | -0.6 | -0.6 | -1.4 | -0.9 | -1.0 |
| Class D | -1.1 | -0.1 | -0.2 | -1.1 | -0.6 | -0.9 |
| Class E |  |  |  |  |  |  |
| All | -1.2 | -0.2 | 0.1 | -1.3 | -0.6 | -0.5 |
| Enc Time[%] | -- |  |  | -- |  |  |
| Dec Time[%] | -- |  |  | -- |  |  |

# Conclusion

This contribution presents experimental results for our proposal on a hardware friendly ROT. With our proposed “m-set” and “n-set” transform matrices, the HF-ROT can achieve very regular data flow and save significant operations, and easy to be efficiently implemented by hardware. At same time, the HF-ROT can achieve similar coding gain compared to original ROT. We recommend adopting this proposal into HM for further study it in CE7. We also recommend the ROT for the next version of the HEVC test-model.

# Patent rights declaration(s)

Hong Kong University of Science and Technology may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] 	JCT-VC, “Test Model under Consideration”, JCTVC-B205, Joint Collaborative Team on Video Coding meeting,  July 2010,  Geneva, Switzerland.

[2] 	“Samsung’s Response to the Call for Proposals on Video Compression Technology”, JCTVC-A124, Joint Collaborative Team on Video Coding Meeting, April 2010, Dresden, Germany.

[3]    F. Bossen, “Common test conditions and software reference configurations,” in JCTVC-C500, Guangzhou, China, Oct. 2010.

[4] 	G. Bjøntegaard, “Calculation of Average PSNR Differences between RD curves,” in ITU-T SC16/Q6, VCEG-M33, Austin, USA, Jan. 2004.

[5] 	“Performance of Rotational Transform (ROT) in TMuC 0.9”, JCTVC-D222, Joint Collaborative Team on Video Coding Meeting, January 2011, Daegu, Korea.