| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D183 |
| --- | --- |

| Title: | CE8 Subset3: Cross verification on Picture Quadtree Adaptive Offset (JCTVC-D122) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Il-Koo Kim Tammy Lee | Email: | ilkoo.kim@samsung.com tammy.lee@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution document contains cross-check results of JCTVC-D122 [1] on Picture Quadtree Adaptive Offset (PAQO) submitted by MediaTek. The cross-check results match those proposed in JCTVC-D122. Attached spreadsheet contains detailed data of the results.

# Picture Quadtree Adaptive Offset (PQAO)

Picture Quadtree Adaptive Offset (PQAO) is a picture-level adaptive offset compensation method to minimize distortion between original and reconstruction picture. After partitioning reconstructed pixels into quadtree in recursive manner, pre-calculated offsets are added to reconstructed pixels using 4 different compensation methods (uniform band offset, non-uniform band offset, cross pattern edge offset and diagonal cross pattern edge offset) in order to reduce the deviation from original pixels.

Encoding of PQAO consists of 3 steps: pixel statistic calculation, rate-distortion optimization and pixel offset compensation. After getting pixel statistics, optimal partitioning structure and type of compensation is determined in recursive manner. Offsets of each type also calculated in the rate-distortion optimization process. At the final stage, offsets are added to reconstructed pixels according to determined structure and type of compensations.

The structural information, type of compensation of each quadtree blocks and offset of each type of compensation are sent to decoder as additional information. At the decoder, based on the received information, pixel offset compensation process is performed in the same way at encoder.

# Result

When compared with the anchor defined in JCTVC-C500, PQAO achieve -0.8%, -1.1%, -1.5%, -1.3%, -2.2 and -2.7% BD-rate reductions for Intra High Efficiency (I-HE), Intra Low Complexity (I-LC), Random Access High Efficiency (RA-HE), Random Access Low Complexity (RA-LC), Low Delay High Efficiency (LD-HE), and Low Delay Low Complexity (LD-LC), respectively. The average encoding and decoding times are103% and 105% for all 6 configurations. Summarized results are shown in Table 1.

Table 1. Summary of BD-rate reduction and encoding/decoding time for 6 configurations

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.3 | 0.1 | 0.1 | -1.9 | 0.1 | 0.1 |
| Class B | -0.6 | 0.1 | 0.1 | -0.8 | 0.2 | 0.2 |
| Class C | -0.9 | 0.0 | 0.0 | -1.0 | 0.2 | 0.2 |
| Class D | -0.8 | 0.0 | 0.0 | -0.8 | 0.1 | 0.1 |
| Class E | -0.9 | 0.2 | 0.2 | -1.5 | 0.3 | 0.3 |
| All | -0.8 | 0.1 | 0.1 | -1.1 | 0.2 | 0.2 |
| Enc Time[%] | 105% |  |  | 107% |  |  |
| Dec Time[%] | 107% |  |  | 107% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.6 | 0.0 | 0.2 | -1.3 | -0.1 | 0.0 |
| Class B | -1.5 | 0.1 | 0.2 | -1.7 | 0.2 | 0.0 |
| Class C | -1.3 | 0.0 | 0.0 | -1.2 | 0.2 | 0.1 |
| Class D | -1.5 | 0.2 | 0.2 | -0.8 | 0.3 | 0.5 |
| Class E |  |  |  |  |  |  |
| All | -1.5 | 0.1 | 0.2 | -1.3 | 0.2 | 0.2 |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 102% |  |  | 108% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.7 | 0.2 | 0.2 | -2.8 | 0.6 | 0.6 |
| Class C | -2.6 | 0.1 | 0.0 | -2.6 | 0.3 | 0.4 |
| Class D | -2.3 | -0.4 | 0.4 | -1.4 | -0.2 | -0.2 |
| Class E | -2.7 | 0.4 | 0.7 | -4.1 | 1.8 | 1.5 |
| All | -2.2 | 0.1 | 0.3 | -2.7 | 0.6 | 0.5 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 104% |  |  |

# Conclusions

PQAO from MediaTek showed -1.6% BD-rate reduction in average with at most 5% encoding and decoding time increase. The BD-rate reduction and encoding/decoding time were confirmed. We’d like to recommend the adoption of PQAO in HEVC Test Model (HM).

# References

- Chih-Ming Fu, Ching-Yeh Chen, Yu-Wen Huang, and Shawmin Lei, CE8 Subtest3: Picture Quadtree Adaptive Offfset, in 4th JCT-VC meeting , no. JCTVC-D122, Jan. 2011.