| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D184 |
| --- | --- |

| Title: | Reduced redundancy of the low complexity entropy coder (LCEC) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hisao Sasai, Takahiro Nishi | Tel: Email: | +81 669009689 sasai.hisao@jp.pansonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

In the current HEVC specification[1], a joint adaptive VLC coding is used for reference frame index and inter prediction direction in the low complexity entropy coder (LCEC). As the total number of reference frame index is known at the picture level, it is redundant to use unlimited number of VLC table index for the coding of reference frame index. This contribution presents an experimental result in which the number of VLC table index is determined by the total number of reference frame index and used stationary at the PU level. Compared to TMuC0.9-hm anchors, such a change leads to 0.3% bit-saving for random-access case and 0.0 % bit-saving for low-delay cases without any significant complexity incensement, on average.

# Introduction

Some of joint coding structures have been introduced in the low complexity entropy coder (LCEC) in order to achieve superior coding efficiency. However, there is still remaining a redundancy in the joint coding of “reference index” and “inter prediction mode”. Specifically, even though the necessary total number of VLC table entry is known at the picture level, but it is not utilized to restrict the number of entry for the VLC table.

# Solutions

Figure 1 shows the overview of the proposed method. In the method, the number of entry in the VLC tables is restricted depending on “Max Range” which indicates the dynamic range of the variable for the target parameters. Specifically, it is calculated from the number of entry in the reference picture list 0/1 and NoBackPredFlag as the steps written in below.

Figure 1: Proposed method for VLC table selection.

Picture Level

For the joint coding of “reference index” and “inter prediction mode”, the dynamic range of the variable length code (m_uiMaxRange) is derived by the following pseudo code:

if(NoBackPredFlag)

m_uiMaxRange = NumRefIdx(REF_PIC_LIST_0)+ NumRefIdx(REF_PIC_LIST_0)* NumRefIdx(REF_PIC_LIST_1);

else

m_uiMaxRange = NumRefIdx(REF_PIC_LIST_0)+ NumRefIdx(REF_PIC_LIST_1)+ NumRefIdx(REF_PIC_LIST_0)* NumRefIdx(REF_PIC_LIST_1);

A VLC table for the joint coding of “reference index” and “inter prediction mode” is selected based on the value m_uiMaxRange. As a result, the last bit of the VLC code for the parameters can be truncated, for example.

block (PU) Level

The target signal, which is a combination of “reference index” and “inter prediction mode”, is coded by using the selected VLC table.

- Experimental results

Coding performance of the proposed method relative to the TMuC 0.9-hm software was checked under the common test condition described in JCTVC-C500[2] .
The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3.

Table 1 : BD-Rate for Simplified VLC table selection (Anchor: TMuC0.9-hm)

Figure 2 : BD-Rate for Random access on each sequence

# Conclusions

In this contribution, a redundancy reduction is proposed for the joint coding of reference frame index and inter prediction mode. An experimental result shows that this modification provides improved coding performance for all test sequences consistently (0.3% in average) without complexity increase. As the changes are quite small and theoretically natural, it is proposed to adopt these changes in HM/WD of HEVC.

# References

- JCT-VC, “WD1: Working Draft 1 of High-Efficiency Video Coding”, JCTVC-C403, JCT-VC 3rd Meeting, Guangzhou, October, 2010.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-C500, JCT-VC 3rd Meeting, Guangzhou, October 2010.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).