| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D185 |
| --- | --- |

| Title: | Simplified Context modeling for Transform Coefficient Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hisao Sasai Takahiro Nishi | Tel: Email: | +81 6900 9689 sasai.hisao@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

Comparing with the AVC specification, increased number of context models is used in the latest HEVC specification. In general, it is desired to avoid unnecessary increase of the implementation complexity. The number of context models affects on the amount of memory size especially for a hardware implementation and computational cost for the initialization of each context model. In this contribution, it is proposed that a simplified context modeling for significant map and last flag. Proposed modifications reduce the number of context model by 46 % (significant map) and 75% (last flag) with 0.1% to 0.2% coding loss relative to the TMuC0.9-hm anchor.

# Introduction

For improved coding efficiency, HHI_TRANSFORM_CODING[1] is taking a highly adaptive context selection approach for significance map and last flag, while increased number of context model is required in comparison with the AVC specification. The number of context model for significant maps and last flags in the TMuC0.9-hm software is shown in Table 1.

In the AVC High profile, “460” indexes are defined for all slice type. In the HEVC software, “758” indexes are defined for each slice type and it is noticed that its 59%, i.e. 448 indexes, is used for Significant Map and Last Flag.

As the number of context model affects on the amount of memory size especially for a hardware implementation and computational cost for the initialization of each context model, we tried to reduce the number to the comparable level with that for the AVC High profile.

Table 1 : Number of the context modeling for Transform Coefficient Coding

# Proposed Solution

To simplify the HHI_TRANSFORM_CODING, we modify the context model derivation process for Significant Map and Last Flag as follows:

Significant Map

For greater block sizes than 8x8, HHI_TRANSFORM_CODING refers significant flags of neighboring frequency components for the higher frequency components, which are referred as top, left and center in Figure 1. In the proposed solution, they are also referred for 8x8 blocks, i.e. done for 8x8, 16x16 and 32x32 blocks. For the higher frequency components, the context index is derived from the count of surrounding significant flags as shown in Figure 2 and Table 2. For greater block size than 8x8, those context models can be grouped since those components have similar characteristics on each condition of the index. On the other hand, for low frequency components (2x2 coefficients), the context index is derived from the position of the coefficients since those components has different characteristics.

Figure 1 : Context Index of the Significant map for the Lowest Frequency components

Figure 2 : Context Index of the Significant map for Higher Frequency components

Table 2 : High Frequency Part Separation

For greater block sizes than 8x8, the context index belongs to each position of coefficients as shown in  Figure 3. As the results of above simplification, the total number of context model for Significant Map becomes “124” (55% of current HEVC) as shown in Table3.

Figure 3 : Context Index of 2x2 and 4x4 Significant map

Table 3 : Number of the context modeling for Significant Map

Last Flag

In current HEVC, the context index is derived from 16 partitioning of the last flag position at even intervals as shown in Figure 4. However, it is not necessary to keep small segmentation on especially higher frequency part since non-zero coefficients are occasionally presented in higher frequency component. In proposed solution, the context index is derived from 4 partitioning of the last flag position  instead of 16 as shown in Figure 5. As the results of this simplification, the total number of context model for Last Flag becomes “56” (25% of current HEVC) as Table 4.

Figure 4 : Context Index separation for Last Flag in current HM.

Figure 5 : Context Index separation for Last Flag in proposed method.

Table 4 : Number of the context modeling for Last Flag

As the results of those simplification for Significant Map and Last Flag, the total number of context model for Significant Map and Last Flag becomes “180” (40% of current HEVC). The total number of context mode in HEVC becomes “490”, and 35% reduction has been achieved by our proposed method.

# Results

The proposed method performance relative to the TMuC 0.9-hm software was checked under the common test condition described in JCTVC-C500[2] .
The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3.

Table 5 : BD-Rate for Simplification of Context model selection for Significant Map

Table 6 : BD-Rate for Simplification of Context model selection for Last Flag

Table 7 : BD-Rate for Simplification of Context model selection for Significant Map and Last Flag

# Conclusion

In this contribution, a simplified context model selection method is proposed. An experimental result shows that the proposed method reduces the number of contest model by 45% for Siginificant Map and 75% for Last Flag respectively with less than 0.1% performance drop in average BD-rate. It is suggested to consider the inclusion of this simplification in the HM/WD of HEVC in order to avoid unnecessary complexity increase for the future deployment of the standard.

# Reference

- JCT-VC, “WD1: Working Draft 1 of High-Efficiency Video Coding”, JCTVC-C403, JCT-VC 3rd  Meeting, Guangzhou, October, 2010.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-C500, JCT-VC 3rd Meeting, Guangzhou, October 2010.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).