| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D186 |
| --- | --- |

| Title: | Unification of Transform Coefficient Coding for non-reference intra block |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hisao Sasai Takahiro Nishi | Tel: Email: | +81 6 6900 9689 sasai.hisao@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

A Separated DC coefficient coding for non-reference intra blocks is applied only in LCEC mode of the TMuC0.9-hm software. In this contribution, a unified solution on the non-reference intra block coding is proposed for both of the two entropy coders. In comparison with the TMuC0.9-hm anchors, the proposed change leads to no significant differences in BD-rate and software execution time.

# Introduction

In the TMuC0.9-hm software, a special coding process is applied to code DC coefficient only for non-reference intra block in LCEC. Figure 1 shows an example of non-reference intra block. On the other hand, in CABAC coder, no special coding process is applied. In general, it is desirable to make the coding process identical as much as possible between the two entropy coders in order to make it possible to re-use the implementation design for plural of deployments. So, we tried to provide a unified process for those coders.

Figure 1 : non-reference intra block

- Solution

For non-reference intra block, the special process has been applied to DC coefficient coding.

Current HEVC software

In case of LCEC, DC coefficient for non-reference intra block is coded as following steps (Figure 2).

step1. 	Pick up DC coefficient value in case of non-reference intra block.

step2.	DC level is coded by using vlc table. DC sign is coded if DC level is not equal to “0”.

step3.	Replace DC coefficient with a value of “1”.

step4.	Code coefficients which are replaced at step 3.

In case of CABAC, nothing special process for non-reference intra block.

Figure 2 : Coding steps on special process in LCEC coder in HEVC.

Proposed Method

In case of LCEC, there is some space to improve coding efficiency. At step 3, a DC value is replaced with “1”. Therefore, the information of value ”1” should be coded (existence and sign) and the last position is kept. It is not necessary to code the value of “1” if the step 3 is changed as following steps (Figure 3). 
By using these steps, the last position is decreased by 1 and no effects to following step4.

step3A.	Stuff the position of DC with remaining coefficients by following scan-order.

step3B.	Stuff the position of the end of block, with a value “0”.

Figure 3 : Coding steps for proposed method.

- Simulation & Results

The proposed method performance relative to the TMuC 0.9-hm software was checked under the common test condition described in JCTVC-C500[2] .
The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3.

To compare proposed method, LCEC method is also tested in high efficiency set.

Table 1 : BD-Rate results (%) for LCEC and proposed method for Intra low complexity set.
( Anchor : no special process for coefficient coding )

Figure 4 : BD-Rate results (%) on each sequence for intra low complexity set.
(Anchor: no special process for coefficient coding)

Figure 5 : BD-Rate results (%) on each sequence for intra low complexity set.
(Anchor: TMuC v0.9-hm Anchor)

Figure 6 : BD-Rate results (%) on each sequence for intra high efficiency set.
(Anchor: TMuC v0.9-hm Anchor)

- Conclusion

In this contribution, unified transform coefficient coding method is proposed for non-reference intra block coding to be used for both CABAC and LCEC. In comparison with the TMuC0.9-hm anchors, the proposed change leads to no significant differences in BD-rate and software execution time. It is suggested that such unified approach for non-reference intra block coding should be discussed in either CE or AdHoc group.

- References

- JCT-VC, “WD1: Working Draft 1 of High-Efficiency Video Coding”, JCTVC-C403, JCT-VC 3rd Meeting, Guangzhou, October, 2010.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-C500, JCT-VC 3rd Meeting, Guangzhou, October 2010.

- Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).