| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D190 WG11 Number: m18949 |
| --- | --- |

| Title: | CE11: Coding Efficiency of Tools in HHI_TRANSFORM_CODING (JCTVC-A116) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Core Experiment |  |  |
| Author(s) or Contact(s): | Vivienne Sze Texas Instruments Inc. | Email: | sze@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

Abstract

The coding efficiency gain of HHI_TRANSFORM_CODING was evaluated with TMuC-0.9 to be between 1.9 to 3.7%.  A breakdown of this gain across the four main tools was measured: adaptive scanning order of significance map (0.2 to 0.3%), HHI context selection of significant_coeff_flag (1.4 to 2.8%), HHI context selection of last_significant_coeff_flag (0 to 0.9%), and HHI context selection of coefficient level (0.2 to 0.7%).  An increase between 13 to 32.7 % of significant_coeff_flag bins is also reported, which increases the workload for the CABAC.  It should be noted that the four tools increase the complexity of context modeling making it more difficult to parallelize for throughput. These results have been verified to be a match with results provided by HHI.

# Introduction

This document reports the performance evaluation results of HHI_TRANSFORM_CODING in the TMuC-0.9 as part of CE11 Error: Reference source not found.

# Tool Description

Transform coefficient coding is a scanning and context modeling scheme that aims to improve coding efficiency using four main modifications:

- Adaptive scanning order of significance map:  The direction of the diagonal sub-scan depends on the location of the majority of significant coefficients, either bottom-left or top-right part of transform block, of the previous diagonal scan (i.e. each diagonal scan can be in any direction)

- Context selection of significant_coeff_flag:  For 8x8 blocks, each 2x2 sub-block uses a separate context for significant_coeff_flag.  For transforms greater than 8x8, the context of significant_coeff_flag depends on up to 10 top and left neighboring coefficient positions.

- Context selection of last_significant_coeff_flag:  The context for last_significant_coeff_flag depends on distance from top-left corner of transform block.

- Proposed context modeling for coefficient levels: Different contexts can be used for each 4x4 sub-block to encode coeff_abs_level_minus_one depending on the statistics of already coded sub-blocks (i.e. number of coefficients whose absolute value is greater than one in the previous sub-block within the same transform block).

## Complexity Analysis

Parallel bin processing has been proposed in [1][3][4].  However, as highlighted in [5][6], the context modeling remains a challenge to parallelize.  The three of the four modifications introduce additional complexity to the context selection; specifically adaptive scanning order and HHI context modeling for significance map when transform >8x8 make it difficult to process multiple significant_coeff_flag elements in parallel since their context depends on up to 10 neighbors.  The dependencies are highlighted in Fig. 1.

A

B

C

D

E

F

G

H

I

X

J

K

Figure 1 –Dependencies of context selection for significant_coeff_flag in HHI_TRANFORM_CODING.

Finally, the HI context modeling for coefficients also introduces dependencies between 4x4 blocks within a transform block, which again makes parallelism difficult.  Due to the increased complexity in context selection, it is important to understand the coding gains of each tool.  Therefore, we will evaluate each independently as described in the next sections.

# Test Conditions

The TMuC-0.9 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations specified in by the Software AhG are followed. The TMuC-0.9 anchor has HHI_TRANSFORM_CODING enabled.  As part of CE11, the number of significant_coeff_flag was counted, and the change relative to TMuC-0.9 anchor is reported. The significant_coeff_flag % was computed using the formula

(# significant_coeff_flag under test - # significant_coeff_flag in anchor) / # significant_coeff_flag in anchor

Since the TMuC-0.9 is the anchor with all HHI_TRANSFORM_CODING tools enabled, a positive % means a reduction in bins.  A positive BD-rate also means a coding gain of the tool.

The various tools in HHI_TRANSFORM_CODING are disabled in TypeDef.h by #define (shown below) provided by HHI. Table 1 shows the configuration for each tool listed in Section 2.

#define CE11_DISABLE_HHI_ADAPTIVE_SCAN          0

#define CE11_DISABLE_HHI_SIG_FLAG_CTX_MOD       0

#define CE11_DISABLE_HHI_LAST_FLAG_CTX_MOD      0

#define CE11_DISABLE_HHI_ABS_LEVEL_M1_CTX_MOD   0

|  | 1 (adaptive scan) | 2 (sig flag) | 3 (last flag) | 4 (coefficient) | Overall gain |
| --- | --- | --- | --- | --- | --- |
| CE11_DISABLE_HHI_ADAPTIVE_SCAN | 1 | 0 | 0 | 0 | 1 |
| CE11_DISABLE_HHI_SIG_FLAG_CTX_MOD | 0 | 1 | 0 | 0 | 1 |
| CE11_DISABLE_HHI_LAST_FLAG_CTX_MOD | 0 | 0 | 1 | 0 | 1 |
| CE11_DISABLE_HHI_ABS_LEVEL_M1_CTX_MOD | 0 | 0 | 0 | 1 | 1 |

Table 1: Configuration used to test each tool in HHI_TRANSFORM_CODING

# Test Results

## Coding gain from Adaptive Scan

As shown in Table 2, Adaptive Scan provides 0.2% to 0.3% gain. The detailed results are shown in JCTVC-D190.xls. However, it causes a 2.2 to 2.8% reduction in the number of significant_coeff_flag bins.

|  | Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A | 0.3 | 0.5 | 0.4 | 2.30% |
| Class B | 0.2 | 0.4 | 0.5 | 2.46% |
| Class C | 0.4 | 0.6 | 0.6 | 2.64% |
| Class D | 0.4 | 0.6 | 0.6 | 2.40% |
| Class E | 0.4 | 0.4 | 0.2 | 3.80% |
| All | 0.3 | 0.5 | 0.5 | 2.72% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 113% |  |  |  |
|  | Random access |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A | 0.2 | 0.6 | 0.6 | 2.09% |
| Class B | 0.2 | 0.2 | 0.5 | 2.09% |
| Class C | 0.3 | 0.3 | 0.3 | 2.34% |
| Class D | 0.2 | 0.4 | 0.6 | 2.15% |
| Class E |  |  |  |  |
| All | 0.2 | 0.3 | 0.5 | 2.17% |
| Enc Time[%] | 92% |  |  |  |
| Dec Time[%] | 96% |  |  |  |
|  | Low delay |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A |  |  |  |  |
| Class B | 0.2 | 0.3 | 0.3 | 2.18% |
| Class C | 0.2 | 0.2 | 0.2 | 2.67% |
| Class D | 0.2 | -0.4 | 0.6 | 2.06% |
| Class E | 0.1 | 0.6 | 0.6 | 4.37% |
| All | 0.2 | 0.2 | 0.4 | 2.82% |
| Enc Time[%] | 92% |  |  |  |
| Dec Time[%] | 95% |  |  |  |

Table 2  Average gain from the HHI adaptive scan order. Positive numbers in the table mean proposed context modeling for significance map provides the coding gain.  Negative sig count indicates increase in number of bins.

## Coding gain from Context Selection of significant_coeff_flag

As shown in Table 3, the HHI context selection of significant_coeff_flag provides 1.4 to 2.8% gain. For detailed results, please refer to JCTVC-D190.xls.  However, it increases the number of significant_coeff_flag bins by 13 to 27%.

|  | Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A | 1.1 | 1.1 | 1.2 | -7.75% |
| Class B | 2.3 | 2.5 | 2.6 | -13.97% |
| Class C | 1.3 | 1.3 | 1.4 | -12.87% |
| Class D | 1.2 | 1.2 | 1.1 | -9.60% |
| Class E | 2.7 | 1.9 | 1.9 | -19.14% |
| All | 1.8 | 1.7 | 1.7 | -12.66% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 112% |  |  |  |
|  | Random access |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A | 0.8 | 1.1 | 1.3 | -8.30% |
| Class B | 1.9 | 2.4 | 1.9 | -18.02% |
| Class C | 1.5 | 1.1 | 0.8 | -20.55% |
| Class D | 1.0 | 0.4 | 0.4 | -17.24% |
| Class E |  |  |  |  |
| All | 1.4 | 1.4 | 1.1 | -16.03% |
| Enc Time[%] | 92% |  |  |  |
| Dec Time[%] | 112% |  |  |  |
|  | Low delay |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A |  |  |  |  |
| Class B | 3.3 | 2.5 | 2.3 | -23.26% |
| Class C | 2.6 | 1.5 | 1.3 | -27.08% |
| Class D | 2.5 | -0.9 | -0.2 | -25.75% |
| Class E | 2.5 | -0.1 | 0.9 | -33.51% |
| All | 2.8 | 0.9 | 1.1 | -27.40% |
| Enc Time[%] | 92% |  |  |  |
| Dec Time[%] | 116% |  |  |  |

Table 3  Average gain from the HHI context selection for significant_coeff_flag. Positive numbers in the table mean proposed context modeling for significance map provides the coding gain.  Negative sig count indicates increase in number of bins.

## Coding gain from Context Selection of last_significant_coeff_flag

As shown in Table 4, the HHI context selection of last_significant_coeff_flag provides 0 to 0.9% gain. For detailed results, please refer to JCTVC-D190.xls. However, it increases the number of significant_coeff_flag bins by 1 to 12%.

|  | Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A | 0.0 | 0.1 | 0.2 | -0.41% |
| Class B | 0.1 | 0.3 | 0.2 | -0.53% |
| Class C | 0.0 | 0.3 | 0.3 | -2.22% |
| Class D | 0.1 | 0.3 | 0.3 | -1.88% |
| Class E | 0.0 | -0.1 | 0.0 | -0.63% |
| All | 0.0 | 0.2 | 0.2 | -1.13% |
| Enc Time[%] | 105% |  |  |  |
| Dec Time[%] | 117% |  |  |  |
|  | Random access |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A | 0.0 | -0.1 | 0.1 | -0.87% |
| Class B | 0.2 | 0.0 | 0.0 | -3.87% |
| Class C | 0.3 | 0.0 | -0.1 | -7.63% |
| Class D | 0.4 | -0.4 | 0.0 | -8.27% |
| Class E |  |  |  |  |
| All | 0.3 | -0.1 | 0.0 | -5.16% |
| Enc Time[%] | 91% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Low delay |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A |  |  |  |  |
| Class B | 0.7 | 0.0 | -0.6 | -8.13% |
| Class C | 1.0 | 0.5 | 0.1 | -13.18% |
| Class D | 1.2 | -2.1 | -1.5 | -16.33% |
| Class E | 0.4 | -2.1 | -1.8 | -11.51% |
| All | 0.9 | -0.8 | -0.9 | -12.29% |
| Enc Time[%] | 92% |  |  |  |
| Dec Time[%] | 96% |  |  |  |

Table 4  Average gain from the HHI context selection for last_significant_coeff_flag. Positive numbers in the table mean proposed context modeling for significance map provides the coding gain.  Negative sig count indicates increase in number of bins.

## Coding gain from Proposed Context Modeling of Coefficient Levels

As shown in Table 5, the HHI context selection of coefficient levels provides 0.2 to 0.7% gain. For detailed results, please refer to JCTVC-D190.xls. However, it causes a 2.6 to 8.8% increase in the number of significant_coeff_flag bins.

|  | Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A | 0.7 | 0.5 | 0.5 | -2.02% |
| Class B | 0.9 | 0.9 | 0.9 | -2.45% |
| Class C | 0.3 | 0.6 | 0.5 | -2.30% |
| Class D | 0.3 | 0.5 | 0.5 | -1.47% |
| Class E | 0.7 | 0.4 | 0.5 | -2.07% |
| All | 0.6 | 0.6 | 0.6 | -2.06% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 118% |  |  |  |
|  | Random access |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A | 0.1 | 0.9 | 0.7 | -2.22% |
| Class B | 0.3 | 1.5 | 1.1 | -2.97% |
| Class C | 0.1 | 1.3 | 1.1 | -4.26% |
| Class D | 0.2 | 1.3 | 1.4 | -4.35% |
| Class E |  |  |  |  |
| All | 0.2 | 1.3 | 1.1 | -3.45% |
| Enc Time[%] | 92% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Low delay |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A |  |  |  |  |
| Class B | 0.5 | 4.2 | 3.8 | -5.32% |
| Class C | 0.7 | 4.0 | 3.9 | -8.11% |
| Class D | 1.0 | 2.8 | 3.4 | -10.05% |
| Class E | 0.9 | 4.8 | 4.5 | -11.62% |
| All | 0.7 | 3.9 | 3.8 | -8.78% |
| Enc Time[%] | 92% |  |  |  |
| Dec Time[%] | 104% |  |  |  |

Table 5  Average gain from the HHI context selection for coefficient levels. Positive numbers in the table mean proposed context modeling for significance map provides the coding gain.  Negative sig count indicates increase in number of bins.

## Total coding gain from HHI_TRANSFORM_CODING

As shown in Table 6 below, the average gain from HHI_TRANSFORM_CODING ranges from 1.9% to 3.7%. However, it causes a 13 to 33% increase in the number of significant_coeff_flag bins.

The detailed results are shown in JCTVC-D190.xls

|  | Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A | 2.1 | 2.4 | 2.5 | -8.63% |
| Class B | 3.2 | 3.7 | 3.9 | -14.83% |
| Class C | 2.0 | 2.5 | 2.5 | -12.83% |
| Class D | 1.9 | 2.4 | 2.3 | -9.30% |
| Class E | 3.7 | 2.4 | 2.7 | -19.20% |
| All | 2.6 | 2.8 | 2.9 | -12.96% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Random access |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A | 1.1 | 1.9 | 1.9 | -9.72% |
| Class B | 2.5 | 3.3 | 3.4 | -20.30% |
| Class C | 1.9 | 2.3 | 1.8 | -22.18% |
| Class D | 1.6 | 1.7 | 1.6 | -19.25% |
| Class E |  |  |  |  |
| All | 1.9 | 2.4 | 2.3 | -17.86% |
| Enc Time[%] | 92% |  |  |  |
| Dec Time[%] | 95% |  |  |  |
|  | Low delay |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A |  |  |  |  |
| Class B | 4.1 | 4.4 | 3.7 | -27.66% |
| Class C | 3.5 | 4.3 | 3.5 | -31.89% |
| Class D | 3.5 | 0.7 | 1.9 | -32.05% |
| Class E | 3.8 | 0.4 | 2.8 | -39.25% |
| All | 3.7 | 2.7 | 3.0 | -32.71% |
| Enc Time[%] | 92% |  |  |  |
| Dec Time[%] | 97% |  |  |  |

Table 6  Average gain from HHI_TRANSFORM_CODING in TMuC-0.9. Positive numbers in the table mean proposed context modeling for significance map provides the coding gain.  Negative sig count indicates increase in number of bins.

# Summary

The evaluation results are summarized as below in the Table 7 below.

| Tool Under Test | Average Gain (BD-rate) | Reduction in % significant_coeff_flag bins |
| --- | --- | --- |
| HHI_TRANSFORM_CODING | 1.9 to 3.7 % | -13 to -32.7% (increase) |
| HHI Adaptive Scan | 0.2 to 0.3% | 2.2 to 2.8% |
| HHI context selection significant_coeff_flag | 1.4 to 2.8 % | -12.7 to -27.4% (increase) |
| HHI context selection last_significant_coeff_flag | 0 to 0.9 % | -1.1 to -12.3% (increase) |
| HHI context selection coefficient level | 0.2 to 0.7 % | -2.1 to -8.8% (increase) |

Table 7.  Summary of CE11 testing on HHI_TRANSFORM_CODING. Positive numbers in the table mean coding gain.  Negative % sig count indicates increase in number of bins.

Note: The decoding times may have been affected by the sig flag count of alternative scans.

# Conclusions

HHI_TRANSFORM_CODING showed higher gains in TMuC-0.9 than TMuC-0.7.  The majority of the coding efficiency gains of HHI_TRANSFORM_CODING are due to the HHI context selection for significant_coeff_flag.  It is important to note that HHI_TRANSFORM_CODING increases the number of significant_coeff_flag by up to 32.7%. Assuming significant_coeff_flag accounts for 1/3 of the bins, this increase in significant_coeff_flag bins increases the CABAC workload by ~10%.

# References

- “CE11: Coefficient scanning and coding”, JCTVC-C511, 3rd JCT-VC Meeting, Guangzhou, CN, October 2010.

- “Description of video coding technology proposal by Fraunhofer HHI”, JCTVC-A116, 1st JCT-VC Meeting, Dresden, DE, April 2010.

- “Video coding technology proposal by RIM”, JCTVC-A120, 1st JCT-VC Meeting, Dresden, DE, April 2010.

- “Video coding technology proposal by Texas Instruments Inc”, JCTVC-A101, 1st JCT-VC Meeting, Dresden, DE, April 2010.

- “Parallel Context Processing techniques for high coding efficiency entropy coding in HEVC”, JCTVC-B088, 2nd. JCT-VC Meeting, Geneva, CH, July 2010.

- “Improving throughput for V2V coding in HEVC”, JCTVC-B036, 2nd. JCT-VC Meeting, Geneva, CH, July 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, 2nd. JCT-VC Meeting, Geneva, CH, July 2010.