| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D191 WG11 number: m18950 |
| --- | --- |

| Title: | CE6: Report and evaluation of new Combined Intra Prediction settings |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE Report |  |  |
| Author(s) or Contact(s): | Marta Mrak, Thomas Davies, David Flynn, Andrea Gabriellini BBC R&D, South Lab Centre House, 56 Wood Lane W12 7SB London, UK | Tel: Email: | marta.mrak@bbc.co.uk |
| Source: | BBC |  |  |

_____________________________

# Abstract

This report presents various flexible implementations of Combined Intra Prediction (CIP) in TMuC 0.9, in the context of CE6 [1]. The presented approach enables selection of CIP settings of different complexities. A selection of CIP settings has been evaluated and the results are presented. The results show that without noticeable increase of complexity, compared to TMuC 0.9, the average BD-rate gain for all intra coded sequences is 0.3 % and 0.7 % for the high-efficiency and low-complexity configurations, respectively. While the gain is negligible for lower resolutions, on higher resolutions it is larger - e.g. the average gain for Class A is 1.2 % and 2.4 % for the high-efficiency and low-complexity configurations, respectively. CIP implementation is discussed and parallelizable implementations are presented

# Combined Intra Prediction

CIP was a part of BBC's response to the Call for Proposals [2]. In later studies [3] using TMuC 0.7 it was shown that CIP can contribute to the BD-rate gains with around 1 % in intra settings, with all other tools used in the same test (e.g. ROT, planar, full RQT,...).

CIP comprises a weighted combination of an angular (closed-loop) prediction together with a local mean prediction (open-loop prediction, OLP). The local mean is constructed as the average of the selected neighboring pixel values. The selected values are defined by an OLP template. In the example from Figure 1, the OLP template consists of three pixels - the left, top-left and above the current pixel.

In the decoder and within the local decoder in the encoder, the values in OLP template are the reconstructed values after inverse quantization and transform. In the encoder, the reconstructed values are not available because quantization and transform of the prediction residue have not yet been performed, so original values are used This local mean prediction is therefore different between forward prediction (encoder only) and reconstruction (encoder and decoder).

Figure 1 Example of Combined Intra Prediction utilizing a local mean and ANG prediction

When CIP is used on a block, for each pixel value within that block the open-loop prediction component is predicted from the values in the OLP template. Figure 2 shows examples of different OLP templates.

| X. |  |
| --- | --- |
| a) |  |
| X. | X. |
| b) | c) |
| X. | X. |
| d) | e) |

Figure 2 Examples of open-loop prediction templates for prediction of pixel values marked with X

While the template from Figure 2.a) enables averaged prediction from 3 directions, as used in [3], other OLP templates can also be used. In b) and c) the prediction from neighboring row and column are depicted and will be discussed later in section 4. In d) and e) the prediction is from neighboring pixels on left or on top of the current pixel. We refer to this type of prediction as to the directional prediction and its choice can be related to the direction of the angular prediction. For example, OLP template d) can be used in the combination with horizontal (ANG) prediction, and OLP template e) can be used in the combination with the vertical (ANG) prediction.

# CIP settings

As in its implementation in [3], application of CIP can be decided for each intra-coded block and signaled in the bit-stream. In that case the decision is done in an additional RD-search which increases the encoder's complexity. It is important to stress that CIP prediction itself has almost no influence on the decoding time, as can be seen from the test results in this document. This is because it only uses averaging of up to 3 neighboring pixel values and the angular prediction, to form the final predicted pixel value. In the rest of this report, CIP complexity is related to the corresponding encoder's complexity.

CIP can be used in its different settings that can be classified as follows:

1) CIP_HC (high complexity CIP);

This setting uses the most comprehensive RD search, taking into account different angular modes, RQTs and all CU components (Y, UV and related signaling). This version of CIP corresponds to the version presented in [3]. Using comprehensive RD search, this setting is expected to deliver highest coding gains.

2) CIP_MC (medium complexity CIP);

This setting involves RD search CIP within the current luma PU only, which is generally faster since chroma modes, other PUs in the current CU, and other CU components are not reassessed with or without CIP. This setting can be configured to provide various trade-offs between between higher complexity and higher gain, and lower complexity and lower gain performances

3) CIP_LC (low complexity CIP);

This setting applies CIP by default on selected PUs (under certain block size), which does not increase the RD search complexity

Selected CIP settings have been tested using TMuC 0.9, under the experimental conditions defined in [1], and the results are reported in the following section. In our implementation the choice of CIP settings can be defined for each slice type and is controlled with 4 macros.

# BD-rate results

The CIP results are presented in comparison to TMuC 0.9 and negative BD-rate values indicate gain introduced using CIP.

## Intra coding

Tables 1-3 include results for intra coding using CIP, demonstrating its flexible setting choices. In all cases where CIP is applied on default on certain block sizes, the directional CIP is used.

Results summarized in Table 1 are obtained for CIP_HC, as defined in the previous section. For CIP_MC (Table 2), CIP has been applied by default on 4x4 and 8x8 PUs, and is optional for 16x16 PUs. The increased encoder complexity, compared to the anchor, is due to the RD search on 16x16 PUs. When CIP is applied by default on 4x4 and 8x8 blocks, as in selected CIP_LC setting (Table 3), some of the gain that can be introduced with more complex settings is still preserved, with the processing times similar to the anchor's processing times.

Table 1 CIP High Complexity (HC) settings

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.2 | -5.7 | -5.7 | -3.7 | -4.5 | -4.3 |
| Class B | -1.3 | -3.3 | -3.4 | -2.3 | -2.8 | -2.8 |
| Class C | -1.4 | -2.2 | -2.3 | -2.1 | -1.8 | -1.8 |
| Class D | -1.3 | -1.9 | -1.9 | -2.1 | -1.6 | -1.6 |
| Class E | -1.8 | -4.4 | -4.3 | -3.3 | -3.5 | -3.6 |
| All | -1.5 | -3.2 | -3.2 | -2.5 | -2.6 | -2.6 |
| Enc Time[%] | 175% |  |  | 207% |  |  |
| Dec Time[%] | 104% |  |  | 102% |  |  |

Table 2 CIP Medium Complexity (MC) settings

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.5 | -5.4 | -5.6 | -2.9 | -6.1 | -6.0 |
| Class B | -0.5 | -2.7 | -2.8 | -0.8 | -4.0 | -4.1 |
| Class C | 0.2 | -2.3 | -2.3 | 0.0 | -2.8 | -2.8 |
| Class D | 0.1 | -2.0 | -2.0 | -0.3 | -2.4 | -2.4 |
| Class E | -0.5 | -3.8 | -3.6 | -1.7 | -5.2 | -5.1 |
| All | -0.3 | -2.9 | -3.0 | -0.9 | -3.8 | -3.8 |
| Enc Time[%] | 107% |  |  | 113% |  |  |
| Dec Time[%] | 102% |  |  | 108% |  |  |

Table 3 CIP Low Complexity (LC) settings

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.2 | -3.8 | -3.9 | -2.4 | -4.5 | -4.3 |
| Class B | -0.5 | -1.8 | -1.9 | -0.6 | -3.0 | -3.1 |
| Class C | 0.1 | -1.8 | -1.8 | 0.0 | -2.3 | -2.4 |
| Class D | 0.0 | -1.6 | -1.7 | -0.3 | -2.0 | -2.1 |
| Class E | -0.4 | -2.6 | -2.4 | -1.3 | -3.5 | -3.6 |
| All | -0.3 | -2.1 | -2.1 | -0.7 | -2.9 | -2.9 |
| Enc Time[%] | 100% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 104% |  |  |

## Inter coding

While this evaluation is presented within the CE that targets intra-only configurations, CIP results for inter coding are also included in this section. In this test, CIP is considered for all blocks in I slices and intra-coded blocks in P and B slices. Different CIP settings are chosen for different slices and results are summarized in Tables 4 and 5.

Results in Table 4 are obtained for CIP_LC for I slices and HC for B slices. Since the complexity of inter RD search is considerably more complex than intra optimization, the processing times using CIP are not significantly higher than the anchor. This is also the case for the choice of setting reported in Table 5 where CIP_HC is applied to all slices.

Table 4 Results for inter configurations and CIP LC/HC

|  | Random access |  |  | Random access LoCo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate |  | V BD-rate |  |  |
| Class A | -0.7 | -1.8 | -1.8 | -0.7 | -2.0 |  | -1.9 |  |  |
| Class B | -0.4 | -1.2 | -1.2 | -0.1 | -1.3 |  | -1.5 |  |  |
| Class C | -0.4 | -1.1 | -0.9 | 0.1 | -1.0 |  | -0.9 |  |  |
| Class D | -0.2 | -0.7 | -0.6 | 0.0 | -0.8 |  | -0.7 |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| All | -0.4 | -1.1 | -1.1 | -0.1 | -1.2 |  | -1.2 |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |  |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate |  | V BD-rate |  |  |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.1 | -0.4 | -0.6 | 0.0 | -0.2 |  | -0.3 |  |  |
| Class C | -0.2 | -0.5 | -0.6 | 0.0 | -0.4 |  | -0.2 |  |  |
| Class D | -0.1 | -0.5 | 0.1 | -0.1 | 0.1 |  | -0.2 |  |  |
| Class E | 0.5 | -0.9 | -0.5 | 0.6 | 0.2 |  | 0.1 |  |  |
| All | 0.0 | -0.5 | -0.4 | 0.1 | -0.1 |  | -0.2 |  |  |
| Enc Time[%] | 101% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |  |  |  |

Table 5 Results for inter configurations and CIP HC/HC

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.0 | -1.8 | -1.5 | -1.2 | -1.6 | -1.6 |
| Class B | -0.7 | -1.0 | -1.0 | -0.7 | -1.1 | -1.2 |
| Class C | -0.9 | -1.0 | -1.0 | -0.7 | -0.9 | -1.0 |
| Class D | -0.7 | -0.8 | -0.3 | -0.6 | -0.7 | -0.6 |
| Class E |  |  |  |  |  |  |
| All | -0.8 | -1.1 | -0.9 | -0.7 | -1.0 | -1.0 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 97% |  |  | 95% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | -0.3 | -0.3 | -0.1 | -0.3 | -0.2 |
| Class C | -0.3 | -0.5 | -0.5 | -0.1 | -0.4 | -0.1 |
| Class D | -0.2 | -0.5 | -0.3 | -0.1 | 0.3 | 0.0 |
| Class E | 0.3 | -0.5 | -0.2 | 0.2 | -0.2 | -0.3 |
| All | -0.1 | -0.5 | -0.4 | -0.1 | -0.2 | -0.2 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 96% |  |  |

# Alternative CIP implementation directions

In a simple software implementation, OLP for a block can be computed in one-by-one fashion. This is depicted in Figure 3.a), for a 4x4 block, where numbers indicate time instances (steps) at which pixel is computed, assuming the OLP template from Figure 2.a). For the same template, the OLP computation can be parallelized, as depicted in Figure 3.b) where the arrows indicate parallel processes. As a result, with N-way parallelism the OLP for a N  N block can be computed in (2  N - 1) steps.

| 0; 1; 2; 3. 4; 5; 6; 7. 8; 9; 10; 11. 12; 13; 14; 15. | 1 |
| --- | --- |
| a) | b) |

Figure 3 Serial (a) and parallel (b) computation of OLP for the template from Figure 2. a)

For the OLP templates that consist of 1 pixel, e.g. Figure 2. d) and e), the processing can be done row by row (or column by column, depending on the template), in parallel which requires N steps for processing of N  N block. The same processing time can be achieved for larger templates. That is demonstrated in Figure 4 where red lines show OLP prediction directions for each pixel.

| 0 0; 0; 0. 0; 1; 1; 1. 0; 1; 2; 2. 0; 1; 2; 3. |  |
| --- | --- |
| a) | b) |

Figure 4 Parallel computation using adaptive templates

Where predictions can be formed in parallel it is possible to perform prediction and reconstruction across a number of pixels at a time using processor intrinsics such as SIMD.

Increasing the processing parallelization capabilities can influence the coding performance. For the same setting as in Table 3 (Intra CIP_LC), the test was carried out using the template from Figure 4. The results are summarized in Table 5. Average BD-rate results are similar to the results for larger template.

Table 6 Results for intra coding configurations with CIP_LC and OLP prediction strategy from Figure 4

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.6 | -3.8 | -3.9 | -2.6 | -4.2 | -3.9 |
| Class B | -0.6 | -1.5 | -1.6 | -0.5 | -2.5 | -2.5 |
| Class C | 0.2 | -1.4 | -1.4 | 0.3 | -1.8 | -1.8 |
| Class D | 0.1 | -1.3 | -1.3 | -0.1 | -1.6 | -1.5 |
| Class E | -0.4 | -2.3 | -2.1 | -1.0 | -2.8 | -2.9 |
| All | -0.3 | -1.8 | -1.8 | -0.6 | -2.3 | -2.3 |
| Enc Time[%] | 100% |  |  | 109% |  |  |
| Dec Time[%] | 103% |  |  | 108% |  |  |

# Conclusions and recommendations

This documents reports flexible implementations of CIP representing a range of different complexity settings.

Presented results show that in HM intra configurations the encoding times can vary between 100 % to 200 % (compared to the TMuC 0.9 anchor), depending on the chosen CIP setting. Generally, higher complexity is associated to larger BD-rate gains.

For the low complexity CIP encoder the average gains are (-0.3, -2.1, -2.1) and (-0.7, -2.9, -2.9), in (Y, U, V) BD-rate, for intra high efficiency and intra low complexity configurations, respectively.

For the high complexity CIP encoder the average gains are (-1.5, -3.2, -3.2) and (-2.5, -2.6, -2.6) for intra high efficiency and intra low complexity configurations, respectively.

From the results it is also evident that on higher resolutions, particularly for Class A sequences, CIP provides larger gains than for lower resolutions.

The results also show that when used in generally more complex HM inter configurations, the average complexity that CIP adds in its high complexity setting, is about 1 %. In that case, the average BD-rate gains for the random access configurations are (-0.8, -1.1, -0.9) and (-0.7, 1.0, -1.0), for high efficiency and low-complexity, respectively. The average BD-rate gains for the low delay configurations are (-0.1, -0.5, -0.4) and (-0.1, -0.2, -0.2), for high efficiency and low-complexity, respectively.

A brief analysis of further CIP parallelized processing capabilities has also been provided, showing that its further simplifications can preserve its average BD-rate gains in intra configurations.

References

[1]	JCTVC-C506: Description of Core Experiment 6: Intra Prediction Improvement

[2]	JCTVC-A125: BBC's Response to the Call for Proposals on Video Compression Technology

[3]	JCTVC-C213: TE12: Additional Results for Combined Intra Prediction

British Broadcasting Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).