| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D192_r2 WG11 Number: m18951 |
| --- | --- |

| Title: | Analysis on the interaction between deblocking filtering and in-loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage Saori Asaka Akiyuki Tanizawa Takashi Watanabe Takeshi Chujoh 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2152 tomoo.yamakage@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution reports some analysis on the relationship for various in-loop filtering techniques, in particular interaction between deblocking filtering and Wiener-based in-loop filtering (QC_ALF).  This item is one of the mandates of In-loop and post-processing filtering ad-hoc group.

In this contribution, a parameter () that controls filter on/off and strength of filtering for deblocking is adjusted for both with and w/o QC_ALF cases.  By comparing the BD-rate gain of each case with the best performing  values, QC_ALF shows additional bitrate reduction of 0.7% for I Only case, 0.5% for Random Access case and 0.7% for Low Delay case for High Efficiency coding condition.

Therefore, it is recommended to use different  values for deblocking filtering according to the usage of QC_ALF so that the default value can provide the best performance.

# Introduction

Multi-input Wiener-based in-loop filtering schemes have been proposed to JCT-VC [1][2].  These schemes utilize pre-deblocked pixels (or inverse DCT pixels and predicted pixles) and post-deblocked pixels.  Those pixels are weighted and accumulated to perform Wiener-based filtering.  These schemes may improve coding efficiency compared to a single input Wiener-based filtering that uses only post-deblocked pixels.

However, it seems not clear why the coding efficiency is improved.  It is expected that there are two potential reasons at least.  One possibility is to increase the number of pixels to be used for Wiener-based filtering.  The other possibility is to adjust the strength of deblocking filtering by Weiner-based filtering coefficients.

In this contribution, an analysis is made for the latter part.  By optimizing the parameter  for deblocking filtering in HEVC Test Mode 1 (HM1), coding efficiency is improved.  To make a fair comparison,  is optimized for with and w/o QC_ALF cases.  Coding efficiency improvement for with QC_ALF case is larger than that for w/o QC_ALF case.  This means that the default  values should be changed according to the usage of QC_ALF.

# Optimization of 

In order to obtain the best performing  values, offsetting Qp is applied when referring to the  values in HM1.  Six High Efficiency cases are tested; (IO, RA, LD) x (with QC_ALF, w/o QC_ALF)

It is observed that the best performing  values are different from each other and from those in HM1.  Figure 1 shows the recommended  values (1 for with QC_ALF and 2 for w/o QC_ALF) for High Efficiency conditions.

Figure 1  Recommended  values for with and w/o QC_ALF

# Experimental results

The simulation was conducted based on High Efficiency conditions in JCTVC-C500 [3] and JCTVC-C508_r1 [4].  Test cases used the recommended  values in Figure 1.  Results against the HM1 anchor for High Efficiency conditions are shown in Table 1.

Table 1  BD-rate Gain (%) for High Efficiency conditions

| High Efficiency |  |  | With QC_ALF (vs  |  |  | W/o QC_ALF (vs 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | IO | RA | LD | IO | RA | LD |
| A | S01 | Traffic | -1.2 | -1.7 |  | -0.1 | -0.6 |  |
|  | S02 | PeopleOnStreet | -1.3 | -0.3 |  | 0.0 | 0.8 |  |
| B | S03 | Kimono | -0.6 | -0.3 | -0.4 | 0.0 | 0.1 | 0.1 |
|  | S04 | ParkScene | -1.2 | -1.1 | -1.5 | -0.2 | -0.3 | -0.3 |
|  | S05 | Cactus | -0.9 | -1.0 | -1.4 | -0.2 | -0.2 | -0.5 |
|  | S06 | BasketballDrive | -0.9 | -0.1 | -0.5 | -0.3 | 0.2 | 0.0 |
|  | S07 | BQTerrace | -0.7 | -0.7 | -1.1 | -0.2 | 0.1 | -0.2 |
| C | S08 | BasketballDrill | -0.9 | -1.1 | -1.4 | -0.4 | -0.6 | -0.9 |
|  | S09 | BQMall | -0.7 | -0.3 | -0.5 | -0.1 | 0.0 | 0.0 |
|  | S10 | PartyScene | -0.4 | -0.2 | -0.3 | -0.1 | -0.1 | 0.0 |
|  | S11 | RaceHorses | -0.6 | 0.0 | -0.2 | -0.1 | 0.4 | 0.2 |
| D | S12 | BasketballPass | -0.7 | 0.0 | -0.2 | -0.2 | 0.2 | 0.3 |
|  | S13 | BQSquare | -0.2 | -0.5 | -0.3 | -0.1 | -0.3 | 0.2 |
|  | S14 | BlowingBubbles | -0.4 | -0.4 | -0.7 | -0.2 | -0.4 | -0.3 |
|  | S15 | RaceHorses | -0.9 | -0.1 | -0.6 | -0.2 | 0.4 | -0.1 |
| E | S16 | Vidyo1 | -0.8 |  | -2.4 | 0.0 |  | -0.9 |
|  | S17 | Vidyo3 | -0.9 |  | -2.7 | -0.2 |  | -0.8 |
|  | S18 | Vidyo4 | -0.9 |  | -2.5 | -0.1 |  | -1.1 |
| Class A |  |  | -1.2 | -1.0 |  | 0.0 | 0.1 |  |
| Class B |  |  | -0.9 | -0.7 | -1.0 | -0.2 | 0.0 | -0.2 |
| Class C |  |  | -0.6 | -0.4 | -0.6 | -0.2 | -0.1 | -0.2 |
| Class D |  |  | -0.6 | -0.3 | -0.4 | -0.2 | 0.0 | 0.0 |
| Class E |  |  | -0.9 |  | -2.5 | -0.1 |  | -0.9 |
| Total |  |  | -0.8 | -0.5 | -1.0 | -0.1 | 0.0 | -0.3 |

Experimental results show that QC_ALF with the 1 values gives 0.8%, 0.5% and 1.0% gain of coding efficiency on average compared to the anchor for I Only, Random Access and Low Delay cases, respectively.  However, w/o QC_ALF with the 2 values gives 0.1%, 0.0% and 0.3% gain of coding efficiency on average compared to the anchor w/o QC_ALF for I Only, Random Access and Low Delay cases, respectively.

To be fair, the gain of the latter condition (w/o QC_ALF) should be subtracted from the gain of the former one (with QC_ALF), which corresponds to 0.7%, 0.5% and 0.7% additional gain for I Only, Random Access and Low Delay cases, respectively.

# Combination results with MQT_ALF

The recommended  values (1) were also applied to MQT_ALF [5].  Table 2 shows the experimental results of MQT ALF and the combination of MQT_ALF and the recommended  values against the anchor of High Efficiency coding condition.

Table 2  BD-rate Gain (%) for MQT_ALF and the combination with 1

| High Efficiency |  |  | MQT_ALF ( |  |  | MQT_ALF (1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | IO | RA | LD | IO | RA | LD |
| A | S01 | Traffic | 0.1 | -0.1 |  | -1.0 | -1.8 |  |
|  | S02 | PeopleOnStreet | 0.0 | 0.0 |  | -1.3 | -0.2 |  |
| B | S03 | Kimono | 0.3 | 0.2 | -0.4 | -0.2 | -0.1 | -0.9 |
|  | S04 | ParkScene | 0.0 | -0.3 | -2.1 | -1.3 | -1.4 | -4.0 |
|  | S05 | Cactus | -0.2 | -0.3 | -2.1 | -0.7 | -1.3 | -3.6 |
|  | S06 | BasketballDrive | 0.2 | -0.1 | -0.6 | -0.7 | -0.3 | -1.1 |
|  | S07 | BQTerrace | 0.1 | -0.8 | -1.3 | -0.6 | -1.8 | -2.8 |
| C | S08 | BasketballDrill | 0.1 | -1.5 | -3.0 | -0.7 | -2.7 | -4.9 |
|  | S09 | BQMall | 0.2 | -0.6 | -2.3 | -0.4 | -1.0 | -3.1 |
|  | S10 | PartyScene | 0.0 | -0.4 | -1.6 | -0.3 | -0.6 | -2.1 |
|  | S11 | RaceHorses | 0.2 | 0.0 | -0.5 | -0.4 | 0.0 | -0.7 |
| D | S12 | BasketballPass | 0.0 | -0.2 | -0.5 | -0.7 | -0.2 | -0.8 |
|  | S13 | BQSquare | -0.5 | -1.6 | -2.6 | -0.7 | -2.0 | -3.4 |
|  | S14 | BlowingBubbles | 0.0 | -0.4 | -1.1 | -0.4 | -1.0 | -1.9 |
|  | S15 | RaceHorses | 0.3 | -0.3 | -1.1 | -0.5 | -0.4 | -1.6 |
| E | S16 | Vidyo1 | 0.4 |  | -2.7 | -0.4 |  | -4.6 |
|  | S17 | Vidyo3 | 0.3 |  | -5.6 | -0.7 |  | -8.7 |
|  | S18 | Vidyo4 | 0.3 |  | -2.1 | -0.6 |  | -4.7 |
| Class A |  |  | 0.1 | 0.0 |  | -1.1 | -1.0 |  |
| Class B |  |  | 0.1 | -0.3 | -1.3 | -0.7 | -1.0 | -2.5 |
| Class C |  |  | 0.2 | -0.6 | -1.9 | -0.5 | -1.1 | -2.7 |
| Class D |  |  | -0.1 | -0.6 | -1.3 | -0.6 | -0.9 | -1.9 |
| Class E |  |  | 0.3 |  | -3.5 | -0.7 |  | -6.0 |
| Total |  |  | 0.1 | -0.4 | -1.9 | -0.7 | -1.0 | -3.1 |

Experimental results show that MQT_ALF with the 1 values gives 0.8%, 0.6% and 1.2% additional gain of coding efficiency on average compared to MQT_ALF with the original  values for I Only, Random Access and Low Delay cases, respectively.

As shown in Tables 1 and 2, it is recommended to modify  values according to the usage of QC_ALF or MQT_ALF.

# Conclusion

In this contribution, a parameter () that controls filter on/off and strength of filtering for deblocking is adjusted for both with and w/o QC_ALF cases.  By comparing the BD-rate gain of each case with the best performing 1 values and 2 values, QC_ALF shows additional bitrate reduction of 0.7% for I Only case, 0.5% for Random Access case and 0.7% for Low Delay case for High Efficiency coding condition.  The coding efficiency improvement is also observed for MQT_ALF.

Therefore, it is recommended to use different default  values for deblocking filtering according to the usage of QC_ALF so that the default value can provide the best performance.

# References

[1]	T. Ikai and T. Yamamoto, “TE10 subtest 2: Parallel adaptive loop filter”, JCTVC-C173, October 2010.

[2]	M. Narroschke, “TE12.5: Results for Adaptive loop filter using prediction and residual”, JCTVC-C214, October 2010.

[3]	F. Bossen, “Common test conditions and software reference configurations”, JCTVC-C500, October 2010.

[4]	T. Yamakage, T. Chujoh, Y.W. Huang, K. Chono, I.S. Chong, T. Yamamoto, J. Lim, B. Jeon, J. Xu, M. Narroschke, “Description of CE8: In-loop filtering”, JCTVC-C508_r1, October 2010.

[5]	C.Y. Chen, C.M. Fu, C.Y. Tsai, Y.W. Huang, S. Lei, M. Karczewicz, I.S. Chong, T. Yamakage, T. Chujoh and T. Watanabe, “CE8 Subtest2: A Joint Proposal on Adaptive Loop Filter by MediaTek, Qualcomm, and Toshiba”, JCTVC-D119, January 2011.

# Patent rights declaration(s)

Toshba Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).