| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D195 WG11 Number: m18955 |
| --- | --- |

| Title: | CE11: Simplified context selection for significant_coeff_flag (JCTVC-C227) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vivienne Sze Texas Instruments Inc. Madhukar Budagavi Texas Instruments Inc. | Email: Email: | sze@ti.com madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

HHI_TRANSFORM_CODING uses a highly adaptive context modeling approach for significance map, where context selection for significant_coeff_flag depends on coefficients in 10 neighboring positions.  While it provides coding gains between 1.4% to 2.8% [2], it greatly increases the complexity of context selection when coding significant_coeff_flag. In this contribution, we propose a simplification of the context selection.  We reduce neighbor dependency from 10 to 8, then to 4. The number of contexts required for significant_coeff_flag is also reduced from 128 to 108.  These modifications were implemented in TMuC-0.9 and their coding efficiencies were evaluated.  Reducing from 10 to 8 has no coding loss, while reducing from 10 to 4 (reducing the dependency by more than half) has a coding loss between 0% to 0.2%. This simplified context selection had a negligible effect on number significant_coeff_flag bins.

# Introduction

For improved coding efficiency, HHI_TRANSFORM_CODING proposes a highly adaptive context selection approach for significant_coeff_flag in the significance map, where the context selection can depend on the value of the coefficients in 10 neighbouring positions (typically in the left, top-left and top directions) in the transform [1].    For instance, in Fig. 1 below, the context for position X can depend on positions A to K (note: only either G or K will be required, depending on the direction of the scan). These dependencies are derived from the source code of HHI_TRANSFORM_CODING in Fig. 2. In this contribution, we propose reducing the number of context dependencies to less than half of that used in HHI_TRANSFORM_CODING[1].  We also reduce the number of contexts that need to be stored for significant_coeff_flag from 128 to 108.

A

B

C

D

E

F

G

H

I

X

J

K

Fig.1 –Dependencies of context selection for significant_coeff_flag in HHI_TRANFORM_CODING.

UInt TComTrQuant::getSigCtxInc ( TCoeff* pcCoeff, const UInt uiPosX, const UInt uiPosY, const UInt uiLog2BlkSize, const UInt uiStride, const bool bDownLeft ) { UInt uiCtxInc = 0; UInt uiSizeM1 = ( 1 << uiLog2BlkSize ) - 1; . . . . . . . . . { const int* pData = &pcCoeff[ uiPosX + uiPosY * uiStride ]; int iStride = uiStride; int iStride2= iStride << 1; UInt uiCnt = ( pData[ -iStride ] ? 1 : 0 ); //F uiCnt += ( pData[ -1 ] ? 1 : 0 ); //I uiCnt += ( pData[ -1 -iStride ] ? 1 : 0 ); //E if( uiPosX > 1 ) { uiCnt += ( pData[ -2 ] ? 1 : 0 ); //H uiCnt += ( pData[ -2 -iStride ] ? 1 : 0 ); //D if( uiPosY < uiSizeM1 ) { uiCnt += ( pData[ -2 +iStride ] ? 1 : 0 ); //J } } if( uiPosY > 1 ) { uiCnt += ( pData[ -iStride2 ] ? 1 : 0 ); //B uiCnt += ( pData[ -1 -iStride2 ] ? 1 : 0 ); //A if( uiPosX < uiSizeM1 ) { uiCnt += ( pData[ 1 -iStride2 ] ? 1 : 0 ); //C } } if( bDownLeft ) { if( uiPosX < uiSizeM1 ) { uiCnt += ( pData[ 1 -iStride ] ? 1 : 0 ); //G } } else { if( uiPosY < uiSizeM1 ) { uiCnt += ( pData[ -1 +iStride ] ? 1 : 0 ); //K } } uiCtxInc = 10 + min<UInt>( 4, ( uiCnt + 1 ) >> 1 ); } return uiCtxInc; }

Figure 2 Source code (TComTrQuant.cpp) to process context selection for proposed context modeling of significance map.  Dependencies are highlighted.

# Simplified Context Selection for Significance Map

We propose removing the first the dependency on A and D as shown in Fig. 3. Furthermore, we propose removing dependencies on B, C, D, H and J as shown in Fig. 4.

B

C

E

F

G

H

I

X

J

K

Fig.3 – Context dependency simplification to reduce neighbour dependency from 10 to 8.

E

F

G

I

X

K

Fig.4 – Context dependency simplification to reduce neighbour dependency from 10 to 4.

# Experiment Results

The TMuC-0.9 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations specified by the Software AhG are followed.  As part of CE11, the number of significant_coeff_flag was counted, and the change relative to TMuC-0.9 anchor is reported. The significant_coeff_flag % was computed using the formula

(# significant_coeff_flag under test - # significant_coeff_flag in anchor) / # significant_coeff_flag in anchor

A negative % means a reduction in bins.  The modification proposed in this has negligible impact on the number of significant_coeff_flag.

Removing dependency on A and D has no coding loss.

|  | Intra (remove 2) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A | 0.0 | 0.0 | 0.0 | -0.18% |
| Class B | 0.0 | 0.0 | 0.0 | -0.29% |
| Class C | 0.0 | 0.0 | 0.0 | -0.55% |
| Class D | 0.0 | 0.0 | 0.0 | -0.07% |
| Class E | 0.0 | 0.0 | -0.1 | 0.02% |
| All | 0.0 | 0.0 | 0.0 | -0.22% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Random access (remove 2) |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A | 0.0 | 0.0 | 0.2 | -0.14% |
| Class B | 0.0 | -0.1 | -0.1 | -0.39% |
| Class C | 0.1 | -0.1 | -0.1 | -0.28% |
| Class D | 0.0 | 0.0 | 0.3 | 0.11% |
| Class E |  |  |  |  |
| All | 0.0 | 0.0 | 0.1 | -0.17% |
| Enc Time[%] | 92% |  |  |  |
| Dec Time[%] | 97% |  |  |  |
|  | Low delay (remove 2) |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A |  |  |  |  |
| Class B | 0.1 | 0.0 | -0.2 | -0.25% |
| Class C | 0.0 | -0.2 | -0.1 | -0.22% |
| Class D | 0.0 | -0.4 | 0.3 | 0.48% |
| Class E | -0.2 | 0.2 | -0.5 | 0.87% |
| All | 0.0 | -0.1 | -0.1 | 0.22% |
| Enc Time[%] | 92% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

Table 1  Average loss from reducing dependency from 10 to 8 for significant_coeff_flag in HHI_TRANSFORM_CODING. Positive numbers in the table means coding loss.

Removing dependency on B, C, D, H and J has coding loss between 0 to 0.2%. Detailed results in CE11-JCTVC-D195.xls

|  | Intra (remove 6) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | Y BD-rate | Y BD-rate | % sig count |
| Class A | 0.2 | 0.2 | 0.2 | -2.06% |
| Class B | 0.3 | 0.3 | 0.3 | -2.65% |
| Class C | 0.1 | 0.1 | 0.1 | -1.36% |
| Class D | 0.1 | 0.1 | 0.1 | -0.99% |
| Class E | 0.2 | 0.2 | 0.2 | -1.72% |
| All | 0.2 | 0.2 | 0.2 | -1.76% |
| Enc Time[%] | 95% |  |  | 95% |
| Dec Time[%] | 98% |  |  | 98% |
|  | Random access (remove 6) |  |  |  |
|  | Y BD-rate | Y BD-rate | Y BD-rate | % sig count |
| Class A | 0.1 | 0.1 | 0.1 | -1.35% |
| Class B | 0.2 | 0.2 | 0.2 | -2.60% |
| Class C | 0.1 | 0.1 | 0.1 | -1.30% |
| Class D | 0.0 | 0.0 | 0.0 | -0.39% |
| Class E |  |  |  |  |
| All | 0.1 | 0.1 | 0.1 | -1.41% |
| Enc Time[%] | 93% |  |  | 93% |
| Dec Time[%] | 99% |  |  | 99% |
|  | Low delay(remove 6) |  |  |  |
|  | Y BD-rate | Y BD-rate | Y BD-rate | % sig count |
| Class A |  |  |  |  |
| Class B | 0.2 | 0.2 | 0.2 | -1.86% |
| Class C | 0.0 | 0.0 | 0.0 | -0.45% |
| Class D | -0.1 | -0.1 | -0.1 | 2.18% |
| Class E | -0.2 | -0.2 | -0.2 | 1.72% |
| All | 0.0 | 0.0 | 0.0 | 0.40% |
| Enc Time[%] | 91% |  |  | 91% |
| Dec Time[%] | 98% |  |  | 98% |

Table 2  Average loss from reducing dependency from 10 to 4 for significant_coeff_flag in HHI_TRANSFORM_CODING. Positive numbers in the table means coding loss.  For significant_coeff_flag count, negative number means reduction in bins.

To measure the gain of HHI context selection for significant_coeff_flag with reduced context selection, compare against TMuC-0.9-anchor with HHI context selection for significant_coeff_flag turned off completely [2]. The simplified context selection still delivers gains of 1.3 to 2.7%.

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | Y BD-rate | Y BD-rate |
| Class A | -0.9 | -1.0 | -1.2 |
| Class B | -1.9 | -2.3 | -2.3 |
| Class C | -1.2 | -1.1 | -1.3 |
| Class D | -1.0 | -1.2 | -1.0 |
| Class E | -2.4 | -1.8 | -1.9 |
| All | -1.5 | -1.5 | -1.6 |
| Enc Time[%] | 95% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Random access |  |  |
|  | Y BD-rate | Y BD-rate | Y BD-rate |
| Class A | -0.7 | -1.0 | -1.2 |
| Class B | -1.7 | -2.4 | -1.9 |
| Class C | -1.4 | -1.1 | -0.8 |
| Class D | -1.0 | -0.4 | -0.1 |
| Class E |  |  |  |
| All | -1.3 | -1.4 | -1.0 |
| Enc Time[%] | 93% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | Y BD-rate | Y BD-rate |
| Class A |  |  |  |
| Class B | -3.0 | -2.6 | -2.8 |
| Class C | -2.5 | -1.7 | -1.3 |
| Class D | -2.5 | 0.2 | 0.3 |
| Class E | -2.6 | 0.4 | -1.4 |
| All | -2.7 | -1.1 | -1.4 |
| Enc Time[%] | 91% |  |  |
| Dec Time[%] | 98% |  |  |

Table 3  Average gain from using simplified HHI context selection for significant_coeff_flag with dependency on only 4 neighbors, versus not have any dependency at all.  Negative numbers in the table means coding gain.  For significant_coeff_flag count, negative number means reduction in bins.

# Summary

In this contribution, we presented modifications to the context selection of significant_coeff_flag in HHI_TRANFORM_CODING by reducing the dependency to less than half with only 0 to 0.2% coding loss (i.e. it still provides 1.3 to 2.7% coding gain, compared to disabling HHI context selection for significant_coeff_flag).   This simplified context selection had a negligible effect on number significant_coeff_flag bins.  The contexts requires for significant_coeff_flag is also reduced from 128 to 108. We propose that this simplification be adopted into the HEVC test model.

# References

- “Description of video coding technology proposal by Fraunhofer HHI”, JCTVC-A116, 1st JCT-VC Meeting, Dresden, DE, April 2010.

- “CE11: Coding Efficiency of Tools in HHI_TRANSFORM_CODING (JCTVC-A116)”, JCTVC-Dxx, 4th Meeting, Daegu, KR, January 2011.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, 2nd. JCT-VC Meeting, Geneva, CH, July 2010.

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).