| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D196 |
| --- | --- |

| Title: | CE6: Cross verification of Toshiba bidirectional intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jani Lainema Kemal Ugur | Email: | jani.lainema@nokia.com kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports Core Experiment 6.a / bidirectional intra prediction cross verification results by Nokia. The results indicate 1.4 % and 2.0 % average bitrate reductions in Intra HE and Intra LC configurations, respectively.

# Results

Summary of the results are shown in the table below and more detailed results can be found in the accompanying Excel sheet. The simulations were run in the 64-bit Linux environment (on 16 core Intel(R) Xeon(R) CPU L5520 @ 2.27GHz nodes).

|  | Intra HE |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.6 | -1.0 | -1.1 | -2.3 | -1.7 | -1.7 |
| Class B | -1.6 | -1.0 | -1.1 | -2.0 | -1.5 | -1.5 |
| Class C | -1.4 | -0.7 | -0.7 | -1.7 | -1.1 | -1.1 |
| Class D | -1.1 | -0.5 | -0.5 | -1.6 | -0.9 | -0.9 |
| Class E | -1.7 | -1.3 | -1.1 | -2.6 | -2.2 | -1.8 |
| All | -1.4 | -0.9 | -0.9 | -2.0 | -1.4 | -1.4 |
| Enc Time[%] | 104 % |  |  | 112 % |  |  |
| Dec Time[%] | 104 % |  |  | 108 % |  |  |

# Findings and recommendations

Results indicate that bidirectional intra prediction as defined in this core experiment provides bitrate reductions in both tested configurations and all tested classes of test sequences. The average luminance bitrate reductions were 1.4 % and 2.0 % in Intra HE and Intra LC configurations, respectively. As bidirectional intra prediction can provide effects similar to smoothing of the reference samples used for intra prediction it may be advisable to study further the interaction and relationship of bidirectional intra prediction and different adaptive intra smoothing (AIS) technologies.

The tested algorithms had relatively minor complexity increase in the HM 0.9 environment. In Intra HE configuration both encoder and decoder runtimes increased 4 %. Whereas, in Intra LC configuration average encoder runtime increased 12% while the decoder runtime increased 8 %.

# Patent rights declaration(s)

Nokia may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).