| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D197 WG11 Number: m18957 |
| --- | --- |

| Title: | Proposition for robust parsing with temporal predictor |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Jung, G. Clare Orange Labs 38-40 rue du Général Leclerc 9279 ISSY LES MOULINEAUX | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange-ftgroup.com gordon.clare@orange-ftgroup.com |
| Source: | Orange Labs |  |  |

_____________________________

# Introduction

As previously noted by Sharp in JCTVC-C257, there is an error resilience issue in the current HEVC design.  The problem is related to the parsing of the motion predictor index when previous reference frames are missing. The existing predictor-index signaling method in combination with the use of a temporal predictor results in the impossibility of parsing the predictor index unless all reference frames are present.

Four basic solutions for making the parsing robust are studied. All of them bring losses between 1.5 and 2%. Another scheme using a modified temporal predictor is proposed. The average loss compared to these basic solutions is reduced.

# Problem statement

In the current HEVC design, motion vector competition is used in order to improve R-D performance for skip, direct and inter blocks.  For each block, the encoder chooses the best predictor in a set of available predictors (best in an R-D sense), and then signals the index of the chosen predictor in the bitstream where necessary. In order to improve the signaling performance of the chosen index, two operations are performed.

The first operation is that the set of predictors is ‘pruned’ – a given predictor is only added to the set if there is no predictor already in the set with the same value.  This results in a smaller set, and signaling a smaller index in a smaller set uses less symbols. The second operation is simply the signaling of the index in the bitstream.  The predictor index is signaled in unary, but in a ‘truncated’ fashion that depends on the size of the set.  . The fact that the size of the set of predictors (and so the signaling of the predictors) relies on the value of the temporal predictor makes the scheme non robust.  With a set size of N, the index x being signaled ranges from 0 through N-1.  To signal x, x 1-symbols are transmitted.  A terminating 0-symbol is emitted if x < N-1, i.e. if x is not the last in the set.  This ‘truncation’ results in less symbols being coded and is particularly effective for small sets.  In the extreme case of a pruned set containing just a single element, no symbols are transmitted in the bitstream, and x is inferred to be 0.

This encoding is shown in the Table below For values of N of 1 through 5.

| Index | Code when N=1 | Code when N=2 | Code when N=3 | Code when N=4 | Code when N=5 |
| --- | --- | --- | --- | --- | --- |
| 0 | (inferred) | 0 | 0 | 0 | 0 |
| 1 |  | 1 | 10 | 10 | 10 |
| 2 |  |  | 11 | 110 | 110 |
| 3 |  |  |  | 111 | 1110 |
| 4 |  |  |  |  | 1111 |

In order to allow the decoder to determine the index x for a given block, it is thus critical that the decoder can determine the size of the set.  This is always possible if all frames are present – all predictors can be correctly constructed.  In the case where a previous reference frame has been lost, however, it is not possible to determine the correct value of a temporal predictor that is derived from that frame. If such a predictor is encountered, the set of predictors cannot be correctly constructed, and the size of the set is not known.  If the size of the set is not known, the index simply cannot be parsed.

# Testing of ‘basic’ robust solutions

Robust reference 0: Remove the set trimming process and replace unavailable predictors with a default value, such as 0.  If the set of predictors is always guaranteed to be the same size, there is no parsing problem. This method has not been tested. It is expected to provide uninteresting results as the set is always maximum-sized and contains redundant values.  It would be worse than robust reference 4, below.

Robust reference 1: “Spatial Only” – The spatial-only predictors depend only on the current frame being parsed and are thus always available.  The pruning of the set by the decoder is guaranteed to produce the same set as found by the encoder. Motion Vector Competition is enabled with the spatial predictors only, to solve the robustness issue. The spatial set is pruned as normal, and the predictor index is signaled as normal.

|  | Random Access | Random Access - LC | Low Delay | Low Delay - LC |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | Y BD-rate | Y BD-rate | Y BD-rate |
| ClassA | 2,5 | 2,7 | - | - |
| ClassB | 1,8 | 1,4 | 1,8 | 3,0 |
| ClassC | 1,6 | 1,7 | 2,3 | 3,3 |
| ClassD | 2,0 | 1,9 | 2,2 | 2,8 |
| ClassE | - | - | 2,5 | 3,0 |
| AVG | 1,9 | 1,8 | 2,2 | 3,0 |
| AVG without classE | 1,9 | 1,8 | 2.1 | 3.0 |

(Reference: CU merge non robust, HM0.9 default configuration, positive values indicate a loss)

Robust reference 2: “Median Only” – Motion Vector Competition is disabled, and the median predictor is used. No pruning or predictor index signaling is required as there is always just a single element in the set of predictors

|  | Random Access | Random Access - LC | Low Delay | Low Delay - LC |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | Y BD-rate | Y BD-rate | Y BD-rate |
| ClassA | 3,4 | 4,7 | - | - |
| ClassB | 2,5 | 3,1 | 2,2 | 3,9 |
| ClassC | 2,4 | 3,4 | 2,8 | 4,4 |
| ClassD | 2,6 | 3,4 | 2,5 | 3,6 |
| ClassE | - | - | 3,0 | 3,6 |
| AVG | 2,6 | 3,5 | 2,6 | 3,9 |
| AVG without classE | 2,6 | 3,5 | 2.5 | 4.0 |

(Reference: CU merge non robust, HM0.9 default configuration, positive values indicate a loss)

Robust reference 3: “T + Median” – The temporal predictor is introduced in a robust way. Only the temporal predictor is used. In the case where the temporal predictor is not available, the spatial median is used instead.  No pruning or predictor index signaling is required as there is always just a single element in the set of predictors.

|  | Random Access | Random Access - LC | Low Delay | Low Delay - LC |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | Y BD-rate | Y BD-rate | Y BD-rate |
| ClassA | 0,9 | 2,2 | - | - |
| ClassB | 1,0 | 2,2 | 0,8 | 1,6 |
| ClassC | 1,6 | 3,2 | 1,5 | 2,5 |
| ClassD | 1,2 | 2,7 | 0,9 | 1,7 |
| ClassE | - | - | -0,5 | -1,3 |
| AVG | 1,2 | 2,6 | 0,8 | 1,3 |
| AVG without classE | 1,2 | 2,6 | 1.0 | 1.9 |

(Reference: CU merge non robust, HM0.9 default configuration, positive values indicate a loss)

Robust reference 4: “Full idx” – A possible solution is to signal the predictor index in a way that does not depend on the size of the set of predictors.  If the correct set is unable to be created by the decoder, the decoder can still correctly parse the index x, even though it cannot later determine the correct predictor value for reconstruction.  The simplest way is to guarantee the presence of the terminating ‘0’ symbol when signaling the predictor index, even when signaling the last element in the set unless the set is at its defined maximum size. Here, Motion Vector Competition is used with the classical spatio-temporal set. The set is pruned as normal, but the predictor index is fully and systematically signaled.  I.e., the signaling of the index is not ‘truncated’ as described above – there is always a terminating ‘0’ symbol even when signaling the use of the last element in the set.  This includes the case where there is a single element in the set.  (In which case previously inferred to be zero, the index 0 is now signaled with only the terminating symbol ‘0’).  It does not include the case where the set is completely full.  The encoding table becomes (with differences from above highlighted in yellow):

| Index | Code when N=1 | Code when N=2 | Code when N=3 | Code when N=4 | Code when N=5 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 |  | 10 | 10 | 10 | 10 |
| 2 |  |  | 110 | 110 | 110 |
| 3 |  |  |  | 1110 | 1110 |
| 4 |  |  |  |  | 1111 |

|  | Random Access | Random Access – LC | Low Delay | Low Delay – LC |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | Y BD-rate | Y BD-rate | Y BD-rate |
| ClassA | 1,6 | 2,3 | - | - |
| ClassB | 1,1 | 2,1 | 0,8 | 2,6 |
| ClassC | 1,2 | 1,9 | 1,1 | 2,3 |
| ClassD | 1,4 | 2,1 | 1,3 | 2,2 |
| ClassE | - | - | 2,5 | 7,2 |
| AVG | 1,2 | 2,1 | 1,3 | 3,3 |
| AVG without classE | 1,2 | 2,1 | 1.1 | 2.4 |

(Reference: CU merge non robust, HM0.9 default configuration, positive values indicate a loss)

Among these simple solutions, robust reference 3 (“T + Median”) is the best.

# Proposal: description and results

## Method 1

The proposed method is based on the Robust Reference 4 (“Full idx”).

The standard set of predictors:

- M (median of A, B, X)

- A (1st left neighbor found)

- B (1st top neighbor found)

- X (a corner, 1st of C, E, D neighbors)

- T (a temporal, found by mapping the origin of the block to a collocated frame)

becomes:

- Tctr (a temporal, found by mapping the center of the block to a collocated frame)

- M (median of A, B, X)

- A (1st left neighbor found)

- B (1st top neighbor found)

- X (a corner, 1st of C, E, D neighbors)

This new set is pruned as normal, but the index is fully and systematically signaled, as described in the description of ‘robust reference 4.’  The parsing of this index then does not depend on the size of the pruned set of predictors.

|  | Random Access | Random Access – LC | Low Delay | Low Delay – LC |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | Y BD-rate | Y BD-rate | Y BD-rate |
| ClassA | -0,8 | 0,7 | - | - |
| ClassB | -0,2 | 1,5 | 0,3 | 2,4 |
| ClassC | 0,1 | 1,3 | 0,5 | 2,0 |
| ClassD | 0,1 | 1,3 | 0,7 | 1,8 |
| ClassE | - | - | 0,2 | 6,3 |
| AVG | -0,1 | 1,3 | 0,4 | 2,9 |
| AVG without classE | -0.1 | 1.3 | 0.5 | 2.1 |

(Reference: CU merge non robust, HM0.9 default configuration, positive values indicate a loss)

# Conclusion

The proposed robust method reduces the loss compared to the basic robust reference methods. In the case of the RAHE configuration, no loss is observed compared to the non robust reference. Consequently, we recommend the adoption of this new configuration that allows to make the parsing of the stream robust in the case of frame losses, with reduced coding efficiency losses.

# Patent rights declaration(s)

Orange Labs may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).