| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D198 |
| --- | --- |

| Title: | CE10: Cross verification of directional intra prediction configurations |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jani Lainema Kemal Ugur | Email: | jani.lainema@nokia.com kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports Core Experiment 10 cross verification results by Nokia. The results demonstrate different complexity – coding efficiency trade-offs obtained by varying the number of intra prediction directions for different PU sizes in the HM 0.9 environment.

# Results

Summary of the results are shown in the table below and more detailed results can be found in the accompanying Excel sheet. The simulations were run in the 64-bit Linux environment (on 16 core Intel(R) Xeon(R) CPU L5520 @ 2.27GHz nodes).

| Block size | Number of Intra Prediction Directions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Reference | Test |  |  |  |  |  |  |  |  |  |  |  |
|  |  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 |
| 4x4 | 17 | 17 | 9 | 9 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 9 | 17 |
| 8x8 | 34 | 17 | 9 | 34 | 17 | 9 | 34 | 34 | 34 | 34 | 34 | 17 | 34 |
| 16x16 | 34 | 17 | 9 | 34 | 34 | 34 | 17 | 9 | 34 | 34 | 34 | 17 | 34 |
| 32x32 | 34 | 17 | 9 | 34 | 34 | 34 | 34 | 34 | 17 | 9 | 34 | 17 | 34 |
| 64x64 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 3 | 5 | 5 |
| Intra | BDBR | 1.8 | 5.1 | 0.6 | 0.6 | 1.2 | 0.4 | 0.7 | 0.1 | 0.1 | 0.0 | 2.6 | -0.2 |
|  | EncTime | 95 % | 89 % | 97 % | 97 % | 95 % | 98 % | 98 % | 99 % | 98 % | 99 % | 92 % | 203 % |
|  | DecTime | 101 % | 101 % | 100 % | 100 % | 100 % | 100 % | 100 % | 100 % | 100 % | 100 % | 101 % | 100 % |
| IntraLoCo | BDBR | 1.9 | 6.3 | 1.1 | 1.2 | 2.2 | 0.3 | 0.5 | 0.0 | 0.0 | 0.0 | 3.4 | -0.7 |
|  | EncTime | 90 % | 79 % | 96 % | 96 % | 91 % | 97 % | 95 % | 97 % | 96 % | 99 % | 86 % | 216 % |
|  | DecTime | 102 % | 101 % | 99 % | 100 % | 101 % | 100 % | 100 % | 100 % | 100 % | 100 % | 100 % | 99 % |

# Findings and recommendations

Results indicate that 17, 34 and 34 prediction directions for PUs of size 4x4, 8x8 and 16x16, provide very good complexity – coding efficiency trade-off. When it comes to larger block sizes it may be desirable to limit the number of directions to 9 and 3 for PUs of size 32x32 and 64x64, respectively (if there is no visual quality degradation associated with the limitations).

It was further found out the current encoder algorithm performs very well compared to the full RDO approach tested in Test #12. The full RDO encoder has effectively double the run time compared to the default configuration, but achieves only 0.2 % and 0.7 % bitrate reductions in Intra HE and Intra LC configurations, respectively.

# Patent rights declaration(s)

Nokia may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).