| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D199 |
| --- | --- |

| Title: | Configurable directional intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Kemal Ugur | Email: | jani.lainema@nokia.com kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes signaling the number of available intra prediction directions for each PU size. The motivation for this is to allow low complexity encoders achieve better compression efficiency as well as improve codec’s adaptivity to different types of content.

# Introduction

Low complexity video encoders may decide to check only a limited number of directional intra prediction methods in order to lower encoding complexity. In these cases there is some sub-optimality in signaling the intra prediction modes, as some of those will never appear in the generated bitstream. In this contribution we study the effect of being able to indicate the number of intra prediction directions checked by the encoder for each of the PU sizes. The performance of signaling the number of modes encoder checks is compared against HM 0.9 codec where similar restriction is applied as an encoder only feature.

# Implementation

In order to signal the maximum number of prediction directions we include a one bit identifier in the SPS (Sequence Parameter Set) to indicate if the maximum number of prediction directions for each PU is identical to the maximum numbers defined in HM 0.9. If the number of prediction directions is different from these defaults values for one or more PU sizes, we signal in SPS with 1 to 3 bits the maximum number of intra prediction directions (3, 5, 9, 17 or 34) for the each PU size separately.

# Results

Summary of the results are shown in the tables below and more detailed results can be found in the accompanying Excel sheet. The simulations were run in the 64-bit Linux environment (on 16 core Intel(R) Xeon(R) CPU E5540 @ 2.53GHz nodes).

In the experiments we use the default maximum number of intra prediction directions for each PU size as defined in HM but clip those to 17, 9, 5 or 3 in the four experiments performed. That is, the following number of intra prediction directions for each PU size is used in different sets of experiments:

|  | Maximum number of prediction directions |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PU size | Anchor | 17 | 9 | 5 | 3 |
| 4x4 | 17 | 17 | 9 | 5 | 3 |
| 8x8 | 34 | 17 | 9 | 5 | 3 |
| 16x16 | 34 | 17 | 9 | 5 | 3 |
| 32x32 | 34 | 17 | 9 | 5 | 3 |
| 64x64 | 5 | 5 | 5 | 5 | 3 |

The Bjøntegaard delta bitrate increases associated with the case where we indicate the maximum number of prediction directions in the bitstream (“Signaling the number” column in the table below) and with the case where an encoder only restriction in HM 0.9 is performed (“Encoder restriction”) are reported below.

| Intra High Efficiency (Luminance BDBR) |  |  |  | Intra Low Complexity (Luminance BDBR) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number of directions | Signaling the number | Encoder restriction | Difference | Number of directions | Signaling the number | Encoder restriction | Difference |
| 17 | 1.8 | 2.5 | -0.7 | 17 | 1.9 | 2.8 | -0.9 |
| 9 | 5.1 | 6.4 | -1.3 | 9 | 6.3 | 8.8 | -2.5 |
| 5 | 9.0 | 10.0 | -1.1 | 5 | 11.5 | 14.9 | -3.4 |
| 3 | 11.0 | 11.8 | -0.8 | 3 | 14.6 | 18.6 | -4.1 |

In Intra HE configuration the average bitrate reduction from being able to indicate the number of intra predictions vary from 0.7 % to 1.3 %, whereas in Intra LC configuration the gain varies from 0.9 % to 4.1 %. We feel signaling the maximum number of intra prediction directions in the bitstream is a useful feature in low complexity encoder environments and propose it to be included in the future HM releases.

# Patent rights declaration(s)

Nokia may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).