| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D200 |
| --- | --- |

| Title: | High layer syntax to improve support for temporal scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce, Danny Hong, Alexandros Eleftheriadis 433 Hackensack Ave Hackensack, NJ 07601 USA | Tel: Email: | +1 732 912 8648 jill@vidyo.com danny@vidyo.com alex@vidyo.com |
| Source: | Vidyo, Inc. |  |  |

_____________________________

# Abstract

High layer syntax changes are proposed to improve support for temporal scalability in the HEVC design.  The contribution proposes adding normative semantics for temporal_id, adding a temporal_switching_point_flag to the NAL unit header, moving temporal_id_nesting_flag to the sequence parameter set, and an SEI message to describe the temporal coding picture structure for the sequence.  The proposed changes bring benefits to bitstream extractors, transraters, and parallel decoding.

# Introduction

The H.264/AVC standard is flexible enough to allow for a variety of patterns of coding pictures and reference pictures.  The hierarchical P and hierarchical B coding patterns used for temporal scalability can be supported using ref_pic_list_modification() and dec_ref_pic_marking()/MMCO syntax elements.

With the HEVC design, there is now an opportunity to specify a cleaner design for improved support of temporal scalability.   Temporal scalability is widely accepted and adopted, and is used in the HEVC test conditions, because of its coding efficiency benefits.

In this contribution we propose:

- Normative semantics for temporal_id

- Moving temporal_id_nesting_flag to the sequence parameter set, and adding a temporal_switching_point_flag  to the NAL unit header, and

- SEI message describing the temporal picture coding structure

These proposals would simplify the operation of bitstream extractors, such as gateways, which attempt to exploit temporal scalable bitstreams.  Parallel decoding capability could be improved, by providing more explicit information to determine when it is unnecessary to wait for the decoding of a previously coded picture.

# Normative semantics for temporal_id

The WD already includes the temporal_id field in the NAL unit header.  We propose normative semantics associated with the temporal_id, disallowing reference picture predictions from higher temporal layers.

The SVC design placed temporal_id in the NAL unit header SVC extension, and required that sub-bitstreams be compliant, but did not have any direct impact on decoding process.

All sub-bitstreams that can be derived using the sub-bitstream extraction process as specified in subclause G.8.8.1 with any combination of values for priority_id, temporal_id, dependency_id, or quality_id as the input shall result in a set of coded video sequences, with each coded video sequence conforming to one or more of the profiles specified in Annex A and Annex G.

A normative restriction in reference picture patterns allows the exploitation of temporal scalability by bitstream extractors, by ensuring that access units containing higher temporal layers may be safely discarded without impacting the decoding of access units containing lower temporal layers.  We propose to change the reference picture list initialization process, so that reference pictures with higher temporal_id values are not assigned indices in the reference picture prediction list, as described in more detail in the section below.    ref_pic_list_modification( ) or dec_ref_pic_marking() will no longer be required for hierarchical picture coding patterns.  The temporal_id syntax element does not change which pictures are actually stored in memory, but merely changes the index assignment.

Encoders are free to avoid the temporal_id coding restrictions by encoding all pictures with temporal_id equal to 0, which imposes no restrictions on selection of reference pictures.

## Proposed text changes for normative semantics for temporal_id

temporal_id specifies a temporal identifier for the NAL unit. The assignment of values to temporal_id is constrained by the sub-bitstream extraction process as specified in subclause XX.

The value of temporal_id shall be the same for all NAL units of an access unit. When an access unit contains any NAL unit with nal_unit_type equal to 5, temporal_id shall be equal to 0.

Pictures with higher values of temporal_id can not be used as reference pictures for pictures with lower values of temporal_id, as specified in the decoding process for reference list construction process in subclause X (8.2.4.2.1 and 8.2.4.2.3 in AVC).

FROM AVC:

8.2.4 Decoding process for reference picture lists construction

8.2.4.2 Initialisation process for reference picture lists

This initialisation process is invoked when decoding a P, SP, or B slice header.

RefPicList0 and RefPicList1 have initial entries as specified in subclauses 8.2.4.2.1 through 8.2.4.2.5. When the number of entries in the initial RefPicList0 or RefPicList1 produced as specified in subclauses 8.2.4.2.1 through 8.2.4.2.5 is greater than num_ref_idx_l0_active_minus1 + 1 or num_ref_idx_l1_active_minus1 + 1, respectively, the extra entries past position num_ref_idx_l0_active_minus1 or num_ref_idx_l1_active_minus1 are discarded from the initial reference picture list.

When the number of entries in the initial RefPicList0 or RefPicList1 produced as specified in subclauses 8.2.4.2.1through 8.2.4.2.5 is less than num_ref_idx_l0_active_minus1 + 1 or num_ref_idx_l1_active_minus1 + 1, respectively, the remaining entries in the initial reference picture list are set equal to "no reference picture".

8.2.4.2.1 Initialisation process for the reference picture list for P and SP slices in frames

This initialisation process is invoked when decoding a P or SP slice in a coded frame.

When this process is invoked, there shall be at least one reference frame or complementary reference field pair that is currently marked as "used for short-term reference" or "used for long-term reference".  Pictures with higher values of temporal_id than the current picture cannot be used for reference, and are not included in the reference picture list.  The reference picture list RefPicList0 is ordered so that short-term reference frames and short-term complementary reference field pairs have lower indices than long-term reference frames and long-term complementary reference field pairs.

The short-term reference frames and complementary reference field pairs are ordered starting with the frame or complementary field pair with the highest PicNum value and proceeding through in descending order to the frame or complementary field pair with the lowest PicNum value, excluding any frame or complementary field pair with a temporal_id value higher than that of the current picture.

The long-term reference frames and complementary reference field pairs are ordered starting with the frame or complementary field pair with the lowest LongTermPicNum value and proceeding through in ascending order to the frame or complementary field pair with the highest LongTermPicNum value, excluding any frame or complementary field pair with a temporal_id value higher than that of the current picture.

8.2.4.2.3 Initialisation process for reference picture lists for B slices in frames

This initialisation process is invoked when decoding a B slice in a coded frame.

For purposes of the formation of the reference picture lists RefPicList0 and RefPicList1 the term reference entry refers in the following to decoded reference frames or complementary reference field pairs.

When this process is invoked, there shall be at least one reference entry that is currently marked as "used for short-term reference" or "used for long-term reference".

Pictures with higher values of temporal_id than the current picture cannot be used for reference, and are not included in either RefPicList0 or RefPicList1.

For B slices, the order of short-term reference entries in the reference picture lists RefPicList0 and RefPicList1 depends on output order, as given by PicOrderCnt( ). When pic_order_cnt_type is equal to 0, reference pictures that are marked as "non-existing" as specified in subclause 8.2.5.2 are not included in either RefPicList0 or RefPicList1.

NOTE 1 – When gaps_in_frame_num_value_allowed_flag is equal to 1, encoders should use reference picture list reordering to ensure proper operation of the decoding process (particularly when pic_order_cnt_type is equal to 0, in which case PicOrderCnt( ) is not inferred for "non-existing" frames).

The reference picture list RefPicList0 is ordered such that short-term reference entries have lower indices than long-term reference entries. It is ordered as follows.

– Let entryShortTerm be a variable ranging over all reference entries that are currently marked as "used for short-term reference" and which have a value of temporal_id equal to or lower than the temporal_id of the current picture. When some values of entryShortTerm are present having PicOrderCnt( entryShortTerm ) less than PicOrderCnt( CurrPic ), these values of entryShortTerm are placed at the beginning of refPicList0 in descending order of PicOrderCnt( entryShortTerm ). All of the remaining values of entryShortTerm (when present) are then appended to refPicList0 in ascending order of PicOrderCnt( entryShortTerm ).

– The long-term reference entries which have a value of temporal_id equal to or lower than the temporal_id of the current picture are ordered starting with the long-term reference entry that has the lowest

LongTermPicNum value and proceeding through in ascending order to the long-term reference entry that has the highest LongTermPicNum value.

The reference picture list RefPicList1 is ordered so that short-term reference entries have lower indices than long-term reference entries. It is ordered as follows.

– Let entryShortTerm be a variable ranging over all reference entries that are currently marked as "used for short-term reference" and which have a value of temporal_id equal to or lower than the temporal_id of the current picture.. When some values of entryShortTerm are present having PicOrderCnt( entryShortTerm ) greater than PicOrderCnt( CurrPic ), these values of entryShortTerm are placed at the beginning of refPicList1 in ascending order of PicOrderCnt( entryShortTerm ). All of the remaining values of entryShortTerm (when present) are then appended to refPicList1 in descending order of PicOrderCnt( entryShortTerm ).

– Long-term reference entries which have a value of temporal_id equal to or lower than the temporal_id of the current picture are ordered starting with the long-term reference frame or complementary reference field pair that has the lowest LongTermPicNum value and proceeding through in ascending order to the long-term reference entry that has the highest LongTermPicNum value.

# Add temporal_switching_point_flag to the NAL unit header and temporal_id_nesting_flag to the Sequence Parameter Set

By imposing the normative restrictions associated with temporal_id as proposed in the preceding section, a bitstream extractor may safely discard pictures from higher temporal layers, without impacting the decoding of lower temporal layers.  However, it does not provide enough information for the bitstream extractor to allow switching between temporal layers.

The SVC design includes the temporal_id_nesting_flag in the Scalability information SEI message, and the tl_switching_point SEI message to enable temporal layer switching.   Because these SEI messages are optional, gateways may not depend upon their being present.  These SEI messages are not part of AVC standard or the current HEVC design, even though they both support temporal scalability.

We propose a cleaner solution to enable temporal layer switching by gateways.   We propose to move the temporal_id_nesting_flag to the Sequence Parameter Set, and define normative semantics associated with it.   We also propose to introduce a temporal_switching_point_flag in the NAL unit header.

If temporal_id_nesting_flag is set to 1, when a lower temporal_id picture is decoded, pictures with higher temporal_id values can no longer be used for prediction.   This is enforced by setting as “unused for reference” all pictures in the reference picture list with higher temporal_id values, which means that those pictures will be removed from reference picture memory.

The use of temporal_id_nesting_flag  impacts all of the pictures and all of temporal layers in a sequence.  More flexibility can be gained by using the temporal_switching_point_flag for individual pictures, enabling encoders to control the tradeoff between multiple reference picture coding efficiency gains and frequency of temporal switching points.

temporal_switching_point_flag will indicate if a particular picture represents a temporal switching point, where higher temporal layers can start being decoded.  This is useful for gateways to know when it is appropriate to start passing through higher layer data when switching to a higher temporal layer is desired.  This flag is also useful for DVR operation and error resiliency.

Consider the Figure 1 below which shows an example  hierarchical P picture coding structure with multiple reference pictures  The first predictor in a prediction list is shown with a solid arrow, and additional predictors allowed by multiple reference picture prediction for only pictures P3 and P6  shown with a dashed arrow.  As shown in Figure 1(a), if multiple reference pictures are used, and temporal_id_nesting_flag is equal to 0, picture P3 can be predicted from pictures P2, P1, or I0, and picture P6 can be predicted from P4, P2, or I0.    As shown in Figure 2(b), if temporal_id_nesting_flag is equal to 1, picture P3 can be predicted from P2 or I0, but not from P1, and picture P6 can be predicted from P4 or I0, but not from P2.

Using temporal_switching_point_flag allows more flexibility.  As shown in Figure 2(c), if temporal_switching_point_flag was equal to 1 for picture P2, but equal to 0 for picture P4,  a switching point from layer 1 to layer 2 is created, but not a switching point from layer 0 to layer 1.  A decoder that was already decoding layers 0 and 1 could add layer 2 starting with the layer 2 coded picture following P2, which is P3.  Picture P3 could be predicted from P2 or I0, but not from P1.  But differently from if the temporal_id_nesting_flag were used, the predictor list for P6 is unchanged, and picture P6 can be predicted from P4, P2, or I0.  Allowing P6 to be predicted from P2with multiple reference pictures potentially improves coding efficiency.

## Proposed syntax changes

In the NAL unit header

| nal_unit( NumBytesInNALunit ) { | C | Descriptor |
| --- | --- | --- |
| forbidden_zero_bit | All | f(1) |
| nal_ref_idc | All | u(2) |
| nal_unit_type | All | u(5) |
| NumBytesInRBSP = 0 |  |  |
| nalUnitHeaderBytes = 1 |  |  |
| if( nal_unit_type = = 1 ¦ ¦ nal_unit_type = = 5 ) { |  |  |
| temporal_id | All | u(3) |
| output_flag | All | u(1) |
| temporal_switching_point_flag | All | u(1) |
| reserved_zero_3bits | All | u(3) |
| nalUnitHeaderBytes += 1 |  |  |
| } |  |  |

temporal_switching_point_flag specifies if the current access point is a temporal switching point allowing the decoding of higher temporal id layers following this access unit.

If temporal_switching_point_flag is equal to 1, all pictures with higher values of temporal_id in the reference picture storage are marked as “unused for reference”, as specified in subclause X.

The value of temporal_switching_point_flag shall be the same for all NAL units of an access unit.

If temporal_id_nesting_flag is equal to 1, temporal_switching_point_flag shall be equal to 1.

NOTE – When starting to decode a high temporal layer, availability of required reference pictures can be guaranteed immediately following an IDR, or a picture with a lower value of temporal_id and temporal_switching_flag equal to 1.

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| profile_idc | 0 | u(8) |
| reserved_zero_8bits /* equal to 0 */ | 0 | u(8) |
| level_idc | 0 | u(8) |
| seq_parameter_set_id | 0 | ue(v) |
| bit_depth_luma_minus8 | 0 | ue(v) |
| bit_depth_chroma_minus8 | 0 | ue(v) |
| increased_bit_depth_luma | 0 | ue(v) |
| ine_bit_depth_chroma | 0 | ue(v) |
| log2_max_frame_num_minus4 | 0 | ue(v) |
| pic_order_cnt_type | 0 | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |  |
| log2_max_pic_order_cnt_lsb_minus4 | 0 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |  |
| delta_pic_order_always_zero_flag | 0 | u(1) |
| offset_for_non_ref_pic | 0 | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | 0 | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |  |
| offset_for_ref_frame[ i ] | 0 | se(v) |
| } |  |  |
| max_num_ref_frames | 0 | ue(v) |
| gaps_in_frame_num_value_allowed_flag | 0 | u(1) |
| temporal_id_nesting_flag | 0 | u(1) |

temporal_id_nesting_flag specifies whether inter prediction is additionally restricted for the target access unit set.

Dependent on temporal_id_nesting_flag, the following applies.

– If temporal_id_nesting_flag is equal to 0, additional constraints may not be obeyed.

– Otherwise (temporal_id_nesting_flag is equal to 1), the following constraint shall be obeyed for layer representations with any combination of dependency_id and quality_id values present in the target access unit set.

For each access unit auA with temporal_id equal to tIdA, an access unit auB with temporal_id equal to tIdB and tIdB  less than or equal to tIdA shall not be referenced by inter prediction when there exists an access unit auC with

temporal_id equal to tIdC and tIdC less than tIdB, which follows the access unit auB and precedes the access unit auA in decoding order.

NOTE 2 – The syntax element temporal_id_nesting_flag is used to indicate that temporal up-switching, i.e., switching from decoding of up to a specific temporal_id tIdN to decoding up to a temporal_id tIdM > tIdN, is always possible.

If  temporal_id_nesting flag is equal to 1 temporal_switching_point_flag shall be equal to 1.

## Proposed semantics changes

8.2.5 Decoded reference picture marking process

This process is invoked for decoded pictures when nal_ref_idc is not equal to 0.

NOTE – The decoding process for gaps in frame_num that is specified in subclause 8.2.5.2 may also be invoked when nal_ref_idc is equal to 0, as specified in clause 8.

A decoded picture with nal_ref_idc not equal to 0, referred to as a reference picture, is marked as “used for short-term reference” or "used for long-term reference". For a decoded reference frame, both of its fields are marked the same as the frame. For a complementary reference field pair, the pair is marked the same as both of its fields. A picture that is marked as "used for short-term reference" is identified by its FrameNum and, when it is a field, by its parity. A picture that is marked as "used for long-term reference" is identified by its LongTermFrameIdx and, when it is a field, by its parity.

Frames or complementary field pairs marked as “used for short-term reference” or as "used for long-term reference" can be used as a reference for inter prediction when decoding a frame until the frame, the complementary field pair, or one of its constituent fields is marked as “unused for reference”. A field marked as “used for short-term reference” or as "used for long-term reference" can be used as a reference for inter prediction when decoding a field until marked as “unused for reference”.

A picture can be marked as "unused for reference" by the sliding window reference picture marking process, a first-in, first-out mechanism specified in subclause 8.2.5.3 or by the adaptive memory control reference picture marking process, a customised adaptive marking operation specified in subclause 8.2.5.4, or by considering the temporal layer prediction restrictions from temporal_switching_point_flag and temporal_id_nesting_flag.

A short-term reference picture is identified for use in the decoding process by its variables FrameNum and FrameNumWrap and its picture number PicNum, and a long-term reference picture is identified for use in the decoding process by its long-term picture number LongTermPicNum. When the current picture is not an IDR picture, subclause 8.2.4.1 is invoked to specify the assignment of the variables FrameNum, FrameNumWrap, PicNum and LongTermPicNum

8.2.5.1 Sequence of operations for decoded reference picture marking process

Decoded reference picture marking proceeds in the following ordered steps.

1. All slices of the current picture are decoded.

2. Depending on whether the current picture is an IDR picture, the following applies.

– If the current picture is an IDR picture, the following applies.

– All reference pictures are marked as "unused for reference".

– Depending on long_term_reference_flag, the following applies.

– If long_term_reference_flag is equal to 0, the IDR picture is marked as "used for short-term reference" and MaxLongTermFrameIdx is set equal to “no long-term frame indices”.

– Otherwise (long_term_reference_flag is equal to 1), the IDR picture is marked as "used for long-term reference", the LongTermFrameIdx for the IDR picture is set equal to 0, and MaxLongTermFrameIdx is set equal to 0.

– Otherwise (the current picture is not an IDR picture), the following applies.

- If temporal_switching_point_flag is equal to 1 or temporal_id_nesting_flag is equal to 1, all reference pictures with value of temporal_id greater than the current temporal_id are marked as “unused for reference”

- If adaptive_ref_pic_marking_mode_flag is equal to 0, the process specified in subclause 8.2.5.3 is invoked.

- Otherwise (adaptive_ref_pic_marking_mode_flag is equal to 1), the process specified in subclause 8.2.5.4 is invoked.

# SEI Message

Although the current HEVC design and the H.264/AVC standard allow for flexible patterns of coded pictures, many encoders, including the JM and HM reference software, generate sequences of coded pictures following a fixed pattern.   Even when a fixed pattern is in used, gateways and decoders are not aware of it.

Knowing the coding structure a priori can help in dropping frames for transraters and bitstream extractors.  Additionally, transraters can optimally choose frames to transcode and minimize drift.  DVRs can also utilize the coding structure information for operations like fast forwarding.  Finally, knowing the structure can enable decoders to process certain frames in parallel.  For example, for the hierarchical P picture coding structure shown below, pictures P1 and P2 can be processed in parallel, then P3 and P4, and so on.

An encoder generates a sequence of coded pictures that consists of one or more GOPs (Group of Pictures).  Here, a GOP is defined as a sequence of coded pictures that start with an I/IDR picture and ends with a picture immediately preceding the next I/IDR picture.  Many encoders generate one or more GOPs of the same structure of coded pictures.  Furthermore, each GOP generated by these encoders typically consists of a smaller fixed pattern that repeats.  The coding structure SEI message describes this smaller, repeating, fixed pattern of a temporal picture coding structure that is used by the encoder.  For example, in the figure shown below, the 3 B picture structure repeats between I and P pictures; this SEI message can be used to describe the repeating structure.  Hereinafter, the first lowest temporal layer picture followed by the repeating set of pictures (in coding order) are referred to as an SOP (Structure of Pictures).  A GOP (Group of Pictures) begins with an I frame, and includes one or more SOPs.  In the example below, there are 4 pictures in the SOP and 3 SOPs in the GOP: the first SOP consists of the first I picture followed by the 3 immediately following B pictures, the second SOP consists of the first P picture followed by the next 3 B pictures, and so on.

## Proposed text changes for SEI message

| coding_structure( payloadSize ) { | Descriptor |
| --- | --- |
| num_pictures_in_sop_minus1 | ue(v) |
| num_sops_in_gop | ue(v) |
| for( i = 0; i < num_pictures_in_sop_minus1; i++ ) { |  |
| primary_pic_type[ i ] | u(2) |
| ref_flag[ i ] | u(1) |
| temporal_num[ i ] | u(3) |
| display_num[ i ] | ue(v) |
| } |  |
| average_frame_rate_flag | u(1) |
| average_bit_rate_flag | u(1) |
| if( average_frame_rate_flag ) |  |
| average_frame_rate | u(16) |
| if( average_bit_rate_flag ) |  |
| for( i = 0; i < NumTemporalLayers; i++ ) |  |
| average_bit_rate[ i ] | u(16) |
| } |  |

num_pictures_in_sop_minus1 specifies the number of pictures in the SOP minus 1.  In the case of all intra or IPPP coding, this value shall be 0.

num_sops_in_gop specifies the number of SOPs in a GOP (between two intras).  In the case of all intra coding, this value shall be 1.  num_sops_in_gop equal to 0 specifies that there is no specific GOP structure and that the next expected I picture is unknown, or that the coded video sequence consists of just one GOP.

primary_pic_type has the same definition as specified in Table 7-2 of WD1 of HEVC [2].

ref_flag equal to 1 specifies that the coded picture is a reference picture.   ref_flag equal to 0 specifies that the coded picture is not a reference picture.

temporal_num specifies the temporal_id value associated with the coded picture.

display_num specifies the display order of the coded picture within the SOP.  Except for the very first picture in the SOP, each picture in the SOP is described in the coding order, which may be different from the display order.  This unique number specifies the display number of the coded picture within the SOP.

average_frame_rate_flag specifies whether the average frame rate (average_frame_rate) is specified.

average_frame_rate specifies the average frame rate in units of frames per 256 seconds of the entire coded sequence.  Using this information and the temporal_num information gathered for each coded picture in the SOP, the frame rate of each temporal layer can be derived.

average_bit_rate_flag specifies whether the average bit rate (average_bit_rate) is specified.

average_bit_rate indicates the average bit rate in units of 1000 bits per second of the temporal layer i.  All NAL units of the temporal layer, including all NAL units of the temporal layer j, where j < i, are taken into account in the calculation.

A coding structure SEI message shall not be present in access units that contain VCL NAL units of a primary coded picture with IdrPicFlag equal to 0.  The set of access units consisting of the access unit associated with the coding structure SEI message and all succeeding access units in decoding order until, but excluding, the next access unit that does not contain any VCL NAL unit of a primary coded picture with IdrPicFlag equal to 0 (if present) or the end of the bitstream (otherwise) is referred to as the target access unit set.  The coding structure SEI message applies to the target access unit set.

## Examples

The following four syntactic elements are specified for each coded picture (in the coding order) in the SOP except for the very first picture in the SOP.  For the very first picture, the following are implied:

- primary_pic_type = 0 or 1

- ref_flag = 1

- temporal_num = 0

- display_num is derived based on the display_num values used by other pictures in the SOP.  For example, if there are 4 pictures in a SOP, then each picture in the SOP is uniquely assigned to one of the values in [0, 3].  The one value that is not assigned to the three explicitly specified pictures is used as the value for the very first picture in the SOP.

In the examples below, the values (primary_pic_type, ref_flag, temporal_num, display_num) are specified as a quadruple.

1.  All intra

num_pictures_in_sop_minus1:	0

num_sops_in gop:			1

NumTemporalLayers is derived to be 1

2. IPPP

num_pictures_in_sop_minus1:	0

num_sops_in gop:			0

NumTemporalLayers is derived to be 1

3. IBBPBBPBBPBBI… (Open GOP)

num_pictures_in_sop_minus1:	2

num_sops_in gop:			4

i = 0: (2 (B), 0, 1, 1)

i = 1: (2 (B), 0, 1, 2)

NumTemporalLayers is derived to be 2

4. IBBPBBPBBPBBI… (Closed GOP)

num_pictures_in_sop_minus1:	2

num_sops_in gop:			4

i = 0: (2 (B), 0, 1, 0)

i = 1: (2 (B), 0, 1, 1)

NumTemporalLayers is derived to be 2

5. Hierachical-P with 3 temporal layers

num_pictures_in_sop_minus1:	3

num_sops_in gop:			0

i = 0: (1 (P), 0, 2, 1)

i = 1: (1 (P), 1, 1, 2)

i = 2: (1 (P), 0, 2, 3)

NumTemporalLayers is derived to be 3

6. Hierachical-B with 4 temporal layers

num_pictures_in_sop_minus1:	7

num_sops_in gop:			2

i = 0: (2 (B), 1, 1, 4)

i = 1: (2 (B), 1, 2, 2)

i = 2: (2 (B), 0, 3, 1)

i = 3: (2 (B), 0, 3, 3)

i = 4: (2 (B), 1, 2, 6)

i = 5: (2 (B), 0, 3, 5)

i = 6: (2 (B), 0, 3, 7)

NumTemporalLayers is derived to be 4

# References

- A. M. Tourapis, K. Sühring and G. Sullivan, “H.264/14496-10 AVC Reference Software Manual (revised for JM17.1),” JVT-AE010 revised, JVT-Manual, London, UK, June, 2009.

- T. Wiegand, W.-J. Han, B. Bross, J.-R. Ohm and G. J. Sullivan, “WD1: Working Draft 1 of High-Efficiency Video Coding,” JCTVC-C403, Guangzhou, CN, October, 2010.

# Patent rights declaration(s)

(NOTE – Activities in the JCT-VC and contributions to the JCT-VC are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-VC proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. An example of an informal IPR notification statement for a contribution is provided below.)

Vidyo, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).