| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D202 |
| --- | --- |

| Title: | 1:2 Spatial Scalability Support for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Danny Hong, Jill Boyce, Alexandros Eleftheriadis 433 Hackensack Ave. Hackensack, NJ 07601, USA | Tel: Email: | +1 (201) 289-8597 danny@vidyo.com jill@vidyo.com alex@vidyo.com |
| Source: | Vidyo, Inc. |  |  |

_____________________________

# Abstract

The H.264 SVC standard has been shown to enable very efficient and robust videoconferencing systems, utilizing temporal and spatial scalability.  This contribution proposes that the HEVC design be extended to incorporate a set of tools from SVC for supporting 1:2 spatial scalability.  Including scalability tools in the initial phase of HEVC enables a cleaner design, avoiding the need to retrofit the system design with ugly backwards-compatible patches, such as the NAL unit header SVC extension and the prefix NAL unit.  This contribution restricts spatial scalability support to 1:2 for simplicity and because of its usefulness in currently deployed applications.  Although 1:2 spatial scalability support for I, P and B pictures is described, because of time constraints, experimental results are only available for I pictures.  In the high-efficiency setting, 2-layer intra only scalable coding yields an average of (14.9%, 17.7%, 17.3%) (Y, Cb, Cr) BD-rate gain and a maximum of (29.8%, 33.7%, 35.4%) BD-rate gain over simulcast.  In the low-complexity setting, 2-layer intra only scalable coding yields an average of (13.7%, 16.7%, 16.6%) BD-rate gain and a maximum of (27.4%, 31.9%, 32.3%) BD-rate gain over simulcast.

# Introduction

Before the introduction of H.264 SVC extension [1], all videoconferencing solutions used MCU (multipoint control unit) in order to support multiple endpoints.  Then, novel solutions utilizing SVC codec and negating the need for costly MCUs have been introduced.  The benefit of using scalable codec is clear [2].  The ability to easily scale a given bitstream, without transcoding, to support various endpoints with different capabilities and network conditions is one of the key benefits over traditional MCU-based systems.  Simulcast can be used instead to send multiple non-scalable bitstreams for various rates and resolutions.  However, as will be shown in this contribution, sending such bitstreams in place of a single scalable bitstream results in significant bit rate overhead.

One of the main reasons for introducing the scalability extension at such an early stage is to design a standard that cleanly and concisely describes both non-scalable and scalable bitstream syntax and decoding process without the need for adding patches such as NAL unit header SVC extension, prefix NAL unit, subset SPS, and etc…  Because H.264 was initially designed without scalability in mind, such patches were necessary, making the H.264 SVC extension harder to understand and implement.

The most widely used scalability features in the videoconferencing industry are temporal scalability and 1:2 spatial scalability.  The HEVC design already supports temporal scalability.  This contribution proposes to adopt, directly from H.264 SVC, a set of tools for supporting 1:2 spatial scalability.  Section 2 describes in high level the tools that need to be adopted, and Section 3 specifies bitstream syntax modifications needed for the spatial scalability support.  Section 4 describes the experimental conditions and results, and concluding remarks are given in Section 5.

# Proposal

This contribution proposes to adopt the H.264 SVC 1:2 spatial scalability tools in the HEVC design.  Specifically, the contribution proposes to include the three inter-layer prediction tools: 1) inter-layer intra prediction, 2) inter-layer motion prediction, and 3) inter-layer residual prediction.  These three are additional predictions available to the enhancement layer prediction units (PUs) and they are signaled using the syntactic elements – base_mode_flag, motion_prediction_flag_l0, motion_prediction_flag_l1, and residual_prediction_flag.

If the current PU is neither skipped (!skipped_flag) nor inferred (!merge_flag) and base_mode_flag is set, then the current PU is coded using information from the base layer (BL).  In the case the corresponding BL samples are intra coded, the reconstructed BL intra samples are upsampled as predictor for the current PU’s samples.  Otherwise, if the corresponding BL samples are inter coded, then the upsampled BL motion information is used as the motion information of the current PU.

In the case the current PU is MODE_INTER coded (!skipped_flag, !merge_flag, and !base_mode_flag), the motion information can be predicted from the corresponding BL motion information by setting motion_prediction_flag_l0 and/or motion_prediction_flag_l1.

Lastly, the current PU’s residue can be predicted from the reconstructed BL’s residue by setting residual_prediction_flag.

# Proposed Syntax

The following changes to the WD1 of HEVC [3] are proposed; changes are highlighted.

7.3.1 NAL unit syntax

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| … |  |
| if( nal_unit_type == 1 ¦ ¦ nal_unit_type == 5 ) { |  |
| temporal_id | u(3) |
| output_flag | u(1) |
| non_base_layer_flag | u(1) |
| reserved_zero_3bits | u(3) |
| if( non_base_layer_flag ) { |  |
| no_inter_layer_pred_flag | u(1) |
| dependency_id_minus1 | u(2) |
| discardable_flag | u(1) |
| reserved_non_base_layer_zero_4bits | u(4) |
| nalUnitHeaderBytes += 1 |  |
| } |  |
| nalUnitHeaderBytes += 1 |  |
| } |  |
| … |  |
| } |  |

7.3.3 Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( deblocking_filter_control_present_flag ) { |  |
| … |  |
| } |  |
| if( !no_inter_layer_pred_flag ) { |  |
| adaptive_base_mode_flag | u(1) |
| if( !adaptive_base_mode_flag ) |  |
| default_base_mode_flag | u(1) |
| if( !default_base_mode_flag ) { |  |
| adaptive_motion_prediction_flag | u(1) |
| if( !adaptive_motion_prediction_flag ) |  |
| default_motion_prediction_flag | u(1) |
| } |  |
| adaptive_residual_prediction_flag | u(1) |
| if( !adaptive_residual_prediction_flag ) |  |
| default_residual_prediction_flag | u(1) |
| } |  |
| } |  |

7.3.6 Coding unit syntax

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize ) |  |
| else { |  |
| if( NumMergeCandidates > 0 ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| … |  |
| } else { |  |
| if( adaptive_base_mode_flag ) |  |
| base_mode_flag | u(1) ¦ ae(v) |
| if( base_mode_flag ) { |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize ) |  |
| transform_tree( x0, y0, log2CUSize, 0, 0 ) |  |
| } else { |  |
| if( slice_type != I ) |  |
| pred_mode | u(1) ¦ ae(v) |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| if( PredMode == MODE_INTRA ) { |  |
| … |  |
| } else if( PredMode = = MODE_INTER ) { |  |
| … |  |
| } else { /* MODE_DIRECT */ |  |
| … |  |
| } |  |
| } /* !base_mode_flag */ |  |
| } |  |
| } |  |
| } |  |

7.3.7 Prediction unit syntax

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| … |  |
| } else { |  |
| if( !base_mode_flag ) { |  |
| if( PredMode == MODE_INTRA ) { |  |
| … |  |
| } else { /* MODE_MERGE, MODE_DIRECT, MODE_INTER */ |  |
| if( merge_flag[ x0 ][ y0 ] && NumMergeCandidates > 1 ) { |  |
| merge_left_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } else if( PredMode == MODE_DIRECT ) { |  |
| … |  |
| } else { |  |
| if( slice_type == B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L1 ) { |  |
| if( adaptive_motion_prediction_flag ) |  |
| motion_prediction_flag_l0[ i ] | ue(1) ¦ ae(v) |
| if( !motion_prediction_flag_l0[ i ] ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0 | ue(v) ¦ ae(v) |
| } |  |
| else { |  |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| } |  |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 ) { |  |
| if( adaptive_motion_prediction_flag ) |  |
| motion_prediction_flag_l1[ i ] | ue(1) ¦ ae(v) |
| if( !motion_prediction_flag_l1[ i ] ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1 | ue(v) ¦ ae(v) |
| } |  |
| else { |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } // if( !base_mode_flag ) |  |
| if( adaptive_residual_prediction_flag && slice_type != I && ( base_mode_flag ¦¦ PredMode != MODE_INTRA ) ) |  |
| residual_prediction_flag | ue(1) ¦ ae(v) |
| } /* ! skip_flag[ x0 ][ y0 ] */ |  |
| } |  |

7.4.1 NAL unit semantics

non_base_layer_flag specifies whether the NAL unit is a base layer NAL unit.  When non_base_layer_flag is equal to 0, then no_inter_layer_pred_flag shall be implied to equal to 1 and DependencyId shall be equal to 0.  When non_base_layer_flag is equal to 1, then DependencyId shall be greater than 0.

no_inter_layer_pred_flag specifies whether inter-layer prediction may be used for decoding the coded slice.  When no_inter_layer_pred_flag is equal to 1, inter-layer prediction is not used for decoding the coded slice.  When no_inter_layer_pred_flag is equal to 0, inter-layer prediction may be used for decoding the coded slice as signalled in the slice header or the PU layer.

dependency_id_minus1 is used to derive the dependency identifier for the NAL unit.  When dependency_id_minus1 is not present in the bitstream, then it is set to -1.  The dependency_id_minus1 value shall be between 0 and 3, inclusive.  This means that up to four enhancement layers are supported.

The variable DependencyId is derived by:

DependencyId = dependency_id_minus1 + 1.

Note: When no_inter_layer_pred_flag is equal to 0, then the coded slice may use inter-layer prediction using the information from the dependency representation corresponding to DependencyId-1.  Due to this rule, there is no need to specify ref_layer_dq_id as in H.264 SVC.

discardable_flag equal to 1 specifies that the current NAL unit is not used for decoding dependency representations that are part of the current coded picture or any subsequent coded picture in decoding order and have a greater value of DependencyId than the current NAL unit. discardable_flag equal to 0 specifies that the current NAL unit may be used for decoding dependency representations that are part of the current coded picture or any subsequent coded picture in decoding order and have a greater value of DependencyId than the current NAL unit.

7.4.3 Slice header semantics

adaptive_base_mode_flag specifies the presence of syntax elements in the slice header and the PU layer.  adaptive_base_mode_flag equal to  0 specifies that default_base_mode_flag is present in the slice header and base_mode_flag is not present in the PU layer.  adaptive_base_mode_flag equal to 1 specifies that specifies that default_base_mode_flag is not present in the slice header and base_mode_flag is present in the PU layer.  When adaptive_base_mode_flag is not present, it shall be inferred to be equal to 0.

default_base_mode_flag specifies how base_mode_flag is inferred when it is not present in the PU layer.  When default_base_mode_flag is not present, it shall be inferred to be equal to 0.

adaptive_motion_prediction_flag specifies the presence of syntax elements in the slice header and the PU layer. adaptive_motion_prediction_flag equal to 0 specifies that default_motion_prediction_flag is present in the slice header and motion_prediction_flag_l0[] and motion_prediction_flag_l1[] are not present in the PU layer.  adaptive_motion_prediction_flag equal to 1 specifies that default_motion_prediction_flag is not present in the slice header and motion_prediction_flag_l0[] and motion_prediction_flag_l1[] are present in the PU layer.  When adaptive_motion_prediction_flag is not present, it shall be inferred to be equal to 0.

default_motion_prediction_flag specifies how motion_prediction_flag_l0[] and motion_prediction_flag_l1[] are inferred when they are not present in the PU layer.  When default_motion_prediction_flag is not present, it shall be inferred to be equal to 0.

adaptive_residual_prediction_flag specifies the presence of syntax elements in the slice header and the PU layer.  adaptive_residaul_prediction_flag equal to 0 specifies that default_residual_prediction_flag is present in the slice header and residual_prediction_flag is not present in the PU layer.  adaptive_residaul_prediction_flag equal  to 1 specifies that default_residual_prediction_flag is not present in the slice header and residual_prediction_flag is present in the PU layer.  When adaptive_residual_prediction_flag is not present, it shall be inferred to be equal to 0.

default_residual_prediction_flag specifies how residual_prediction_flag is inferred when it is not present in the PU layer.  When default_residual_prediction_flag is not present, it shall be inferred to be equal to 0.

7.4.6 Coding unit semantics

base_mode_flag equal to 1 specifies that the PU is encoded using inter-layer prediction.  The decoding process using inter-layer prediction is specified in subclause XXX.

When base_mode_flag is not present, base_mode_flag shall be inferred as follows.

- If skip_flag is equal to 1 or merge_flag is equal to 1, the value of base_mode_flag is inferred to be equal to 0.

- Otherwise, the value of base_mode_flag is inferred to be equal to default_base_mode_flag.

7.4.7 Prediction unit semantics

motion_prediction_flag_l0[ i ] equal to 1 specifies that an alternative motion vector prediction process as specified in subclause XXX is used for deriving the list 0 motion vector of the PU and that the list 0 reference index of the PU is inferred as specified in subclause XXX.

When motion_prediction_flag_l0[ i ] is not present, motion_prediction_flag_l0[ i ] is inferred to be equal to default_motion_prediction_flag.

motion_prediction_flag_l1[ i ] has the same semantics as motion_prediction_flag_l0[ i ], with l0 and list 0 replaced by l1 and list 1, respectively.

residual_prediction_flag equal to 1 specifies that the residual signal of the current PU is predicted as specified in subclause XXX using the dependency representation corresponding to DependencyId-1.  residual_prediction_flag equal to 0 specifies that the residual signal of the current PU is not predicted.

When the syntax element residual_prediction_flag is not present, residual_prediction_flag shall be inferred as follows.

- If all of the following conditions are true, residual_prediction_flag is inferred to be equal to default_residual_prediction_flag:

–	slice_type is not equal to I,

–	base_mode_flag is equal to 1 or PredMode != MODE_INTRA.

–	Otherwise, residual_prediction_flag is inferred to be equal to 0.

# Experimental Results

TMuC 0.9 is modified to support 2 layer (one base layer and one enhancement layer) 1:2 spatial scalability for the intra only case.  The modification includes adding an extra mode (I_BL) when encoding enhancement layer PUs in MODE_INTRA.  When encoding the PUs, the cost of I_BL is compared with the best intra cost.  When the I_BL mode is chosen, the corresponding reconstructed base layer samples are upsampled (using the same upsampler as the one described in H.264 SVC) and used as the predictor.  The source code with the described modification is included as the zipped file “JCTVC-D202-TMuC0.9-Vidyo.zip”.  The changes can be identified by searching for the string “VIDYO_SVC”.

The experiments are performed for the 2 spatial layer case on all the test sequences of the 5 classes in both high-efficiency and low-complexity settings [4].  For the Class B sequences, the last 8 rows of pixels are discarded to make the base layer resolution be a multiple of 8 pixels.  Hence, the cropped sequences are 1920x1072 for the enhancement layer and 960x536 for the base layer.

For the enhancement layer, the QP values specified in [4] are used.  In one experiment, the base layer is encoded using the same QP as the enhancement layer – referred to as using QP offset of 0.  The results of using QP offset of 0 are provided in the worksheets of the Excel file “JCTVC-D202.xls” with tabs titled “QP offset = 0”.  In another experiment, the base layer is encoded using QP offset of 2, i.e., the base layer QP is 2 less than the enhancement layer QP.  The results of using QP offset of 2 are provided in the worksheets of the same Excel file with tabs titled “QP offset = 2”.

In each “Summary” tab of the Excel file, three tables are provided.  The first table titled “Total Scalable vs. Total Simulcast” compares the enhancement layer (EL) quality per total bits spent between scalable coding and simulcast.  Here, total bits correspond to the bits spent for both the base layer (BL) and the EL.  Note that, with the current implementation, the BL of scalable coding is encoded exactly the same way as single layer encoding (the BL of simulcast); hence, the BL quality is not compared.  The results for the case where the same QP is used for encoding both the BL and the EL are copied in Table 1.  The results for the case where QP offset of 2 is used are copied in Table 2.  As can be seen from the table, in the high efficiency setting, 2-layer intra only scalable coding yields an average of 14.9% Y BD-rate gain and a maximum of 29.8% Y BD-rate gain over simulcast.

Table 1: BD-rate gain of scalable coding over simulcast (EL QP = BL QP)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -19.4 | -21.2 | -21.9 | -17.8 | -20.0 | -20.2 |
| Class B | -15.2 | -18.1 | -17.6 | -13.1 | -15.0 | -14.9 |
| Class C | -7.9 | -10.5 | -9.7 | -7.1 | -8.8 | -8.6 |
| Class D | -6.5 | -8.4 | -7.6 | -6.0 | -8.1 | -7.8 |
| Class E | -15.3 | -17.0 | -16.6 | -13.3 | -14.5 | -14.5 |
| All | -12.1 | -14.4 | -14.0 | -10.7 | -12.6 | -12.4 |
| Max | -25.4 | -28.1 | -29.6 | -21.7 | -25.4 | -26.3 |

Table 2: BD-rate gain of scalable coding over simulcast (EL QP = BL QP + 2)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -22.8 | -25.1 | -26.2 | -21.9 | -25.1 | -25.8 |
| Class B | -18.4 | -21.9 | -21.7 | -16.7 | -19.6 | -19.7 |
| Class C | -10.3 | -13.1 | -12.3 | -9.3 | -11.8 | -11.6 |
| Class D | -8.4 | -10.3 | -9.6 | -7.7 | -10.8 | -10.5 |
| Class E | -18.7 | -21.8 | -21.2 | -16.9 | -20.5 | -20.2 |
| All | -14.9 | -17.7 | -17.3 | -13.7 | -16.7 | -16.6 |
| Max | -29.8 | -33.7 | -35.4 | -27.4 | -31.9 | -32.3 |

Another interesting comparison is scalable vs. single layer coding to estimate the overhead of scalable coding.  The second table titled “Total Scalable vs. Top Layer of Simulcast” in the “Summary” tab of the Excel file summarizes this overhead.  Finally, we also compare just the enhancement layer of scalable coding vs. single layer coding and the results are summarized in the third table titled “Top Layer of Scalable vs. Top Layer of Simulcast”.

The results show that scalable coding yields better coding gain for larger resolution sequences.  Additionally, slightly better gain is seen in the high-efficiency setting than in the low-complexity setting.  This may be due to better entropy coding of the base_mode_flag.  Lastly, using QP offset of 2 yields better result than QP offset of 0 because the base layer samples are better coded, and as a result, better predict the enhancement layer samples.

# Conclusion

This contribution introduces modifications to HEVC to support 1:2 spatial scalability by adopting some of the H.264 SVC tools.  Experimental results show a clear improvement in coding efficiency vs. simulcast.  Incorporating scalability features during the first phase of the HEVC development ensures a clean scalability design and eliminates the need to retrofit the design later to add scalability support.

# References

- ITU-T Recommendation H.264, “Advanced Video Coding for Generic Audiovisual Services,” March 2010.

- A. Eleftheriadis, M. R. Cinvalar and O. Shapiro, “Multipoint Videoconferencing with Scalable Video Coding,” International Packet Video Workshop, Hangzhou, CN, April, 2006.

- T. Wiegand, W.-J. Han, B. Bross, J.-R. Ohm and G. J. Sullivan, “WD1: Working Draft 1 of High-Efficiency Video Coding,” JCTVC-C403, Guangzhou, CN, October, 2010.

- F. Bossen, “Common Test Conditions and Software Reference Configurations,” JCT-VC, Doc JCTVC-C500, Guangzhou, CN, October, 2010.

# Patent rights declaration(s)

(NOTE – Activities in the JCT-VC and contributions to the JCT-VC are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-VC proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. An example of an informal IPR notification statement for a contribution is provided below.)

Vidyo, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).