| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, Korea, 20-28 January 2011 | Document: JCTVC-D204-rev1 WG11 Number: m18964 |
| --- | --- |

| Title: | CE6: Cross-check report on combined intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Report and Proposal] |  |  |
| Author(s) or Contact(s): | Keiichi Chono Kenta Senzaki Hirofumi Aoki Junji Tajime Yuzo Senda 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 431 7595 chono@ct.jp.nec.com k-senzaki@bp.jp.nec.com hao@bu.jp.nec.com tajime@ak.jp.nec.com y-senda@bc.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution reports the performances of three Combined Intra Prediction (CIP) schemes with different complexity configurations proposed by BBC: High complexity CIP (HCIP), Medium complexity CIP (MCIP), and Low complexity CIP (LCIP). Simulation results show that:

- For high efficiency all intra case, BD-Rates improvements of HCIP, MCIP, and LCIP are -1.53%, -0.32%, and -0.30%, respectively, and encode time increases of those are 85%, 7%, and -3%, respectively;

- For low complexity all intra cases, BD-Rates improvements of HCIP, MCIP, and LCIP are -2.51%, -090%, and -0.71%, respectively, and encode time increases of those are 108%, 12%, and 1%, respectively.

If CIP is considered as an intra tool of HM ver.2, the following further studies are recommended:

- Force CIP-off on intra PUs in which CIP side information is not signaled in order to allow encoders to avoid picture quality degradations associated with open loop intra prediction;

- Improve the throughput of CIP’s pixel reconstruction process by relaxing its pixel-by-pixel dependency, for example, by not using the weighted combination at block boundaries which are used for the predictions of the subsequence intra PUs.

- Introduction

Combined Intra Prediction (CIP) is an open loop intra prediction which comprises a weighted combination of an Angular intra prediction together with a pixel-by-pixel local mean prediction. The local mean prediction is the average of pixel values to the left, top-left and above the current pixel. These values are original values in the encoder, whereas they are the reconstructed values after inverse quantization and transform in the local decoder of the encoder and the decoder. The CIP description of TMuC document JCT-VC B205 claims that the CIP provides pixel-by-pixel adaptation and improves intra prediction, especially in large blocks where a directional intra prediction may not work well.

This contribution studies the performances of three CIP schemes with different complexity configurations proposed by BBC: High complexity CIP (HCIP), Medium complexity CIP (MCIP), and Low complexity CIP (LCIP). The high-level description of different CIP schemes, simulation results, and remarks are presented in the following sections.

- High-level descriptions of HCIP, MCIP, and LCIP

HCIP signals CIP side information (CIP on/off flag) in all intra CUs, and therefore it requires additional R-D cost computations of each of CUs with two PU sizes (2Nx2N and NxN). MCIP signals CIP side information only in 16x16 intra PUs, and it forces CIP on 4x4/8x8 intra PUs and disables CIP of 32x32/64x64 intra PUs. Its CIP on/off decision is conducted during the intra mode decision process of 16x16 intra PUs; the additional encoder complexity is reasonably small relative to that of HCIP’s. LCIP does not signal CIP side information, and it forces CIP on 4x4/8x8 intra PUs and disables CIP of other size intra PUs. LCIP does not require additional computation for CIP decision. The three CIP schemes are summarized in Table 1.

Table 1: Summary of HCIP, MCIP, and LCIP. “Adaptive” means that CIP on/off is controlled by encoder. No CIP” means that CIP is not used. “Force CIP” means that CIP is used.

From Table 1, expected encode time increases associated with CIP on/off decision of HCIP and MCIP are roughly 100% (=100*(10-5)/5) and 20% (=100*(6-5)/5). (Note that the actual encode time increase of MCIP should be than 20% since its CIP decision is conducted during the intra mode decision process of 16x16 intra PUs.)

- Simulation

3.1. Coding conditions

Three CIP schemes with different complexity configurations were evaluated according to JCTVC-C500 [1] and JCTVC-C506 [2]. The results for TMuC0.9 without CIP and with different complexity CIPs are denoted by Anchor, High complexity CIP (HCIP), Medium complexity CIP (MCIP), and Low complexity CIP (LCIP).

3.2. Computing platforms

Two 64-bit executables for encoding and decoding were generated by using Microsoft Visual C++ 2008. They ran on Windows-based computing platforms listed below. Note that as inferred from the list, CPU automated over-clocking happened for some results. Individual encode and decode times can be significantly varied.

| Computing platform | Usage |
| --- | --- |
| Windows 7 64-bit Professional Core i7 2.8GHz and Memory 12GB | Up to 4 processes for encoding and decoding ran at the same time. |

3.3. Results

Attached three Excel sheets, NEC-HCIP-results.xls, NEC-MCIP-results.xls, and NEC-LCIP-results.xls, show the detailed results of HCIP, MCIP, and LCIP, respectively. Table 2 shows the summary of those results.

Table 2: Summary of HCIP, MCIP, and LCIP results.

Simulation results show that:

- For high efficiency all intra case, BD-Rates improvements of HCIP, MCIP, and LCIP are -1.53%, -0.32%, and -0.30%, respectively, and encode time increases of those are 85%, 7%, and -3%, respectively;

- For low complexity all intra cases, BD-Rates improvements of HCIP, MCIP, and LCIP are -2.51%, -090%, and -0.71%, respectively, and encode time increases of those are 108%, 12%, and 1%, respectively;

- Significant decode time increase is not observed in all CIP schemes (Note that 7% increase in MCIP can come from CPU automated over-clocking and disk-access associated with YUV file writing).

- Remarks

From significant gains observed in all CIP results for ClassA, MCIP and LCIP look good design in terms of the balance of performance and complexity. However, the MCIP and LCIP designs that force CIP on particular intra PUs are undesirable because the CIP is an open loop intra prediction that can cause picture quality degradations on reconstructed images which do not appear when closed loop intra prediction is used. Without a means of controlling CIP on/off PU-by-PU, the MCIP and LCIP designs require picture-level CIP on/off control in order to avoid such picture quality degradations, and that can double encode time. Although the HCIP design can avoid such picture quality degradations by PU-level CIP on/off control, its current encoder complexity seems to be unacceptably high.

As for CIP decoder complexity, its impact on decode time is small as expected from its required arithmetic operations deduced from the CIP description of TMuC document JCT-VC B205. However, it should be asserted that the use of CIP can affect the throughput of pixel reconstruction processes due to its pixel-by-pixel dependent process.

Based on the remarks, if CIP is considered as an intra tool of HM ver.2, the following further studies are recommended:

- Force CIP-off on intra PUs in which CIP side information is not signaled;

- Improve the throughput of CIP’s pixel reconstruction process by relaxing its pixel-by-pixel dependency, for example, by not using the weighted combination at block boundaries which are used for the predictions of the subsequence intra PUs.

- Conclusions

This contribution has presented the high-level description of different CIP schemes and simulation results.If CIP is considered as an intra tool of HM ver.2, the following further studies are recommended:

- Force CIP-off on intra PUs in which CIP side information is not signaled picture quality degradations associated with open loop intra prediction;

- Improve the throughput of CIP’s pixel reconstruction process by relaxing its pixel-by-pixel dependency, for example, by not using the weighted combination at block boundaries which are used for the predictions of the subsequence intra PUs.

- Reference

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C500, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

- JCT-VC, “Core Experiment 6: Intra Prediction Improvement”, JCTVC-C506_r1, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).