| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, Korea, 20-28 January 2011 | Document: JCTVC-D205-rev1 WG11 Number: m18965 |
| --- | --- |

| Title: | CE6: Cross-check report on differential coding of intra mode |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Report and Proposal] |  |  |
| Author(s) or Contact(s): | Keiichi Chono Kenta Senzaki Hirofumi Aoki Junji Tajime Yuzo Senda 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 431 7595 chono@ct.jp.nec.com k-senzaki@bp.jp.nec.com hao@bu.jp.nec.com tajime@ak.jp.nec.com y-senda@bc.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution reports cross-check results of CE6 Differential Coding of Intra Modes (DCIM). The cross-check task was successfully completed and it is verified that simulation results match those provided by Sharp. If DCIM is considered as an intra tool of HM ver.2, it is recommended that its slope computation is simplified.

- Introduction

This contribution describes implementation analysis and simulation results of DCIM described in JCTVC - C169 [1] and JCTVC-c176 [2].

# Implementation analysis

The following DCIM implementations are observed from the software provided by Sharp:

- DCIM is used for 4x4, 8x8, 16x16, and 32x32 Intra PU partitions;

- The numbers of DCIM predictors are set to 7, 7, 7, and 3 for 4x4, 8x8, 16x16, and 32x32 Intra PU partitions, respectively;

- DCIM intra predictor is derived from slopes of neighboring edge information; slope computation uses floating-point operations (including division process) which shall be simplified to LUT implementation, for example;

- DCIM with bi-directional intra prediction is introduced as suggested in its CE6 description;

- Chroma DCIM, which allows applying different intra predictions to U and V components, is introduced as suggested in its CE6 description.

It is confirmed that the implementation of DCIM is compliant to its CE6 description. However, its slope computation shall be simplified by means of LUT since floating-point operation is prohibitively complex and difficult to guarantee the exact matches between different computing platforms.

- Simulation

DCIM was evaluated according to JCTVC-C500 [3] and JCTVC-C506 [4]. In addition to its All intra coding results, Random access and Low delay coding results were obtained. Furthermore, DCM’s performance against cropped SHV sequences provided by NHK was evaluated.

Software provided by Sharp was successfully compiled by Microsoft Visual C++ 2008, and its 64-bit encoder and decoder executables were run on Windows-based computing platforms listed below. Note that as inferred from the list, CPU-automated over-clocking happened for some results. Individual encode and decode times can be significantly varied.

| Computing platform | Usage |
| --- | --- |
| Windows 7 64-bit Professional Intel Xeon 3.33GHz and Memory 32GB | Up to 12 processes for encoding and decoding ran at the same time. |

Attached two Excel sheets, NEC-DCIM-results.xls and NEC-DCIM-SHV-results.xls, show the detailed results for common test sequences and cropped SHV sequences. Table 1 and Table 2 show the summary of those results. It was confirmed that BD-PSNR/Rate values are the same as those of Proponent. It is confirmed that the simulation results are accurate.

Table 1: Summary of DCIM results for common test sequences.

Table 2: Summary of DCIM results for cropped SHV test sequences.

2.4. Visual quality

NEC checked DCIM’s results of BasketballDrive and BasketballDrill sequences encoded by using Low complexity Random access results: The reason for selecting those sequences is that each of them shows the best BD-rate improvements among its Class; the reason for choosing Low complexity Random access results is that All intra results were heavily degraded by intra flicker. 

The summary of the tests is as follows:

- Visual appearances of Anchor and DCIM are different, especially in flooring areas. However, significant visual quality improvements and degradations by DCIM were not observed in play-backed videos. Note that since DCIM's bit rates is lower than that of corresponding Anchor's, this observation implies that DCIM can attain comparable visual quality against Anchor with less bit rates.

- By freezing video playback and comparing individual frames, significant visual quality improvements by DCIM were observed in BasketballDrive QP=37 and BasketballDrill QP=32 results: As for the BasketballDrive result, artificial vertical lines appear on the wooden wall areas of the background in Anchor, whereas those do not appear in DCIM. As for the BasketballDrill results, a diagonal line of the basketball backboard goal is distorted in Anchor, whereas that is finely reconstructed in DCIM. (The screen shots are found in “NEC-DCIM-pics.pdf”)

The visual quality tests show that DCIM has potential to improve the visual quality of some video sequences by using accurate intra prediction direction with less signaling overhead.

- Conclusion

Base on the cross-verification results, NEC has confirmed that Sharp’s results are accurate.

- Acknowledgement

Authors acknowledge to Dr. Ichigaya in NHK who kindly offered us the cropped versions of SHV test sequences.

- Reference

- T. Yamamoto and Y. Yasugi, "Analysis and improvement of differential coding of intra modes", JCTVC-C176, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

- E. Maani, L. Dong, and W. Liu, "TE6.b: Experimental results of DCIM", JCTVC-C169, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

- JCT-VC, “Core Experiment 6: Intra Prediction Improvement”, JCTVC-C506_r1, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C500, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).