| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, Korea, 20-28 January 2011 | Document: JCTVC-D206 WG11 Number: m18996 |
| --- | --- |

| Title: | CE8: Cross-check report on SKKU/SKT deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Report] |  |  |
| Author(s) or Contact(s): | Keiichi Chono Kenta Senzaki Hirofumi Aoki Junji Tajime Yuzo Senda 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 431 7595 chono@ct.jp.nec.com k-senzaki@bp.jp.nec.com hao@bu.jp.nec.com tajime@ak.jp.nec.com y-senda@bc.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution reports the cross-verification results of SKKU/SKT deblocking filter. The cross-verification tasks have been successfully completed and it is confirmed that the results match those provided by proponent.

- Introduction

This contribution describes implementation and simulation results of SKKU/SKT’s deblocking technique proposed in JCTVC-C130 [1].

- Implementation analysis

The deblocking technique uses 7 block-strength values and introduces another weak filtering process. The 7 block-strength values are used for selecting a filtering process and adjusting its tC parameter; the introduced weak filtering process is based on the filter used in Chroma filtering process of TMuC0.9, and it is selected when both block-boundary pixels, P and Q, are intra coded, P and Q have the same IntraPredMode, P and Q have similar IntraPred directions, and either of P or Q has significant coefficient. No filtering process is applied when P and Q are intra coded, P and Q have the same IntraPredMode, P and Q have similar IntraPred directions, and neither of P nor Q has significant coefficient.

It is confirmed that the implementation of SKKU/SKT’s deblocking technique is compliant to the description of JCTVC-C130 [1].

# Simulation

3.1. Coding conditions

SKKU/SKT deblocking filter was evaluated according to JCTVC-C500 [2] and JCTVC-C508 [3]. Two 64-bit executables for encoding and decoding were generated by using Microsoft Visual C++ 2008. They ran on Windows-based computing platforms listed below. As inferred from the list, CPU automated over-clocking happened for some encoding results. Therefore individual time of encoding and decoding can be significantly varied.

| Computing platform | Usage |
| --- | --- |
| Windows 7 64-bit Professional Intel Xeon 3.33GHz and Memory 32GB | Up to 12 processes for encoding and decoding ran at the same time. |

3.2. Simulation results

Attached two Excel sheets, NEC_SKKU_CE8_results.xls and NEC_SKKU_CE8_additional_results.xls, contain the detailed results of common and additional common test conditions. Table 1 and Table 2 show the summary of those results. It was confirmed that BD-PSNR/Rate values are the same as those of SKKU/SKT’s and that encode and decode times are similar to those of SKKU/SKT’s. (NEC’s encode and decode times for additional common test condition are slightly higher than those of SKKU/SKT. It is expected that the small difference came from CPU-automated over-clocking and disk access fro writing yuv files.)

Table 1: Summary of results for common test conditions.

Table 2: Summary of results for additional common test conditions in CE8 Subtest1.

3.4. Subjective quality analysis

Anchor and Proposal results are arranged in side-by-side as shown in Figure 1 so that the differences can be easily detected. Viewing test with professional monitors (SONY SRM L560 and BVM L231) was conducted for all results excluding Class A and D, and significant improvement and degradation by the proposal were not observed in the playback video. Accurate edge reconstruction is observed in some frames of BQMall (IO LC QP=37) by comparing the individual frames of Anchor and Proposal results. The example of improvements is shown in Figure 2 and 3.

Figure 1	Side-by-side arrangement (Left: Anchor, Right Proposal)

Figure 2	BQMall results (IO LC QP=37) of 253rd frame。

Figure 3	Zoom of the red-circled regions of Figure 2

# Conclusion

Base on the cross-verification results, NEC has confirmed that SKKU/SKT’s results are accurate.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- J. Yang, K. Won, B. Jeon (SKKU), J. Lim, J. Song (SKT), “TE10 subtest 1: Results of intra deblocking filter testing by SKKU/SKT,” Document: JCTVC-C130, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C500, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

- T. Yamakage et al., “Description of CE8: In-loop filtering,” Document: JCTVC-C508, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.